3GPP TSG-RAN WG2 Meeting #128	R2-2410113

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding inter-CU LTM, there are still some open issues on the details of inter-CU LTM in both non-DC and DC scenarios. In this contribution, we would like to share our opinions on the following remaining issues for inter-CU LTM.

- Security key handling for inter-CU LTM in non-DC case;

- Coexistence of one-shot LTM and subsequent LTM;

- Inter-CU MCG LTM.

- Discussion

Security key handling for inter-CU LTM in non-DC case

In Rel-18, subsequent LTM for intra-CU scenario is supported and security key is maintained upon an LTM cell switch. For inter-CU LTM, the handover parameters exchanged between gNBs, and then needs to be transmitted over Uu interface. Frequent update of handover parameters results in high inter-node singling overhead. By supporting subsequent LTM, the signalling overhead for over Xn and Uu interface can be reduced to a certain extent. Thus, subsequent LTM for inter-CU scenario will also be supported in Rel-19.

To support inter-CU subsequent LTM, how to handling the security key update is the most essential issue to be solved, which also involves coordination with RAN3/SA3. In RAN2#125bis meeting, RAN2 sent an LS R2-2403929 on the security handling for inter-CU LTM in non-DC cases to SA3 for further evaluation of potential solutions. In reply LS R2-2409534, the following information are provided by SA3.

Copied from R2-2409534

The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like,

-	Security threat of the unprotected MAC CE to deliver the NCC value.

-	Sending NCC value per candidate cell via RRC message during LTM preparation phase.

As mentioned by SA3, the most preferred solution in SA3 is providing the NCC value to be used by the UE for key derivation to UE via RRC signalling at the next inter-CU LTM cell switch, which is secure but need to initial RRC reconfiguration procedure. For MAC CE based solution, SA3 understands there may be security threat of the unprotected MAC CE to deliver the NCC value. From network operation’s perspective, security threats are more critical than signalling overhead. Moreover, only updating the NCC value by RRC may not cause too much signalling overhead. And the solution of sending NCC value per candidate cell via RRC message during LTM preparation phase can be treated as a compromise solution, which could also be considered.

Proposal 1: For security key handling for inter-CU LTM in non-DC case, RAN2 further down-selects from the following RRC based solutions.

- Option 1: The UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch.

- Option 2: Sending NCC value per candidate cell via RRC message during LTM preparation phase.

Coexistence of one-shot LTM and subsequent LTM

In Rel-16/17, the feature of CPAC is introduced. To reduce signalling overhead, subsequent CPAC is introduced in Rel-18. Since the procedure of CPAC and subsequent CPAC are introduced separately in different release, the coexistence issue was discussed in Rel-18. It is agreed that coexistence of CPAC and subsequent CPAC for the same candidate SN is not supported.

For LTM, one-shot LTM and subsequent LTM may influence the signalling interaction among the network nodes. It is better to clarify the support of subsequent LTM is mandatory or optional for a certain candidate gNB or SN.

Taking inter-SN SCG LTM as an example, Fig.1 illustrates a potential stage-2 signalling flow for SN-initiated inter-SN SCG LTM based on the agreements made in previous RAN2 meetings. In step 5~6, the MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. Then the source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN. If the support of subsequent LTM is mandatory, step 7~8 shall be mandatory to transfer the common CSI resource configuration and the collected information of candidate cells to each candidate SN. The candidate SN(s) responds with the updated candidate SCG configuration to the MN. If the support of subsequent LTM is optional, step 7~8 could be optional as well.

Fig.1 Stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed

Proposal 2: RAN2 clarifies the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Inter-CU MCG LTM

Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN or release the SN. For inter-CU MCG LTM with SCG unchanged or released, we can follow the similar principle that the target MN decides whether to keep or release the SN. For the case that the target MN decides to change the SN, the handling of SCG configuration could be up to NW implementation (e.g. release or reconfiguration), as agreed in RAN2#127 meeting.

Proposal 3: Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline.

Proposal 4: For inter-CU MCG LTM with SCG unchanged or released, the target MN decides whether to keep or release the SN.

- Conclusion

In this contribution, we discuss inter-CU LTM related aspects in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For security key handling for inter-CU LTM in non-DC case, RAN2 further down-selects from the following RRC based solutions.

- Option 1: The UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch.

- Option 2: Sending NCC value per candidate cell via RRC message during LTM preparation phase.

Proposal 2: RAN2 clarifies the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Proposal 3: Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline.

Proposal 4: For inter-CU MCG LTM with SCG unchanged or released, the target MN decides whether to keep or release the SN.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

3GPP TS 38.300 18.1.0: " NR; NR and NG-RAN Overall Description; Stage 2".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB