3GPP TSG-RAN WG2#128												     R2-2410125

Orlando, FL, US, 18 – 22 Nov 2024

Agenda Item:	8.2.3

Source: 	NEC

Title:	        Ambient-IoT Paging

Document for:	 Discussion

- Introduction

In TSG RAN Meeting #102, a new SID of studying on solutions for Ambient IoT in NR has been approved [1]. Paging will be supported as a basic function to enable DO-DTT and DT data transmission. In RAN2#127bis, the following agreements were made:

RAN2#127bis - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA. - For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

This contribution provides our views on the paging of ambient-IoT.

- Discussion

  - Paging contents for RA type determination

In previous meeting RAN2 agreed that “For all use cases, device determines the random access type from the paging message”, and the information used for random access type determination can be explicitly or implicitly. From our perspective, at least an explicit information can be introduced to indicate whether RA resources indicated in paging is for contention-based or contention-free. This especially applicable to the cases that the paging message is for more than one device (e.g., paging message contains group id, all devices, or multiple devices). While for the cases that paging message only includes identity for a single device, the random access resources indicated in paging is for contention-free random access.

For cases that paging message targeting for more than one device, introduce information to indicate whether RA resources indicated in paging is for CBRA or CFRA.

For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

On top of that, for contention-based random access resources, device may further determine whether to use 2-step (carry random id with upper layer data directly) or 3-step (carry random id without upper layer data) according to RA type information in paging message.

And for contention-free random access resources, since there will be no collision between different devices, performing 2-step RA procedure brings more resource utilization efficiency. Moreover, for CFRA, performing 2-step RA, which means providing random id with upper layer data in the first D2R message, makes the random access procedure designed for CBRA and CFRA to be unified.

For CBRA resources, introduce information in paging message to further indicate 2-step or 3-step random access procedure.

For CFRA resources, devices perform 2-step random access.

  - Respond based on data to report

Considering use cases of reaching devices for collecting data, i.e., sensor data, it is possible that only part of devices have available data. If all the devices respond upon being paged (especially respond to the A-IoT paging message triggering an inventory and command procedure), some of them performing a successful random access-like procedure may  be found with empty senor data buffer.

In some use cases it is possible that not all of the devices being paged need to respond.

Large device populations will bring more collisions in the access stage, causing more delay or access failures. Therefore, for devices without available data to report, it is better not to respond the paging even if it is being paged. In such a case we think allowing intended devices to determine whether to respond to the A-IoT paging message triggering an inventory and command procedure is beneficial. In other words, device(s) can determine whether to respond the paging procedure not only based on the indicated device/group identifier, but also based on whether there’s available data to be reported.

Based on configuration of the A-IoT paging message triggering, device(s) can determine whether to respond the paging procedure based on whether there’s available data to be reported.

  - Paging contains group ID

Support of identifying group of devices leads to how to group devices. We think at least following aspects can be considered:

- A static grouping information. This may base on particular provider or operator, or particular product type;

- Group information decided by A-IoT NAS layer (if supported);

- Group information decided by A-IoT AS layer. Initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information. For example, a device, whose group information is set to “group 1”, should only respond to further commands for “group 1”.

Consider how to group devices from following aspects:

- Group information based on particular provider or operator, or particular product type, which may be a static grouping information;

- Group information decided by NAS layer;

- Group information based on paging: initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information.

Paging message may include group ID that maps to multiple devices. For a group of devices which are selected by the same group ID, at least part of device ID may be common with each other, for example, some initial bits. During random access, it may not very optimized because common bits are not useful for contention resolution.

For devices in a single device group, at least part of device ID may be common with each other because they can be masked by a same group ID.

For devices masked by the same group ID, avoid use common bits for contention resolution.

  - Determination of transmission/access occasion

In RFID, within an access round triggered by “Query” command, determination of transmission/access occasion is based on reception of “QueryRep” command which instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately. Like “QueryRep” command, a transmission/access occasion start message is supported for determination of transmission/access occasion.

Like “QueryRep” command in RFID, a transmission/access occasion start message is supported for determination of transmission/access occasion.

The transmission/access occasion start message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

RAN1#116bis agreed to “Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.” If RAN1 agrees to support frequency-domain multiple access of D2R transmissions, then whether to support individual transmission/access occasion start message for multiple frequency-domain accesses of D2R transmission needs further discussions.

Moreover, RAN1#117 agreed to “Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.” The time count purposes listed in [3] are

•	e.g., device can sleep or skip monitoring PRDCH for a time duration based on reader’s indication or based on the scheduling/timing relations, TDMA of multiple D2R transmissions from different devices by the same R2D transmission etc.

•	Clock assumptions, accuracy

•	Device power consumption, complexity

If RAN1 agrees to support device behaviour to sleep or skip monitoring PRDCH (e.g., the transmission/access occasion start message) for a time duration, then whether to include indicate the cumulative number of past transmission/access occasion in the transmission/access occasion start message needs further discussions.

Regarding the detailed content of transmission/access occasion start message, RAN2 to wait for more RAN1 progresses on frequency-domain multiple access and device behaviour to sleep or skip monitoring PDSCH during a time duration.

  - Multiple-reader issue

As per the general scope of SID [1], the following coverage requirement is considered:

General Scope - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

Given such a small coverage and the need for seamless coverage, a region may be covered or served by multiple readers.

An A-IoT operating area may be covered or served by multiple readers.

In RFID technology, inventory process comprising an interrogator and an associated tag population. An interrogator chooses one of four sessions and inventories tags within that session. From the tag perspective, each session may associate to different interrogators or the same interrogator with different inventory service. In this way sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes, using an independent inventoried flag for each process. The interrogator and associated tag population operate in one and only one session for the duration of an inventory round.

RFID sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes.

When we come to A-IoT device, whether or how do A-IoT devices handle multiple processes with the same or different readers may need to be considered.

RAN2 to study whether or how do A-IoT devices handle multiple processes with the same or different readers.

- Conclusion

Observation 1:	In some use cases it is possible that not all of the devices being paged need to respond.

Observation 2:	For devices in a single device group, at least part of device ID may be common with each other because they can be masked by a same group ID.

Observation 3:	An A-IoT operating area may be covered or served by multiple readers.

Observation 4:	RFID sessions allow tags to keep track of their inventoried status separately for each of four possible time-interleaved inventory processes.

Proposal 1:	For cases that paging message targeting for more than one device, introduce information to indicate whether RA resources indicated in paging is for CBRA or CFRA.

Proposal 2:	For cases that paging message only includes identity for a single device, RA resources indicated in paging is for CFRA.

Proposal 3:	For CBRA resources, introduce information in paging message to further indicate 2-step or 3-step random access procedure.

Proposal 4:	For CFRA resources, devices perform 2-step random access.

Proposal 5:	Based on configuration of the A-IoT paging message triggering, device(s) can determine whether to respond the paging procedure based on whether there’s available data to be reported.

Proposal 6:	Consider how to group devices from following aspects:

a)	Group information based on particular provider or operator, or particular product type, which may be a static grouping information;

b)	Group information decided by NAS layer;

c)	Group information based on paging: initial trigger message(s) may be used to set group information. Device may be identified to be in different groups and further access the reader according to the group information.

Proposal 7:	For devices masked by the same group ID, avoid use common bits for contention resolution.

Proposal 8:	Like “QueryRep” command in RFID, a transmission/access occasion start message is supported for determination of transmission/access occasion.

Proposal 9:	The transmission/access occasion start message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

Proposal 10:	Regarding the detailed content of transmission/access occasion start message, RAN2 to wait for more RAN1 progresses on frequency-domain multiple access and device behaviour to sleep or skip monitoring PDSCH during a time duration.

Proposal 11:	RAN2 to study whether or how do A-IoT devices handle multiple processes with the same or different readers.

- References

[1] RP-240826, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] Report of 3GPP TSG RAN WG2 meeting #127bis, ETSI MCC

[3] R1-2405381, FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT