3GPP TSG-RAN WG2 Meeting #128	R2-2410139

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda item:	8.13.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on control plane and QoS handling for NR sidelink multi-hop relay

Document for:	Discussion and Decision

1 Introduction

In RAN2#127 meeting, RAN2 discussed the issues about control plane procedures and achieved the following agreements [1]:

The following connections are assumed as a baseline to be needed: From last Relay UE perspective: - A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and - A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED. From intermediate relay UE perspective (including first relay UE): - A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and - An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED. FFS what RRC states are supported for the intermediate relay UE. From U2N Remote UE perspective: - A direct (non-relayed) PC5 connection with Intermediate Relay UE, and - An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED. The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18). The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

In this contribution, we will continue discuss the control plane procedures and QoS handling to support multi-hop L2 U2N relay.

2 Discussion

## 2.1 Architecture

For L2 U2N multi-hop relay, there seem to be two main approaches for implementing multi-hop U2N relays. Approach 1 is a fully U2N based approach. The network directly controls each of the intermediate relay UEs using dedicated Uu RRC signalling and all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED. Approach 2 is still U2N-based but with some elements of U2U. Only the last relay UE needs to be controlled using dedicated RRC signalling. The intermediate relays could be in coverage or out of coverage, in any RRC state, in the same or different gNBs. Each intermediate relay is acted as a U2U relay UE.

In the email discussion [2], RAN2 has discussed the pros/cons and the key issues associated with each approach. In our view, R19 multi-hop U2N relay suggest to support Approach 1. In Approach 1, it is aligned with R17 single-hop U2N relay principle that all the involved UEs are connected to the same gNB and controlled by the gNB and has less impact on the specification. While in Approach 2, both U2N and U2U are involved to support multi-hop L2 U2N relay. Except for the last relay UE, intermediate relay UEs are not controlled by gNB which lead to a lot of problems, such as how to configure the local UE ID for the remote UE and how to configure the RLC channel configuration for a relay UE that is in RRC_IDLE/RRC_INACTIVE.

Proposal 1: Support multi-hop U2N relay Approach 1 which the NW needs to directly control each of intermediate relay UEs via Uu RRC signalling and all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED.

## 2.2 Connection establishment procedure

If Approach 1 is adopted, the connection establishment procedure of single-hop U2N relays can be as a baseline for multi-hop U2N relays. As in single-hop relay, connection establishment in the remote UE first requires that each intermediate relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. For all the relay UEs, upon reception of a message on SL-SLB0, they perform connection establishment as though they are acting as a remote UE.

Proposal 2: The connection establishment procedure of single-hop U2N relays can be as a baseline for multi-hop U2N relays if Approach 1 is adopted.

## 2.3 QoS split

In Rel-17 single-hop L2 U2N relay, it is gNB implementation to handle the QoS split over Uu and PC5 for the end-to-end QoS enforcement of a particular session established between Remote UE and network.

Specifically, in remote UE service establishment procedure, CN provides end-to-end QoS requirements of remote UE’s service to gNB. The end-to-end QoS requirements can be understood as QoS parameters between gNB and remote UE, which should be satisfied by Uu link and PC5 link in U2N relay. From the perspective of gNB, it knows the Uu link condition and the PC5 link condition through the measurement mechanisms, so it is able to know how to satisfy the end-to-end QoS requirement by the two links. Based on this, gNB may divide the end-to-end QoS into per hop-by-hop QoS by implementation and make sure each hop-by-hop QoS to be satisfied by Uu link and PC5 link.

In Rel-19 multi-hop L2 U2N relay, the work starts with one additional hop relay firstly. Compared to a single-hop L2 U2N relay, the difference is that there are two relay UEs between gNB and remote UE. The QoS split mechanism can still apply to multi-hop U2N relay.

Proposal 3: For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation can handle the end-to-end QoS split over multi-hop.

To help gNB to perform QoS split or adjust QoS split, UE(s) need to report measurements or other assistance information to the gNB. The existing SL measurement report and CBR measurement reports can be used by gNB to understand PC5 link conditions and determine QoS configuration.

Proposal 4: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

## 2.4 QoS configuration

Once QoS split is done by gNB, it is necessary to convey related QoS configurations to remote UE and/or relay UE. As in single-hop L2 U2N relay, gNB directly configures remote UE and first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling. Moreover, QoS configuration for remote UE and relay UE (first and last) on PC5 hop are configured per PC5 RLC bearer.

Proposal 5: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 6: QoS configuration is configured per PC5 RLC bearer.

## 2.5 QoS flow control

One of the objective in the WID is to specific mechanism supporting QoS handling for multi-hop relaying. However, QoS is likely to be corrupted when network congestion occurs. Packet loss in that case causes TCP to slow down. Thus, some kind of flow control mechanism is required to avoid too packets to be pushed to the overloaded node. Such as, relay UE (first or last) can transmit flow control indication to remote UE for controlling traffic flow when relaying in UL, or relay UE (first or last) can transmit flow control indication to gNB for controlling traffic flow when relaying in DL.

Proposal 7: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

3 Conclusion

In this contribution we discuss the QoS handling for multi-hop relay, with the following proposals:

Proposal 1: Support multi-hop U2N relay Approach 1 which the NW needs to directly control each of intermediate relay UEs via Uu RRC signalling and all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED.

Proposal 2: The connection establishment procedure of single-hop U2N relays can be as a baseline for multi-hop U2N relays if Approach 1 is adopted.

Proposal 3: For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation can handle the end-to-end QoS split over multi-hop.

Proposal 4: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

Proposal 5: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 6: QoS configuration is configured per PC5 RLC bearer.

Proposal 7: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

- References

- Draft report of 3GPP TSG RAN WG2 #127, RAN2 chair (interDigtal), Maastricht, Netherlands, Aug 19th – 23rd, 2024.

- [POST127][402][Relay] Multi-hop relay control plane, InterDigtal, Maastricht, Netherlands, Aug 19th – 23rd, 2024.