3GPP TSG-RAN WG2 Meeting #128 	R2-2410140

Orlando, USA, 18th – 22th Nov, 2024

Agenda Item:	8.2.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on paging procedure of AIoT

Document for:	Discussion and Decision

Introduction

In the RAN2 #125bis and #127-bis meeting, some progress was made on AIoT paging as follows:

RAN2 #125bis Agreements 1. Legacy paging message for device will not be supported. 2. Legacy paging occasion and legacy DRX for the device is not supported. This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well). 3. RAN2 assumes that the device will not support tracking/RAN area update procedure. 4. For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2 RAN2 #126 Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details. RAN2 #127 Agreements Paging Triggers/Retransmission: 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices Definition of the access occasion: => wait for further RAN1 progress on indication of the start of access occasion. RAN2 #127 Agreements 1 The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. 2 Based on this information the device determines whether to skip sending the response to paging. 3 From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. 4 If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA 5 For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, the message content of AIoT paging and paging response issues will be analyzed. And we provide our opinions.

Discussion on paging

2.2 Message content

- Resource allocation

In previous meeting, RAN2 has agreed that AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

For contention-based random access, AIoT paging message can indicate the shared resource. The last meeting in RACH section, we has agreed that as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It can be further discussed if this is applicable to FDMA case. And in RAN1 meeting, they agreed that a R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). For FDMA, some frequency resource can be indicated in R2D trigger message, for example Y frequency resource. The X time domain resource(s) refer to the number of green time resource blocks in a grey block in Figure 1 [1]. But there is no agreement about how to determine the access occasion.

Figure 1 the overall framework example of slotted ALOHA random access

In our opinion, AIoT paging message can indicate the total number of “slot” for the D2R response message, for example AIoT paging message includes a parameter Q (like RFID mechanism, the total number of ‘slot’ is 2Q), or a parameter N indicating the total number of “slot”. The only difference between the two parameter is that the indication methods are different and the occupied bits are different. The parameter Q can only indicate available ‘slot’ resources in multiples of 2 by using fewer bits. And the parameter N can indicate any number of slots but takes up more bits.

The “Slot” means X*Y resources indicated by R2D trigger message. This method can decouple the spec impact of RAN1 and RAN2. RAN2 standardizes the number of “slot” and RAN1 is to indicate the location of time-frequency resources within each slot. Multiple R2D trigger messages are required to indicate X*Y access occasions because large SFO of AIoT device limits the value of X, and the frequency-shit capability of AIoT device limits the value of Y. So AIoT paging only needs to indicate the number of ‘slot’. The location indication of access occasions within one ‘slot’ depends on RAN1.

Proposal 1: For contention-based RA, AIoT paging message indicates the total number of “slot” for the D2R response message. E.g., a parameter Q (like RFID mechanism), or a parameter N indicating the number of “slot”. The location indication of access occasions within a ‘slot’ depends on RAN1.

For Contention-free RA, dedicated resource (s) can be configured for one device or multiple devices. When the resource(s) are configured for multiple devices, multiple devices IDs should be included to map to dedicated resources. The mapping rules can be pre-defined. For example, dedicated resources are mapped to each device in order of the device ID.

Proposal 2: For contention-free RA, AIoT paging message includes dedicated access occasion resource for corresponding device.

- Random Access type

The last meeting, RAN2 agreed that for all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2-step, 3-step, CFRA) or try to unify the design for 2-step and 3-step. For implicitly indication, CBRA and CFRA may be distinguished by whether the reader allocates dedicated resources or shared resources. But for 2-step CBRA and 3-step CBRA, implicitly indicating the RACH type through resource allocation is inaccurate because the response resources for 2-step RACH can also be small, e.g. due to target coverage/data rate, energy consumption/device availability, etc. the TB size may be small. So at least for 2-step CBRA and 3-step CBRA, AIoT paging message should indicate the RACH type.

Proposal 3: At least for 2-step CBRA and 3-step CBRA, AIoT paging message should indicate the RACH type.

## 2.2 Paging Response

RAN2 #127-bis agreement: The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase.

In last meeting, we agreed that AIoT paging message can include information to avoid duplicate response from the device to a reader. FFS how to indicate. There are two options according to the current contributions: 1) session ID in AIoT paging message; 2) NDI-like information in AIoT paging message.

For option 1, In SA2, there are solutions that AIoT service request can be uniquely identified by session ID, which means that the same AIoT service request is associated with a same session ID [2]. The intention is that one AIoT service may be transferred to multiple readers. And multiple readers will trigger paralleled paging procedure with a same high layer session ID. From device side, it only respond to the first paging message it receives, instead of repeatedly responding to paging messages carrying the same session ID.

For option 2, if there is multiple different parallel service, the NDI indication information cannot enable the device to correctly identify the subsequent paging of different services.

Proposal 4: AIoT paging message includes the upper layer session ID defined by SA2 without introducing additional RAN session ID.

Conclusions

In this contribution, we discuss the Rel-19 paging procedure and functionality. And the proposals are given as follows:

Proposal 1: For contention-based RA, AIoT paging message indicates the total number of “slot” for the D2R response message. E.g., a parameter Q (like RFID mechanism), or a parameter N indicating the number of “slot”. The location indication of access occasions within a ‘slot’ depends on RAN1.

Proposal 2: For contention-free RA, AIoT paging message includes dedicated access occasion resource for corresponding device.

Proposal 3: At least for 2-step CBRA and 3-step CBRA, AIoT paging message should indicate the RACH type.

Proposal 4: AIoT paging message includes the upper layer session ID defined by SA2 without introducing additional RAN session ID.

References

[1] R2-2407985, Report of [POST127][033][AIoT] Random Access.

[2] TR 23.700-13, SA2 Technical report of Study on architecture support of ambient power-enabled IoT.