3GPP TSG-RAN WG2#128	R2-2410146

Orlando, USA, Nov. 18th – 22nd, 2024

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Discussion on on-demand SIB1 procedure for NES

Document for:	Discussion and decision

# Background

In RAN2#127bis meeting, RAN2 made some agreements as follows [1].

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we would like to discuss and provide our views on the on-demand SIB1 operation from RAN2 perspective.

# Discussion

## UL WUS procedure

### UL WUS configuration

UL WUS configuration by RRC release message

In RAN2#126, the following issue was discussed and RAN2 agreed whether or not to adopt this as an option in stage 3.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3

In [2], signalling overhead might be reduced by using RRC release message. However, if the NW does not broadcast the UL WUS configuration, the NW cannot update UL WUS configuration after the NES UE is in the idle/inactive mode. In addition, the NES UE re-selected from a neighboring cell to Cell A does not have a valid UL WUS configuration. So, it imposes another mechanism to provide the UL WUS configuration, e.g., periodical SI update. Therefore, it is suspicious this option can reduce overhead of broadcast signalling for UL WUS configuration.

Proposal 1:	RRC release message assisted intra-cell UL WUS is not supported.

Contents of UL WUS configuration

In RAN1#118bis meeting, there are following agreements on the parameters included in the UL WUS configuration.

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

In the last RAN2 meeting, it was agreed that the NES cell’s UL WUS configuration is included in a new SIB (including its own UL WUS configuration). Therefore, the new SIB in Cell A or in the NES cell should include the above RAN1 agreed parameters for UL WUS configuration.

Proposal 2:	The parameters agreed by RAN1 to be included in UL WUS configuration are included in the new SIB for UL WUS configuration.

### UL WUS transmission

Triggering condition for UL WUS

For the agreement “A cell for which SIB1 request configuration is available, can periodically broadcast SIB1”, it means the NES UE may attempt to receive SIB1 if the NES UE already has UL WUS configuration corresponding to the NES cell.

For example, in case of return from out of coverage or cell selection during RRC re-establishment, if the NES UE has UL WUS configuration corresponding to the selected NES cell, there is no reason to prevent SIB1 reception of NES cell. One question is whether the NES UE can trigger UL WUS transmission to the cell if SIB1 is not broadcasted, e.g., due to on-demand mode. In this case, it should not be a trigger of UL WUS transmission, because UL WUS configuration may be changed but the NES UE does not know it.

Proposal 3:	For return from out of coverage or RRC re-establishment, the NES UE does not trigger UL WUS transmission, even if the NES cell is selected and the NES UE has a corresponding UL WUS configuration.

### NES cell configuration

MIB configuration for NES cell

In the last meeting, RAN2 agreed “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell” but it was not yet discussed how the NES UE checks the SIB1 broadcast status before UL WUS transmission. To achieve this, some companies assumed the current SIB1 broadcast status should be indicated by MIB.

In RAN1#118bis meeting, RAN1 agreed a specific K_SSB value is used in MIB of the NES cell.

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

RAN1 is still discussing whether the pdcch-ConfigSIB1 can be used to identify the frequency location of the nearest CD-SSB (if K_SSB > 23 for FR1 and K_SSB > 11 for FR2 but K_SSB is not equal to 30 in FR1 or 14 in FR2) or repurposed to identify the frequency location of SSB for Cell A (if K_SSB is equal to 30 in FR1 or 14 in FR2). In other words, although the ssb-SubcarrierOffset and the pdcch-ConfigSIB1 can be provided by the UL WUS configuration from Cell A, there is still no spare bit in NES cell’s MIB.

Observation 1:	Based on the RAN1 discussion, it is not possible in MIB to explicitly indicate the current SIB1 broadcast status of NES cell.

Observation 2:	For NES cell, RAN1 may use the K_SSB values with 23 < K_SSB ≤ 31 for FR1 and 11 < K_SSB ≤ 14 for FR2.

Based on the above observations, Rel-19 NES UE can ignore the K_SSB value indicating ‘no SIB1’ if UL WUS configuration for the NES cell is available, whereas Rel-19 NES UE is barred by the K_SSB value indicating ‘no SIB1’ as in legacy if UL WUS configuration for the NES cell is not available. It is up to RAN1 whether an actual K_SSB value is provided by UL WUS configuration.

Proposal 4:	Rel-19 NES UE can ignore the K_SSB value if UL WUS configuration for the NES cell is available.

Proposal 5:	RAN2 to confirm Rel-19 NES UE is barred by K_SSB value with K_SSB > 23 for FR1 and K_SSB > 11 for FR2 as in legacy if UL WUS configuration for the NES cell is not available.

Besides that, RAN2 discussed whether the legacy UEs can access the NES cell if NES cell starts to transmit SIB1 periodically, but there is no agreement so far. Table.1 summarises access restriction corresponding to SIB1 status based on RAN1 agreements.

Table.1 - Access restriction corresponding to SIB1 status

| Access restriction |  | MIB | SIB1 |
| --- | --- | --- | --- |
| 1 | Bar legacy UE only, Rel-19 NES UE is allowed to access | No SIB1 indication (K_SSB > 23 for FR1 and K_SSB > 11 for FR2) | No transmission (On-demand transmission) |
| 2 | Bar legacy UE only, Rel-19 NES UE is allowed to access | No SIB1 indication (K_SSB > 23 for FR1 and K_SSB > 11 for FR2) | Ongoing on-demand transmission within time window |
| 3 | Bar legacy UE only, Rel-19 NES UE is allowed to access | No SIB1 indication (K_SSB > 23 for FR1 and K_SSB > 11 for FR2) | Periodical transmission |
| 3A | Legacy and Rel-19 NES UE are allowed to access (normal operation) | No barring | Periodical transmission |

When SIB1 is periodically broadcasted, the NES cell may maintain K_SSB value in MIB to bar the legacy UEs (Case 3) or the NES cell may change K_SSB value in MIB to alleviate access restriction (Case 3A). If the NW should always change K_SSB value whenever SIB1 is periodically broadcasted, the UE is aware of the current SIB1 broadcast status by MIB itself. However, when the NES cell is prevented by the excluded list in SIB3/4, the legacy UEs still don’t consider the NES cell as a candidate until the SI is updated. Thus, it should be carefully discussed whether MIB barring would be alleviated due to SIB1 broadcast status. Another option is that UL WUS configuration contains the SIB1 broadcast status per the NES cell. But as SIB1 broadcast status may be changed dynamically, it is not suitable to include it in UL WUS configuration. As a result, if K_SSB value is maintained to bar the legacy UEs, to be aware of the SIB1 broadcast status, the NES UE should attempt to receive SIB1 in NES cell directly.

Proposal 6:	RAN2 to discuss the following options for checking SIB1 broadcast status before UL WUS transmission:

- Option 1: the value of the ssb-SubcarrierOffset in MIB implicitly indicates SIB1 broadcast status in NES cell (i.e., K_SSB ≤ 23 for FR1 and K_SSB ≤ 11 for FR2),

- Option 2: the NES UE should attempt to receive SIB1 from the NES cell directly based on the corresponding UL WUS configuration.

### On-demand SIB1 reception

On-demand SIB1 reception

So far in RAN1, there are following agreements on PDCCH monitoring occasions for on demand SIB1:

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

On top of that, in RAN1#118bis meeting, there are following agreement on PDCCH monitoring occasions for RAR:

Agreement Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

In our understanding, the configuration of CORESET #0/SS#0 and the time window for type 0 PDCCH monitoring occasions for RAR and on demand SIB1 can be further discussed by RAN1. RAN2 can wait for RAN1 conclusion and then discuss how to include the required configuration in the corresponding signalling.

Proposal 7:	RAN2 to wait for the RAN1 conclusion on the configuration of RAR reception and SIB1 reception.

On-demand SIB1 update in NES cell

In RAN2#127 meeting, it was agreed that UE keeps SIB1 updated while on the NES cell via regular SI modification procedure. An issue that can be further discussed is how long the network should transmit the on-demand SIB1 from the network modification period, as shown in the following figure.

Figure 1. On-demand SIB1 update in the NES cell

In our view, the network can transmit the on-demand SIB1 during the whole next modification period (i.e., legacy SI update) or in a time window with length equal to the RAN1 defined time window for type 0 PDCCH monitoring occasions for on-demand SIB1 reception. Regardless of the options, the UE can assume the updated SIB1 is transmitted from the next modification period. If the UE is unable to acquire the updated SIB1, the UE should consider the NES cell as barred, because the UL WUS configuration may be obsoleted due to SI update.

Proposal 8:	RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

## Access restriction for legacy UEs

Cell selection

As described in sub-section 2.1.3, the NES cell supporting on-demand SIB1 needs to set a specific value for the ssb-SubcarrierOffset (i.e., K_SSB > 23 for FR1 and K_SSB >11 for FR2). The legacy UE does not camp on the NES cell because these values mean that this cell does not provide SIB1 from Rel-15. Thus, no further mechanism is needed for barring the legacy UEs.

Cell re-selection

In order to prevent the legacy UE from re-selecting the Rel-19 NES cell, the current excluded cell list (i.e., IntraFreqExcludedCellList or InterFreqExcludedCellList) can be used. It is beneficial to reduce the attempts of SSB/SIB1 acquisition for legacy UEs. Hence, the excluded cell list should be considered.

However, Rel-19 NES UE should be allowed to access the Rel-19 NES cell even if the cell is listed in the excluded cell list. One solution is the Rel-19 NES UE can consider the listed cell in the excluded cell list as candidate if the corresponding UL WUS configuration is obtained from Cell A. Otherwise (i.e., if the UE does not have the corresponding UL WUS configuration), the cell(s) listed in the excluded cell list would not be considered as candidate, as in legacy. Another solution is that a new excluded cell list is additionally provided for Rel-19 NES UE to indicate the excluded cell(s).

Comparing the two solutions above, both solutions can work for the legacy UEs but the second solution may need more signalling overhead. For example, if NW would like to indicate cell #m is not considered as candidate for both the legacy and Rel-19 NES UEs, the NW has to include the cell #m into both the legacy and new excluded cell lists. Therefore, the first solution is preferred from the signalling overhead perspective.

Proposal 10: The Rel-19 NES UE can consider the cell in the legacy excluded cell list as candidate, if the UE has the UL WUS configuration corresponding to the cell in the excluded cell list.

# Conclusion

In this contribution, the following proposals and observations were made:

UL WUS configuration by RRC release message

Proposal 1:	RRC release message assisted intra-cell UL WUS is not supported.

Contents of UL WUS configuration

Proposal 2:	The parameters agreed by RAN1 to be included in UL WUS configuration are included in the new SIB for UL WUS configuration.

Triggering condition for UL WUS

Proposal 3:	For return from out of coverage or RRC re-establishment, the NES UE does not trigger UL WUS transmission, even if the NES cell is selected and the NES UE has a corresponding UL WUS configuration.

MIB configuration for NES cell

Observation 1: Based on the RAN1 discussion, it is not possible in MIB to explicitly indicate the current SIB1 broadcast status of NES cell.

Observation 2: For NES cell, RAN1 may use the K_SSB values with 23 < K_SSB ≤ 31 for FR1 and 11 < K_SSB ≤ 14 for FR2.

Proposal 4:	Rel-19 NES UE can ignore the K_SSB value if UL WUS configuration for the NES cell is available.

Proposal 5:	RAN2 to confirm Rel-19 NES UE is barred by K_SSB value with K_SSB > 23 for FR1 and K_SSB > 11 for FR2 as in legacy if UL WUS configuration for the NES cell is not available.

Proposal 6:	RAN2 to discuss the following options for checking SIB1 broadcast status before UL WUS transmission:

- Option 1: the value of the ssb-SubcarrierOffset in MIB implicitly indicates SIB1 broadcast status in NES cell (i.e., K_SSB ≤ 23 for FR1 and K_SSB ≤ 11 for FR2),

- Option 2: the NES UE should attempt to receive SIB1 from the NES cell directly based on the corresponding UL WUS configuration.

On-demand SIB1 reception

Proposal 7:	RAN2 to wait for the RAN1 conclusion on the configuration of RAR reception and SIB1 reception.

On-demand SIB1 update in NES cell

Proposal 8:	RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 9:	The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

Cell re-selection

Proposal 10:	The Rel-19 NES UE can consider the cell in the legacy excluded cell list as candidate, if the UE has the UL WUS configuration corresponding to the cell in the excluded cell list.

# References

- R2-2409211, Report from session on V2X/SL, R19 NES and MOB

- R2-2404634, Apple