3GPP TSG-RAN WG2 Meeting # 128	R2-2410149

# Orlando, USA,  November 18th - 22th 2024

Agenda Item:	8.13.3

Source:	Ericsson

Title:	Discussion on control plane procedures

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on control plane procedures have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

We discuss aspects on control plane procedures in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

## 2.1 Agreements

Agreements:

RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms.

RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

RAN2 has already agreed to minimize the impact of hop count on the multi-hop relay mechanisms, and design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

Given the above agreements in mind, we further elaborate our views on the issues regarding control plane procedure.

An example of the multi-hop L2 U2N relay is illustrated in Figure 1.

Figure 1: an example of a multi-hop L2 U2N relay.

According to the WID [1], RAN2 will specify service continuity for intra-gNB scenarios. It means that Remote UE, and all Relay UEs for the same path are served by the same gNB. The first question that RAN2 needs to answer is whether Intermediate Relay UE(s) can be served in a different cell than U2N Relay UE. There are two scenarios expected.

Scenario 1: Intermediate Relay UE(s) and U2N Relay UE are in the same serving cell.

Scenario 2: Intermediate Relay UE(s) and U2N Relay UE are in different serving cells of the same gNB. With more additional Intermediate Relay UEs on the same path, Intermediate Relay UEs may be in different cells.

Scenario 2 may require additional efforts for RAN2 than Scenario 1. However, we think the additional efforts are manageable.

Therefore, we would like to RAN2 to confirm this before executing any detailed study.

RAN2 to work on two scenarios including:

  - Scenario 1: Intermediate Relay UE(s) and Last  Relay UE are in the same serving cell.

  - Scenario 2: Relay UEs can be in different serving cells of the same gNB.

## 2.2 RRC connection management

After a Remote UE has completed the discovery procedure to reach the target U2N Relay UE, the Remote UE needs to establish its own PDU sessions/DRBs with the network before user plane data transmission. The NR sidelink PC5 unicast link establishment procedures can be used to setup a secure unicast link between Remote UE and Intermediate Relay UE, and between Intermediate Relay UE and U2N Relay UE, before Remote UE establishes a Uu RRC connection with the network via U2N Relay UE. Same approach can be used for NR sidelink PC5 unicast link establishment procedure to setup a secure unicast link between Intermediate Relay UE and another Intermediate Relay UE, before Remote UE establishes a Uu RRC connection with the network via U2N Relay UE when we consider 2 additional Relays scenario in later meetings. In this case, the NR sidelink PC5 unicast link establishment procedures would need to be performed independently for each hop. However, the NR sidelink PC5 unicast link establishment procedures will propagate back towards the network, until it reaches the L2 U2N Relay.

The Remote UE selects all Relay UEs after Relay (re)selection, i.e., selects the path to reach U2N Relay UE. After that, the Remote UE can initiate the PC5 unicast link establishment towards the selected Intermediate Relay UE. However, in order to trigger the Intermediate Relay UE to initiate the PC5 unicast link establishment towards its next Relay UE (e.g., U2N Relay UE), the Intermediate UE needs to know which UE is selected as the U2N Relay UE).

To establish PC5 unicast link between a selected Intermediate Relay UE and a selected last Relay UE, the Intermediate Relay UE needs to be aware of which UE has been selected as the last Relay UE.

It seems that this issue has been also somewhat discussed in SA2. As shown in the below, how path information is applied in link management, has been captured in SA2 conclusions for multi-hop U2N relay (see clause 8.1 Conclusion for K1#1 in TR 23.700-03).

-	When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

It is sufficient for each Relay UE to apply the path information in the unicast link establishment towards the U2N Relay UE. The path information is built by Remote UE. therefore, Remote UE would need to signal each Relay UE of the path information. In addition, the link establishment procedure involving the path information for L3 multi-hop relay, being discussed in SA2 can be reused for L2 multi-hop U2N relay. Since the link establishment procedure is common for L2 multi-hop U2N relay and L3 multi-hop U2N relay.

As captured in TR 23.700-03, for multi-hop U2N relay, Remote UE performs link management procedure to reach last Relay UE according to the path information which is an ordered list of User Info ID of Relays in the selected path.

As captured in TR 23.700-03, the L2 link establishment procedure involving the path information for multi-hop U2N relay, being discussed in SA2 is also applicable for L2 multi-hop relay. Since the link establishment procedure is common for L2 multi-hop relay and L3 multi-hop relay.

After relay (re)selection, Remote UE can build the path information for the selected path based on the selected Relays towards the selected U2N Relay UE.

After relay (re)selection, remote UE builds the path information based on selected Relays for the selected path towards last Relay UE.

Upon initiating the link establishment procedure for the first PC5 hop, Remote UE sends the DCR including the path information to Intermediate Relay UE. After reception of the DCR message from Remote UE, the Intermediate Relay UE stores the path information. When Intermediate Relay UE determines to initiate the link establishment procedure towards the selected U2N Relay UE according to the path information, Intermediate Relay UE includes the path information (without any update) in the DCR and sends the DCR to the selected U2N Relay UE. Upon reception of the DCR, the selected U2N Relay UE may also store the path information, based on which the U2N Relay UE knows that the Remote UE has selected it to be the U2N Relay UE. After that, Remote UE and the U2N Relay UE can follow the legacy procedure (i.e., as captured in clause 16.12.5.1 of TS 38.300) can be reused to establish RRC connection for Remote UE towards the gNB serving the U2N Relay UE.

During the L2 link establishment procedure, Remote UE sends the DCR including the path information to the selected Intermediate Relay UE for the first hop link establishment. The selected Intermediate Relay UE sends the DCR including the path information to the selected Relay UE for the second hop link establishment. The procedure repeats until reaching the last relay UE on the selected path.

RAN2 can assume the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. Based on this, it is recommended that RAN2 sends LS to SA2 indicating this assumption to trigger SA2 further work on L2 multi-hop U2N relay.

RAN2 assumes the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. RAN2 sends LS to SA2 indicating this.

It may be beneficial for the gNB to be aware of the path information for the selected path. In Rel-17 L2 U2N relay, it was the relay UE to report to the gNB of the remote UE ID via SidelinkUEInformationNR. Similarly, in case of multi-hop U2N relay, the L2 U2N Relay UE can report the path information (provided by upper layer) to the gNB via SidelinkUEInformationNR. It is expected that the gNB (as described in our accompany paper [2], it is assumed all Relay UEs are served by the same gNB) can use the path information at least for the below purposes

- Allocate local ID for Remote UE

- configure (PC5 or Uu) RLC channels for relaying SRB1/2 purpose

- configure (PC5 or Uu) RLC channels for relaying DRBs purpose

It is beneficial for L2 U2N Last Relay UE to inform the gNB of the path information for the selected path.

Last Relay UE signals the gNB of the path information (provided by upper layer) for the selected path via SidelinkUEInformationNR..

## 2.2 RRC state

In the legacy, both U2N/U2U remote UE and U2N/U2U relay UE can be in any RRC state (i.e., RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The RRC states combination of Remote UE in RRC_CONNECTED and relay UE in RRC_IDLE is not supported. The RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_INACTIVE is not supported. The same rules would be applicable for Remote UE and Last Relay U2N Relay.

Same as in legacy, when L2 U2N Remote UE is in RRC_CONNECTED, L2 U2N Last Relay UE also needs to be in RRC_CONNECTED.

Since L2 Intermediate Relay UE may be out of coverage. It seems unnecessary for Intermediate Relay UE to be mandated in RRC_CONNECTED. This issue is also being discussed in the email discussion [Post127][402][Relay] Multi-hop relay control plane (InterDigital).

For the email discussion, approach 1 and approach 2 will be used as follows:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC, i.e., Intermediate Relay UEs are required to be in RRC_CONNECTED.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC, i.e., Intermediate Relay UEs are not mandated to be in RRC_CONNECTED.

Companies have raised pros and cons for the two approaches.

In our view, although both approaches are feasible to implement multi-hop U2N relay, Approach 1 has clear limitation including

- during relay discovery, relay (re)selection phase, first relay UE, intermediate relay UE(s) are required to select the same last relay UE as the remote UE for their own relay connection, this would add additional design complexity/restrictions for RAN2 and SA2.

- The gNB needs to maintain SRAP configuration and local IDs for each Intermediate relay UE. The gNB operational complexity for relay operation is increased. Collision probability of local IDs is also increased due to increased number of the remote UE contexts.

- the failure probability for remote UE to find/establish a path to the network/gNB is much increased. Remote UE may fail to find a path due to restriction of the intermediate relay UE (e.g., the intermediate UE’s serving cell needs to be the same as the last relay UE, the intermediate UE’s serving cell needs to support U2N relay, the latency incurred due to intermediate relay UE’s connection setup/path switch to the serving cell of the last relay UE etc).

- the remote UE connection establishment always triggers connection establishment in each of the relay UEs (if they are in IDLE/INACTIVE), which results in higher signaling overhead and long latency. This is because that each intermediate relay UE goes to RRC_CONNECT after its adjacent parent relay UE goes to RRC_CONNECTED first. It may even occur that an intermediate relay UE’s RRC_CONNECTION establishment may fail due to its parent relay UE takes too long time to establish its RRC_CONNECTION. In this case, remote UE may need to trigger reselection of relay UEs, which adds further design complexity.

- All relay UEs need to be served in the same cell, i.e., the serving cell of the Last Relay UE, which adds too much restriction.

- For all relay UEs except the last relay UE, they need to be configured with a remote UE Uu DRB configuration and SRAP configuration to act as a remote UE (even without having any Uu traffic towards the gNB). In the legacy framework and procedure (Uu and Rel-17 U2N), a UE (remote UE) can not setup a connection to the gNB, with only SRB, the UE needs to have at least one DRB (e.g., default DRB/best effort DRB), in order to establish a PDU session. In this case, all relay UE’s except the last relay would then need to have to establish at least one best effort DRB, although they don’t have any own traffic to the network. this would be wasteful for the resources. So, basically, RAN2 needs to study two options

  - Option 1: each relay UE except the last relay needs to establish at least a best effort DRB, although the relay UE has no own Uu traffic.

  - Option 2: each relay UE except the last relay UE only needs to establish SRBs with DRBs, which would need additional spec changes to allow this.

On the contrary, Approach 2 gives much better flexibility. Relay UEs are allowed to belong to different cells. In addition, intermediate relay UEs are not mandated to be in RRC_CONNECTED either. Due to each intermediate relay UE is not mandated to be in RRC_CONNECTED, high signaling overhead and long latency for Remote UE’s RRC connection setup towards the gNB can be avoided. Thus, with Approach 2, multi-hop U2N relay can be supported for more use cases and in larger deployment scope.

For Approach 1, during relay discovery, relay (re)selection phase, first relay UE, intermediate relay UE(s) are required to select the same last relay UE as the remote UE for their own relay connection, this would add additional design complexity/restrictions for RAN2 and SA2.

For Approach 1, the gNB needs to maintain SRAP configuration and local IDs for each Intermediate relay UE. The gNB operational complexity for relay operation is increased. Collision probability of local IDs is also increased due to increased number of the remote UE contexts.

For Approach 1, the failure probability for remote UE to find/establish a path to the network/gNB is much increased. Remote UE may fail to find a path due to restriction of the intermediate relay UE (e.g., the intermediate UE’s serving cell needs to be the same as the last relay UE, the intermediate UE’s serving cell needs to support U2N relay, the latency incurred due to intermediate relay UE’s connection setup/path switch to the serving cell of the last relay UE etc).

For Approach 1, the remote UE connection establishment always triggers connection establishment in each of the relay UEs (if they are in IDLE/INACTIVE), which results in higher signaling overhead and long latency. It may even occur that an intermediate relay UE’s RRC_CONNECTION establishment may fail due to its parent relay UE takes too long time to establish its RRC_CONNECTION. In this case, remote UE may need to trigger relay reselection, which adds further design complexity.

For Approach 1, All relay UEs need to be served in the same cell, i.e., the serving cell of the Last Relay UE, which adds too much restriction.

For Approach 1, for all relay UEs except the last relay UE, they need to be configured with a remote UE Uu DRB configuration and SRAP configuration to act as a remote UE (even without having any Uu traffic towards the gNB).

Therefore, we think Approach 2 shall be adopted, meanwhile Approach 1 shall be ruled out in this release.

To down-select Approach 2 between Approach 1 and Approach 2, i.e., L2 U2N Intermediate Relay UEs are not mandated to be in RRC_CONNECTED.

## 2.3 E2E QoS Split

End-to-end QoS is guaranteed in relaying by splitting the QoS requirements (i.e., latency) between the different hops. In approach 1, each relay UE is in RRC_CONNECTED when the U2N Remote UE is RRC_CONNECTED.  It is therefore natural that the gNB performs the QoS splitting for each hop, considering this was done for single hop U2N relays where the L2 U2N relay also needed to be RRC_CONNECTED.

In approach 2, only the Last Relay UE needs to be in RRC_CONNECTED.  Since the Uu hop is managed by the network, it should be the network to determine the QoS split (i.e., the portion of the latency) associated with the Uu hop.

For Approach 2, it is the network to determine the QoS split (i.e., the portion of the latency) associated with the Uu hop.

One remaining question for Approach 2 is how to perform the splitting over the path between the Last Relay UE and the U2N Remote UE.  If the relays are all in RRC_CONNECTED, the situation is the same as the assumption for approach 1, and the network can perform the splitting.  On the other hand, if the relays are in RRC_IDLE/RRC_INACTIVE/OOC, it would be possible to use the U2U mechanism in Rel18 and leave the splitting to the relay UE implementation.

For approach 2, there are two options expected for QoS split of each link of the path between the Last Relay UE and the U2N Remote UE.

Option 1: Network performs the split.

Option 2: Relay UE serving that link, performs the split

Option 1 is more simple way, and Remote UE’s serving gNB should be an anchor node, it can provide PC5 RLC Channel directly and forwarded to the intermediate Relay UE. Option A is more like with approach 1, the only difference is how to provide configuration to the intermediate relay UE, using Uu RRC connection or PC5 RRC connection.

Option 2 is extended by existing U2U relay. Actually, SA2 already agreed Option 2 for L3 based multi-hop U2N relay, it can be reused for L2 relay.

Therefore, we think both options area feasible.

For Approach 2, RAN2 to discuss it is the network (gNB) or the relay UE serving that link, to perform QoS split of each link of the path between the L2 U2N Last Relay UE and the L2 U2N Remote UE.

## 2.3 Other aspects

In legacy, after Uu RLF is declared, the L2 U2N Relay UE takes the following action on top of the actions described in clause 9.2.7:

-	a PC5-RRC message can be used for sending an indication to its connected L2 U2N Remote UE(s), which may trigger RRC connection re-establishment for L2 U2N Remote UE; or

-	indicating to upper layer to trigger PC5 unicast link release.

Upon detecting PC5 RLF, the L2 U2N Remote UE may trigger RRC connection re-establishment.

The above legacy procedure can be reused for L2 U2N Last Relay UE and L2 U2N Remote UE with the below enhancements

- An indication on detection of Uu RLF at L2 U2N Last Relay UE needs to be forwarded to connected L2 U2N Remote UE(s) and L2 U2N Intermediate Relay UEs; how to signal the indication to all connected L2 U2N Remote UE(s) are FFS, since the per hop legacy PC5 RRC cannot be used.

- L2 U2N Relay UE need to signal L2 U2N Intermediate Relay UE(s).

L2 U2N Relay UE cannot use per hop legacy PC5 RRC to inform L2 U2N Remote UE of detected Uu RLF event.

Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE signals an indication to its connected L2 U2N Remote UE(s), which may trigger RRC connection re-establishment for L2 U2N Remote UE. FFS on the signaling details.

Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE signals an indication to L2 U2N Intermediate Relay UE(s). FFS on the signaling details.

Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE indicates to upper layer to trigger PC5 unicast link release towards adjacent L2 U2N Intermediate Relay UE.

In Rel-17, the L2 U2N Remote UE may perform the following actions during the RRC connection re-establishment procedure:

-	If only suitable cell(s) are available, the L2 U2N Remote UE initiates RRC re-establishment procedure towards a suitable cell;

-	If only suitable L2 U2N Relay UE(s) are available, the L2 U2N Remote UE initiates RRC re-establishment procedure towards a suitable relay UE's serving cell via selected suitable L2 U2N Relay;

-	If both a suitable cell and a suitable relay are available, the L2 U2N Remote UE can select either one to initiate RRC re-establishment procedure based on implementation.

For multi-hop relay, upon triggering of RRC connection re-establishment, Remote UE can perform similar actions as in legacy.

The legacy L2 U2N Remote UE actions during the RRC connection re-establishment procedure can be reused for L2 multi-hop U2N relay.

In Rel-17, the L2 U2N Remote UE in RRC_CONNECTED can use the on-demand SIB framework to request the SIB(s) via L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

In Rel-17, Remote UE needs to provide paging assistance information (e.g., its ID and paging DRX cycle) to a Relay UE so that the Relay UE can monitor Remote UE’s POs. The Relay UE sends the paging messages to Remote UE via PC5-RRC.

For multi-hop relay, Remote UE connects to the network via one or multiple Intermediate Relay UEs and one U2N Relay UE. In such case, it is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell of the same gNB than L2 U2N Relay UE.

It is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell than L2 U2N Relay UE.

Similar as Uu RLF, the legacy per hop PC5-RRC signalling cannot be directly applied by L2 U2N Remote UE and L2 U2N Relay UE to signal information on system information or paging between each other because Remote UE and they are separated by more than one hop.

RAN2 to discuss how to signal system information and/or paging information between L2 U2N Relay UE and L2 U2N Remote UE.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	To establish PC5 unicast link between a selected Intermediate Relay UE and a selected last Relay UE, the Intermediate Relay UE needs to be aware of which UE has been selected as the last Relay UE.

Observation 2	As captured in TR 23.700-03, for multi-hop U2N relay, Remote UE performs link management procedure to reach last Relay UE according to the path information which is an ordered list of User Info ID of Relays in the selected path.

Observation 3	As captured in TR 23.700-03, the L2 link establishment procedure involving the path information for multi-hop U2N relay, being discussed in SA2 is also applicable for L2 multi-hop relay. Since the link establishment procedure is common for L2 multi-hop relay and L3 multi-hop relay.

Observation 4	After relay (re)selection, remote UE builds the path information based on selected Relays for the selected path towards last Relay UE.

Observation 5	During the L2 link establishment procedure, Remote UE sends the DCR including the path information to the selected Intermediate Relay UE for the first hop link establishment. The selected Intermediate Relay UE sends the DCR including the path information to the selected Relay UE for the second hop link establishment. The procedure repeats until reaching the last relay UE on the selected path.

Observation 6	It is beneficial for L2 U2N Last Relay UE to inform the gNB of the path information for the selected path.

Observation 7	For Approach 1, during relay discovery, relay (re)selection phase, first relay UE, intermediate relay UE(s) are required to select the same last relay UE as the remote UE for their own relay connection, this would add additional design complexity/restrictions for RAN2 and SA2.

Observation 8	For Approach 1, the gNB needs to maintain SRAP configuration and local IDs for each Intermediate relay UE. The gNB operational complexity for relay operation is increased. Collision probability of local IDs is also increased due to increased number of the remote UE contexts.

Observation 9	For Approach 1, the failure probability for remote UE to find/establish a path to the network/gNB is much increased. Remote UE may fail to find a path due to restriction of the intermediate relay UE (e.g., the intermediate UE’s serving cell needs to be the same as the last relay UE, the intermediate UE’s serving cell needs to support U2N relay, the latency incurred due to intermediate relay UE’s connection setup/path switch to the serving cell of the last relay UE etc).

Observation 10	For Approach 1, the remote UE connection establishment always triggers connection establishment in each of the relay UEs (if they are in IDLE/INACTIVE), which results in higher signaling overhead and long latency. It may even occur that an intermediate relay UE’s RRC_CONNECTION establishment may fail due to its parent relay UE takes too long time to establish its RRC_CONNECTION. In this case, remote UE may need to trigger relay reselection, which adds further design complexity.

Observation 11	For Approach 1, All relay UEs need to be served in the same cell, i.e., the serving cell of the Last Relay UE, which adds too much restriction.

Observation 12	For Approach 1, for all relay UEs except the last relay UE, they need to be configured with a remote UE Uu DRB configuration and SRAP configuration to act as a remote UE (even without having any Uu traffic towards the gNB).

Observation 13	L2 U2N Relay UE cannot use per hop legacy PC5 RRC to inform L2 U2N Remote UE of detected Uu RLF event.

Observation 14	It is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell than L2 U2N Relay UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to work on two scenarios including:

a.	Scenario 1: Intermediate Relay UE(s) and Last  Relay UE are in the same serving cell.

b.	Scenario 2: Relay UEs can be in different serving cells of the same gNB.

Proposal 2	RAN2 assumes the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. RAN2 sends LS to SA2 indicating this.

Proposal 3	Last Relay UE signals the gNB of the path information (provided by upper layer) for the selected path via SidelinkUEInformationNR..

Proposal 4	Same as in legacy, when L2 U2N Remote UE is in RRC_CONNECTED, L2 U2N Last Relay UE also needs to be in RRC_CONNECTED.

Proposal 5	To down-select Approach 2 between Approach 1 and Approach 2, i.e., L2 U2N Intermediate Relay UEs are not mandated to be in RRC_CONNECTED.

Proposal 6	For Approach 2, it is the network to determine the QoS split (i.e., the portion of the latency) associated with the Uu hop.

Proposal 7	For Approach 2, RAN2 to discuss it is the network (gNB) or the relay UE serving that link, to perform QoS split of each link of the path between the L2 U2N Last Relay UE and the L2 U2N Remote UE.

Proposal 8	Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE signals an indication to its connected L2 U2N Remote UE(s), which may trigger RRC connection re-establishment for L2 U2N Remote UE. FFS on the signaling details.

Proposal 9	Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE signals an indication to L2 U2N Intermediate Relay UE(s). FFS on the signaling details.

Proposal 10	Upon detection of Uu RLF at L2 U2N Last Relay UE, L2 U2N Last Relay UE indicates to upper layer to trigger PC5 unicast link release towards adjacent L2 U2N Intermediate Relay UE.

Proposal 11	The legacy L2 U2N Remote UE actions during the RRC connection re-establishment procedure can be reused for L2 multi-hop U2N relay.

Proposal 12	RAN2 to discuss how to signal system information and/or paging information between L2 U2N Relay UE and L2 U2N Remote UE.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.

[2] R2-2407035, “discussion on discovery and relay (re)selection”, 3GPP TSG-RAN WG2 #127, Maastricht, Netherlands, 19th – 23th August 2024.