3GPP TSG-RAN WG2 Meeting #128	R2-2410173

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:		7.0.2.13

Source:				ASUSTeK

Title:		Discussion on supporting 8Tx in MAC specification

Document for: 	Discussion and Decision

- Introduction

8Tx UL has been introduced by RAN1 in release 18 MIMO. In the last meeting, an agreement was made that for uplink spatial multiplexing, a DCI schedules a single uplink grant for two-TB transmission for a HARQ process:

For uplink grant modelling of HARQ operation for uplink spatial multiplexing, a DCI schedules a single uplink grant for two-TB transmission for a HARQ process. Detailed changes to the MAC spec can be further checked in the next meeting.

In this contribution, we discuss changes needed in the MAC spec to support 8Tx (2-TB) transmission based on the agreement.

- Discussion

Based on the agreement, below shows analysis on changes needed to support 2-TB transmission in one HARQ process:

NDI consideration for different TBs

Due to HARQ process having more than one NDI for multiple TBs, MAC specification needs to take multiple NDIs into consideration when determining whether the NDI(s) are toggled. For example, in the current UL grant reception procedure in section 5.4.1, the UE considers the NDI for a HARQ process to have been toggled if an uplink grant is for MAC entity’s C-RNTI and the previous uplink grant for the same HARQ process is associated with configured uplink grant:

### 5.4.1 UL Grant reception 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

Therefore, when the uplink grant is received and the condition is fulfilled, the UE should consider both NDIs to have been toggled for the HARQ process.

Observation 1: The UE should consider both NDIs of a 2-TB uplink grant to be toggled when previous uplink grant is an configured uplink grant.

Per-TB new transmission determination

In the current specification, the HARQ entity considers whether an uplink grant is a new transmission or retransmission in a per-HARQ process manner, including determining whether an uplink grant is for a new transmission by considering whether the HARQ buffer is empty:

#### 5.4.2.1 HARQ Entity … Each HARQ process supports one or two TBs. … For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or … 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: … 2> else (i.e. retransmission): …

In 8Tx UL transmission, one HARQ process handles transmissions of two TBs, and the corresponding HARQ buffer is used to store two MAC PDUs generated for the two-TB transmission. As a DCI indicates two NDIs for two TBs, it is possible that a DCI can indicate a new transmission for a first TB and a retransmission for a second TB by assigning corresponding NDI values for each TB.

Observation 2: A single HARQ buffer of a HARQ process is used for storing two MAC PDUs for two-TB transmission.

Observation 3: For a 2-TB transmission in one Cell, the DCI can indicate a new transmission and a retransmission for the two TBs respectively.

Therefore, an issue might occur when using the existing procedure in the current spec to determine new transmission or retransmission for an uplink grant, when there’s one MAC PDU waiting for retransmission stored in the HARQ buffer and the NW intends to schedule a two-TB transmission for retransmission of the MAC PDU and a new transmission of a second TB, as shown in Figure 1. The UE does not consider the NDI_2 of the uplink grant to be toggled, as there’s no previous uplink grant for the second TB to compare with, and also the HARQ buffer is not empty for the HARQ process, so the UE does not obtain a MAC PDU for this second TB transmission.

Figure 1

Per-TB HARQ buffer handling

In the current specification, the HARQ buffer of a HARQ process is flushed when a new transmission is indicated but no MAC PDU has been obtained for the uplink grant:

#### 5.4.2.1 HARQ Entity … For each uplink grant, the HARQ entity shall: … 3> if a MAC PDU to transmit has been obtained: … 3> else: 4> flush the HARQ buffer of the identified HARQ process.

This is to avoid unintended (re)transmission of outdated data stored in the HARQ buffer that might be misread by NW as new data. In 8Tx UL, another issue would occur if the entire HARQ buffer is flushed for the HARQ process as it could affect the transmission of the other TB that shares the same HARQ buffer of the HARQ process.

Observation 4: For a 2-TB transmission in a HARQ process, the UE should handle data associated with the two TBs in one HARQ buffer separately.

Ensuring proper 2-TB generation

In LTE, when the UE is configured to transmit up to two TBs at a same timing and receives two UL grants for two TBs, the UE needs to generate both MAC PDUs once one MAC PDU is to be generated. More specifically, according to TS 36.321 as below, when the PHY is configured for uplink spatial multiplexing, the MAC can receive two UL grants for two TBs for the same TTI. And if at least one MAC PDU is to be generated for this TTI, the MAC generates MAC PDUs corresponding to all UL grants for this TTI (even if a condition for UL skipping may be fulfilled).

Observation 5: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE is indicated to transmit 2 TBs for a TTI.

5.4.1 UL Grant reception In order to transmit on the UL-SCH the MAC entity must have a valid uplink grant (except for non-adaptive HARQ retransmissions) which it may receive dynamically on the PDCCH or in a Random Access Response or which may be configured semi-persistently or preallocated by RRC or provided by RRC for transmission using PUR (see clause 5.4.7). To perform requested transmissions, the MAC layer receives HARQ information from lower layers. When the physical layer is configured for uplink spatial multiplexing, the MAC layer can receive up to two grants (one per HARQ process) for the same TTI from lower layers. … ### 5.4.3 Multiplexing and assembly #### 5.4.3.1 Logical channel prioritization The Logical Channel Prioritization procedure is applied when a new transmission is performed. … If the MAC PDU includes only the MAC CE for padding BSR or periodic BSR with zero MAC SDUs and there is no aperiodic CSI requested for this TTI, as specified in TS 36.213 [2], the MAC entity shall not generate a MAC PDU for the HARQ entity in the following cases: - in case the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI; or - in case the MAC entity is configured with skipUplinkTxSPS and the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's Semi-Persistent Scheduling C-RNTI or by the MAC entity's UL Semi-Persistent Scheduling V-RNTI; or - in case the grant indicated to the HARQ entity is a configured uplink grant activated by the MAC entity's AUL C-RNTI; or - in case the grant indicated to the HARQ entity is a preconfigured uplink grant. NOTE 1a: If at least one MAC PDU is to be generated for the HARQ entity for this TTI, the MAC entity generates MAC PDUs corresponding to all UL grants indicated to the HARQ entity for this TTI.

The intention for adding the NOTE in LTE is that PHY is unable to perform corresponding transmission properly when MAC generates only one TB for UL spatial multiplexing, as a two TB transmission is scheduled by DCI and expected by eNB. When UL spatial multiplexing is configured, MAC can receive two dynamic UL grants at a given TTI and PHY would expect to receive two TBs from MAC, as PHY is unable to perform corresponding transmission if only one TB is generated from MAC. Without NOTE 1a, whether to generate MAC PDU is determined separately for each MAC PDU, so that the HARQ entity would generate only one MAC PDU if there is insufficient available data at the TTI. The Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured, resulting in only one MAC PDU being generated.

Observation 6: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

As a result, this NOTE is also needed for 8Tx UL in NR. Even if one UL grant is used to schedule two TBs, whether to generate MAC PDU is still determined separately for each MAC PDU so that the same issue in LTE UL spatial multiplexing occurs. That is, in NR, when UE receives one UL grant for two TBs and there is insufficient available data, the Multiplexing and assembly entity would not generate the MAC PDU comprising pure padding when skipUplinkTxDynamic is configured and thus results in only one MAC PDU generated. Therefore, to address the same issue, it is proposed to add a similar note in NR as shown in CR R2-2410174. The difference between legacy LTE NOTE and the proposed change is due to lack of concept of “TTI” in NR, and we also try to cover the case where a DCI schedules a new transmission and a retransmission.

On the other hand, since this issue may not be RAN2’s area of expertise, if RAN1 input is required for making the final decision for this change, an LS can also be sent for clarification on this issue in NR.

Uplink grant priority determination

In the current specification, the MAC entity can be configured with lch-basedPrioritization, and the priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed or have data available that can be multiplexed in the MAC PDU. With the introduction of 2-TB transmission, the uplink grant should consider priorities of LCHs that can be multiplexed in either of the TBs.

Observation 7: the UE should consider logical channel data for both MAC PDUs when determining priority of an uplink grant scheduling 2-TB.

Based on the above analysis, a CR has been prepared in R2-2410174 and corresponding changes can be adopted to support UL spatial multiplexing  in MAC specification.

Proposal 1: Adopt changes in R2-2410174 to support UL spatial multiplexing in MAC specification.

- Conclusion

In this contribution, we have following observations and proposal:

Observation 1: The UE should consider both NDIs of a 2-TB uplink grant to be toggled when previous uplink grant is an configured uplink grant.

Observation 2: A single HARQ buffer of a HARQ process is used for storing two MAC PDUs for two-TB transmission.

Observation 3: For a 2-TB transmission in one Cell, the DCI can indicate a new transmission and a retransmission for the two TBs respectively.

Observation 4: For a 2-TB transmission in a HARQ process, the UE should handle data associated with the two TBs in one HARQ buffer separately.

Observation 5: In LTE, the UE generates both MAC PDUs if one MAC PDU is to be generated, when the UE is indicated to transmit 2 TBs for a TTI.

Observation 6: The intention for the NOTE 1a in LTE is that PHY is unable to perform corresponding transmission properly with only one TB when two TBs are expected for UL spatial multiplexing.

Observation 7: the UE should consider logical channel data for both MAC PDUs when determining priority of an uplink grant scheduling 2-TB.

Proposal 1: Adopt changes in R2-2410174 to support UL spatial multiplexing in MAC specification.

- Reference