3GPP TSG-RAN WG2 Meeting #128	R2-2410181

Orlando, USA, Nov 18th – 22nd, 2024	Revision of R2-2408360

Agenda Item:	8.9.2

Source:			ASUSTeK

Title:	Discussion on assistance information for S&F

Document for: 	Discussion and Decision

# Introduction

In previous RAN2 meetings, we have made the following progress on information for Store & Forward operation. In RAN2 #125bis meeting:

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

In RAN2 #126 meeting:

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

In RAN2 #127 meeting:

Agreements:

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

In RAN2 #127bis meeting:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this contribution, we would like to further discuss the assistance information provided to the UE for S&F operation.

# Discussion

Store and Forward (S&F) operation in IoT NTN is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. As specified in TR 22.865 [1], S&F could be considered as another operation mode (as shown in Figure 1 below) in addition to the “normal/default satellite operation” mode (as shown in Figure 2 below). In the agreements of the last meeting, the two operation modes was called as “S&F mode” and “real-time mode”.

Figure 1: “S&F” mode [1]

Figure 2:“Real-time/normal/default” mode [1]

Since the UE behaviour when the UE is in S&F operation mode should be different from UE behaviour when the UE is in real-time mode, the UE needs to know whether the current serving network is operating in which mode. In the previous meetings, it was agreed that a S&F operation indication would be provided by SIB to inform the UE(s) whether S&F mode is applied on the serving satellite.

The IoT NTN network may switch between these two different operation modes. For example, when feeder link is available for a satellite, the IoT NTN network could operate in real-time mode. And when feeder link becomes unavailable, the IoT NTN network could switch to S&F mode. Since when to switch operation mode may depend on satellite movement or network decision, the switching should be predictable. It is beneficial to indicate the time information about when to switch operation mode between S&F operation mode and real-time mode to the UE. In RAN2 #127 meeting, it has been discussed in [2] whether the UE is informed of the remaining time of the satellite’s current operation mode (real-time / S&F). And in RAN2 #127bis meeting, it also has been discussed in [3][4] that some time information (e.g., starting time and duration) of S&F mode could be provided to the UE.

With the time information, the UE could know when the S&F operation would be started or stopped. For example, based on the time information, the UE could determine whether to perform some non-delay-tolerant transmission, which is not suitable in S&F mode. The UE could also use the time information to determine whether to establish a connection in the current cell or select a cell.

Proposal 1: In addition to S&F operation indication, the network could provide the UE time information about when to switch to/from S&F mode.

# Conclusion

We have the following proposal for S&F mode indication:

Proposal 1: In addition to S&F operation indication, the network could provide the UE time information about when to switch to/from S&F mode.

# Reference

- 3GPP TR 22.865 v19.2.0, “Study on satellite access Phase 3.”

- R2-2407237, “Support for store and forward in IoT NTN”, Ericsson.

- R2-2408017, “RAN2 Aspect for S&F Operation”, vivo.

- R2-2408591, “Support of S&F operation in IoT NTN”, Apple.