3GPP TSG-RAN WG2 Meeting #128	R2-2410184

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.13.3

Source:	ASUSTeK

Title:	Remaining issues on SRB0 message forwarding in multi-hop U2N Relay

Document for:	Discussion and Decision

# 1	Introduction

RAN2#127 made the following agreements on hop-by-hop for SRAP:

Agreements:

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

[POST127][402][Relay] Multi-Hop Relay Control Plane [1] discusses the following two approaches for multi-path U2N Relay:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

This document further discusses the following issues on the baseline connection establishment procedure in [1] for multi-hop U2N Relay with Approach 1:

    - Whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.

    - Whether the Last Relay UE can send SUI message on behalf of all other relay UEs.

    - Whether the SRAP header is present over each PC5 hop between two relay UEs for L2 U2N Remote UE's message on SRB0.

# 2	Discussion

2.1	Current conclusion on the baseline connection establishment procedure with Approach 1

The current conclusion on the baseline connection establishment procedure with Approach 1 in [1] is quoted below:

Proposal 1 – In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. - The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. [FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.] - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. The first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the First Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Intermediate Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from U2N Remote UE on the specified PC5 Relay RLC channel. The Intermediate Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Intermediate Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Last Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from the First Relay UE on the specified PC5 Relay RLC channel. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate Relay UE on the specified PC5 Relay RLC channel. In each of the previous sub-steps, if a given relay UE and its parent relay UE both need to enter RRC_CONNECTED, the given relay UE cannot do so until the parent relay UE has completed its own RRC connection establishment. The Last Relay UE receives SRB0 relaying Uu Relay RLC channel configuration for the Intermediate Relay UE from gNB. The Intermediate Relay UE receives SRB0 relaying Relay RLC channel configuration for the First Relay UE from gNB. The gNB configures SRB0 (for U2N Remote UE) relaying RLC channel to the first Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified PC5 Relay RLC channels over each of the PC5 links. [FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.] …

2.2	Whether connection establishment of an Intermediate Relay UE is captured in specification as connection establishment of a remote UE or a relay UE

In TS 23.304 [2], the Layer-2 link establishment procedure specified in clause 6.4.3.1 for the unicast mode of ProSe Direct communication over PC5 reference point is reused for the 5G ProSe UE-to-Network Relay. In clause 6.4.3.1 of TS 23.304 [2], the Layer-2 link is established between two peer UEs (UE1 and UE2). The Layer-2 link establishment procedure for the 5G ProSe UE-to-Network Relay is then specified as below:

#### 6.4.3.6 Layer-2 link management over PC5 reference point for 5G ProSe UE-to-Network Relay The Layer-2 link procedures over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 can be used for the PC5 reference point between 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay, with the following differences and clarifications: - The Layer-2 link modification procedure is applicable to ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay, other procedures are applicable to both ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay and ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay. - The UE oriented Layer-2 link establishment is used with UE-1 representing the 5G ProSe Remote UE and UE-2 representing the 5G ProSe UE-to-Network Relay. For other procedures either UE-1 represents the 5G ProSe Remote UE and UE-2 represents the 5G ProSe UE-to-Network Relay, or UE-1 represents the 5G ProSe UE-to-Network Relay and UE-2 represents the 5G ProSe Remote UE. I.e. the Layer-2 link establishment is initiated by the 5G ProSe Remote UE, while other procedures may be initiated either by the 5G ProSe Remote UE or by the 5G ProSe UE-to-Network Relay. …

Observation 1: In TS 23.304, the Layer-2 link establishment procedure specified for the unicast mode of ProSe Direct communication over PC5 reference point is reused for the 5G ProSe UE-to-Network Relay, with one of the two peer UEs representing the U2N Relay UE.

We think the above way of modelling can reduce specification efforts and also make future maintenance easier. Thus, we suggest to mimic this way of modelling for specifying the connection establishment of an Intermediate Relay UE i.e. reuse the connection establishment of a L2 U2N Remote UE, with the L2 U2N Remote UE representing the Intermediate Relay UE.

Proposal 1:  In Approach 1, specify the connection establishment of an Intermediate Relay UE by reusing the connection establishment of a L2 U2N Remote UE, with the L2 U2N Remote UE representing the Intermediate Relay UE.

2.3	Whether the Last Relay UE can send SUI message on behalf of all other relay UEs

As highlighted in Step 2 of the baseline connection establishment procedure [1], it is FFS whether the Last Relay UE can send SUI message on behalf of all other relay UEs.

According to Proposal 1 in the conclusion, connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. Given this conclusion, each relay UE in the relay path can send its own SUI message to the gNB via its parent relay UE(s) just like Remote UE RRC message forwarding by the U2N Relay UE in single-hop U2N Relay. In general, it is preferable to reuse the legacy mechanism.

Thus, we propose:

Proposal 2:  In Approach 1, each Relay UE in the relay path sends its own SUI message (if necessary) to the gNB for supporting multi-hop L2 U2N Relay, just like Remote UE RRC message forwarding by the U2N Relay UE in single-hop U2N Relay.

2.4	Whether the SRAP header is present over each PC5 hop between two relay UEs for L2 U2N Remote UE's message on SRB0

(header)

SRAP PDU

(no header)

SRAP PDU

Remote UE

gNB

U2N Relay

Figure 1. SRAP header for SRB0 message in single-hop L2 U2N Relay

As shown in Figure 1, for L2 U2N Remote UE's message on SRB0 in single-hop U2N Relay (according to [3]), the SRAP header is not present over PC5 hop, while the SRAP header is present over Uu hop for both DL and UL. It is not clear whether the SRAP header is present over the PC5 hop between relay UEs in the relay path of multi-hop L2 U2N Relay.

Observation 2: It is not clear whether the SRAP header is present over each PC5 hop between two relay UEs in the relay path of multi-hop L2 U2N Relay.

Basically, the L2 U2N Relay UE in single-hop U2N Relay may receive the local ID of the Remote UE from the gNB in the RRCReconfiguration message which carries the SRB0 relaying Uu Relay RLC channel configuration so that the L2 U2N Relay UE can include the SRAP header (i.e. UE ID of the Remote UE) in the SRAP PDU with SRB0 message sent over the Uu hop.

According to the baseline connection establishment procedure [1], each relay UE receives SRB0 relaying Uu/PC5 Relay RLC channel configuration from the gNB. Similarly, the RRCReconfiguration message carrying the SRB0 relaying PC5/Uu Relay RLC channel configuration may also include the local ID of the Remote UE. In this situation, the First Relay UE and the Intermediate Relay UE can include the SRAP header (i.e. UE ID of the Remote UE) in the SRAP PDU with SRB0 message sent over the PC5 hop so as to enable the Last Relay UE to include the SRAP header in the SRAP PDU with SRB0 message sent over the Uu hop as shown in Figure 2.

Proposal 3:  In Approach 1, the gNB provides the local ID of the Remote UE and the SRB0 relaying PC5/Uu Relay RLC channel configuration to each Relay UE in RRCReconfiguration message.

Proposal 4:  In Approach 1, for L2 U2N Remote UE's message on SRB0, the SRAP header is present over each PC5 hop between Relay UEs in the relay path of multi-hop L2 U2N Relay.

(no header)

SRAP PDU

(header)

SRAP PDU

(header)

SRAP PDU

(header)

SRAP PDU

Remote UE

gNB

First Relay

Intermediate Relay

Last Relay

Figure 2.   SRAP header for SRB0 message in multi-hop L2 U2N Relay

# 3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1: In TS 23.304, the Layer-2 link establishment procedure specified for the unicast mode of ProSe Direct communication over PC5 reference point is reused for the 5G ProSe UE-to-Network Relay, with one of the two peer UEs representing the U2N Relay UE.

Observation 2: It is not clear whether the SRAP header is present over each PC5 hop between two relay UEs in the relay path of multi-hop L2 U2N Relay.

Proposal 1:  In Approach 1, specify the connection establishment of an Intermediate Relay UE by reusing the connection establishment of a L2 U2N Remote UE, with the L2 U2N Remote UE representing the Intermediate Relay UE.

Proposal 2:  In Approach 1, each Relay UE in the relay path sends its own SUI message (if necessary) to the gNB for supporting multi-hop L2 U2N Relay, just like Remote UE RRC message forwarding by the U2N Relay UE in single-hop U2N Relay.

Proposal 3:  In Approach 1, the gNB provides the local ID of the Remote UE and the SRB0 relaying PC5/Uu Relay RLC channel configuration to each Relay UE in RRCReconfiguration message.

Proposal 4:  In Approach 1, for L2 U2N Remote UE's message on SRB0, the SRAP header is present over each PC5 hop between Relay UEs in the relay path of multi-hop L2 U2N Relay.

# References

[1] R2-240xxxx, Report of [POST127][402][Relay] Multi-Hop Relay Control Plane

[2] TS 23.304-j10, Proximity based Services (ProSe) in the 5G System (5GS) (Release 19)

[3] TS 38.300-i30, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)