3GPP TSG-RAN WG2 #128 meeting	R2-2410189

# Orlando, the USA, 18th Nov – 22nd Nov, 2024

Source:		NEC

Agenda item:		8.7.4.2	DSR enhancements

Title:		Considerations on DSR enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#127bis meeting, agreements [1] on DSR enhancements for XR were made as below:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, we will further discuss the remaining issues related to the enhanced DSR.

# 2. Discussion

In the RAN2 #127bis meeting, Companies were encouraged to provide their analysis on 1) the impact of setting the triggering threshold to value lower than the largest reporting threshold on DSR procedure; 2) whether legacy and enhanced DSR can be configured/used at the same time.

For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

We believe that these two issues are correlated. If there is at least one LCG configured with multiple reporting thresholds, there are 2 options to consider regarding whether legacy DSR should be reported:

- Option 1: Only enhanced DSR is reported.

- Option 2: Legacy and enhanced DSR are configured/used at the same time.

First of all, we would like to emphasize that any chosen option should ensure all the delay information of delay-critical data is reported.

Proposal 1: Any enhancements on DSR should ensure all the delay information of delay-critical data is reported.

Figure 1: Issue of Option 1 (i.e., only enhanced DSR is reported)

If Option 1 is adopted, the implementation may be straightforward. However, with the introduction of new triggering threshold and reporting thresholds, we must address the following issues as illustrated in Figure 1:

- Issue 1-1: If the triggering threshold (e.g., T1) of the enhanced DSR is lower than the largest configured reporting threshold (e.g., T2) or legacy remainingTimeThreshold (e.g., T3), no DSR shall be triggered when there is only delay-crititcal data with remaining time above T1. This will lead to situations where there is delay-critical data, but no DSR is triggered.

- Issue 1-2: If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the legacy remainingTimeThreshold (e.g., T3), the enhanced DSR may fail to cover the delay information of all delay-critical data, for example, the delay information with the remaining time range [T2, T3) will be missed.

Proposal 2: If only enhanced DSR is reported, the following issues illustrated in Figure 1 should be considered:

- Issue 1-1: If the triggering threshold (e.g., T1) of the enhanced DSR is lower than the largest configured reporting threshold (e.g., T2) or legacy remainingTimeThreshold (e.g., T3), no DSR shall be triggered when there is only delay-crititcal data with remaining time above T1.

- Issue 1-2: If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the legacy remainingTimeThreshold (e.g., T3), the delay information with the remaining time range [T2, T3) will be missed.

Figure 2: Issue of Option 2 (i.e., legacy and enhanced DSR coexist)

If Option 2 is adopted, the following issues should be considered:

- Issue 2-1: Whether to cancel the legacy DSR upon triggering the enhanced DSR.

  - If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the remainingTimeThreshold (e.g., T3) of legacy DSR, there will be unreported residual delay-critical data with remaining time range [T2, T3) after reporting of enhanced DSR.

      - If legacy DSR is cancelled upon triggering the enhanced DSR, then the delay information of the unreported residual delay-critical data will be missed.

      - If legacy DSR is not cancelled upon triggering the enhanced DSR, and there is unreported residual delay-critical data after reporting of enhanced DSR, then legacy DSR shall also be reported. This, however, would lead to reporting redundant delay information with the remaining time range [0, T2).

- Issue 2-2: Whether at most one DSR MAC CE or both types can be contained in a MAC PDU.

  - Given that legacy DSR and enhanced DSR are mainly different in granularity, we prefer that a MAC PDU contains at most one DSR MAC CE, and enhanced DSR with finer granularity should be prioritized over legacy DSR.

Proposal 3: If legacy and enhanced DSR coexist, the following issues illustrated in Figure 2 should be considered:

- If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the remainingTimeThreshold (e.g., T3) of legacy DSR, and

  - If legacy DSR is cancelled upon triggering the enhanced DSR, then the delay information of the unreported residual delay-critical data with the remaining time range [T2, T3) will be missed.

  - If legacy DSR is not cancelled upon triggering the enhanced DSR, the reported legacy DSR may lead to reporting redundant delay information with the remaining time range [0, T2).

Proposal 4: At most one DSR MAC CE can be contained in a MAC PDU. If legacy and enhanced DSR coexist, enhanced DSR with finer granularity shall be prioritized over legacy DSR.

Based on the analysis, if the largest configured reporting threshold is lower than the legacy remainingTimeThreshold, there will be unreported residual delay-critical data or redundant delay information regardless of the option chosen. Therefore, we propose that the largest configured reporting threshold of enhanced DSR should be equal to the legacy remainingTimeThreshold. If it’s agreed, we can further discuss the impact on legacy DSR.

Proposal 5: To avoid unreported residual delay-critical data or reporting redundant delay information, the largest configured reporting threshold of enhanced DSR should be equal to the legacy remainingTimeThreshold.

The legacy DSR in Rel-18 does not inform the network about the status of non-delay-critical data, as a result, the network cannot schedule it accordingly. For the resources scheduled based on the legacy DSR, if non-delay-critical data of LCH1 has a higher priority than delay-critical data of LCH2, non-delay-critical data of LCH1 shall be multiplexed first and the remaining resources may not be sufficient for the transmission of delay-critical data. Although BSR reports the total buffer size, the timing gap between DSR and BSR reports may lead to outdated and inaccurate information when scheduling delay-critical data.

Moreover, it was agreed in the RAN2 #127 meeting that the network shall configure multiple remaining time thresholds for reporting multiple pairs of remaining time and buffer sizes per LCG. It is evident that packets with remaining time above the maximum configured threshold can also be reported, which may encompass both non-delay-critical and low-importance data. Based on it, the network can obtain comprehensive insights into the entire buffer and consider both delay-critical and non-delay-critical data when allocating resources.

Proposal 6: The buffer size of non-delay-critical data (e.g., packets with remaining time above the maximum configured threshold) is reported in the enhanced DSR.

# Summary

This contribution provides our analysis on the enhancement of DSR, and has the following proposals:

Proposal 1: Any enhancements on DSR should ensure all the delay information of delay-critical data is reported.

Proposal 2: If only enhanced DSR is reported, the following issues illustrated in Figure 1 should be considered:

- Issue 1-1: If the triggering threshold (e.g., T1) of the enhanced DSR is lower than the largest configured reporting threshold (e.g., T2) or legacy remainingTimeThreshold (e.g., T3), no DSR shall be triggered when there is only delay-crititcal data with remaining time above T1.

- Issue 1-2: If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the legacy remainingTimeThreshold (e.g., T3), the delay information with the remaining time range [T2, T3) will be missed.

Proposal 3: If legacy and enhanced DSR coexist, the following issues illustrated in Figure 2 should be considered:

- If the largest configured reporting threshold (e.g., T2) of the enhanced DSR is lower than the remainingTimeThreshold (e.g., T3) of legacy DSR, and

  - If legacy DSR is cancelled upon triggering the enhanced DSR, then the delay information of the unreported residual delay-critical data with the remaining time range [T2, T3) will be missed.

  - If legacy DSR is not cancelled upon triggering the enhanced DSR, the reported legacy DSR may lead to reporting redundant delay information with the remaining time range [0, T2).

Proposal 4: At most one DSR MAC CE can be contained in a MAC PDU. If legacy and enhanced DSR coexist, enhanced DSR with finer granularity shall be prioritized over legacy DSR.

Proposal 5: To avoid unreported residual delay-critical data or reporting redundant delay information, the largest configured reporting threshold of enhanced DSR should be equal to the legacy remainingTimeThreshold.

Proposal 6: The buffer size of non-delay-critical data (e.g., packets with remaining time above the maximum configured threshold) is reported in the enhanced DSR.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0