3GPP TSG-RAN WG2 #128	R2-2410193

Orlando, US, November 18 – 22, 2024

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Functionality aspects for A-IoT

Document for:	Discussion/Decision

# 1	Introduction

This paper discusses aspects on the functionality required for A-IoT devices, including security related questions to SA3, the need for resource allocation, segmentation/reassembly (pending RAN1 progress), QoS handling, higher layer repetition, BSR and SR. Note that in parts were referring to “Reader”, this would apply to either a gNB or an Intermediate node as Reader, described in SID and elsewhere.

# 2	Discussion

## 2.1 	Registration and Device Disabling

It is beneficial to have registration for A-IoT devices, if they are capable. Keeping device context helps avoid the need of setting up security, authentication, device ID allocation every time device accesses the network. Thus, we assume that A-IoT devices need to register to network to get authorized to receive services associated with considered use cases and there is device context with a minimal set of key information established and stored at network and device side.

It is preferable to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

Capture in TR 38.769 that RAN2 assumes the existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on identification, authorization, and authentication.

The discussion in RAN2 has mainly been focusing on the two use cases of inventory and command. However, RAN2 impact from A-IoT involves other procedures including disabling devices and registration of devices, both of which are part of the key issue 2 of SA2 study (as in TR23.700-13). As for the registration procedure, from a RAN2’s perspective, similar to legacy 3GPP systems, it can be assumed that there is a need for signaling procedures over air interface to support device identification reporting from A-IoT device, provisioning of temporary CN allocated device ID, possible security information/setup including authentication. We think the three-step baseline procedure being defined in current TR38.760-13, clause 6.3 supports also procedure steps for a registration procedure. For example, the authentication steps can be integrated in the same Random Access procedure, i.e., steps A, B, C in the overall AS procedures The similar applies to the device disabling.

Capture in TR38.76 how the overall AS procedure (three-step baseline) can support procedural steps for registration and device disabling.

RAN2 can assume registration can be performed in connection to inventory or command procedures to reduce signaling overhead. For example, Figure 1 below shows an example of registration procedure which is performed in connection to the initial inventory procedure for reduced signaling.

RAN2 assumes registration can be performed in connection with inventory procedure.

Figure 1: Message flow for registration procedure (in connection with inventory procedure)

Steps 1-6: Similar to inventory only procedure. However, there may need an indication in A-IoT paging to indicate to target devices to perform registration in connection with the inventory, if they are capable. We think this is applicable to other procedures/use cases as well.

As an example, device can report in step 5 its registration capability, which helps CN to determine if the registration in next steps 7-10 can be performed for the device. However, CN can also check device capability from e.g., UDM based on device ID provided in step 5. Hence, the registration procedure is not mandatory for all A-IoT devices, i.e., steps 7-10 are in dashed lines and textboxes.

Step 7: In case the device is registration capable, the CN perform authentication, temporary device ID allocation and security determination.

Step 8: the gNB/UE reader and device perform relevant procedures over air interface to exchange upper layers (e.g., A-IoT NAS) information related to authentication, security setup, and temporary device ID provisioning.

Step 9: the CN registers device information (e.g., into UDM).

Step 10: the device, the gNB/intermediate UE, CN store device context information for subsequent communications. As described in command only procedure, the temporary device ID can be used to start a subsequent command procedure and security can be used to protect command data without establishing/setup again.

Liaison with SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P1, P2, P3 in the TR.

## 2.2	Segmentation

In previous meetings it was agreed that segmentation will be studied in MAC layer assuming it is required (pending further RAN1 indications) and that a solution should be in common between R2D and D2R although the need for segmentation R2D will be discussed later. Segmentation will not use additional buffer to store the separate segments, but just a register to store a pointer to the next byte that should be sent, and that a few-bit indication will be used to control the transmission of segments.

The overall mechanism is “Stop & Wait”, meaning that the transmitter waits for the receiver before continuing with the next segment or possibly retransmit the previous segment.

It remains to be discussed the detail of the feedback message and what information it delivers. How many bits and which information exactly is delivered depends on the desired level of functionality that segmentation will have and to what extent the system is able to recover from error cases, for instance if the message or the feedback signal is not received correctly.

In the agreements it is captured that “An indication is used to indicate to reader whether the data is segmented and whether it is last segment”. Nevertheless, if the data is not segmented, this is equivalent to the first segment being also the last segment. On the other hand, if the device indicates that this is not the last segment, it is obvious for the reader that data is segmented. Therefore, only 1 bit is necessary to satisfy the agreement.

The device sends a 1-bit indication indicating whether the current transmission is the last segment or not.

Figure 2 a) and b) show the normal functioning of the segmentation procedure under these assumptions, and how the device keeps track of the offset in the buffer (pointer or “ptr”). It is assumed that the trigger from the reader contains a New Data Indicator (NDI) that is set to 1 to request the next segment or 0 to request the retransmission of the previous one.

It is important to notice that if we assume that each segment is sent upon a trigger from the reader (e.g.: UL Grant + ACK/NACK) two failures can happen: either the device receives the UL Grant but the reader does not receive the segment, or the device does not receive the UL Grant. In both cases, the reader will not detect any transmission in the UL resources reserved for the segment transmission, and thus in both cases the reader should behave in the same manner, for instance by sending a NACK. The device updates the buffer pointer “ptr” when the previous transmission has been acknowledged (NDI=1)

The reader does not have a way to distinguish if a failure was caused by missing reception of the UL Grant or missing reception of the data segment.

The problem with this is represented in Figure 2 where in c) the device correctly receives the trigger message (and updates its buffer pointer) but the transmission of the segment fails. After NDI=0 has been sent the device just retransmits the previous segment and everything works as intended. On the other hand, if the trigger transmission fails (case d)) the device does not update the buffer pointer, so when the next NDI=0 is transmitted, the device would retransmit segment i, but the reader actually received segment i and is now expecting a retransmission of segment i+1. Therefore, in the reader buffer segment I will be appended twice.

| Device Pseudo-code: On Upper Layer Trigger received: - Build Reply - ptr := 0 TBS_prev := 0 - Process UL Grant (below) On UL Grant (TBS=x) received: - If NDI==1: - ptr := ptr + TBS_prev - if ptr == end_of_buffer: - Completed - else: - TBS_prev := x - Transmit bit from ptr to ptr + x-1. a) | b) |
| --- | --- |
| c) | d) |

Figure 2: Simple segmentation procedcure and error cases: a) pseudo code at device side, b) normal functioning, c) failed data transmission error case d) failed trigger transmission error case

There are two possible solutions to this problem. The first is to assume that UL Grant is sent with a more robust MCS so that case d) is not actually possible (or if the SINR is not sufficient to receive the UL Grant correctly, the device is considered out of coverage).

Otherwise, there could be a mechanism to check the validity of the received data once the last segment has been received. For instance, a CRC can be included in the overall transmission.

Capture in the TR the following options for handling segmentation error cases:

        - A mechanism to check the validity of the overall transmission, after the last segment is received, is supported

        - The trigger command may be sent with a more robust MCS than the data and/or a device is forbidden to operate if the signal quality is not sufficient

Finally, it is worth noticing that passive devices that will likely have enough energy for the whole duration of the session will not have issues in losing the stored data due to lack of energy.

On the other hand, active devices may in fact lose energy during the transaction. In this case, assuming NVM is used for this feature, all information regarding the ongoing segmentation procedure will be lost. The reader will likely detect a failure and may decide to try again later. If the device is never able to transmit all the segments it may be considered simply out of coverage because unable to operate. Note that this will not be different from the case of a single segment (of the same size).

A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

Another open issue is whether the protocol stack should have an additional layer on top of MAC or no. The previous agreements suggest that segmentation should be part of MAC layer, but this has never been assessed properly.

In general, segmentation (and possibly other features) and MAC functions will require control elements (i.e.: the NDI and “more segments” bits as illustrated above). If the number of needed MAC and segmentation control elements is m and s respectively, having them in separate layers will require an overhead of at least log2m+1+log2s bits to encode them (+1 to handle a dedicated logical channel directed to the upper layer), while if they are all handled by the same layer, they would require an overhead of log2m+s. In general, handling control elements in a single layer will require a smaller number of bits for header/encoding.

Merging segmentation in MAC implies a smaller overhead in terms of encoding of control elements.

In terms of complexity, having many functions in the MAC will increase its complexity. Since this would cause a considerable re-design of MAC layer on network side as functions that typically are handled by other layers (such as segmentation in RLC) would be placed into MAC, whereas separating functional responsibility could simplify the design and specification of layers.

Merging segmentation in MAC implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Capture in the TR pros and cons of having segmentation (and possibly other features) in MAC vs. in a new AS layer on top of MAC (See TP in Annex A)

## 2.3	Repetition

RAN1 has made the below agreement regarding repetition in previous meetings:

Agreement

Study D2R transmission in the physical layer using repetition

- Note: Discussions regarding higher-layer repetitions are up to RAN2.

Agreement

Define repetition types for study purposes as follows:

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times

  - NOTE: Equivalent to extending the duration of each chip by Rchip times

Agreement

- For D2R, study at least block-level and bit-level repetition type 1 and type 2.

First, it is worth clarifying that repetitions in the context of RAN2 refers to the AS, excluding PHY layer, to autonomously decide to retransmit or repeat a MAC PDUs when necessary.

In other words, it means that the PHY layer may implement some forms of repetitions (as per agreements) which means that a certain success probability is guaranteed from the PHY layer. At the same time the layers above AS (e.g.: the application itself) may decide to improve reliability in other ways but this is out of 3GPP scope. Likewise other mechanisms managed by CN are out of RAN2 scope.

In addition, retransmissions or reattempts can be interpreted as a form of a repetition mechanism. This is an important aspect to note, especially given RAN2 has agreed to not support HARQ and ACK for A-IoT and thus an alternative mechanism may be required.

Depending on RAN1 decisions, a certain success probability is guaranteed in the worst-case coverage scenario (denoted afterwards as “PHY success probability”)

Repetitions to further improve reliability managed by application or CN are out of RAN2 scope.

Both blind repetitions and reattempts are considered as mechanisms to improve reliability

As a first step to discuss the need for repetitions we should first agree if a success probability target should be guaranteed by AS or not. Obviously, if it is not defined it can be argued that no repetition mechanism is needed on top of RAN1 repetition schemes. Otherwise, we can do a more proper analysis. This target may be more qualitative, but for the sake of discussion let’s assume it can be defined numerically (denoted afterwards as “AS success probability”.

The rationale to decide whether repetitions are needed or not should be that a device in the worst-case scenario (i.e.: cell edge and with the largest TBS) the success probability of a transmission should be equal or higher the “AS success probability”.

If the “PHY success probability” is greater or equal the “AS success probability” then further AS mechanisms to improve reliability are not needed. Otherwise, they are.

In case PHY layer does not implement repetitions, or the “PHY success probability” is lower than the “AS success probability” target, RAN2 should study a repetition mechanism to achieve the target.

Repetitions or Reattempts are needed in RAN2 scope if the AS success probability target is lower than the guaranteed PHY success probability in worst-case scenario.

It may be convenient to start studying these mechanisms while waiting for more detailed RAN1 evaluations and agreements on the matter.

RAN2 has yet to study repetitions mechanism in the hypothesis that PHY layer repetitions are not enough to satisfy a success probability target at AS level.

Since data delivery will likely be part of RACH procedure, a simple RAN2 mechanism to implement repetitions may be performing multiple RACH attempts, similarly to what happens in legacy. Further mechanisms do not seem necessary now.

RAN2 to consider multiple RACH attempts as one mechanism to implement repetitions

## 2.4	Device Addressing and AS identifier handling

The UE should be able to determine if a message received on the shared DL channel is directed to itself or not.

In this section, we call “address” the information needed to distinguish transmission from/to different devices from MAC perspective. (i.e.: it is out of scope higher-layer perspective such as NAS, Application, or inventory as a whole).

There are two main problems to solve regarding the device addressing:

- Which information is used as address (including considerations on scope of the address and collision probabilities)

- How the address is used

### 2.4.1	Which information is used as address

In general, the address should be a piece of information that is agreed between the reader and device (provisioning) and for which its validity is clear for both nodes. In the last meeting it was also agreed that this address should be “short” although it is still to be studied if it should be based on part of the device ID or not.

In assessing each possible option, we have to consider the following aspects:

- Security and Privacy: can the information used as address be shared with the involved nodes? Should it be used in its encrypted form or can be used in clear text?

- Uniqueness: is the address unique within a certain area or time scope? Could there be unacceptable collisions with other devices?

- Provisioning: how is the address provisioned (i.e.: agreed between device and network)

- Usage of memory: should the address be kept in volatile registers or written in NVM? How often? This affects energy consumption and complexity.

RAN2 to determine which information should be used as address based on security, privacy, uniqueness, provisioning, and usage of NVM.
FFS other aspects.

We can start by assessing the uniqueness of the address, meaning that within a certain scope in time and space there should not be excessive collisions between users.

There are 3 main strategies on how the address may be generated:

- Randomly generated by the device: in this case there is a collision probability which depends on how many devices are sharing the same address scope and how many bits compose the address.

- Generated from the APP ID or another CN ID: it could be generated from the b bits that have a higher chance of being unique. This case is equivalent to a randomly generated address although this property depends on how many bits are taken and how the original bits were generated. If this is not possible a hashing operation can still be applied to compute a pseudo-random, but deterministic, sequence of bits that can be used as the address which would have random properties.

- Allocated by the network AS: in this case it is ensured that the address is unique, but it requires additional signalling overhead for the address provisioning. In this case the maximum number of devices that can share the address scope is exactly 2b.

It is worth noticing that if the address is randomly generated, there is a probability of collision. Regardless of how negligible it may be, the system should still be able to recover from this.

A classical example is the contention resolution ID in legacy. There is a non-zero probability that two devices select the same RACH resource and random ID in Msg3 and they would both win the contention without reader/network knowing this. This case is typically considered negligible, but if this happens the network would shortly after verify the 5G-TMSI sent in Msg3 and Msg5 understanding that something went wrong and terminating the procedure. A similar recovery should be considered for A-IoT.

A network-assigned address guarantees uniqueness but requires signalling overhead for its provisioning.

A randomly generated address, or address calculated as function of the Device ID or an already assigned CN ID, does not require provisioning overhead but has a collision probability. Regardless of how negligible this probability is, the system should be able to recover from this.

If within a certain scope (time or space) there are N devices with a randomly generated address composed by b bit, the probability that a collision happens can be estimated following the “birthday problem” approach: the probability of a collision is the probability that in all of the possible pairs of devices they have a different address.

This probability is:

pc=1-1-12bNN-12

If we define a maximum collision probability (i.e.: a definition for “negligible”) we can determine the minimum number of bits given the number of competing devices:

1-1-12bNN-12<pmax→b>-log21-1-pmax2N(N-1)

Figure 3: (left) Collision probability as function of address length and number of competing users. (right) Minimum number of address bits as function of number of competing users and maximum collision probability requirement or if address is network allocated

It is possible to see that the sufficient number of bits so that this probability is negligible depends on how many devices are competing in the same scope (space or time). It’s clear though that a network-assigned ID would require between 33% and 50% of the bits required by a randomly selected address.

The number of bits for which the collision probability is negligible depends on the number of devices competing in the same address scope.

A network-assigned address would require from 33% to 50% of the bits of a randomly generated address with the same number of competing devices

Assuming that a network-allocated address would be provisioned in Msg2 (similar to TC-RNTI in legacy) the additional overhead of the provisioning would be repeated once for each Msg2. At the same time, if the address is used later for resource allocation, the impact of having a longer randomly generated address would also be repeated once for every UL/DL grant. So it’s not obvious which option has the smallest overall overhead. This can be studied in the WI phase.

It’s not obvious which, between network-allocated and randomly generated address, has the smallest overall signalling overhead

Capture in the TR the following options for how the AS address is generated

        - Randomly generated by the device (e.g.: same as contention resolution ID promoted to address after contention resolution)

        - Allocated by the network in Msg2

        - Generated based on Device ID in such a way that it’s quasi-random

Figure 3 represents our best understanding of the various phases of the whole inventory procedure and presents some definitions used in the remainder of this document. Specifically, the inventory procedure is composed by a sequence of “Access rounds”, each with a smaller number of devices as they get inventoried in previous access rounds and with a shorter duration. Each access round is composed by a number of “Access Occasions”, each containing a number of random resources that can be multiplexed in time or frequency. In each access occasion the whole CBRA takes place from Msg0 to Msg3 or Msg4 transmission. At the beginning of the Access round all the remaining devices choose a random resource in which to transmit Msg1 and will wait for the Access Occasion containing that resource. Note that possible registration etc, is not taken into account here.

Figure 4: Representation and definitions of the various phases of the inventory procedure as a whole

Depending on the expected scope of the address we can estimate the number of competing devices in the worst-case scenario and consequently the number of bit necessary to minimize the collision probability.

In particular, it can be shown that the number of random resources to be configured to maximise the throughput is approximately equal to the number of competing devices, an assumption that will be used hereby.

- Access Occasion: in this case the address is valid only within the access occasion (i.e.: from Trigger until the last message (e.g.: Msg4)). In an access occasion we can expect a limited number of competing devices (< 10) which means that 4 bits are necessary for addressing if the address is network allocated, or 12 bits if the address is randomly generated and the collision probability should be < 1%. In this case, the main drawback is that the next access occasion cannot start until all operations on the devices in the current access occasions are completed. If only one of these devices requires a subsequent DL command, the inventory is stalled until this is completed. With this scope, the utility of an address is to dynamically allocate resources for Msg3 and Msg4.

- Access Round: in this case the address is valid for a whole access round which would allow for parallel inventory of other devices, while other devices complete a subsequent command. In the worst-case scenario, the first access round of the inventory, all devices will try to access, which means that the number of competing devices can be up to 10k. This means that a network-allocated address would require 14 bit, while a randomly generated address with collision probability < 1% would require 32 bit. In this case it would be possible to continue longer transactions with the devices by dynamically allocating resources for those messages while other devices continue with the CBRA.

- Whole inventory: in this case the address is valid for the whole duration of the inventory procedure and across different access rounds. Since the worst-case access round may contain all the devices to be inventoried, it is equivalent as this case, so the same results apply.

- Long term / multiple inventories: in this case the address remains valid long term, similarly to what happens in legacy to devices that are in INACTIVE state with an allocated I-RNTI. In this case it would be possible to perform another inventory or command much later without the need for a complete initial access, but this address would need to be saved in NVM, since energy may be depleted, and a there would be a larger number of competing devices (all the devices in the cell). If we assume 50k devices this means 16 bits are needed for a network allocated address, and 37 bits for a randomly generated address with < 1% collision probability.

The following table summarizes the pros and cons of these 4 options.

Table 1: Summary of pros and cons of different address scopes

Maintaining an address in NVM for multiple rounds seems too complex for the target level of complexities, but the other three options seem reasonable and should be captured in the TR. Moreover, maintaining the address for the whole inventory seems equivalent to maintaining it only for the access round, so this latter option can be avoided.

Capture in the TR as possible options for the address scope the access occasion (smaller address but limited functionality) and the whole inventory procedure (larger address but more flexibility). See the TP in Annex B.

### 2.4.2	Ways to support addressing

Assuming that an address has been agreed in this section we discuss how it may be used to univocally determine which device is supposed to decode a certain R2D transmission or which device transmitted in a certain D2R resource.

There are two general ways. With explicit addressing a data transmission will contain the address as part of control information. The receiver of this message will decode the entire message, including the address control information. Otherwise with implicit addressing some separate signalling, containing the address, will indicate which device is supposed to receive/use some upcoming resources. In this case the data transmission itself will not contain the address.

Which option is more efficient depends on the expected message exchange. For instance, explicit addressing requires larger data transmission but no additional signalling, while the opposite is true for implicit addressing, therefore explicit addressing may be more suited for short sessions, while implicit for longer sessions.

Within the explicit addressing, the address may be included at different layers. For instance, if PHY layer will include a control part, this could contain the address. Otherwise, the address may be included as a MAC CE or part of the MAC subheader or even added at upper layers.

Within the implicit addressing, we distinguish one-shot and persistent implicit addressing.

One-shot implicit addressing consists in providing an UL/DL Grant containing the address and a description of a specific resource which will be used by the indicated device for a single transmission. This is analogous of legacy PDCCH followed by a PDSCH or PUSCH transmission.

Figure 5: Basic functioning of one-shot implicit addressing.

Persistent implicit addressing consists in a DL control signal blocking until further notice all the resources (or all resources in a specific frequency range in case of FDMA) for a specific device. In this case all data transmissions in this reserved band will belong to the same device. This option is more similar to how RFID works where once a slot begins with QueryRep all transmissions will belong to the same user until the following QueryRep starts a new slot.

Figure 6: Basic functioning of persistent implicit addressing.

Considering current progress on other agenda items, such as RACH, it is likely that either one-shot or persistent implicit addressing can fit nicely in the agreed mechanism. So, we propose to focus more on these.

Capture in the TR that resources can either reserved via an UL/DL grant containing the device address, or by reserving indefinitely a frequency resource using an address until a further reservation is made (see Text Proposal in Annex B)

### 2.4.3	Note on Logical Channels and modelling

RAN1 has agreed that PHY layer will support PRDCH for DL data transmission, PDRCH for UL data transmission, and not to support a broadcast physical channel (PBCH). This does not necessarily imply a certain set of supported transport channels are concluded for use in the interface between PHY and MAC layers.

In legacy NR the MAC PDUs received from the BCH are processed assuming a transparent MAC PDU format which does not add any overhead information. This is typically used for MIB delivery. In addition, in NR, PCH is used for paging message delivery. Whether the BCH and the PCH is needed would be depending on RAN1 decision.

DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

It is important to notice, though, that some messages in DL should be transmitted in a broadcast manner. An example of this is the initial trigger message targeting all devices in reader coverage. For modelling purposes, with no necessary implication on device complexity, it is convenient to define a Broadcast control channel (BCCH). For messages delivered through BCCH the addressing problem explained in this section does not apply as all devices in coverage area are supposed to decode the message. Notice that even if a message via BCCH is broadcast from MAC point of view, the NAS PDU may still contain a command directed to a specific device. Simply, if an address is not available at the moment, all devices should decode the message and determine if they are the intended receiver at a higher layer than MAC.

DL-DTCH (mapped to DL-SCH), BCCH (mapped to DL-SCH), UL-DTCH (mapped to UL-SCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Only messages delivered through DTCH are affected by the addressing problem from MAC perspective.

## 2.5	Other MAC aspects

### 2.5.1	Energy Status Report

In the last meeting it was agreed to capture in the TR a 1-bit Energy Status Report (ESR) in D2R messages. It remains to be decided whether this indication is reader-controlled or not.

It is worth spending some time in understanding in which cases this indication is useful.

Let’s assume that the expected device behaviour after ESR is sent is to stop the ongoing session. In this case the reader will avoid allocating further resources which is the main benefit of ESR. It is worth mentioning that this benefit largely depends on how precisely the device can determine the amount of energy left and how much energy these measurements consume (which could be not negligible) compared to just let the device work until it switches off. It is very likely that the small loss due to the lack ESR is compensated by avoiding stopping prematurely or doing frequent measurements.

How precisely the energy measurement is performed and how much energy it consumes may have a relevant impact on the usefulness of ESR.

As an example, shown in Figure 6, if performing energy measurements consumes a relevant amount of energy, there might be cases where a device may not be able to complete a session whereas if ESR and relative measurements are ignored that would be possible.

Figure 7: Example of case where not implementing ESR is more convenient than doing so

If the expected device behaviour is to stop the ongoing procedure after sending ESR, depending on precision and energy consumption of the energy measurement, there are cases where not sending ESR would allow the completion of the procedure instead

Since it was agreed to capture in the TR the use cases, it is worth capturing also this aspect.

Capture in the use case discussion in the TR the impact of measurement precision and energy consumption and the possibility that not sending ESR is more beneficial

An obvious solution is to allow the device to continue the procedure after it sent the ESR, but in this case another problem arises. If the following message transmission fails the reader has no way to determine if the failure was due to energy depletion or due to a regular radio link failure, and thus it has no way to perform a different action, which also makes the ESR less useful. Moreover, for R2D transmission the reader may also not have a way to determine if the transmission was successful or not (if ACK are not used, pending other agreements)

If the expected device behaviour is to continue the procedure after sending ESR, in case of a failure the reader has no way to determine the reason and thus act differently.

In conclusion the only useful case for ESR is if the expected behaviour after sending ESR is to stop the ongoing procedure, so that reader can avoid allocating further resources, and that the energy measurement itself is sufficiently precise and requires negligible amount of energy, which is conditional to the device capabilities.

Therefore, we propose to have a reader-enabled ESR which can be enabled when these conditions are met.

Capture in the TR that the expected behaviour after sending ESR is to stop the ongoing procedure.

Capture in the TR that ESR is enabled by the network.

Now the question is how the reader would signal that ESR is enabled since System Information will not be present. These are the possible options:

- ESR Enable bit is included in paging message (enabled per inventory procedure)

- ESR Enable bit can be included in any R2D transmission, possibly after reader becomes aware of device capabilities (enable per Request-Reply message pair)

- ESR is enabled based on device capabilities without need for signalling

- A combination of the above (e.g.: ESR is enabled in paging message, but only the devices with sufficient capabilities are allowed to use it)

In general we don’t see a need to signal explicitly if ESR is enabled or not, the reader may simply ignore it as if it was disabled, although this would consume more radio resources. For the Study Item it is sufficient to capture these options

Capture in the TR that reader can enable ESR in the following ways (See TP in Annex C):
- In paging message (per inventory)
- In any R2D transmission (per request/reply)
- Allowed based on device capabilities without explicit enable signal
- A combination of these options

Finally, it is worth reflecting on whether a device with not enough energy to at least complete a minimum set of operations should be allowed in the network. The harvesting conditions of a device will not likely change a lot between different sessions, so it is very likely that a device that failed a minimum set of operations will fail again multiple times, consuming radio resources. It is possible that a device in such conditions should be considered as “out of coverage” despite being technically in radio coverage.

Consider a device without enough energy to complete a minimum set of operations as “out of coverage” and bar it from connecting to the reader

### 2.5.2	Octet vs. Bit aligned MAC PDU

In the previous meeting some companies proposed the concept of not having octet aligned MAC PDU (or upper layer SDUs). It may be that the amount of control information to be sent is very low, just a few bits. In such case it might be beneficial to avoid octet alignment to avoid padding bits.

This discussion of course depends on how flexible and generic the TBS allocation is expected to be, also in future extensions.

Nevertheless, typically the length of a SDU is given in octet in NR. Avoiding octet-alignment means that now the length should be given in bits, which would increase the amount of signaling bits for indicating the TBS size.

- As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Moreover, considering device limitations, memory works by addressing words of various length, typically a byte. Therefore, it might create excessive complexity on device side to have bit-aligned buffers and stored data.

Also, current network design assumes octet alignment. Changing this may create excessive implementation effort on network side.

Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Include in the TR a recommendation that bit-aligned MAC PDUs is not supported (See TP in Annex D).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is preferable to register device context information for subsequent communication as there is no need to perform identification, authentication, security setup every time when a service procedure (e.g., inventory or command) is initiated.

Observation 2	The reader does not have a way to distinguish if a failure was caused by missing reception of the UL Grant or missing reception of the data segment.

Observation 3	Merging segmentation in MAC implies a smaller overhead in terms of encoding of control elements.

Observation 4	Merging segmentation in MAC implies a higher complexity in terms of modelling and lower reusability of network-side implementation of such layer.

Observation 5	Depending on RAN1 decisions, a certain success probability is guaranteed in the worst-case coverage scenario (denoted afterwards as “PHY success probability”)

Observation 6	Repetitions to further improve reliability managed by application or CN are out of RAN2 scope.

Observation 7	Repetitions or Reattempts are needed in RAN2 scope if the AS success probability target is lower than the guaranteed PHY success probability in worst-case scenario.

Observation 8	A network-assigned address guarantees uniqueness but requires signalling overhead for its provisioning.

Observation 9	A randomly generated address, or address calculated as function of the Device ID or an already assigned CN ID, does not require provisioning overhead but has a collision probability. Regardless of how negligible this probability is, the system should be able to recover from this.

Observation 10	The number of bits for which the collision probability is negligible depends on the number of devices competing in the same address scope.

Observation 11	A network-assigned address would require from 33% to 50% of the bits of a randomly generated address with the same number of competing devices

Observation 12	It’s not obvious which, between network-allocated and randomly generated address, has the smallest overall signalling overhead

Observation 13	Only messages delivered through DTCH are affected by the addressing problem from MAC perspective.

Observation 14	How precisely the energy measurement is performed and how much energy it consumes may have a relevant impact on the usefulness of ESR.

Observation 15	If the expected device behaviour is to stop the ongoing procedure after sending ESR, depending on precision and energy consumption of the energy measurement, there are cases where not sending ESR would allow the completion of the procedure instead

Observation 16	If the expected device behaviour is to continue the procedure after sending ESR, in case of a failure the reader has no way to determine the reason and thus act differently.

As a result, It is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Observation 17	Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture in TR 38.769 that RAN2 assumes the existence of “registration procedure (e.g., authorization and authentication)” catering for A-IoT requirements on identification, authorization, and authentication.

Proposal 2	Capture in TR38.76 how the overall AS procedure (three-step baseline) can support procedural steps for registration and device disabling.

Proposal 3	RAN2 assumes registration can be performed in connection with inventory procedure.

Proposal 4	Liaison with SA2, SA3 and CT1 asking for feedback while indicating the above statements captured in P1, P2, P3 in the TR.

Proposal 5	The device sends a 1-bit indication indicating whether the current transmission is the last segment or not.

Proposal 6	Capture in the TR the following options for handling segmentation error cases:

a.	A mechanism to check the validity of the overall transmission, after the last segment is received, is supported

b.	The trigger command may be sent with a more robust MCS than the data and/or a device is forbidden to operate if the signal quality is not sufficient

Proposal 7	A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

Proposal 8	Capture in the TR pros and cons of having segmentation (and possibly other features) in MAC vs. in a new AS layer on top of MAC (See TP in Annex A)

Proposal 9	Both blind repetitions and reattempts are considered as mechanisms to improve reliability

Proposal 10	RAN2 has yet to study repetitions mechanism in the hypothesis that PHY layer repetitions are not enough to satisfy a success probability target at AS level.

Proposal 11	RAN2 to consider multiple RACH attempts as one mechanism to implement repetitions

Proposal 12	RAN2 to determine which information should be used as address based on security, privacy, uniqueness, provisioning, and usage of NVM. FFS other aspects.

Proposal 13	Capture in the TR the following options for how the AS address is generated

a.	Randomly generated by the device (e.g.: same as contention resolution ID promoted to address after contention resolution)

b.	Allocated by the network in Msg2

c.	Generated based on Device ID in such a way that it’s quasi-random

Proposal 14	Capture in the TR as possible options for the address scope the access occasion (smaller address but limited functionality) and the whole inventory procedure (larger address but more flexibility). See the TP in Annex B.

Proposal 15	Capture in the TR that resources can either reserved via an UL/DL grant containing the device address, or by reserving indefinitely a frequency resource using an address until a further reservation is made (see Text Proposal in Annex B)

Proposal 16	DL-SCH (mapped to PRDCH) and UL-SCH (mapped to PDRCH) is the minimum set of transport channels provided by PHY layer.

Proposal 17	DL-DTCH (mapped to DL-SCH), BCCH (mapped to DL-SCH), UL-DTCH (mapped to UL-SCH) logical channels are defined at least for modelling purposes (not necessarily with complexity impact).

Proposal 18	Capture in the use case discussion in the TR the impact of measurement precision and energy consumption and the possibility that not sending ESR is more beneficial

Proposal 19	Capture in the TR that the expected behaviour after sending ESR is to stop the ongoing procedure.

Proposal 20	Capture in the TR that ESR is enabled by the network.

Proposal 21	Capture in the TR that reader can enable ESR in the following ways (See TP in Annex C): - In paging message (per inventory) - In any R2D transmission (per request/reply) - Allowed based on device capabilities without explicit enable signal - A combination of these options

Proposal 22	Consider a device without enough energy to complete a minimum set of operations as “out of coverage” and bar it from connecting to the reader

Proposal 23	Include in the TR a recommendation that bit-aligned MAC PDUs is not supported (See TP in Annex D).

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

RAN2#125bis Chair Notes, April 2024

RAN1#116bis R19 Ambient IoT Chair notes, April 2024

TS 22.369, “Service requirements for ambient power-enabled IoT”, 3GPP, March 2024

R3-24xxxx, “AIoT RAN CN interface details”, Ericsson, August 2024

# Annex A.	Text Proposal on protocol stack

Start of Change

### 6.3.2	Protocol stack and functionality aspects

(… omitted …)

NOTE 1:	Based on the study of the required functionalities, it can be further discussed on whether a new AS protocol on top of A-IoT MAC layer is needed.

Figure 6.3.2-1 Protocol stack for A-IoT radio interface between A-IoT device and reader

As to the A-IoT required functionalities, the following functionalities are supported:

-	A-IoT paging (see clause 6.3.3)

-	A-IoT random access procedure (see clause 6.3.4)

-	A-IoT data transmission (see clause 6.3.5)

-	A-IoT segmentation (see clause 6.3.5)

-	A-IoT device configuration (see clause 6.3.x)

It can be discussed if all these features should belong to the A-IoT MAC layer or if an additional layer should be defined above A-IoT MAC. The following considerations should be taken into account:

- Merging all functionalities in A-IoT MAC requires on average a smaller control signalling overhead, but the resulting layer has higher complexity and monolithic structure.

- Separating some functionalities (e.g.: A-IoT segmentation) in an additional layer requires on average a larger control signalling overhead but each layer has low complexity. In particular, A-IoT MAC would contain no functionalities that are supported by a different layer in legacy

End of Change

# Annex B.	Text Proposal on device addressing

Start of Change

### 6.3.5	A-IoT data transmission

### (… omitted …)

From higher layer perspective, it is assumed that “AS ID” (if defined according to the design in clause 6.1) is used at least for purpose of D2R scheduling and R2D reception. From higher layer perspective, it is assumed that this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. It can be further discussed if this “AS ID” can be based on partial upper layer device ID. It can be further discussed on the length of this “AS ID”. From higher layer perspective, following options are possible for this “AS ID” (it is aimed to define one common design for all access procedures in sub-clause 6.3.4, if technically possible):

-	Option 1: randomly generated by the device a random ID (if used in first D2R message it can be reused) can be reused;

-	Option 2: the reader assigns this “AS ID” to the device. It can be further discussed via which R2D message.

-	Option 3: based on part of the upper layer device ID (provided the resulting AS ID has quasi-random properties)

The needed number of bits for the AS ID depends on the number of competing devices, which depends on the validity duration of the AS ID itself, the chosen option about how the AS ID is generated, and the accepted collision probability in case the AS ID is not unique (e.g.: if randomly generated).

Notice that in case of randomly generated AS ID composed by b bits, with uniform distribution, the probability that two or more devices choose the same AS ID among N devices is:

pc=1-1-12bNN-12

It is possible to define a maximum collision probability, which leads to a minimum number of bits for the AS ID.

Figure 6.3.5.x: (left) Collision probability as function of address length and number of competing users. (right) Minimum number of address bits as function of number of competing users and maximum collision probability requirement or if address is network allocated

We can identify the following cases (assuming 1% collision probability for Options 1 and 3):

- If the validity of the AS ID is the CBRA, only few devices (<10) are competing for an address. This means that very few bits are needed, but it is not possible to use the AS ID in subsequent transactions. For Options 1 and 3, 12 bit AS ID is needed, while for Option 2 only 4 bits are needed

- If the validity of the AS ID is a full inventory round, up to 10000 devices may be competing for an address. This means that more bits are needed, but the AS ID can be used for subsequent transactions without the need of restarting the CBRA. For Options 1 and 3 32 bit AS ID is needed, while for Option 2 only 14 bits are needed

- If the validity of the AS ID is long-term across multiple inventory rounds, more devices may be competing for an address. Other than more bits needed it may be necessary to store the address in NVM which results in more energy consumption and complexity.

For A-IoT the address used for scheduling purposes shall not be included in the data transmission, but in a separate control signalling instead.

One solution, denoted as “One-shot addressing” is where a R2D/D2R grant transmitted by the reader contains the AS ID and a specific resource definition (e.g.: time, frequency, MCS, TBS, etc). A new grant is provided to schedule every new data transmission.

As a second solution, denoted as “persistent addressing”, consists in reserving a set of resources for an indefinite amount of time through a grant message containing the AS ID and information about the resources, until a new grant is provided to reserve the resources to a new device or to free them.

End of Change

# Annex C.	Text Proposal on Energy Status Report

Start of Change

### 6.3.5	A-IoT data transmission

(… omitted …)

The use of the following assistance information is studied (the need of each is still to be decided):

-	The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure.

-	The A-IoT device may report a 1-bit energy status indication to the reader in a D2R message. It can be further discussed if and how the reader control this indication of such report.

-	The reader may take this indication into account for the remaining/follow-up procedure (up to the reader implementation), e.g., the reader may choose not to transmit the subsequent messages for some time, or the reader may choose to not do anything in particular, etc.

It is noted that performing the energy measurement to generate this indication may be not precise for some less complex devices. Moreover, the measurement itself may consume a significant amount of energy. Therefore, there are cases where using the energy status report may prevent the completion of a certain procedure, while no performing any measurement would actually allow the completion of the same procedure due to not wasting energy in performing the measurements. An example is showed in Figure 6.3.5.y.

Figure 6.3.5.y: Example of negative impact of energy status report

It is suggested that this indication is not mandatory, as its usefulness depends on the scenario and the device capabilities; in particular, on the precision of the measurement and the amount of energy required to perform the measurement which may be not negligible. Considering that broadcasted System Information is not supported for A-IoT, the following options may be considered:

- The energy status report is enabled in the paging message for all targeted devices.

- The energy status report is enabled on a D2R basis via the preceding R2D transmission.

- The energy status report is allowed if the device capabilities are sufficiently high (i.e.: the negative impact of measurement precision and energy consumption are negligible)

End of Change

# Annex D.	Text Proposal on MAC PDU alignment

Start of Change

### 6.3.5	A-IoT data transmission

(… omitted …)

MAC should support MAC CEs and multiplexing function at least to support the inclusion of both upper layer SDUs and MAC CE in the same transport block. The MAC PDU should also be octet-aligned to maintain the subheader size small and to allow an easier use of memory.

End of Change

# Annex E.	Text Proposal on Registration

Start of Change

### 6.3.1	General aspects and overall procedure

The study aims that the design on the A-IoT radio interface between reader and A-IoT device is common for Topology 1 and Topology 2. The difference of topologies is transparent to the A-IoT devices and has no impact on A-IoT devices. Unless explicitly stated, the descriptions in clause 6.3 apply to all A-IoT device types and both Topology 1 and Topology 2.

Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

The overall AS procedures can be formulated as:

-	Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond. See clause 6.3.3.

NOTE 1:	In the clause 6.3, the term of “A-IoT paging message” is equal to the “(initial) trigger message”. For simplification, only the former is used.

-	Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. See clause 6.3.4 (and 6.3.5).

-	Step C1: Possible R2D data transmission (e.g., for sending the command). See clause 6.3.5.

-	Step C2: Possible D2R data transmission (e.g., the corresponding response to command). See clause 6.3.5.

Above AS procedure can support indoor inventory and indoor command use cases by the following manners:

-	For the detailed use case of “inventory-only”, it is supported by the procedure with Step A and Step B as baseline.

-	For the detailed use case of “inventory and command”, it is supported by the procedure with Step A, Step B, Step C1 and Step C2, as baseline.

NOTE 2:	For the use case of “inventory and command”, it does not imply that the A-IoT paging message includes both the inventory and command and it does not imply the inventory and command are received by the reader at the same time from upper layer.

-	For the detailed use case of “command-only”:

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [7] and [8]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

In addition, above AS procedure can support registration procedure and disabling procedure by the following manners:

-	For the detailed use case of “registration”, it can be supported by incoporating signaling for authentication procedure (one-way or mutual authentication) into Step A and/or Step C. It is assumed that Step C may have more transmissions than Step C2. Possible CN allocated temporary device ID is realized via Step C1.

-	For the detailed use case of “disabling”, it can be supported by the baseline procedure Step A, Step B, Step C1, and Step C2. It is assumed the disabling command is only performed by the A-IoT device when the commanding network (reader) is considered authentic.

NOTE X:	RAN2 assumes the existence of registration procedure (e.g., authorization and authentication) catering for A-IoT requirements on identification, authorization, and authentication. RAN2 further assumes registration can be performed in connection to inventory or command procedures or in a standalone fashion.

End of Change