3GPP TSG-RAN Meeting #128	R2-2410198

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.7.3

Source:	Ericsson

Title:	RRM Measurement Gap/Restrictions

Document for:	Discussion, Decision

# 1	Introduction

The following are the agreements from RAN2#128 on measurement gap enhancements.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

At the same time, RAN1 already progressed on solutions of measurement gap enhancements. In this contribution, we discuss the above agreements considering the latest RAN1 agreements.

# 2	Discussion

## 2.1 	RAN1 Discussions

There have been extensive discussions on all possible alternatives to avoid scheduling restriction including the DCI and RRC based solution. From the previous RAN1 meeting #118, discussions concluded with the following working assumption that DCI based dynamic solution (Alt 1-1) is agreed to be supported.

Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

- RAN1 agreement selected the DCI based solution instead of RRC based solution.

The decision to skip scheduling restrictions should be based on UE’s instantaneous conditions e.g., radio environment, channel conditions and mobility. As only the DCI based solution adapts scheduling restrictions on a particular occasion i.e., dynamically, it is more flexible and reliable than the RRC based solution cannot dynamically adapt scheduling restrictions. Thus, the expected performance of DCI based dynamic scheduling restriction is better than the RRC based semi-static solution.

- DCI based dynamic solution is more effective, efficient, and reliable than the RRC based semi-static solution.

With the above understanding and based on RAN1’s evaluation, it is not necessary to wait for further evaluation from RAN4 and RAN2 should follow RAN1’s suggestion to consider only the DCI-based dynamic solution. It was also agreed in the last meeting that RAN2 will focus its work on supporting the solution chosen from RAN1/RAN4. Hence, RAN2 should not consider other solutions.

Follow RAN1’s solution for the DCI based skipping solution.

## 2.2 	Discussions of the Impact on DSR/CG/DRX

### 2.2.1	DSR

DSR is triggered by MAC, but its actual transmission is completely up to the availability of radio resource which is dynamically controlled by physical layer signaling. At the same time, the DCI based solution will be integrated in scheduling DCI which means gNB will anyway send a grant with skipping indication. Therefore, if a gNB expects or wants to schedule a resource for DSR but it overlaps with a certain measurement gap/scheduling restriction, it can simply additionally indicate ‘skipping’ in the scheduling DCI for the DSR report. This does not require any specification change.

- Scheduling DCI with skipping indication is sufficient to ensuring a grant for DSR so that no specification impact is expected.

No specification impact is foreseen for measurement gap skipping on DSR.

### 2.2.2	CG

For XR traffic, the gNB is fully aware of the traffic patterns and periodicities with the XR awareness enhancements in Rel-18 and hence, can determine if there is a need to skip the measurement gap.

- gNB is fully aware of the traffic patterns and periodicities for XR traffic and can determine if there is a need to skip the measurement gap.

For CG, the baseline assumption is that network configuration can already ensure that there is either no overlap between CG and measurement gaps or the overlap can be kept to a minimum such that the XR data rate is unaffected.

- For CG, network configuration can ensure there is no overlap between CG and measurement gaps.

Some companies argue that if a UE wants to use a CG occasion overlapping with the measurement gap (for e.g., buffer status report (BSR) of a new traffic arrival) i.e., autonomous UE decision should be supported. However, such approach is very risky since the gNB expects that if a CG and measurement gap occasion overlap, the corresponding CG occasion will be skipped. If a UE is allowed to autonomously skip MG without informing gNB it would result in lost transmissions or in our view an unacceptable increase of the blind decodings at the gNB

- Autonomous UE skipping of measurement gaps for overlapping CG occasions is very risky since a gNB assumes there is no transmission during measurement gaps or would lead to an unacceptable increase of the blind decodings at the gNB.

If the gNB wants to enable use of the resource configured for the CG which overlaps with a measurement gap, gNB can send a dynamic grant with DCI indicating skipping the corresponding gap. This dynamic grant that overlaps with the CG occasion will override the CG.

- DCI based solution can override a CG occasion overlapping with measurement gap to ensure needed uplink transmissions.

Based on our observations we propose:

No enhancements are needed to support CG using the DCI based skipping solution.

### 2.2.3	DRX

Some companies also showed an interest to study potential impact on DRX for the measurement gap skipping. In principle, DRX configuration is independent from measurement gap configuration and their patterns are implementation dependent. If skipping indication needs to be sent, it means that there is a need for scheduling data, so that PDCCH should be transmitted only during when DRX active time. Otherwise, a UE will not monitor any scheduling DCI regardless of skipping indication. Therefore, adding skipping indication in scheduling DCI will not have any impact on DRX operation. The UE configured with DRX can operate as in legacy without being expected to monitor the channel outside the DRX ON duration.

No specification impact is foreseen on the impact of DRX on measurement gap skipping.

## 2.3	Discussion on UAI

Only remaining issue in RAN2 to support the DCI based skipping indication is supporting UE assistance information (UAI). RAN1 also extensively discussed the usefulness of new UAI related to channel conditions, traffic, UE mobility, the measurement occasions. Then, it was agreed that UAI related to channel conditions, traffic, UE mobility is not needed. The UAI related to the measurement occasions may be useful if RAN4 requires to avoid too excessive skipping. Therefore, whether to support UAI related to the measurement occasions requires a progress in RAN4 discussion and we have a following proposal.

RAN2 should wait for RAN4 progress to know if information in the UAI is needed to enable skipping.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	RAN1 agreement selected the DCI based solution instead of RRC based solution.

Observation 2	DCI based dynamic solution is more effective, efficient, and reliable than the RRC based semi-static solution.

Observation 3	Scheduling DCI with skipping indication is sufficient to ensuring a grant for DSR so that no specification impact is expected.

Observation 4	gNB is fully aware of the traffic patterns and periodicities for XR traffic and can determine if there is a need to skip the measurement gap.

Observation 5	For CG, network configuration can ensure there is no overlap between CG and measurement gaps.

Observation 6	Autonomous UE skipping of measurement gaps for overlapping CG occasions is very risky since a gNB assumes there is no transmission during measurement gaps or would lead to an unacceptable increase of the blind decodings at the gNB.

Observation 7	DCI based solution can override a CG occasion overlapping with measurement gap to ensure needed uplink transmissions.

The observations lead to the following proposals:

Proposal 1	Follow RAN1’s solution for the DCI based skipping solution.

Proposal 2	No specification impact is foreseen for measurement gap skipping on DSR.

Proposal 3	No enhancements are needed to support CG using the DCI based skipping solution.

Proposal 4	No specification impact is foreseen on the impact of DRX on measurement gap skipping.

Proposal 5	RAN2 should wait for RAN4 progress to know if information in the UAI is needed to enable skipping.