3GPP TSG-RAN WG2 #127	R2-2410201

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.13.4

Source:	Ericsson

Title:	Service Continuity for Multi-Hop Relays

Document for:	Discussion, Decision

# 1 	Introduction

A new WI was agreed in RAN#104 plenary for NR sidelink multi-hop relays and the following is the objective for service continuity for multi-hop U2N relays:

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

# 2 	Discussion

On the issue of supporting service continuity in a multi-hop relaying scenari,o largely depends on the two approaches as discussed in the post meeting email discussion in [1] as follows:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

As the approach influences the outcome, RAN2 should first settle on this issue before proceeding to discuss the details of service continuity.

RAN2 to first settle on the approach before proceeding with discussing the details of service continuity.

As described in the objective, the existing framework should be used in the design of the multi-hop indirect to direct/single-hop path switch.

The measurement events are an important part of the existing framework. For single-hop relays, events X1/X2 were specified for indirect to direct (i2d) path switch scenario while events Z1/Z2 were defined for indirect to indirect (i2i) path switch. For the multi-hop indirect to the direct path case events X1/X2 can be used as the baseline but might need to be modified depending on the definition of the serving relay UE. The same goes for the multi-hop indirect to single-hop indirect path switch case.

Events X1/X2 and Z1/Z2 can be used as the baseline for measurement configuration for multi-hop indirect to direct path switch and multi-hop indirect to single-hop indirect path switch respectively.

# 3 	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to first settle on the approach before proceeding with discussing the details of service continuity.

Proposal 2	Events X1/X2 and Z1/Z2 can be used as the baseline for measurement configuration for multi-hop indirect to direct path switch and multi-hop indirect to single-hop indirect path switch respectively.

# 4	References

[1] Report of [POST127][402][Relay] Multi-hop relay control plane, RAN2#128, Orlando, USA.