3GPP TSG-RAN WG2 #128	Tdoc R2-2410202

Orlando, US, November 18 – 22, 2024

Agenda Item:	8.12.3

Source:	Ericsson

Title:	Impacts from other NR MIMO Phase 5 objectives

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the impacts from NR MIMO phase 5 related to the following objectives:

- Specify enhancement to facilitate UE-initiated/event-driven beam management

- Specify CSI support for up to 128 CSI-RS ports

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions

# 2	Discussion

An LS was received from RAN1 with RRC parameters for MIMO (R1-2407285).

This section discusses the impacts related to the LS.

## 2.1	Background UE-initiated beam management

Ever since Rel-15, NR supports intra-cell and inter-cell mobility. Inter-cell mobility, also known as L3 mobility, relies on event-driven driven reporting, which is used as a basis for HO signalling. The procedures related to intra-cell mobility are called beam management, and involve periodic, semi-persistent and aperiodic beam measurements, which are used as a basis for beam indication signalling from the NW to the UE.

For intra-cell mobility, there is no event-driven reporting specified. Event-driven, or UE-initiated, reporting could reduce the signalling overhead while maintaining reasonably fast discovery of changing propagation conditions resulting from UE movement across the Tx beam pattern. When the UE discovers that the propagation conditions have changed, the UE would notify the NW and send a report. Based on the report, the NW would send a beam indication to the UE.

RAN1 has agreed to support the following events:

Event-1: Quality of the current beam is worse than a certain threshold.

Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality

RAN1 has defined “current beam” based on the indicated TCI state.

The main motivation for UE-initiated reporting is to reduce overhead, i.e., to avoid transmitting measurement reports that would not result in a beam switch: ideally, each transmitted report should result in a beam switch. This means there must be a mechanism to avoid transmitting reports based on measurement errors or fading dips/peaks. To avoid such spurious reports, RAN1 made the following agreement:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

RAN1 has also lately agreed to some more details to the UE-initiated/event-driven beam reporting such as reuse of the CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report and aspects related to pre-configured Type-1 CG PUSCH resources (amongst other) to carry the beam report. However, as there are several outstanding details to the aspects that may also have RAN2 impact, RAN2 should wait for RAN1's progress discussing those apart from discussing the general principle on where the event valuation and reporting can be specified, see sect. 2.2.3.

### 2.1.1	UE-initiated beam management in RAN2

Currently multiple beam operation including beam management is an important procedure to support mobility on beam level. The current functionality, like beam failure detection and recovery is a closely related set of PHY/MAC procedures. This includes counters for BFR where L1 provides indications to MAC which MAC uses to determine when beam failure should be declared. That is, currently associated actions for beam management are initiated in MAC to maintain TRP(s) and UL beams for DL/UL transmission and reception. In fact, the beam failure recovery procedure is a component of beam management itself.

The RAN1 agreement for Rel-19 beam management (see 2 Background above), when looking at the Event 2 basic function, where a better beam trigger is based on a counter, it seems natural that also that triggering event for the associated L1 indications can be captured in MAC similarly as e.g. beam failure in legacy etc.

Additionally, comparing the functionality w.r.t specifying event triggering and associated reporting procedure are very similar between LTM and UE-initiated beam management.

To note, RAN2 has made agreements for LTM that entails that MAC layer handles the event evaluation and measurement report trigger, together with the clarification that the entire evaluation procedure is handled in MAC based on latest L1 measured results.

While in the end, UE-initiated beam management and LTM may use different signalling methods (MAC CE vs UCI) the fundamental properties of the corresponding event evaluation and measurement report trigger for UE-initiated beam management seems similarly rather straightforward to be specified in the MAC.

To keep this existing separation and benefit from aligned event triggering and reporting in core specification, we propose:

The event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

RAN2 to inform RAN1 on its preference of specifying the triggering event determination for Event 2.

### 2.1.2	Signaling aspects for UE-initiated beam reporting

From the RRC parameters list it was left up to RAN2 to decide on how to capture enabledCurrentBeamReport-r19:

| CSI-ReportConfig | enabledCurrentBeamReport-r19 | New | This parameter is to enable or disable whether current beam is always reported. - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported l - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. | Up to RAN2 |
| --- | --- | --- | --- | --- |

Since the parameter should have an enable/disable behaviour, this can be achieved in RRC by having an optional need R field where presence means the feature is enabled and absence means the feature is not enabled/released.

enabledCurrentBeamReport-r19 is defined as an optional need R field.

## 2.2	CSI support for up to 128 CSI-RS ports

For CSI-Type-I/II, multiple parameter details were left up to RAN2. Firstly, group based CBSR is described as below for CSI-typeI:

| n1-n2-typeI-r19 | new |  | Supported values of (N1,N2): 48 ports: (8,3), (6,4) 64 ports: (16,2), (8,4) 128 ports: (16,4), (8,8) | up to RAN2 |
| --- | --- | --- | --- | --- |
| typeI-CBSR-r19 | new |  | Group-based CBSR (hard restriction only): (N1O1N2O2)/(X1X2)-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD basis vectors along the N2 direction - The CBSR is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| valueOfX1-typeI-CBSR-r19 | new |  | X1 from one of the supported values of (X1,X2): (1,1), (2,1), (2,2), (4,1), (4,2), (4,4) | 1,2,4 |
| valueOfX2-typeI-CBSR-r19 | new |  | X2 from one of the supported values of (X1,X2): (1,1), (2,1), (2,2), (4,1), (4,2), (4,4) | 1,2,4 |

As already done for Rel-18, the CBSR configuration can be embedded in the N1-N2 configuration e.g.:

n1-n2-codebookSubsetRestriction-r19                 N1-N2-CBSR-r19

N1-N2-CBSR-r19 ::=     CHOICE {

eight-three-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction1-r19},

six-four-r19               CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction1-r19},

sixteen-two-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction2-r19},

eight-four-r19             CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction2-r19},

sixteen-four-r19           CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction3-r19},

eight-eight-r19            CHOICE {no-cbsr-r19 NULL, X1-X2-Restriction3-r19}

}

To account for the X1-X2 grouping, different IEs can be used based on the bit string size required for each (N1, N2) pair e.g.:

X1-X2-Restriction1-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (384)),

two-one-r19            BIT STRING (SIZE (192)),

two-two-r19            BIT STRING (SIZE (96)),

four-one-r19           BIT STRING (SIZE (96)),

four-two-r19           BIT STRING (SIZE (48)),

four-four-r19          BIT STRING (SIZE (24))

}

X1-X2-Restriction2-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (512)),

two-one-r19            BIT STRING (SIZE (256)),

two-two-r19            BIT STRING (SIZE (128)),

four-one-r19           BIT STRING (SIZE (128)),

four-two-r19           BIT STRING (SIZE (64)),

four-four-r19          BIT STRING (SIZE (32))

}

X1-X2-Restriction3-r19 ::=     CHOICE {

one-one-r19            BIT STRING (SIZE (1024)),

two-one-r19            BIT STRING (SIZE (512)),

two-two-r19            BIT STRING (SIZE (256)),

four-one-r19           BIT STRING (SIZE (256)),

four-two-r19           BIT STRING (SIZE (128)),

four-four-r19          BIT STRING (SIZE (64))

}

For CSI-Type-I, CBSR configuration is embedded in the N1-N2 configuration. X1-X2 grouping is defined for each N1-N2 configuration.

For power scaling, a similar X1-X2 group handling was defined in RAN1:

| typeI-powerScaling-r19 | new |  | 3-bit scaling factor for each group of X1X2: 3(N1O1N2O2)/(X1X2)-bit power scaling factors in total - the power scaling scheme is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| --- | --- | --- | --- | --- |
| valueOfX1-typeI-powerScaling-r19 | new |  | X1 from one of the supported values of (X1,X2): (2,1), (2,2), (4,1), (4,2), (4,4), (8,1) | 2,4,8 |
| valueOfX2-typeI-powerScaling-r19 | new |  | X2 from one of the supported values of (X1,X2): (2,1), (2,2), (4,1), (4,2), (4,4), (8,1) | 1,2,4,8 |

A similar structure as for CBSR grouping can be used, except that there is no need to configure N1-N2 again, and thus the choice structure within each field of N1-N2-CBSR-r19 is not needed.

typeI-powerScaling-r19                              TypeI-N1-N2-PowerScaling-r19           OPTIONAL  -- Need R

TypeI-N1-N2-PowerScaling-r19 ::=     CHOICE {

eight-three-r19            X1-X2-PowerScalingGroup1-r19,

six-four-r19               X1-X2-PowerScalingGroup1-r19,

sixteen-two-r19            X1-X2-PowerScalingGroup2-r19,

eight-four-r19             X1-X2-PowerScalingGroup2-r19,

sixteen-four-r19           X1-X2-PowerScalingGroup3-r19,

eight-eight-r19            X1-X2-PowerScalingGroup3-r19

}

X1-X2-PowerScalingGroup1-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (576)),

two-two-r19            BIT STRING (SIZE (288)),

four-one-r19           BIT STRING (SIZE (288)),

four-two-r19           BIT STRING (SIZE (144)),

four-four-r19          BIT STRING (SIZE (72)),

eight-one-r19          BIT STRING (SIZE (144))

}

X1-X2-PowerScalingGroup2-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (768)),

two-two-r19            BIT STRING (SIZE (384)),

four-one-r19           BIT STRING (SIZE (384)),

four-two-r19           BIT STRING (SIZE (192)),

four-four-r19          BIT STRING (SIZE (96)),

eight-one-r19          BIT STRING (SIZE (192))

}

X1-X2-PowerScalingGroup3-r19 ::=     CHOICE {

two-one-r19            BIT STRING (SIZE (1536)),

two-two-r19            BIT STRING (SIZE (768)),

four-one-r19           BIT STRING (SIZE (768)),

four-two-r19           BIT STRING (SIZE (384)),

four-four-r19          BIT STRING (SIZE (192)),

eight-one-r19          BIT STRING (SIZE (384))

}

For CSI-Type-I power scaling, X1-X2 grouping is defined for each N1-N2 configuration.

For ng-n1-n2, the L1 parameter tables states the following:

| ng-n1-n2-r19 | new |  | Supported values of (Ng, N1,N2): 48 ports: (2,4,3), (2,6,2),(2,12,1) 64 ports: (2,8,2), (2,16,1), (4,4,2), (2,4,4), (4,8,1) 128 ports: (4,4,4), (4,16,1), (4,8,2) | up to RAN2, (direct extension of the legacy design of ng-n1-n2 in R15 typeI-multiPanel) |
| --- | --- | --- | --- | --- |

As already stated in the L1 parameter table above, this parameter should be an extension of the legacy ng-n1-n2 in R15 typeI-multiPanel, hence the same structure can be used e.g.:

typeI-MultiPanel-r19                                    SEQUENCE {

ng-n1-n2                                                CHOICE {

two-four-three-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (192)),

two-six-two-TypeI-MultiPanel-Restriction                 BIT STRING (SIZE (192)),

two-twelve-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (48)),

two-eight-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (256)),

two-sixteen-one-TypeI-MultiPanel-Restriction             BIT STRING (SIZE (64)),

four-four-two-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (128)),

two-four-four-TypeI-MultiPanel-Restriction               BIT STRING (SIZE (256)),

four-eight-one-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (32)),

four-four-four-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (256)),

four-sixteen-one-TypeI-MultiPanel-Restriction            BIT STRING (SIZE (64)),

four-eight-two-TypeI-MultiPanel-Restriction              BIT STRING (SIZE (256))

}

}

The field ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel.

For CSI-typeII, a group-based CBSR was also defined:

| n1-n2-typeII-r19 | new |  | Supported values of (N1,N2): 48 ports: (8,3), (6,4) 64 ports: (16,2), (8,4) 128 ports: (16,4), (8,8) | up to RAN2 |
| --- | --- | --- | --- | --- |
| typeII-CBSR-r19 | new |  | Group-based CBSR (hard restriction only): (N1N2)/(X1X2)-bit CBSR where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD basis vectors along the N2 direction - The CBSR is optional | up to RAN2, Bitmap size depends on (N1,N2) and (X1,X2) |
| valueOfX1-typeII-CBSR-r19 | new |  | X1 from one of the supported values of (X1,X2): (1,1), (2,1), (4,1) for (N1,N2)=(8,3), (1,1), (2,1), (2,2) for (N1,N2)=(6,4), or (1,1), (2,1), (2,2), (4,1), (4,2) for (N1,N2)=(16,2), (8,4), (16,4), or (8,8) | 1,2,4 |
| valueOfX2-typeII-CBSR-r19 | new |  | X2 from one of the supported values of (X1,X2): (1,1), (2,1), (4,1) for (N1,N2)=(8,3), (1,1), (2,1), (2,2) for (N1,N2)=(6,4), or (1,1), (2,1), (2,2), (4,1), (4,2) for (N1,N2)=(16,2), (8,4), (16,4), or (8,8) | 1,2 |

All N1/N2 values and X1/X2 values above are contained within those of CSI-typeI. Hence the same IEs defined for CSI-typeI can be used for CSI-typeII. However, CSI-typeII has further restrictions as described above i.e. X2 values can only include 1 or 2 (while CSI-typeI can have value 4) and not all X1, X2 groups are applicable to all N1, N2 configurations – such restrictions can be clarified in field description.

The same IE defined for CBSR CSI-typeI is also used for CBSR CSI-typeII.

For CSI-RI, the following was left up to RAN2:

| cri-typeI-SinglePanel-ri-restriction-r19 | New |  | resource-specific RI restriction (8-bit bitmap per resource, where the 8-bit bitmap is similar to (or the same as) typeI-SinglePanel-ri-restriction) , Rel-15 Type-I-based | up to RAN2 |
| --- | --- | --- | --- | --- |
| cri-typeII-ri-restriction-r19 | New |  | resource-specific RI restriction (4-bit bitmap per resource, where the 4-bit bitmap is similar to (or the same as) typeII-ri-restriction) , Rel-16 eType-II based | up to RAN2 |

Those can be defined as legacy RI restrictions as also indicated in the L1 parameter list. They can also be bundled in a choice structure e.g.:

cri-RI-Restriction-r19              CHOICE {

typeI-SinglePanel-RI-Restriction-r19    BIT STRING (SIZE (8)),

typeII-r19-RI-Restriction-r19           BIT STRING (SIZE (4))

}                                                                                                           OPTIONAL   -- Need R

The fields cri-typeI-SinglePanel-ri-restriction-r19 and cri-typeII-ri-restriction-r19 are defined in the same way as typeI-SinglePanel-ri-restriction and typeII-ri-restriction, respectively.

## 2.3	UE reporting enhancement for CJT

It was left up to RAN2:

| referenceAntennaPort | New |  | The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource | up to RAN2 |
| --- | --- | --- | --- | --- |
| subbandSize | New |  | Supported sub-band size(s) {1, 2, 4, 8, 16} PRB for SB PO reporting | {1,2,4,8,16} |
| po-ReportingBand | New |  | configuration of which NSB-P sub-band(s) that the UE reports | up to RAN2 |

It should be noted that referenceAntennaPort is still under discussion in RAN1, and thus there is no need to discuss it in RAN2 at this point. Similarly, po-ReportingBand is also still under discussion in RAN1 and can be postponed.

## 2.4	Non-coherent UL codebook for 3-antenna-port UE

The RRC parameter “[4portSRS_3TX]” is shown in the RAN1 LS on MIMO RRC parameters R1-2407285 as conditional upon whether the proposal in the comment field was agreed, as can be seen in the LS excerpt below. However, the agreement was made in RAN1#118, and so the parameter will be specified.  Exactly how it should be defined from a RAN2 perspective requires further discussion.

| Parameter name in the spec | New or existing? | Description | Value range | Comment |
| --- | --- | --- | --- | --- |
| [4portSRS_3TX] | New | [1 bit RRC parameter to indicate that the configured 4-port SRS resource is configured for a 3TX UE] | single bit | Note: The parameter is conditioned on endorsement of the following proposal,[Proposal: For a UE reporting a UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled.] |

The Rel-19 MIMO WID objectives for 3 Tx (copied below) preclude enhancing SRS resources in order to minimize the specification impact and effort to implement the 3Tx feature.  Therefore, no 3 port SRS was defined; instead, a 4 port SRS resource is used and port 1003 (that can carry SRS and/or PUSCH) is disabled.  This means that the “[4portSRS_3TX]” parameter is configured together with a 4 port SRS to indicate that the UE should use 3 Tx UL MIMO, including transmitting only 3 of the 4 SRS ports, not transmitting PUSCH on port 1003, and using the 3 Tx codebook.

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

An SRS resource with nrofSRS-Ports = ports4 and the 3 Tx UL MIMO parameter  “[4portSRS_3TX]” should be both be configured to enable 3 Tx UL MIMO operation

The parameter “[4portSRS_3TX]” determines whether port 1003 is disabled for both PUSCH and SRS.

RAN1 did not suggest a parent IE for “[4portSRS_3TX]”, but since the parameter describes an SRS resource configuration, it should be a field within SRS-Resource, and not a value of (a Rel-19 version of) nrofSRS-Ports. RAN1 suggested a value range of 1 bit for “[4portSRS_3TX]”, however our understanding of the intention is to say if 3 Tx operation is configured or not, so the value range should be “ENUMERATED { enabled }”.

The L1 parameter [4portSRS_3TX] should be a field within SRS-Resource defined as “ENUMERATED { enabled }”. This field is only configured if nrofSRS-Ports is set to “ports4”.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	An SRS resource with nrofSRS-Ports = ports4 and the 3 Tx UL MIMO parameter  “[4portSRS_3TX]” should be both be configured to enable 3 Tx UL MIMO operation

Observation 2	The parameter “[4portSRS_3TX]” determines whether port 1003 is disabled for both PUSCH and SRS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

Proposal 2	RAN2 to inform RAN1 on its preference of specifying the triggering event determination for Event 2.

Proposal 3	enabledCurrentBeamReport-r19 is defined as an optional need R field.

Proposal 4	For CSI-Type-I, CBSR configuration is embedded in the N1-N2 configuration. X1-X2 grouping is defined for each N1-N2 configuration.

Proposal 5	For CSI-Type-I power scaling, X1-X2 grouping is defined for each N1-N2 configuration.

Proposal 6	The field ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel.

Proposal 7	The same IE defined for CBSR CSI-typeI is also used for CBSR CSI-typeII.

Proposal 8	The fields cri-typeI-SinglePanel-ri-restriction-r19 and cri-typeII-ri-restriction-r19 are defined in the same way as typeI-SinglePanel-ri-restriction and typeII-ri-restriction, respectively.

Proposal 9	The L1 parameter [4portSRS_3TX] should be a field within SRS-Resource defined as “ENUMERATED { enabled }”. This field is only configured if nrofSRS-Ports is set to “ports4”.