3GPP TSG-RAN WG2 Meeting #128	R2-2410209

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM measurement gaps/restrictions related enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements on RRM measurement gaps/restrictions related enhancements were made in RAN2#127bis:

RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In this paper we further discuss RAN2 impacts of RRM measurement gaps/restrictions related enhancements with focus on FFSs from RAN2#127bis.

# 2	Discussion

## 2.1	Impact of gap/restriction skipping on DSR operation

The potential impacts of cancellable MG/scheduling restrictions to the DSR procedure were discussed in RAN2#127bis meeting. Most companies seemed of the opinion that network can anyway configure the remaining time threshold considering the potential impact of (cancellable) MG/scheduling restrictions. However, it was also agreed that cancellable MG/scheduling restrictions may have an impact on the remaining time calculation/definition.

Based on Rel-18 specifications, data is determined as delay-critical by comparing its remaining time till PDCP discard timer expiry to a configured threshold value. The remaining time till PDCP discard timer is expected to reflect the available time for the corresponding PDCP SDU to be scheduled for transmission (before it is discarded).

However, in proximity of a cancellable MG or time window with scheduling restrictions, the time available for scheduling may vary depending on whether a MG/scheduling restriction is skipped (see Figure 2.1-1).

Therefore, RAN2 could discuss whether the remaining time calculation/definition should be modified depending on whether a cancellable MG/scheduling restriction is skipped. E.g., at least one of the following solutions could be considered:

- In proximity of a MG/scheduling restriction, the remaining time calculation used for DSR triggering is modified to take into consideration that the data cannot be scheduled during the MG/scheduling restriction.

- Upon receiving an indication that an upcoming MG/scheduling restriction is cancelled, the UE updates the remaining time calculation when triggering DSR.

For example, the UE determines based on the current value of the remaining time and on the value of the configured remaining time threshold for DSR triggering, that a DSR will be triggered within an upcoming MG/scheduling restriction occasion. Then the UE modifies the remaining time calculation to ensure that a DSR is triggered prior to the start of the MG/scheduling restriction. Upon receiving an indication that the corresponding MG/scheduling restriction occasion is skipped/cancelled, the UE further updates the remaining time calculation for DSR triggering. In this way, the UE can guarantee DSR transmission is not unnecessarily delayed when a MG/scheduling restriction occurs without at the same time causing too early DSR reporting when a MG/scheduling restriction is cancelled.

Proposal 1: RAN2 discusses the impacts of cancellable MG/scheduling restriction on remaining time calculation/definition. The remaining time calculation for DSR triggering is dynamically updated to consider the status of a MG/scheduling restriction (skipped/not-skipped) so that a DSR can be transmitted when intended by configuration.

Figure 2.1-1: measurement gap skipping impact on remaining time

## 2.2	Impacts of gap/restriction skipping on DRX operation

The potential impacts on DRX from cancellable MG/scheduling restrictions were also discussed in RAN2#127bis.

According to current DRX procedure, if the UE is not in active time, the UE does monitor PDCCH even if a measurement gap and or scheduling restriction is indicated as skipped/cancelled. The inactivity timer might keep the UE awake, but usually it is not configured that long to cover offset plus the gap as otherwise it may impact the UE power consumption whenever a new transmission is scheduled as shown in figure 2.2-1.

Offset (e.g. 4ms)

DCI

skip command

Skipped gap, (e.g. 6ms)

Inactivity timer > 10ms to cover the skipped gap

Figure 2.2-1: inactivity timer long enough to cover potential skipped gap

Proposal 2: The UE shall consider the duration of a measurement gap or scheduling restriction period indicated to be skipped as active time without relying on configuring long enough inactivity timer.

Moreover, as the measurement configuration needs to take SSB occasions into account while OnDuration mainly concerns traffic patterns and QoS requirements, it is not possible to always align DRX and MG/SMTC configurations to make sure measurement occasions come at least an offset after OnDuration.

Figure 2.2-2: Scheduling restrictions due to RRM measurements overlapping with DRX OnDuration

If a MG occasion/scheduling restriction window overlaps with (or is too close to) the DRX OnDuration, there might be no opportunity during which the UE monitors PDCCH to receive a MG/scheduling restriction skip indication. This means, at the start of the OnDuration, the network cannot transmit a DCI skip indication to the UE to enable the transmission of any buffered XR data packet received when the UE was sleeping, since the UE is going to prioritize RRM measurements over PDCCH decoding. Therefore, the network risks violating the PDB for data buffered during the inactive time. This problem is illustrated in Figure 2-2-2: an XR frame arrives outside the UE active time and the overlap between the next measurement opportunity and the next OnDuration prevents the network from transmitting a DCI skip indication to cancel the measurement opportunity, and as the buffered XR data cannot be scheduled. An additional scheduling delay is introduced, and this may cause violating the PDB, especially if negligible PDB is left for possible retransmissions. Please note this equally applies to buffered XR data packets received while the UE is outside of active time prior to DRX OnDuration overlapping with a MG/scheduling restriction occasion, as well as to XR data packets that may arrive during the DRX OnDuration overlapping with a MG/scheduling restriction occasion.

Observation 1: When the measurement opportunity overlaps with (or is too close to) DRX OnDuration, the network may not be able to send DCI skip indication to enable e.g. scheduling of buffered data received when the UE was inactive.

A simple solution may consist in extending the DRX active time according to the timing of the measurement opportunity (and its scheduling restriction) to give the network the opportunity to transmit DCI skip indication. For example, as illustrated in Figure 2.2-3 (a), the OnDuration of the DRX cycle could extended “to the right” because the next window of scheduling restrictions overlaps with the OnDuration of the next DRX cycle and there has been no scheduling activity during the OnDuration. In this way there will be more time for the network to serve any XR frame that arrives before the beginning of the next window of scheduling restrictions when the UE may have already entered inactive state. Another alternative (b) consists in waking up the UE earlier than the start of the next OnDuration to monitor for potential DCI skip indication, thus extending the active time “on the left”.

Figure 2.2-3: Extension of DRX active time, where the UE extends the active time if DRX OnDuration overlaps with (or is too close to) windows with scheduling restrictions.

Proposal 3: RAN2 to evaluate rules for extending the UE active time when a measurement opportunity window overlaps with (or is too close to) the DRX OnDuration, e.g. by extending the OnDuration or by waking up the UE earlier than the start of the OnDuration for potential DCI skip command monitoring.

# 3	Conclusion

RAN2 impacts of RRM measurement gaps/restrictions related enhancements on DSR and DRX are discussed in this contribution with the following observation and proposals proposed:

Proposal 1: RAN2 discusses impacts of cancellable MG/scheduling restriction on remaining time calculation/definition. The remaining time calculation for DSR triggering is dynamically updated to consider the status of a MG/scheduling restriction (skipped/not-skipped) so that a DSR can be transmitted when intended by configuration.

Proposal 2: The UE shall consider the duration of a measurement gap or scheduling restriction period indicated to be skipped as active time without relying on configuring long enough inactivity timer.

Observation 1: When the measurement opportunity overlaps with (or is too close to) DRX OnDuration, the network may not be able to send DCI skip indication to enable e.g. scheduling of buffered data received when the UE was inactive.

Proposal 3: RAN2 to evaluate rules for extending the UE active time when a measurement opportunity window overlaps with (or is too close to) the DRX OnDuration, e.g. by extending the OnDuration or by waking up the UE earlier than the start of the OnDuration for potential DCI skip command monitoring.