3GPP TSG-RAN WG2 Meeting #128	R2-2410239

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

The revised Rel-19 NES WID has been approved with the following objective.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

This paper will outline our further views on on-demand SIB1 for UEs in RRC idle/inactive mode.

Discussion

OD-SIB1 request and failure

RAN2#126 meeting reaches an agreement that the UE considers the OD-SIB1 cell as barred once the RA procedure related to the OD-SIB1 request fails. It remains undecided whether this behaviour should be specified or left to UE implementation.

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

RAN2#127bis reuses the rule of the existing Msg1-based OD-SIB procedure to define the RACH failure related to the OD-SIB1 request. Specifically, the UE determines that a RACH failure has occurred when it reaches the maximum preamble transmission number. Consequently, the MAC will indicate the RACH failure related to the OD-SIB1 request to RRC.

Agreements on OD-SIB1

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

Some companies believe the UE should immediately consider a cell as barred once the UE’s upper layer receives a failure indication. However, we believe the UE may still have an opportunity to acquire the SIB1 from this OD-SIB1 cell if the SIB1 is about to broadcast or is currently being broadcasted by the request from other NES UE(s). In this case, the UE would not need to bar this OD-SIB1 cell, as it can still acquire the SIB1. Therefore, we do not need to mandatorily require the UE to bar the cell, instead, we suggest allowing some flexibility for the UE implementation in this regard.

- Even if an RA procedure fails related to the OD-SIB1 request from one UE, the SIB1 of the OD-SIB1 cell may be about to broadcast or be currently broadcasting due to the request from other NES UE(s).

Leave it to the UE implementation to determine whether the associated OD-SIB1 cell is considered barred in the case that the RA procedure fails related to the UE’s OD-SIB1 request, i.e. the UE may or may not consider this OD-SIB1 cell is barred.

RAN2#127bis also made the following conclusion.

Agreements on OD-SIB1

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

We understand that implementing a checking mechanism would help avoid unnecessary OD-SIB1 request triggering and reduce signalling overhead when the SIB1 of the specific OD-SIB1 cell is currently being broadcasted. We suggest leaving it to the UE implementation how to check if the SIB1 is being broadcasted for the reasons below.

- It is challenging to include the broadcasting status of SIB1 in the MIB of the OD-SIB1 cell due to the limited reserved bit in MIB and the tight dependence on RAN1.

- It is not feasible for Cell A to indicate the broadcasting status of SIB1 of the OD-SIB1 cell, because of the frequent and dynamic exchanges between Cell A and the OD-SIB1 cell, along with the un-synchronization issues resulting from exchange delay.

- It is difficult to configure an appropriate checking duration for each UE, as the gNB cannot predict when an individual UE will trigger the OD-SIB1 request. The uncertainty makes it challenging to set a suitable duration to avoid sending the OD-SIB1 request.

Leave it to the UE implementation how to verify if the cell's SIB1 is currently being broadcasted or provided on demand before requesting SIB1 of that cell.

Prevent the legacy UE from accessing the OD-SIB1 cell

RAN2 previously discussed the impact on the legacy UE and concluded to prevent or deprioritise the legacy UE camping on Cell A from attempting to switch to the OD-SIB1 cell.

Agreements on on-demand SIB1 in RAN2#126

Legacy UE impact

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Agreements on OD-SIB1 in RAN2#127bis

4.	NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

The intention is to implement individual cell reselection operations for both legacy UE and NES UE. This will involve deviating from the current design of broadcasting unified reselection parameters for all UEs camping on this cell. Otherwise,

- The NES UE will find it hard to reselect the OD-SIB1 cell if the frequency priority of the OD-SIB1 cell is lower than that of Cell A. Or,

- The legacy UE will waste power and delay the reselection to a suitable cell if the frequency priority of the OD-SIB1 cell is higher than that of Cell A.

In addition to using an individual excluded cell list, providing different cell reselection priorities via SIB is another way to implement the differentiated handling for legacy UE and NES UEs, especially for the case that a specific frequency is dedicated for the OD-SIB1 cells. This approach allows the legacy UE to deprioritize the NES frequency while enabling the NES UE to handle the NES frequency normally. As a result, the legacy UE does not need to perform measurement when the quality of the serving cell is good enough. One may argue that the dedicated RRC signalling could provide different NES frequency priorities to the NES UE, but the frequency priorities provided via dedicated signalling may be deleted or preserved in some cases, e.g., T320 expire or the UE is in camped on any cell state.

To implement differentiated cell reselection handling for the legacy UE and NES UE, the network can configure separate cell reselection frequency priority in SIB for legacy and R19 NES UEs.

The WUS configuration provision

In RAN2#126, it has been concluded that the RRC release message can be an option for WUS configuration provision.

7. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

In legacy, the RRC release message is used to provide the resource configuration used for the idle or inactive UEs for the cell re-selection procedure. The cell reselection-related information remains valid until a specific timer expires. After that, the UE applies the cell reselection-related information that has been broadcasted. This principle can also be applied to the UL WUS configuration provided via the RRC release message, which helps reduce the signalling overhead caused by periodic SIB transmission. In this case, the UE applies the UL WUS configuration provided via the RRC release message before a proper timer expires and applies the UL WUS configuration provided via SIB later.

The UL WUS configuration can also be provided via the RRCRelease message.

Conclusion

We have the following observations:

Observation 1	Even if an RA procedure fails related to the OD-SIB1 request from one UE, the SIB1 of the OD-SIB1 cell may be about to broadcast or be currently broadcasting due to the request from other NES UE(s).

We have the following proposals:

Proposal 1	Leave it to the UE implementation to determine whether the associated OD-SIB1 cell is considered barred in the case that the RA procedure fails related to the UE’s OD-SIB1 request, i.e. the UE may or may not consider this OD-SIB1 cell is barred.

Proposal 2	Leave it to the UE implementation how to verify if the cell's SIB1 is currently being broadcasted or provided on demand before requesting SIB1 of that cell.

Proposal 3	To implement differentiated cell reselection handling for the legacy UE and NES UE, the network can configure separate cell reselection frequency priority in SIB for legacy and R19 NES UEs.

Proposal 4	The UL WUS configuration can also be provided via the RRCRelease message.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR