3GPP TSG-RAN WG2 Meeting #128	R2-2410241

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Operation of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved. SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where a separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency, especially in wide-area macro networks where many UEs are in power-limited conditions in the UL direction. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in  RP-241614.

RAN2 start discussing on SBFD random access procedures and in RAN2#127bis, and the following agreemenst are made:

Agreements RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In this contribution, we further discuss different aspects of the SBFD random access operation.

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. As mentioned in section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

In section 3, we present the various aspects of random access in RRC Connected mode, and in RRC Idle/Inactive mode, and the conclusion is provided in section 4.

# Various Aspects of SBFD random access

In this section, we discuss the RAN2 aspects of the SBFD random access procedure for RRC_CONNECTED and IDLE/INACTIVE operations.  RAN1 agreements on SBFD random access is listed in Appendix 1.

## PRACH resource configuration for RRC_CONNECTED UEs

To enable random access for SBFD symbols in SBFD-aware UEs, the gNB must set up a new set of PRACH resources which includes RO configuration, SSB-RO mapping, the random access configuration table, and PRACH repetition.

In RAN1#118bis, the following agreements are made on the PRACH resource configuration:

Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - FFS: Possible modifications

It seems straightforward to reinterpret that for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which fall outside the UL subband after applying offset are considered as invalid ROs for SBFD UEs.

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs that fall outside the UL subband after applying offset are considered invalid ROs for SBFD UEs.

On the FFS part of option 2, since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough.

Proposal 2 For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters. Only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

## RACH Occasion selection and prioritization

For both RACH configuration Option 1 and Option 2 agreed in RAN1, SBFD-aware UEs will have the possibility of transmitting RACH on legacy ROs and SBFD ROs, while legacy UEs would only be capable of using legacy ROs.

In the last meeting, we agreed on the UE selection between the legacy and the additional ROs based on certain specified/configured conditions/prioritizations.

RAN2#127bis - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

In the following we present some of these conditions:

RO selection based on load balancing and coverage [Option 1 and Option 2]:

For load balancing purposes, it is beneficial that SBFD-aware UEs prioritize as much as possible RACH transmissions on SBFD ROs and avoid consuming the quite-limited capacity of legacy RACH occasions. Considering that UE-to-UE CLI may only be a problem in SBFD ROs but not in legacy ROs, and may be especially critical in cell-edge areas (where the UL transmission power is high, while the DL received signal power is low), the selection between legacy ROs and SBFD ROs may also be based UE’s coverage conditions. For instance, a cell-centre UEs transmit RACH on SBFD ROs, while a cell-edge SBFD-aware UE uses ROs belonging to the legacy RACH configuration.

Observation 1: For CBRA and CFRA, from the SBFD-aware UE perspective and to have better load balancing across different RACH resources,  SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions.

Proposal 3: For RACH configuration Option 1 and Option 2, an SBFD aware UE selects RO type for Msg1 transmission based on UE’s coverage condition. Details are FFS.

RO selection based on on PRACH preamble format [Option 2]:

Due to the support of two PRACH formats with different lengths, SBFD-aware UEs have the possibility to select a suitable preamble format based on its location. In Figure 2, we show an example of two SBFD-aware UEs. One is in cell range 1, and other is in cell range 2. For UE1 (cell range 1) to access the NW, the short-length PRACH format is better adapted due to its location near the NW. On the contrary, UE2 is in cell range 2, requiring a long-length PRACH format to be able to access the NW. Due to the support of two PCIs in Option 2, these two UEs can select the best PRACH format types based on their location. However, these UEs are in initial access and have no information or indication about their location in reference to the NW. One way to overcome this issue is to measure the SS-RSRP, e.g., in Figure 2, UE1 selects the best SSB (SSB#1) with the best received power, and, since the SS-RSRP is higher than a defined SS-RSRP threshold to switch to the long-length PRACH format, UE1 uses ROs configured by the legacy RACH configuration. Following, UE2 selects the best SSB (SSB#6), with a received power lower than the defined SS-RSRP threshold; therefore, it decides to enable the additional ROs configured with the additional RACH configuration.

Proposal 4:  For Option 2, in case two RACH configurations with two different PRACH formats lengths, SBFD aware UEs selects the best PRACH format length configured by the legacy or the additional RACH configuration for the initial access based at least on SSB-RSRP threshold(s). Details FFS.

Figure 2 SBFD aware UEs select the best PRACH preamble format based on their location regarding the NW.

## RA-RNTI collision in SBFD

For the additional RACH configuration in option 2, designed for SBFD-aware UEs, a challenge is avoiding RA-RNTI confusion caused by having two sets of RACH resources. The RA-RNTI is used by a UE to receive the Random Access Response (RAR) once the PRACH is transmitted. Different RACH occasions within a system frame are assigned unique RA-RNTI values to ensure that the RAR for a given PRACH transmission is directed only to the UE that used that specific RACH occasion. If two RACH occasions share the same RA-RNTI value, there is a risk that two UEs using different RACH occasions could incorrectly act on the same RAR message.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations may overlap with the legacy ROs in flexible symbols.

In this scenario, collision can be avoided by defining a new rule for indexing of f_id when there is overlapping without modifying the existing RA-RNTI formula. Therefor we propose:

Proposal 5: When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

## Early Indication for SBFD RA procedure

In RAN2, it is agreed that if an additional RO is selected by an SBFD-aware UE, early identification via Msg1 is possible from the network’s point of view for this UE without any specification impact. It is also agreed that SBFD will not be introduced as a new feature combination in the current PRACH preamble partitioning framework.

For the Option 1 PRACH configuration, it is possible that a legacy RO can be selected by both SBFD UEs and legacy UEs for Msg1 transmission. In this scenario, the network will not be able to differentiate between the SBFD UE and legacy UEs.

Observation 3: It is not possible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the legacy RACH resources used by the UEs.

The question now is, given that early indication through preamble partitioning is not supported, the benefit of early indication via Msg3 for subsequent Msg5 scheduling is limited. The reduction in latency would be minimal, as the UE will ultimately report the capability to the gNB in Msg5.

Proposal 6: RAN2 to discuss the advantage of Msg-3 based early indication.

# Conclusion

Observation 1: For CBRA and CFRA, from the SBFD-aware UE perspective and to have better load balancing across different RACH resources,  SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations may overlap with the legacy ROs in flexible symbols.

Observation 3: It is not possible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the legacy RACH resources used by the UEs.

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which falls outside the UL subband after applying offset  is considered as invalids ROs for SBFD UEs

Proposal 2: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters. Only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

Proposal 3: For RACH configuration Option 1 and Option 2, an SBFD aware UE selects RO type for Msg1 transmission based on UE’s coverage condition. Details are FFS.

Proposal 4:  For Option 2, in case two RACH configurations with two different PRACH formats lengths, SBFD aware UEs selects the best PRACH format length configured by the legacy or the additional RACH configuration for the initial access based at least on SSB-RSRP threshold(s). Details FFS.

Proposal 5: When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

Proposal 6: RAN2 to discuss the advantage of   Msg-3 based early indication.