3GPP TSG-RAN WG2 Meeting #128	R2-2410255

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SSB SCell operation

WID/SID:	Netw_Energy_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we will discuss the aspects related to the on-demand SSB SCell operation which is one of the objectives of the R19-network energy savings work item

# 2	On-demand SSB SCell Operation

## 2.1	Scenarios and signalling

RAN1 has made following agreements regarding on-demand SSB:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE FFS: Whether the RRC is newly introduced or existing RRC is reused

Based on the agreements in RAN1#117, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission. Our current understanding is these signaling are applicable to Scenario#2, Scenario#2A as depicted in the figure 1 below, where:

- For Scenario#2: the on-demand SSB transmission can be indicated by gNB at the time instance T1 (when UE receives SCell configuration via RRC message) or during the time period between T1 and T2. From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

- For Scenario#2A: the on-demand SSB transmission can be indicated by gNB at the time instance T2 (when UE receives SCell activation via MAC CE command). From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

For the RAN2 discussion we prefer to consider Scenario#2 and Scenario#2A for both Case#1(without always-on SSB) and Case#2(with always-on SSB).

Figure 1: Scenarios and indication for on-demand SSB transmission.

RAN2 could possibly start the discussion on how the on-demand SSB configurations that can be provided to the UE to support these scenarios.

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

During the RAN1#118-bis, it has also been agreed to provide the on-demand SSB configuration via RRC signaling and may include at least the parameters below

- Frequency of the on-demand SSB

- SFN offset, half frame index, SSB positions within an on-demand SSB burst

- Periodicity of the on-demand SSB

- If number N of on-demand SSB bursts may be configured is under FFS in RAN1

Also, there are more than one SSB transmission patterns supported in RAN1 as listed below

- Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

- Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

- Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Depending on the SSB pattern to be transmitted, we believe the configurations may have to include additional parameters for example,

- The periodicity parameter is required for all the on-demand transmission options.

- Number of SSB bursts is required for the Option 3

- time duration for which on-demand SSB are transmitted for Option 2 and Option 4

- alternate or a second periodicity for Option 4.

- and may also require a corresponding SMTC configuration.

In general, on-demand SSB configuration may be provided via the Serving Cell Configuration. As per our understanding, the set of parameters required for the on-demand SSB operation may be kept minimal. As indicated in Figure 3 two options may be considered.

- the minimal set consisting of mandatory parameters for OD-SSB operation that can work on top of the legacy SSB configuration.

- the full set of parameters for OD-SSB operation that would replace the legacy SSB configuration.

Figure 2: On-demand SSB configuration structure

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration. Discuss the possibility of defining a minimal set for OD-SSB operation that may work on top of the legacy configuration or a full set of parameter for OD-SSB operation that replace the legacy configuration.

It may be likely needed to have multiple of OD-SSB configuration per SCell – or at least signaling should be taken so that this is the case.

SSB-Pattern-Config :: SEQUENCE {

pattern-id INTEGER (1..M)

ssb-Periodicity-r19 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

activeDuration ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640} OPTIONAL,

numberOfBurst INTEGER (1..N)

ssb-Frequency ARFCN-ValueNR,

subcarrierSpacing SubcarrierSpacing,

ssb-PositionsInBurst CHOICE {

shortBitmap BIT STRING (SIZE (4)),

medium Bitmap BIT STRING (SIZE (8)),

longBitmap BIT STRING (SIZE (64))

} OPTIONAL,

ss-PBCH-BlockPower INTEGER (-60..50) OPTIONAL,

}

It is worth noting that the objective for SSB adaptation also considers the RRC configuration enhancements and it may be possible that the on-demand SSB configuration framework is also applicable to the SSB adaptation. RAN2 may take this into account while discussing the RRC configurations for on.demand SSB.

From previous RAN2 discussions, we have the following agreement:

Agreements on OD-SSB Scell (RAN2 #127) OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Agreements on OD-SSB Scell (RAN2 #127-bis) - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE

Further for MAC CE design, RAN1 has agreed to support OD-SSB activation before SCell activation as well as upon SCell activation and also the possibility of configuring multiple configurations at least containing periodicity of the OD-SSB, sfn-offset, half-frame-index, but whether also for any other parameters is still FFS.

From RAN2 signalling point of view, the simplest way would be to indicate in the MAC CE the SSB configuration IDs configured in RRC and the RRC configuration can accommodate any parameter that RAN1 concludes to support multiple candidate values.

Proposal 2: The different periodicities are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 3: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

It should be also noted that in AI 8.5.4 for SSB adaptation in time domain, quite similar behaviour can be considered as for OD-SSB i.e. by merely just changing the periodicity of the SSB burst and not fully turning on/off the SSB. From RAN2 perspective these two features could be rather same in the end i.e., different SSB periodicity is activated with MAC CE thus it might be good idea to consider these two features simultaneously when designing MAC CE.

Proposal 4: Take into account in MAC CE design for OD-SSB also SSB adaptation in time domain feature for activating different periodicities

On L3 measurements based on on-demand SSB, RAN1 thinks that RAN2 should lead the discussions. Some agreements were made during RAN2#127 meeting and RAN2 is required to study how UE performs L3 measurements for OD-SSB.

Agreements on OD-SSB Scell (RAN2 #127) Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration

Our understanding is that on-demand SSB based L3 measurements and reporting may depend on the on-demand SSB pattern.

The MeasConfig framework, shown in Figure 3, configures the MeasObject. This object defines the SSB frequency to measure and the timing for the measurement is provided by the SMTC1.

For case#2, when always-on SSB and on-demand SSB are transmitted on the same frequency, the UE measures ssbFrequency in SMTC1 as per the legacy behaviour. When on-demand SSB is activated via MAC-CE, the SMTC associated with the on-demand SSB configuration may be used for the measurement.

For case#1, if the MeasObject is configured, as per the legacy behaviour, it should be ensured that RAN2 agreement not measuring non activated SSBs is achieved in the signaling as well. Upon activation of on-demand SSB via MAC-CE, the new SMTC may be applied.

Figure 3: MeasConfig framework

This requires to align the SMTC window with the on-demand SSB transmission.. The SMTC configuration may be explicitly configured for each on-demand SSB configuration or derived based on the on-demand SSB periodicity.

Figure 4: on-demand SSB and SMTC

Proposal 5: Consider dedicated SMTC configuration for each on-demand SSB configuration.

Proposal 6: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

And as we discuss in our R2-24010257 for SSB adaptation there seems to be lots of commonalities between two features especially in RAN2. Thus it seems logical that whenever RAN2 makes designs for MAC/RRC they should be done by jointly taking both objectives into account to avoid unnecessary complexity/duplication of behaviour in RAN specifications.

Proposal 7: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

During the RAN1#118 meeting it has been agreed that L1 measurement based on on-demand SSB may be supported. The L1 measurements are configured using the CSI-reporting framework where the resources to be measured are included. The UE may be configured with CSI-ResourceConfig that contains,

- A list of NZP CSI-RS resource set and/or SSB resource set provided by nzp-CSI-RS-ResourceSetList and/or csi-SSB-ResourceSetList

- Or a list of CSI-IM resource set provided by csi-IM-ResourceSetList

The SSB resources are provided as the SSB-Index that maps to a SSB configuration.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17        SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF ServingAdditionalPCIIndex-r17  OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

It is uncertain whether UE would measure single SSB(e.g always-on) or multiple SSBs(on-demand SSB and always-on SSB) when on-demand SSB based L1 measurements enabled. This could affect the current CSI reporting framework and needs to be discussed in RAN1 before RAN2 can consider signaling enhancements.

Proposal 8: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.

# 3	Conclusion

Observation 1: During the RAN1#117 meeting, both RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration. Discuss the possibility of defining a minimal set for OD-SSB operation that may work on top of the legacy configuration or a full set of parameter for OD-SSB operation that replace the legacy configuration.

Proposal 2: The different periodicities are configured as different OD-SSB configurations in RRC and MAC indicates which OD-SSB configuration ID to be activated, which is forward compatible if other parameters also introduced in RAN1.

Proposal 3: The new MAC-CE for OD-SSB includes a bitmap for the SCell(s) with OD-SSB to be activated and the indication of the activated SSB configuration ID for each SCell.

Proposal 4: Take into account in MAC CE design for OD-SSB also SSB adaptation in time domain feature for activating different periodicities

Proposal 5: Consider dedicated SMTC configuration for each on-demand SSB configuration.

Proposal 6: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

Proposal 7: When designing on-demand SSB and SSB adaptation activation MAC CE/RRC signaling they should be done jointly in RAN2

Proposal 8: RAN2 to wait for RAN1 decision in the signaling enhancement needed for the CSI-Reporting framework.