3GPP TSG-RAN WG2 Meeting #128	R2-24100256

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

In RAN2#127bis, the following agreements were made:

Agreements on OD-SIB1

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2 – For information only

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 3	Access restriction for legacy UEs and NES UE behavior

RAN2#127bis agreed about the following:

1. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. 2. NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

One will set ssb-SubcarrierOffset (K_SSB) value for NES cell with OD-SIB1 operation considering latest RAN1 agreement:

- 23< K_SSB <31 for FR1 and 11< K_SSB <15 for FR2

With these values the cell is referred as non-Cell defining SSB. However, 38.300 specifies the following for cell selection and cell reselection for NCD-SSB:

Cell selection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4):

Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

This mean legacy UE will not select or re-select a NES with OD-SIB1 operation if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.  But it is not fully clear if all legacy UEs are actually implemented like this in real life so that should be verified before we can progress much on this . Thus it would be good if such a discussion would happen first before we actually can continue. This topic seems to be under discussion in this RAN WG meeting round in RAN1 thus it might be good idea to wait for them to conclude before RAN2 continues discussing details on determing legacy UE handling in NES cells

Proposal 1: RAN2 should wait RAN1 to progress their understanding of legacy UE handling of 23< K_SSB <31 for FR1 and 11< K_SSB <15 for FR2

In theory legacy UEs should not be camping on these kind of cells but there are discussions ongoing in RAN1 if this is actually the case:

Observation 1: For a cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell

But NES UE naturally should be able to camp and it can be distinguished by UE having acquired SIB1 request configuration earlier.

| UE operation when trying to camp on Cell indicating K_SSB > 23 for FR1 and K_SSB > 11 for FR2 |  |  |  |
| --- | --- | --- | --- |
| UE type | Cell selection | Cell re-selection | Barring |
| Legacy UE and NES UE without WUS configuration | N/A | N/A | May treat the cell as barred as no way to acquire SIB1. For K_SSB <30 for FR1 and K_SSB <14 for FR2, UE determine the nearest SS/PBCH based on a GSCN offset (see 38.213) associated to CD-SSB For K_SSB = 30 for FR1 and K_SSB = 14 for FR2, UE determines that the SS/PBCH associated to the cell is not carrying information about SIB1. |
| NES UE with WUS configuration | N/A | YES | NO |

Proposal 2: If NES UE has SIB1 request configuration of a cell, NES UE may camp on a cell with NCD-SSB located on the synchronization raster and acquire SIB1 with on-demand fashion even if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.

Observation 2: With 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 for FR2, UE is redirected to another SS/PBCH to acquire SIB1, and it shall not assume for UE to be unable to acquire SIB1. Also in TS 38.304, it stated that the UE “may” exclude the barred cell, meaning that it does not mandate legacy UEs to bar a cell with these values of K_SSB.

Observation 3: For a NES cell with K_SSB = 30 for FR1 and K_SSB = 14 for FR2, legacy UE can determine that NES cell is with SIB1-less operation and UE may treat the cell as barred for 300s.

Naturally same would occur for NES UE if it has not acquired SIB1 request configuration prior trying to camp on a cell indicating NCD-SSB. But after a while when UE acquire SIB1 request configuration it does not make much sense to bar the cell anymore.

Proposal 3: If NES UE has SIB1 request configuration of a NES cell and NES UE already barred the celldue to being unable to acquire SIB1, NES UE should treat the NES cell as not barred anymore.

UE camping on CellA and receiving via SIB, e.g. SIB3 can be informed via legacy reselection control parameter e.g. intra/interFreqExcludedCellList or separate reselection priorities to prevent legacy UEs to re-select to NES cells (OD-SIB1 cells). To avoid preventing NES UEs to re-select NES cells, one would need to allow NES UE to consider reselections to OD-SIB1 cells

Proposal 4: NES UEs should be allowed to reselect to cells that are prevented from legacy UEs with both excluded cell list and reselection priorities

# 4	Broadcast status for OD-SIB1

RAN2#127bis agreed about the following:

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

RAN1#118bis agreed about the following:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

-	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

-	UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

NES cell is a NCD-SSB cell as RAN1 agreed that K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster. Based on RAN1 agreement, NES cell identification by NES UE is not possible based on PBCH/MIB only of a NES cell.

Observation 2: Legacy UE and NES UE treat the NES cell as NCD-SSB.

NES UE identification of NES cell:

- If NES UE detects the NES cell before camping on Cell A, legacy UEs and NES UEs behaves the same way with respect to NES cell.

- If NES UE camps on cell A firstly, NES UE can acquire WUS configuration carried by SIBx of Cell A. NES UE can identify the cell as OD-SIB1 cell based on WUS configuration received from Cell A.

Observation 4: based on RAN1 #127bis agreement, a NES UE cannot identify a cell with on-demand SIB1 operation based only on PBCH/MIB of the cell.

SIB1 broadcasted or provided on-demand:

The following RAN2 agreement has been made in last meeting:

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

A cell for which SIB1 request configuration is available may be included in the WUS configuration A NES UE receiving the WUS configuration from cell A, may assume that the NES is with on-demand SIB1 operation. Based on above RAN2 agreement, a NES cell for which SIB1 request is available:

- Status 1: May periodically broadcast SIB1.

- Status 2: May provide SIB1 on-demand.

To allow a NES UE which has SIB1 request configuration of a NES cell, to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell, two options can be used:

- Option 1: The PBCH/MIB of the NES cell needs to be reflecting the SIB1 status (like in legacy). A cell broadcasting SIB1, shall have a K_SSB <= 23 for FR1 and K_SSB <=11 for FR2 if it is transmitted on sync raster. In addition, the PBCH/MIB payload shall contain a valid pdcch-ConfigSIB1 pointing to type0-PDCCH.

  - Observation 5: This means that a NES cell when switching to SIB1 broadcasting mode, shall switch from a NCD-SSB to a CD-SSB.

- Option 2: The PBCH/MIB contents of the NES cell are kept unchanged (i.e. NCD-SSB is indicated always )whatever the cell is broadcasting SIB1 or providing SIB1 on-demand, i.e., K_SSB does not change, and NES cell is associated a NCD-SSB. UE which has SIB1 request configuration, may try to perform blind monitoring of SIB1 may be broadcast at least based on WUS configuration received from Cell A.

  - Observation 6: This would basically makes legacy UEs not being able to camp on cell even if it broadcasts SIB1.

Option 1: For a NES cell switching from a NCD-SSB to a CD-SSB, legacy UEs can select/re-select the cell unless legacy UE already barred this cell. This option mandates NES UE to re-acquire MIB before trying to acquire SIB1. This does not seem to be issue as UE acquire MIB anyway whenever selecting/reselecting to cell – only case this could have some impact is the scenario when UE needs to reacquire SIB1 while camping on a cell.

Option 2 is relying on a NES UE which has SIB1 request configuration of a NES cell, to monitor type0 PDCCH occasions basically in blind fashion as cell may or may not be transmitting SIB1.

In our view whenever cell is broadcasting a SIB1 it should be possible to legacy UEs to camp on such a cell. It would seem pretty suboptimal design that NW is broadcasting SIB1 but UEs cannot camp on the cell – especially as we have quite a big population of legacy UEs i.e. we would not consider option 2 as feasible alternative.

Proposal 5: One needs to design SIB1 sending status so that whenever cell is broadcasting SIB1 it is possible for legacy/non-NES UE to select/re-select a NES cell

This could be achieved by agreeing following:

Proposal 6 : NES UE with SIB1 request configuration of a cell assumes that the cell, with SSB containing K_SSB > 23 for FR1 and K_SSB >11 for FR2, will acquire SIB1 via on-demand SIB1 procedure

Proposal 7: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy

For NES UEs reselecting to NES cell (cell for which UE has SIB1 request configuration) it should not initiate SIB1 requests if the cell is currently doing regular SIB1 broadcast (as in legacy). NW may need to change SIB1 broadcast status abruptly due to load reasons and the updated SIB1 request information has not been yet synchronized to all cell As (cell providing SIB1 requst configuration for NES cell). Thus it is imperative and logical that of course in this kind of situation UE behaves as the legacy UE

Proposal 8: NES UE should acquire broadcast SIB1 similarly as legacy UEs if the SIB1 is being broadcast in the cell

One could consider optimizing unnecessary SIB1 requests by NES UEs e.g. if cell is not broadcasting SIB1 but providing them in different manner for other UEs who have already requested SIB1, the new UE should not unnecessarily send SIB1 requests in the cell:

Proposal 8: RAN2 to consider that if UE has SIB1 request configuration of a cell, UE needs to monitor if SIB1 is currently being transmitted for that cell,e.g. requested by other UEs, before requesting SIB1 of that cell..

# 5	RAR response handling

RAN1 consider dedicated PRACH resources for SIB1 request. Dedicated PRACH resources are not referring to UE-specific PRACH resources but rather PRACH resources dedicated to on-demand SIB1 request.

For PRACH resources for UL WUS, RAN2 should consider on-demand SI procedure as starting point. For on-demand SI, the network may configure the UE with dedicated RA resources in SI-RequestConfig within SI-Scheduling Info IE in SIB1. For on-demand SIB1, the dedicated RA resources could be part of the WUS configuration. Any of time, frequency and/or PRACH preamble resources may be used as dedicated PRACH resources to allow full flexibility to the network.

If common PRACH preamble is used for all UEs requesting on-demand SIB1 operation, this can allow a RAR response that can be interpreted/decoded by all UEs. Thus, a UE intended to transmit UL WUS for requesting SIB1, can monitor if any RAR response is sent by network to check if on-demand SIB1 is already requested by other UEs. However, if dedicated ROs, time/frequency resources are considered as dedicated PRACH resources and it is up to UE to select a preamble, RAR response is a dedicated response that can be decoded only by UE requesting on-demand SIB1.

Observation 7: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

It might be difficult though with current RAR monitoring based on RA-RNTI of the PRACH occasions as the UE would need to monitor all the potential RA-RNTIs of PRACH for OD-SI before WUS transmission. Common RA-RNTI would be required to enable this. Common RA-RNTI would also help network scheduling if multiple UEs are requesting SIB1 simultaneously. Thus we see it is beneficial to design common RA-RNTI.

Observation 8: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

Proposal 9:  It should be possible to configure common RNTI for RAR monitoring of OD-SIB1 requests

# 6	RACH failure

In RAN2#127bis, we had following discussion:

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

SI request procedureis  initiated like this (for normal SIBs):

From RRC (see the NOTE)  – It is left to UE implementation when it retries SI request in case of RACH failure:

It seems valid alternative to do the same for SIB1 request procedure. And if SIB1 request is deemed as failure by UE then of course UE may treat cell as barred as in legacy.

Proposal 10: In case of RACH failure for SIB1 request it is left to UE implementation to decide when SIB1 request fails or to retry SIB1

# 7	Provisioning of UL WUS configuration

RAN2 agreed that multiple Cell A can be associated to one NES cell. In addition, one Cell A can assist multiple NES cells. However, there is no guarantee that the deployment ensure:

- Full overlapping between NES cell and Cell A, e.g. NES cell coverage is within Cell A coverage.

- All neighbor cells of a NES cell can be assumed as Cell A that provision UEs with WUS configuration.

A UE within NES cell coverage, make measurements of this cell based on SS/PBCH. Based on these measurements the NES cell maybe considered as strongest candidate for reselection. What will happen if UE reselects to another cell X not providing WUS configuration for cell A and after a while UE reselects to cell A? So it seems quite likely that unless NW provides WUS configuration in each and every cell with overlapping coverage with NES cell there can be cases that UE does not have WUS configuration unless we allow UE to apply WUS configuration even if has been acquired some time ago. For simplicity it may be best just to say that UE will only consider WUS configuration valid in direct succeeding cell selection/reselection from cell A (the cell where UE acquired WUS configuration). And in order to diminish possibility to provide configuration in multiple cells NW can optimize reselection parameters so that it maximises chances for first reselecting to Cell A and then to NES cell. Basic principle for allowing these NES specific reselection handling was already agreed in RAN2#127bis:

Proposal 11: UE considers WUS configuration only valid in directly succeeding cell selection/reselection from cell A (the cell where UE acquired WUS configuration).

# 8	Conclusions:

Proposal 1: RAN2 should wait RAN1 to progress their understanding of legacy UE handling of 23< K_SSB <31 for FR1 and 11< K_SSB <15 for FR2

Observation 1: For a cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell

Proposal 2: If NES UE has SIB1 request configuration of a cell, NES UE may camp on a cell with NCD-SSB located on the synchronization raster and acquire SIB1 with on-demand fashion even if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.

Observation 2: With 23 < K_SSB < 30 for FR1 and 11 < K_SSB < 14 for FR2, UE is redirected to another SS/PBCH to acquire SIB1, and it shall not assume for UE to be unable to acquire SIB1. Also in TS 38.304, it stated that the UE “may” exclude the barred cell, meaning that it does not mandate legacy UEs to bar a cell with these values of K_SSB.

Observation 3: For a NES cell with K_SSB = 30 for FR1 and K_SSB = 14 for FR2, legacy UE can determine that NES cell is with SIB1-less operation and UE may treat the cell as barred for 300s.

Proposal 3: If NES UE has SIB1 request configuration of a NES cell and NES UE already barred the celldue to being unable to acquire SIB1, NES UE should treat the NES cell as not barred anymore.

Proposal 4: NES UEs should be allowed to reselect to cells that are prevented from legacy UEs with both excluded cell list and reselection priorities

Observation 2: Legacy UE and NES UE treat the NES cell as NCD-SSB.

Observation 4: based on RAN1 #127bis agreement, a NES UE cannot identify a cell with on-demand SIB1 operation based only on PBCH/MIB of the cell.

  - Observation 5: This means that a NES cell when switching to SIB1 broadcasting mode, shall switch from a NCD-SSB to a CD-SSB.

  - Observation 6: This would basically makes legacy UEs not being able to camp on cell even if it broadcasts SIB1.

Proposal 5: One needs to design SIB1 sending status so that whenever cell is broadcasting SIB1 it is possible for legacy/non-NES UE to select/re-select a NES cell

Proposal 6 : NES UE with SIB1 request configuration of a cell assumes that the cell, with SSB containing K_SSB > 23 for FR1 and K_SSB >11 for FR2, will acquire SIB1 via on-demand SIB1 procedure

Proposal 7: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy

Proposal 8: NES UE should acquire broadcast SIB1 similarly as legacy UEs if the SIB1 is being broadcast in the cell

Proposal 8: RAN2 to consider that if UE has SIB1 request configuration of a cell, UE needs to monitor if SIB1 is currently being transmitted for that cell,e.g. requested by other UEs, before requesting SIB1 of that cell..

Observation 7: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS

Observation 8: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

Proposal 9:  It should be possible to configure common RNTI for RAR monitoring of OD-SIB1 requests

Proposal 10: In case of RACH failure for SIB1 request it is left to UE implementation to decide when SIB1 request fails or to retry SIB1

Proposal 11: UE considers WUS configuration only valid in directly succeeding cell selection/reselection from cell A (the cell where UE acquired WUS configuration).