3GPP TSG-RAN WG2 Meeting #128	R2-24010260

Orlando, USA, 18 – 22 November 2024

Agenda item:	7.8.1

Source:	Nokia

Title:	carrierBandwidth configuration for less than 5Mhz

WID/SID:	NR_FR1_lessthan_5MHz_BW-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In last RAN2 meeting we sent LS to RAN4 regarding carrierBandWidth configuration in R2-2409409. In order to hopefully find common understanding how UE handles carrierBandwidth in case less than 5Mhz operation we would like to provide status of RAN4 specification.

# 2	Discussion

In RAN4 it has been specified for band n100 following:

- PCell or SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

- PCell or SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

Therefore, UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of 15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Thus it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Observation 1: It is possible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

The LS exchange was initiated from the paper R2-2409055 in which it was stated:

As noted for observation 1 – in RAN4 specification it is already clear that if 15/25PRB are indicated in carrierBandWidth then UE can determine while operating on band n100 that instead 12/20PRB is applicable. Thus we propose

Proposal 1: Keep existing LessThan5Mhz condition

# 3	Conclusion

Observation 1: It is possible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

Proposal 1: Keep existing LessThan5Mhz condition