3GPP TSG-RAN WG2 Meeting #128	R2-2410271

Orlando, USA, Nov. 18th – 22nd, 2024                         Revision of R2-2408304

Agenda item:		8.9.3

Source:		Lenovo

Title:		EDT for uplink capacity enhancement in NTN

Document for:		Discussion

- Introduction

The latest RAN2#127bis meeting made the following agreement regarding uplink capacity enhancements in Rel-19 IoT NTN:

- RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution we discuss the potential issues and solutions for EDT enhancements in IoT NTN.

- Discussion

EDT is a feature supported in LTE protocols to allow UE data transmission and reception without entering CONNECTED statue to reduce UE power consumption, which can be characterized into MO-EDT and MT-EDT.

- MO-EDT allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure, and it is triggered when the upper layers have requested the establishment or resumption of the RRC Connection for Mobile Originated data and the uplink data size is less than or equal to a size threshold indicated in the system information.

- MT-EDT is intended for a single downlink data transmission during the random access procedure, and it is initiated by the MME if the UE and the network support MT-EDT and there is a single DL data transmission for the UE.

For Rel-19 NTN enhancements, considering the need of supporting massive capacity in terms of number and types of UE, there is a need to identify methods to reduce the necessary uplink and downlink signaling. From this perspective there are two directions that are to be studied: Msg3 transmission without Msg1/2 and reduced overhead reduction for Msg4 delivery.

## Msg3 transmission without Msg1/2 for EDT

The procedures of legacy MO-EDT for Control Plane CIoT EPS/5GS optimisations (i.e., CP-EDT) are shown in Figure 1 as a starting point for discussion. The legacy MO-EDT for User Plane CIoT EPS/5GS optimisations (i.e., UP-EDT) have similar procedures with some differences in contents of specific messages. Meanwhile the MT-EDT can be considered as a paging procedure plus the MO-EDT procedures. In general, it is obvious that the entire random access procedures, including Msg1(preamble) transmission, Msg2(RAR) reception, Msg3 transmission and Msg4 reception, are necessary for legacy EDT. For Rel-19 IoT NTN aiming to achieve Msg3 transmission without Msg1(preamble)/Msg2(RAR), issues are to be identified and solutions are needed.

Figure 1. MO-EDT for Control Plane CIoT EPS Optimisation

In legacy EDT, the UE relies on transmission of EDT-dedicated preamble to request for EDT, and eNB can identify a request for EDT based on that an EDT-dedicated preamble is used. If Msg1(preamble) transmission is omitted for EDT, new schemes are needed for UE to request for EDT operation and for eNB to identify a request for EDT operation.
Observation 1: If Msg1 transmission is omitted for EDT, RAN2 at least need to consider how UE requests for an EDT operation and how eNB identifies it.

A following issue is that, the UE relies on reception of Msg2(RAR) to acquire the UL resource for the followed Msg3 transmission in legacy EDT operation, as there is no resource pre-configuration for the UE. If Msg2(RAR) reception is omitted for EDT, how to configure UE with the corresponding resource for Msg3 transmission needs to be resolved. RAN2#127bis meeting agreed that at least system information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. The CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level.

Observation 2: If Msg2 reception is omitted for EDT, at least system information can be used to configure UE with necessary UL resource for Msg3 transmission.

For the initiation of an EDT operation, legacy EDT scheme only considers one metric of data size while the channel quality (e.g., signal strength) can be guaranteed by random access mechanisms (e.g., power ramping). If Msg1(preamble) is omitted for EDT, how to ensure that the channel quality is sufficient to transmit Msg3? Moreover, for NTN deployed with LEO satellites, the timing of cell change (e.g., cell stop serving time or feeder link switch time) may also have impact on the decision of initiating an EDT operation.

Observation 3: If Msg1 transmission and Msg2 reception are omitted for EDT, RAN2 may need to consider additional metrics for EDT initiation e.g., to guarantee successful Msg3 transmission.

Based on the above analysis, there are several issues to be resolved if Msg1 transmission and/or Msg2 reception are omitted for EDT in NTN, including initiation, identification and UL resource configuration for an EDT operation without Msg1/Msg2.

Proposal 1: RAN2 to discuss the following issues on EDT without Msg1/2 for NTN:

- UL resource configuration in system information for an EDT operation without Msg1/2;

- UE request and eNB identification for an EDT operation;

- Metrics of initiating an EDT operation without Msg1/2.

## Efficient delivery of Msg4 for EDT

In legacy EDT the delivery of Msg4 (e.g., RRCEarlyDataComplete for CP-EDT) is unicast via dedicated RRC signalling, and its included fields (e.g., dedicatedInfoNAS, extendedWaitTime, idleModeMobilityControlInfo, redirectedCarrierInfo) are specified as optional. From network perspective eNB can reduce overhead simply by omitting some unnecessary fields.

Observation 5: By implementation, an eNB can at least reduce the overhead for Msg4 delivery by restricting the optional fields in Msg4.

Moreover, these fields of Msg4 are not all UE-specific e.g., extendedWaitTime, idleModeMobilityControlInfo, and redirectedCarrierInfo could be common for multiple or even all UEs in the cell, and dedicatedInfoNAS could also be common when it is empty or zero (i.e., no DL data for EDT from core network). Therefore it is not necessary to always unicast all these fields with duplicated signalling overhead. Based on the above analysis, at least the following two directions can be considered for efficient Msg4 delivery:

- Case A: At least part of Msg4 is common for multiple UEs initiated EDT in the cell. In this case the common part of Msg4 may be delivered in a non-unicast manner.

- Case B: No DL data or other configuration is to be delivered to UE (i.e., empty Msg4). In this case an alternative indication other than Msg4 via RRC can be considered e.g., the Layer 1 or MAC CE acknowledgement used for PUR.

Observation 6: Enhancements for efficient Msg4 can consider non-unicast or non-RRC delivery.

Therefore the following directions can be further discussed:

Proposal 2: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).

- Conclusion

In this contribution we discuss the potential issues and solutions for EDT enhancements in IoT NTN. The following observations are given:

Observation 1: If Msg1 transmission is omitted for EDT, RAN2 at least need to consider how UE requests for an EDT operation and how eNB identifies it.

Observation 2: If Msg2 reception is omitted for EDT, at least system information can be used to configure UE with necessary UL resource for Msg3 transmission.

Observation 3: If Msg1 transmission and Msg2 reception are omitted for EDT, RAN2 may need to consider additional metrics for EDT initiation e.g., to guarantee successful Msg3 transmission.

Observation 4: If Msg1 transmission and Msg2 reception are omitted for EDT, UE in NTN needs to guarantee UL synchronization to transmit Msg3 directly.

Observation 5: By implementation, an eNB can at least reduce the overhead for Msg4 delivery by restricting the optional fields in Msg4.

Observation 6: Enhancements for efficient Msg4 can consider non-unicast or non-RRC delivery.

And it is proposed that:

Proposal 1: RAN2 to discuss the following issues on EDT without Msg1/2 for NTN:

- UL resource configuration in system information for an EDT operation without Msg1/2;

- UE request and eNB identification for an EDT operation;

- Metrics of initiating an EDT operation without Msg1/2

Proposal 2: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).