3GPP TSG-RAN WG2 #128	R2-2410276

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	LCM for UE-sided model in beam management

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss LCM issues related to UE-sided model for beam management use case.

# Discussion

## Configuration for training data collection

RAN1#118 Chair Notes Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

In the last RAN1 meeting, RAN1 had one working assumption that the associated ID is at least configured within CSI framework. In our understanding, this will apply to both AIML training phase and inference phase. For example, during the data collection for training, the gNB is expected to configure the set A and set B beam resources for UE to measure together with an associated ID within CSI framework. Then, to configure AI inference, gNB can configure a CSI report configuration for beam prediction, and configures an associated ID for the CSI report configuration for the UE to determine an applicable AI/ML functionality for inference operation and the UE may assume that the RS configured as set B has the similar properties as that for training.

RAN1 assumes the associated ID is at least configured within CSI framework together with set A and set B beam configuration for both training and inference.

During the data collection approach discussion, even though how 3GPP CN can be involved (e.g., Option 1b, 2, 3) needs further SA2 discussion, Option 1a that UE will collect data and transfer to OTT server via application layer is by default supported since the collected data is transferred over application layer without 3GPP impact. On the other hand,  another aspect that needs further RAN2 discussion is how does the gNB provide required training data configuration to UE.

Following the RAN1 assumption above, if the concerned training data, such as set A/B beam resource configuration and associated ID, is configured within CSI framework, gNB can only provide the necessary configuration in response to explicitly request (e.g., via RRC UAI message) from UE. Additionally, UE needs to also indicate the data collection is for which AI functionality among those supported functionalities that have been previously reported and other necessary information (e.g., specific set A and set B association), such that gNB can provide the required configuration accordingly.

RAN2 assumes that UE may explicitly request gNB (e.g., via RRC UAI message) for configuration related to training data collection (e.g., set A/B beam resource configuration and associated ID).

The RRC message requesting configuration for training data collection, also contains necessary information of the AI functionality to be trained (among the supported AI functionalities), such that gNB can provide the configuration accordingly.

Following RAN1 working assumption, if the CSI configuration framework can be used to configure both AI training data collection and AI inference, from RRC signalling design point of view, the purpose (i.e., if it is for AI training data collection or for AI inference) has to be clearly indicated to align the behaviour of UE and gNB. For example, if the set A/B beam resource configuration is for AI training data collection, UE will expect gNB to send reference signals on all set A beams for UE measurement.

If the CSI configuration framework is used for both training data collection configuration and inference configuration, in RRC configuration message design, it has to be clearly distinguished if the provided configuration is for training data collection or for inference.

## 2.2	UAI Configuration

It is quite common understanding that UAI based framework will be used for UE to proactively report the applicability related info according to the gNB configuration (i.e., in OtherConfig). Given that gNB might only be interested in specific UE-sided AI functionalities based on the demand of the gNB, it is reasonable to support the configuration of the UE by the gNB (i.e., via OtherConfig) to report applicability related info pertaining to a subset of the supported UE-side AI functionalities (i.e., as indicated in UECapabilityInformation).

gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among the supported ones (i.e., as indicated in UECapabilityInformation).

Another aspect related to the UAI configuration for applicability related info report is the concept of prohibit timer. In the legacy, prohibit timer is usually configured to avoid frequent report of UAI. Then the question is whether the concept of prohibit timer is needed for the applicability related info report as well, e.g., after the UAI is triggered in the following two cases, should UE starts a prohibit timer in case:

- Case#1: UAI triggered due to a non-applicable UE sided AI functionality becoming applicable

- Case#2: UAI triggered due to an applicable UE sided AI functionality becoming now non-applicable

In case#1, after sending the UAI, the UE sided AI functionality could be activated by NW. Then it is quite critical to not put any restriction (e.g. prohibit timer) on the next UAI report for UE to timely notify gNB about the non-applicability at any time to deactivate the UE sided AI functionality before too late. In this sense, it would be better not to restrict the report of non-applicability using a prohibit timer.

In case#2, after sending the UAI, the UE sided AI functionality could be deactivated and UE may use another UE sided AI functionality or fallback to non-AIML method. A prohibit timer can be used in this case to avoid UE immediately sending a new UAI indicating the deactivated AI functionality now becomes applicable again, which may lead to frequent or unnecessary UE sided AI functionality activation/deactivation. And it does not harm if the UE continues using the non-AIML method for a while.

UE can send UAI indicating a UE sided AI functionality becomes non-applicable at any time without restriction (e.g., prohibit timer).

UE can be configured with a prohibit timer that after UE reports via UAI that a UE sided AI functionality becomes non-applicable, UE will not send another UAI message for the same UE-sided AI functionality again within a time period.

## Activating a non-applicable AI functionality

Even if gNB may only activate an AI functionality that was reported to be applicable, it is still possible that the AI functionality becomes non-applicable when UE receives the activation command, e.g., due to the change of UE condition or other reasons and the new applicability report is not yet sent to gNB.

Even if gNB only activates an AI functionality that was reported to be applicable, it is still possible that the AI functionality becomes non-applicable when UE receives the activation command, e.g., due to the change of UE condition or other reasons and the new applicability report is not yet sent to gNB.

In our view and observation of companies proposals, there are essentially two options

- Option 1: after receiving an activation command from gNB, UE does not activate if the concerned AI functionality becomes non-applicable

- Option 2: after receiving an activation command from gNB, UE still activates even if the concerned AI functionality becomes non-applicable

In our view, activating a non-applicable AI functionality should be prevented as much as possible since it will produce inaccurate prediction that may harm the air interface performance and will waste the computing effort at UE side. How to support Option 1 will depend on the other discussion about the activation and deactivation signaling that RAN2 will wait for RAN1 reply, but it could be beneficial to agree Option 1 as principle from RAN2 point of view.

When receiving an AI functionality activation command from gNB, ifthe concerned AI functionality is currently non-applicable, UE will not activate the AI functionality.

## Configuration for AI functionality during handover

As discussed in RAN2 so far, a UE can be configured by gNB to report the applicability information for UE sided AI functionality under the connected cell, and then gNB can decide to activate the AI functionality if applicable. In the meanwhile, in the handover scenario, when UE is handed over from source cell to target cell, whether and how to configure the UE to activate an AI functionality in the target cell should be discussed by RAN2.

Without any further enhancement, the target cell can always configure the UE to report the applicability information in the handover command sent to UE. Then UE will report the applicability information to target cell either in RRCReconfigurationComplete message or in UAI after UE executes the handover and connects to the target cell only after that target cell can configure the UE to activate an AI functionality using another RRC reconfiguration message. However, in this approach, UE shall first deactivate all AI functionalities when it connects to the target cell and wait for the activation configuration from the target cell which creates a time gap during which UE cannot use any AI functionality immediately after handover.

Without enhancement, a target cell/gNB cannot activate a UE sided AI functionality in the handover command, which results in a gap that UE cannot immediately use any AI based beam prediction method after the handover.

To ensure the continuity of using AI based beam prediction during handover, RAN2 is suggested to discuss and support methods that can allow target cell/gNB to activate the UE sided AI functionality in the handover command. One straightforward thinking is to let the source cell/gNB configure the UE to report the applicability information related to AI functionalities (e.g., of target cell/gNB’s interest) in a different cell (e.g., candidate target cells) before triggering the handover.

On the other hand, what should be exchanged between source and target gNBs and what should be provided to UE for the applicability information reporting strongly depends on the on-going RAN1 discussion about the applicability determination.

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference.

RAN2 supports configuring and activating a UE-side AI functionality in the handover command from target cell/gNB.

Before triggering the handover, the source cell/gNB can configure the UE to report the applicability information related to AI functionalities (e.g., of target cell/gNB’s interest) for a different cell (e.g., candidate target cells). FFS details pending on RAN1 progress.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	RAN1 assumes the associated ID is at least configured within CSI framework together with set A and set B beam configuration for both training and inference.

Observation 2	Even if gNB only activates an AI functionality that was reported to be applicable, it is still possible that the AI functionality becomes non-applicable when UE receives the activation command, e.g., due to the change of UE condition or other reasons and the new applicability report is not yet sent to gNB.

Observation 3	Without enhancement, a target cell/gNB cannot activate a UE sided AI functionality in the handover command, which results in a gap that UE cannot immediately use any AI based beam prediction method after the handover.

Based on the discussion above, we propose:

Proposal 1	RAN2 assumes that UE may explicitly request gNB (e.g., via RRC UAI message) for configuration related to training data collection (e.g., set A/B beam resource configuration and associated ID).

Proposal 2	The RRC message requesting configuration for training data collection, also contains necessary information of the AI functionality to be trained (among the supported AI functionalities), such that gNB can provide the configuration accordingly.

Proposal 3	If the CSI configuration framework is used for both training data collection configuration and inference configuration, in RRC configuration message design, it has to be clearly distinguished if the provided configuration is for training data collection or for inference.

Proposal 4	gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among the supported ones (i.e., as indicated in UECapabilityInformation).

Proposal 5	UE can send UAI indicating a UE sided AI functionality becomes non-applicable at any time without restriction (e.g., prohibit timer).

Proposal 6	UE can be configured with a prohibit timer that after UE reports via UAI that a UE sided AI functionality becomes non-applicable, UE will not send another UAI message for the same UE-sided AI functionality again within a time period.

Proposal 7	When receiving an AI functionality activation command from gNB, ifthe concerned AI functionality is currently non-applicable, UE will not activate the AI functionality.

Proposal 8	RAN2 supports configuring and activating a UE-side AI functionality in the handover command from target cell/gNB.

Proposal 9	Before triggering the handover, the source cell/gNB can configure the UE to report the applicability information related to AI functionalities (e.g., of target cell/gNB’s interest) for a different cell (e.g., candidate target cells). FFS details pending on RAN1 progress.

# Reference