3GPP TSG-RAN WG2 Meeting #128	R2-2410290

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.13.3

Source:	Nokia

Title:	SRAP impacts on MH relay

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc discuss the SRAP impact of the MH relay solution with one or more hops.

# 2	Discussion

As we describe in R2-2410288 reusing the legacy U2N procedures has advantages in terms of at least backwards compatibility and may increase the possibility that a last relay UE can be found. In order to ensure similar advantages for the Rel-17/18 Remote UE, some enhancements should be added to meet the Rel-19 objectives. Considering the below architecture for L2 MH relay operation, we see that at least SRAP enhancements are needed in order to enable gNB support.

Figure 1: Example of protocol stack architecture for L2 MH U2N relay connection between Remote UE and gNB

From 38.351, we see that the SRAP header is used to determine the UE ID and Bearer ID, and map this to the egress RLC channel by adding and removing the SRAP header to the data packet.

### 6.2.2 Data PDU Figure 6.2.2-1 shows the format of the U2N SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to U2N SRAP SDU except those for SRB0 delivered over PC5 interface. Figure 6.2.2-1: U2N SRAP Data PDU format with SRAP header Figure 6.2.2-2 shows the format of the U2N SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to U2N SRAP SDU for SRB0 delivered over PC5 interface. Figure 6.2.2-2: U2N SRAP Data PDU format without SRAP header

Observation 1: Some SRAP enhancements are required at least for the intermediate UE and gNB to support MH relay with legacy remote/relay UEs

Proposal 1: RAN2 to agree to study SRAP impact on intermediate UE(s) and gNB for supporting legacy relay/remote UE(s).

Adding one or more intermediate UEs requires the gNB and at least intermediate UE to perform the same mapping procedure on the SRAP layer at least when;

- Acting as a U2N relay for the at least a Rel-17/18 Remote UE, and

- Acting as a U2N Remote UE seen from at least a Rel-17/18 last Relay UE

Note that operation may be different when one or more UEs are all Rel-19 UEs. A simple way, backwards compatible way of handling the mapping of RLC channels seen from an intermediate UE would be to reuse the SRAP header concept from legacy, and adding another SRAP header per hop on top of the data packet already including an SRAP header.

Observation 2: The SRAP header concept can be reused to map per-hop ingress and egress RLC channels even for intermediate UEs.

Proposal 2: An SRAP header can be added per hop to map ingress and egress RLC channels.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Some SRAP enhancements are required at least for the intermediate UE and gNB to support MH relay with legacy remote/relay UEs

Proposal 1: RAN2 to agree to study SRAP impact on intermediate UE(s) and gNB for supporting legacy relay/remote UE(s).

Observation 2: The SRAP header concept can be reused to map per-hop ingress and egress RLC channels even for intermediate UEs.

Proposal 2: An SRAP header can be added per hop to map ingress and egress RLC channels.