3GPP TSG-RAN WG2#128											      R2-2410293

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Consideration on downlink coverage enhancement

Document for:	 Discussion and Decision

- Introduction

In RAN1 #127bis [1], the following agreements were reached regarding DL coverage enhancement.

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

In this contribution, we will discuss cell bar mechanism and cell reselection and provide our views.

- Discussion

Cell bar mechanism

According to the agreement, for UE supporting DL-CE, a barring mechanism is needed. Then it means that a new bar indication for Rel-19 UE supporting DL-CE needs to be introduced. Similar to legacy NTN bar indication, if the new bar indication is set to not barred, the UE shall consider this cell is allowed to access for Rel-19 UE supporting DL-CE. Otherwise, if the new bar indication is set to bar or the bar indication is not present, which means this cell is considered as barred.

Proposal 1: A new bar indication for Rel-19 UE supporting DL-CE can be introduced.

Proposal 2: Similar to legacy NTN bar indication, the UE supporting DL-CE shall

- consider the cell as barred if the new introduced bar indication is set to barred or not present

- consider the cell as not barred if the new introduced bar indication is set to not barred

Cell reselection

For cell reselection, since Rel-19 UEs not supporting DL-CE and legacy UEs do not support the feature of DL CE, these UEs may not reselect to cells operating with DL CE. In order to achieve this, the following two options may be considered:

Option 1: introducing a new excluded list for UE supporting DL-CE

Option 2: introducing a new cell reselection priority for UE supporting DL-CE

In our understanding, supporting the same feature for a frequency should be a typical scenario, for example, DL CE feature should be supported for all cells of one frequency. Cell reselection priority of Option 2 above is per frequency. For UE not supporting DL-CE and legacy UE, the lower priority may be used, i.e. legacy cell reselection priority and/or cell reselection sub-priority may be used. While for UE supporting DL-CE, higher priority may be used, i.e. new introduced cell reselection priority. Then, for UEs supporting DL-CE, it would be easier to reselect to cells operating with DL CE due to higher priority, which is beneficial. Therefore, we prefer to introduce a new cell reselection priority for UE supporting DL-CE, i.e. option 2.

Proposal 3: A new cell reselection priority should be introduced for UE supporting DL-CE to determine cell reselection list.

Cell/beam DTX

Due to the limited power and limited feeder link bandwidth, only limited beams can be simultaneously active. In RAN1 #116[2], some agreements about simulation assumptions were reached, for example, only 10.02 and 1.5% beams can be active in case of 1058 and 800 beams in total for FR1 and FR2 respectively, i.e. only a small portion of beams can be simultaneously active.

Besides, RAN1 also raised that three beam states can be used for system level study based on analytical evaluation. Wherein, N1 state means that there is no any served signals in this area covered by N1 beam footprints, i.e. the NW cannot transmit any signals during this state. N3 state is similar to active period of cell DTX of NES or there is no cell/beam DTX, i.e. the normal transmissions can be performed during this state. For N2 state, there is only common messages according to RAN1’s agreement, i.e. only some necessary information for cell discovery and initial access can be transmitted, such as SSB, SIB1, SIB19, paging and messages of random access procedure.

Then, in order to make flexible and dynamic power allocation among the different satellite beams for DL coverage enhancement, beam hopping mechanism may be considered for all satellite beams, which means that some beams are active and some beams are non-active within a certain duration. Cell DTX mechanism introduced in Rel-18 NES may be baseline. A possible cell/beam level pattern for a beam transaction among the N1, N2/N3 beam states is shown in Figure 1 below. In this figure, cell/beam is in non-Active state within N1 duration and cell/beam is in Semi-Active state within N2 duration. While during N3 state, cell/beam is in Active state.

Figure 1 Cell/beam DTX mechanism based on three beam states

Proposal 4: Introduce cell/beam DTX mechanism to support beam hopping.

- Conclusion

This contribution provides our analysis and views for the DL-CE, the following proposals are listed below:

Proposal 1: A new bar indication for Rel-19 UE supporting DL-CE can be introduced.

Proposal 2: Similar to legacy NTN bar indication, the UE supporting DL-CE shall

- consider the cell as barred if the new introduced bar indication is set to barred or not present

- consider the cell as not barred if the new introduced bar indication is set to not barred

Proposal 3: A new cell reselection priority should be introduced for UE supporting DL-CE to determine cell reselection list.

Proposal 4: Introduce cell/beam DTX mechanism to support beam hopping.

- References

[1] Draft_RAN2_127b_Meeting_Report_v1.

[2] Draft_Minutes_report_RAN1#116_v010.