3GPP TSG-RAN WG2 Meeting #128	R2- 2410297

Orlando, US, 18-22 August, 2024

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we investigate connection procedures, approach 1 and approach 2, discussed in the long email discussion “[Post127][402][Relay] Multi-hop relay control plane”. In addition, we would like to discuss how to perform the notification message delivery, paging monitoring and SIB delivery for remote UE(s) in multi-hop relay operation.

2.	Discussion

2.1 Comparing control procedure between approach 1 and approach 2

Two approaches of control procedure for multi-hop relay operation was discussed in the long email discussion “[Post127][402][Relay] Multi-hop relay control plane”. We think that some issues need to be further clarified with our insight.

For approach 1, one of the raised issue was how to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing (A1.7 in the [Post127][402][Relay] email discussion). In the case of SL operation for the licenced band, we understand that if a cell does not support SL, the UE cannot perform SL operation by using pre-configuration within the cell coverage. This is because the use of pre-configuration will cause interference to the cell.  As the same principle, if a cell does not support multi-hop relay, we wonder whether a UE can perform multi-hop relay operation by using pre-configuration within the cell regardless of the RRC state of the intermediate Relay UE. If multi-hop operation is not allowed in a cell, we believe that the intermediate Relay UE may not act as an intermediate Relay UE by using pre-configuration within a cell. Based on this understanding, we need to check further verify whether approach 2 has any benefit in terms of intermediate Relay mobility.

Proposal 1: For the issue A1.7, we need to check whether the intermediate Relay UE can perform multi-hop relay operation by using pre-configuration within a cell that does not support multi-hop relay operation or SL operation. In the case of legacy SL operation, the UE cannot perform SL operation by using pre-configuration within a cell that does not support SL operation due to interference.

- Issue A1.7: How to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing

For approach 2, one of the raised issues was that approach 2 cannot support the agreed WI Objective for Scenario C and D (A2.6 in the [Post127][402][Relay] email discussion). We think this may be a critical issue. In the case C and D, when the remote UE reports the candidate first intermediate Relay UE to the gNB, the gNB should exclude the path if the one of intermediate Relay UE in the target path is in RRC_IDLE/INACTIVE. In approach 1, there is no possibility for the intermediate Relay UEs belonging to the target path to be in RRC_IDLE/INACTIVE when the candidate first intermediate Relay UE is in RRC_CONNECTED. However, in approach 2, even if the candidate first intermediate Relay UE is in RRC_CONNECTED, the other intermediate Relay UEs belonging to the target path may be RRC_IDLE/INACTIVE. The gNB may trigger the intermediate Relay UEs in RRC_IDLE/INACTIVE in the target path to be in RRC_CONNECTED before sending the handover message to the Remote UE. How the gNB triggers the intermediate Relay UE may be another issue. This is because the gNB may not be aware of the existence of the IDLE/INACTIVE intermediate Relay UEs belonging to the target path. This method will increase the path switching latency of the multi-hop relay. It seems difficult to selecta target path where all the intermediate Relay UE(s) are in RRC_CONNECTED in approach 2.

Proposal 2: For the issue A2.6, it looks difficult to support service continuity for the scenario C and D of the WI objective by using the approach 2.

- Issue A2.6: Approach 2 cannot support the agreed WI Objective for supporting Scenario C and D

For the approach 2, we think it’s not clear in terms of how to assign the local ID and how to use the measurement value for QoS split. Until the current release, there is no multi-hop U2U scheme, we may have to discuss lots of things to apply multi-hop U2U likely scheme to the multi-hop U2N relay operation. Some companies said that what the intermediate Rely UE can be in RRC_CONNECTED in the different cell from the last Relay UE is benefit in approach 2 in the aspect of intermediate relay selection. However, as we mentioned before, we wonder whether the intermediate Relay UE can use pre-configuration if the intermediate Relay UE is in the cell that does not support SL or multi-hop relay operation. On the other hand, if the cell of the intermediate Relay UE supports multi-hop SL relay, the intermediate Relay UE will act as a last Relay UE. We think this kind of scenario looks like unclear and a corner case.

Proposal 3: The RRC_CONNECTED intermediate Relay UE being in the different cell from the last Relay UE seems unclear and a corner case.

- If the CONNECTED cell of the intermediate Relay UE supports multi-hop SL Relay, the intermediate Relay UE should act as a last Relay UE.

- If the CONNECTED cell of the intermediate Relay UE does not support multi-hop SL Relay or SL operation, the intermediate Relay UE should not act as an intermediate Relay UE in the cell.

Regarding local ID assignment, it is unclear how the local ID is assigned in approach 2. If the gNB assigns the local ID to the Remote UE, it’s unclear how the Remote UE delivers it to the gNB. If the Remote UE attached to the first intermediate Relay UE, the Remote UE has to perform local ID request to the gNB. This is because the gNB does not know whether the Remote UE is attached to the first intermediate Relay UE. However, it’s not clear how the local ID assignment request message is delivered to the gNB. Similarly, even if the gNB assigns the local ID for the Remote UE, it’s not clear how to deliver it to the Remote UE. This local ID request and assignment may need to be performed before end-to-end connection establishment between the gNB and the Remote UE. To do this, several methods can be discussed. However, before starting this discussion, the benefit should be clear.

In approach 1, the condition that all the intermediate Relay UE must be in RRC_CONNECTED can be advantageous in some parts. In principle, the gNB can control all the intermediate Relay UE efficiently. In terms of the signalling overhead, we are not sure that approach 2 signalling overhead much less than the approach 1. This is because as long as the gNB configures the local ID, the Remote UE sends the local ID request message to the gNB and the gNB sends the assigned local ID to the Remote UE. Also, when one SL hop configuration is changed due to a Remote UE attachment, the entire SL configuration may be changed. In this case, the SL signalling is increased such as the Uu signalling is increased in approach 1.

Proposal 4: In approach 2, if the local ID is assigned by gNB, the following will be issued.

- How to the local ID request message can delivered to the gNB via multi-hop U2U operation.

- How to the assigned local ID information can be delivered to the Remote UE via multi-hop U2U operation.

In the L3 multi-hop U2N relay operation, the QoS splitting is performed by the gNB. The gNB performs QoS split between Uu hop and SL hop. Some companies believe that this approach can be directly applied to L2 multi-hop U2N relay operation. However, it is worth considering whether the QoS split method used in L3 U2N multi-hop relay should be applied as it is to the L2 multi-hop U2N relay operation. In the case of L3 relay QoS split, we cannot find the consideration of the measurement result. In L3 relay, the gNB cannot configure measurement report to the last/intermediate Relay UE or Remote UE. Although the gNB receives the measurement report, there is no way for the gNB to use the measurement result. The QoS split in L3 multi-hop relay is performed at the CN.

However, in the case of L2 muti-hop U2N relay, the gNB can configure measurement report to the last/intermediate Relay UE or Remote UE. And the gNB can use the measurement report in L2 multi-hop U2N relay operation. We assume that the gNB can perform QoS splitting based on the measurement report in L2 multi-hop U2N relay operation. To do this, the gNB needs to know the total link quality. The QoS split should be performed based on the relative link quality. The QoS split should not be determined solely based on the absolute quality of the Uu link quality alone. For example, if the Uu link quality and SL quality are very good, the split PDB value can be evenly distributed. However, if the Uu link quality is similar to the previous example and the SL quality is poor, then the Uu PDB should be assigned a shorter period, and the SL PDB should be assigned a longer period than in the first example. For the Uu hop QoS split, the gNB should know the entire link quality. Following the same principle, no matter which intermediate Relay UE (including the last Relay UE) performs the QoS split, the intermediate Relay UE (including the last Relay UE) should be aware of the entire link quality.

Observation 1: In L3 multi-hop U2N relay operation, the CN performs QoS split between Uu link and SL. The gNB doesn’t seem to have any role for the QoS split (i.e., it doesn’t use the measurement report).

Observation 2: In L2 multi-hop U2N relay operation, the gNB can perform QoS split based on the measurement report from the last/intermediate Relay UE and Remote UE.

Observation 3: For the gNB to perform QoS split well in L2 multi-hop U2N relay operation, the gNB needs to know the overall link quality. Even though the QoS split is for Uu link and SL.

Proposal 5: For any approach, the QoS split must not be performed on the basis of the absolute link quality value. i.e., for the QoS split, the gNB or intermediate Relay UE (including the last Relay UE) performing the QoS split must know not only the adjacent link quality but also the overall link quality.

- It’s not clear how the gNB or intermediate Relay UE can know the overall link quality in approach 2.

Until now, we don’t find any clear benefits of the control procedure of the approach 2. In terms of latency, we think it doesn’t matter in the case of approach 1. Because the case of the initial connection setup of the initial remote UE causes long latency, especially, when many intermediate Relay UEs are in RRC_IDLE/INACTIVE. This may be a rare case for the most other (not initial) Remote UEs. If all the intermediate Relay UE is in RRC_CONNECTECED, we are not sure that the latency in approach 1 will be longer than in approach 2 for establishing the connection setup. Further, the short latency is not the objective of this WI. Whichever approach is chosen, the coverage extension may be possible. In terms of signaling overhead, it’s also unclear which approach is better. This is because the local ID request and assignment procedure should be performed in both approaches, and the reconfiguration for each hop should be performed regardless of the approach when new remote UE attached.

Proposal 6: The benefits of approach 2 in terms of latency and signalling overhead are not clear. This is because local ID assignment and QoS splitting procedure that cause additional Uu/PC5 signalling have not been discussed yet.

Proposal 7: Considering the simple spec impact and the late start of WI, the Rel-17 based connection establishment procedure (i.e., approach 1) can be a baseline for multi-hop U2N relay.

2.2 Notification message delivery

The notification message can be generated at the last Relay UE due to a Uu link problem such as the legacy Rel-17 U2N Relay. The generated notification message is delivered to the connected intermediate Relay UE. When the intermediate Relay UE receives a notification message form the last or the parent intermediate Relay UE, the intermediate Relay UE can generate the notification message. In this case, the cause value of the notification message can be the value contained in the received notification message. For example, if the intermediate Relay UE receives a notification message from the last Relay UE due to the Uu link problem, the cause value of the notification message generated by the intermediate Relay UE can be the Uu link problem even the intermediate Relay UE can monitor the PC5 link. Or the we can make new cause value such as due to the receipt of the notification message.

Proposal 8: The last Relay UE generates a notification message for the same reason as the legacy Rel-17 U2N relay (i.e., Uu RLF, HO, cell reselection, Uu RRC failure).

Proposal 9: If a notification message is generated due to a Uu link problem at the last Relay UE, the notification message should be delivered to the connected intermediate Relay UE(s).

Proposal 10: When the intermediate Relay UE receives the notification message from its parent intermediate Relay UE/last Relay UE, the intermediate Relay UE generates the notification message and sends it to the connected intermediate Relay UE(s) or Remote UE(s). The cause value can be the same as the value contained in the received notification message.

When the intermediate Relay UE detects an SL-RLF or PC5 release between the last/parent intermediate Relay UE and itself, it generates a notification message. The notification message is delivered to the child intermediate Relay UEs or Remote UEs. However, if the intermediate Relay UE detects an SL-RLF or PC5 release between the Remote UE/child intermediate Relay UE and itself, the intermediate Relay UE reports it to the gNB via the SUI. It is assumed that the intermediate Relay UE is in RRC_CONNECTED.

Proposal 11: When the intermediate Relay UE detects a PC5 RLF to the parent link, the intermediate Relay UE generates notification message and sends it to the intermediate Relay(s) or Remote UE(s) in the child link.

Proposal 12: When the intermediate Relay UE detects a PC5 RLF for the child link, the intermediate Relay UE reports it to the gNB via the SUI.

2.3 Paging monitoring/SIB delivery

It is not certain that the intermediate L2 U2N Relay UE has a reliable Uu link quality to monitor the paging or receive SIB information for the L2 U2N Remote UE. However, the last L2 U2N Relay UE must have reliable Uu link quality. Therefore, the last L2 U2N Relay UE should be responsible for paging monitoring and SIB delivery for the L2 U2N Remote UE and the intermediate L2 U2N Relay UE(s). When the intermediate Relay UE in RRC_IDLE/INACTIVE receives the message for paging monitoring and SIB request message from the Remote UE, the intermediate Relay UE delivers it toward the last Relay UE. Similarly, when the intermediate Relay UE in RRC_IDLE/INACTIVE receives the message for paging and requested/updated SIB information from the last Relay UE, the intermediate Relay UE delivers it toward a Remote UE. To do this, the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE may need to know the direction of the message when receiving a request message for paging monitoring or SIB delivery from the L2 U2N Remote UE. Also, the intermediate L2 U2N Relay UE may have to know which SL connection is toward to the last L2 U2N Relay UE.

Proposal 13: The last L2 U2N Relay UE is responsible for paging and SIB monitoring for the intermediate L2 U2N Relay UEs and L2 U2N Remote UEs in RRC_IDLE/INACTIVE.

Proposal 14: When the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE receives a message for paging monitoring or SIB request from the L2 U2N Remote UE, the intermediate L2 U2N Relay UE delivers it toward the last L2 U2N Relay UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the last L2 U2N Relay UE.

Proposal 15: When the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE receives a message for paging or requested/updated SIB information from the L2 U2N last Relay UE, the intermediate L2 U2N Relay UE delivers it toward the L2 U2N Remote UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the correct L2 U2N Remote UE.

When the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE receives SIB request messages from several L2 U2N Remote UEs, the intermediate L2 U2N Relay UE can combine the received requested SIB types. The intermediate Relay UE sends the combined SIB request messages to the last L2 U2N Relay UE. The last L2 U2N Relay UE in RRC_CONNECTED can request the SIBs to the gNB. Or, the last L2 U2N Relay UE in RRC_IDLE/INACTIVE can monitor the SIBs and then deliver them to the intermediate L2 U2N Relay UE. When the intermediate L2 U2N Relay UE receives the SIBs from the last L2 U2N Relay UE, the intermediate L2 U2N Relay UE delivers them to the L2 U2N Remote UE that requested them. If the intermediate L2 U2N Relay UE has already stored the latest updated SIBs, the intermediate L2 U2N Relay UE may deliver them to the L2 U2N Remote UE without requesting SIBs to the last L2 U2N Relay UE.

Proposal 16: The intermediate L2 U2N Relay UE can combine the types of request SIBs received from multiple U2N Remote UE(s), and deliver them to the last U2N Relay UE.

Proposal 17: If the intermediate U2N Relay UE already has the latest updated SIBs matching the L2 U2N Remote UE is request, the intermediate U2N Relay UE may deliver the stored SIBs to the L2 U2N Remote UE without forwarding the SIB request message to the last U2N Relay UE.

When the RRC_CONNECTED intermediate Relay UE performing SIB request or paging monitoring for the Remote UE becomes RRC_IDLE/INACTIVE, the intermediate Relay UE sends the stored requested SIB type and/or the information for paging monitoring information received from the Remote UE to the parent intermediate Relay UE/last Relay UE. This is because the RRC_CONNECTED intermediate Relay UE performed the SIB request and paging monitoring instead of the Remote UE, but the intermediate Relay UE can no longer perform this work. However, the Remote UE still expects to receive the updated SIB and paging.

If the intermediate Relay UE in RRC_IDLE/INACTIVE changed to RRC_CONNECTED, the intermediate Relay UE can send a message to its parent intermediate Relay UE/last Relay UE to de-configure SIB request and paging monitoring for the Remote UE. The intermediate Relay UE in RRC_CONNECTED can perform the SIB request or paging monitoring via its indirect Uu link instead of its parent intermediate Relay UE. The intermediate Relay UE turned in RRC_CONNECTED can perform SIB request and paging monitoring for the Remote UE. The parent intermediate Relay UE/last Relay UE doesn’t need to perform SIB request or paging monitoring for a Remote UE anymore.

Proposal 18: When the intermediate Relay UE in RRC_CONNECTED performing SIB request or paging monitoring for the Remote UE becomes in RRC_IDLE/INACTIVE, the intermediate Relay UE sends the stored requested SIB type and/or the information for paging monitoring receiving from the Remote UE to the parent intermediate Relay UE/last Relay UE.

Proposal 19: When the RRC_IDLE intermediate Relay UE becomes RRC_CONNECTED, the intermediate Relay UE can send a message to its parent intermediate Relay UE/last Relay UE to de-configure SIB request and paging monitoring for the Remote UE.

3.	Conclusion

Proposal 1: For the issue A1.7, we need to check whether the intermediate Relay UE can perform multi-hop relay operation by using pre-configuration within a cell that does not support multi-hop relay operation or SL operation. In the case of legacy SL operation, the UE cannot perform SL operation by using pre-configuration within a cell that does not support SL operation due to interference.

- Issue A1.7: How to handle intermediate Relay UE mobility in case that intermediate Relay UE has its service ongoing

Proposal 2: For the issue A2.6, it looks difficult to support service continuity for the scenario C and D of the WI objective by using the approach 2.

- Issue A2.6: Approach 2 cannot support the agreed WI Objective for supporting Scenario C and D

Proposal 3: The RRC_CONNECTED intermediate Relay UE being in the different cell from the last Relay UE seems unclear and a corner case.

- If the CONNECTED cell of the intermediate Relay UE supports multi-hop SL Relay, the intermediate Relay UE should act as a last Relay UE.

- If the CONNECTED cell of the intermediate Relay UE does not support multi-hop SL Relay or SL operation, the intermediate Relay UE should not act as an intermediate Relay UE in the cell.

Proposal 4: In approach 2, if the local ID is assigned by gNB, the following will be issued.

- How to the local ID request message can delivered to the gNB via multi-hop U2U operation.

- How to the assigned local ID information can be delivered to the Remote UE via multi-hop U2U operation.

Observation 1: In L3 multi-hop U2N relay operation, the CN performs QoS split between Uu link and SL. The gNB doesn’t seem to have any role for the QoS split (i.e., it doesn’t use the measurement report).

Observation 2: In L2 multi-hop U2N relay operation, the gNB can perform QoS split based on the measurement report from the last/intermediate Relay UE and Remote UE.

Observation 3: For the gNB to perform QoS split well in L2 multi-hop U2N relay operation, the gNB needs to know the overall link quality. Even though the QoS split is for Uu link and SL.

Proposal 5: For any approach, the QoS split must not be performed on the basis of the absolute link quality value. i.e., for the QoS split, the gNB or intermediate Relay UE (including the last Relay UE) performing the QoS split must know not only the adjacent link quality but also the overall link quality.

- It’s not clear how the gNB or intermediate Relay UE can know the overall link quality in approach 2.

Proposal 6: The benefits of approach 2 in terms of latency and signalling overhead are not clear. This is because local ID assignment and QoS splitting procedure that cause additional Uu/PC5 signalling have not been discussed yet.

Proposal 7: Considering the simple spec impact and the late start of WI, the Rel-17 based connection establishment procedure (i.e., approach 1) can be a baseline for multi-hop U2N relay.

Proposal 8: The last Relay UE generates a notification message for the same reason as the legacy Rel-17 U2N relay (i.e., Uu RLF, HO, cell reselection, Uu RRC failure).

Proposal 9: If a notification message is generated due to a Uu link problem at the last Relay UE, the notification message should be delivered to the connected intermediate Relay UE(s).

Proposal 10: When the intermediate Relay UE receives the notification message from its parent intermediate Relay UE/last Relay UE, the intermediate Relay UE generates the notification message and sends it to the connected intermediate Relay UE(s) or Remote UE(s). The cause value can be the same as the value contained in the received notification message.

Proposal 11: When the intermediate Relay UE detects a PC5 RLF to the parent link, the intermediate Relay UE generates notification message and sends it to the intermediate Relay(s) or Remote UE(s) in the child link.

Proposal 12: When the intermediate Relay UE detects a PC5 RLF for the child link, the intermediate Relay UE reports it to the gNB via the SUI.

Proposal 13: The last L2 U2N Relay UE is responsible for paging and SIB monitoring for the intermediate L2 U2N Relay UEs and L2 U2N Remote UEs in RRC_IDLE/INACTIVE.

Proposal 14: When the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE receives a message for paging monitoring or SIB request from the L2 U2N Remote UE, the intermediate L2 U2N Relay UE delivers it toward the last L2 U2N Relay UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the last L2 U2N Relay UE.

Proposal 15: When the intermediate L2 U2N Relay UE in RRC_IDLE/INACTIVE receives a message for paging or requested/updated SIB information from the L2 U2N last Relay UE, the intermediate L2 U2N Relay UE delivers it toward the L2 U2N Remote UE.

- The intermediate L2 U2N Relay UE has to know which SL connection is toward the correct L2 U2N Remote UE.

Proposal 16: The intermediate L2 U2N Relay UE can combine the types of request SIBs received from multiple U2N Remote UE(s), and deliver them to the last U2N Relay UE.

Proposal 17: If the intermediate U2N Relay UE already has the latest updated SIBs matching the L2 U2N Remote UE is request, the intermediate U2N Relay UE may deliver the stored SIBs to the L2 U2N Remote UE without forwarding the SIB request message to the last U2N Relay UE.

Proposal 18: When the intermediate Relay UE in RRC_CONNECTED performing SIB request or paging monitoring for the Remote UE becomes in RRC_IDLE/INACTIVE, the intermediate Relay UE sends the stored requested SIB type and/or the information for paging monitoring receiving from the Remote UE to the parent intermediate Relay UE/last Relay UE.

Proposal 19: When the RRC_IDLE intermediate Relay UE becomes RRC_CONNECTED, the intermediate Relay UE can send a message to its parent intermediate Relay UE/last Relay UE to de-configure SIB request and paging monitoring for the Remote UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.