3GPP TSG-RAN WG2 Meeting #128	R2- 2410299

Orlando, US, 18-22 August, 2024

Agenda Item	: 8.13.4

Source	: LG Electronics Inc.

Title	: Discussion on service continuity for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the service continuity for muti-hop U2N relay operation, especially, for the first priority case in the WID[1].

2.	Discussion

2.1 Service continuity for intra-gNB multi-hop indirect to direct path

According to the WID multi-hop Relay, RAN2 handles the following cases with first priority:

- Case A) Intra-gNB multi-hop indirect to direct path switching using existing framework

- Case B) Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

It can be the same operation as the legacy Rel-17 path-switching. In the legacy Rel-17 U2N path switching, the service continuity of Remote UE was supported when the target path was direct path or single-hop indirect path. Therefore, we believe that the same procedure can be applied to the above two cases. The difference with the legacy Rel-17 U2N service continuity is that the source path has a multi-hop U2N connection. After the Remote UE completes the path switching to the target path, the gNB releases the configuration related to the Remote UE for the intermediate Relay UEs in the source path. The release procedure for the intermediate Relay UEs in the source path can be gNB’s implementation.

Proposal 1: To support case A, the legacy HO command is reused. Upon receiving the HO command, the UE performs the legacy UE behaviours toward the target cell.

Proposal 2:  To support case B, the legacy path switching command and procedure are reused for the Remote UE and the target Relay UE. Upon receiving the path switching command, the UE performs legacy UE behaviour toward the target Relay UE. The target Relay UE can be in any RRC state.

Proposal 3: it is up to gNB how to release the last/intermediate Relay UEs on the source multi-hop indirect path.

RAN2 had a long email discuss in “[Post127][402][Relay] Multi-hop relay control plane”. In the email discussion, we compared the pros and cons between approach 1 and approach 2 for the control procedure. According to the objective in the Rel-19 multi-hop relay WI, the intermediate Relay UE belonging to the target path should be in RRC_CONNECTED state to support the second priority path switching case as C and D. In approach 2, the intermediate Relay UE can be in RRC_IDLE/INACTIVE even if the Remote UE or other intermediate Relay UE is in RRC_CONNECTED. Comparing approach 2, in the approach 1, all the intermediate Relay UEs should be in RRC_CONNECTED if the first intermediate Relay UE is in RRC_CONNECTED. So, in approach 1, when the candidate first Relay UE broadcast discovery message, all the other intermediate Relay UEs towards for the target gNB should be in RRC_CONNECTED. It’s easy to satisfy sthe constraints in the WID that all the intermediate Relay UEs and last Relay UE should be in RRC_CONNECTED belonging to the target path. However, in approach 2, it doesn’t seem easy to meet this constraint. Even if the candidate first intermediate Relay UE is in RRC_CONNECTED, the other intermediate Relay UEs belonging to the target path can be in RRC_IDLE/INACTIVE.

Proposal 4: If approach 1 is agreed in the control plane, it will be easier to support the case C and D.

3.	Conclusion

Proposal 1: To support case A, the legacy HO command is reused. Upon receiving the HO command, the UE performs the legacy UE behaviours toward the target cell.

Proposal 2:  To support case B, the legacy path switching command and procedure are reused for the Remote UE and the target Relay UE. Upon receiving the path switching command, the UE performs legacy UE behaviour toward the target Relay UE. The target Relay UE can be in any RRC state.

Proposal 3: it is up to gNB how to release the last/intermediate Relay UEs on the source multi-hop indirect path.

Proposal 4: If approach 1 is agreed in the control plane, it will be easier to support the case C and D.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.