3GPP TSG-RAN WG2 Meeting #128	R2-2410304

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.9.4

Source:	Huawei, HiSilicon, Turkcell

Title:	Further considerations on PWS support for NB-IoT

Document for:	Discussion and Decision

# Introduction

In RAN plenary #105 meeting, the objective regarding the support of PWS for NB-IOT was approved [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In the RAN2#127bis meeting, the following agreements were achieved:

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this contribution, we provide our consideration on the remaining issues to support PWS for NB-IoT.

# Discussion

## 2.1 PWS indications in Paging-NB

Per current specification TS 36.331, the E-UTRAN notifies the RRC Idle and RRC connected UEs about the on-going broadcasted PWS notification, i.e. etws-Indication and cmas-Indication, following the principle as below:

- For RRC Idle UEs, E-UTRAN sends the Paging message to UEs except for NB-IoT UEs, and send the Direct Indication Information message to BL UEs and UEs in CE.

- For RRC connected UEs, E-UTRAN sends the Paging messaging to UEs except for NB-IoT UEs, BL UEs and UEs in CE, and sends the Direct Indication Information message to UEs in CE.

In the last RAN2 meeting, it was agreed to enhance the NB Direct Indication Information message by introducing etws-Indication and cmas-Indication to support PWS for NB-IoT UEs. Meanwhile, there were different views whether these indications should also be included in the Paging-NB message. In our understanding, the main purpose of this discussion is to support PWS for NB-IoT UEs. Since this is already supported with enhancement to the direct indication information for NB, there is no strong motivation to further enhance Paging-NB which adds extra complexity to NB-IoT UEs.

Proposal 1: Not introduce the etws-Indication and cmas-Indication in the Paging-NB message.

## 2.2 SIB acquisition after PWS indication

An FFS from the last RAN2 meeting is about clarification of the immediate acquisition of PWS messages after receiving the PWS notification from an NB-IoT Cell. According to the current LTE mechanism, upon reception of etws-Indication/cmas-Indication by an eMTC UE, the UE re-acquires the SystemInformationBlockType1 immediately without waiting until the next system information modification period boundary, and acquires the SystemInformationBlockType10/11/12 if present. So here for NB-IoT, the principle should be similar.

Proposal 2: After reception of PWS indication, NB-IoT UE re-acquires SIB1-NB immediately without waiting until the next system information modification period boundary, and acquires the PWS SIBs if present.

## 2.3 PWS reception in RRC connected mode

Since the direct indication information for NB can be received by NB-IoT UEs in Connected mode, it is obvious PWS indication can also be supported for RRC CONNECTED NB-IoT. Regarding the PWS SIB acquisition, in current RRC spec, it is specified:

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT).

Following similar principle, after receiving the PWS indication in in Connected mode, NB-IoT UEs can read the related SIBs without state transition.

Proposal 3: RAN2 confirms NB-IoT UEs in RRC Connected mode can receive PWS related system information without state transition.

Besides, when introducing PWS reception in RRC connected mode for Rel-16 eMTC, the following capabilities were introduced:

etws-CMAS-RxInConnCE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

etws-CMAS-RxInConnCE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

And the following RRC indication was also introduced:

[[	ce-ETWS-CMAS-RxInConn-r16				ENUMERATED {true}		OPTIONAL	-- Need OR

]],

Therefore, similarly we should introduce a capability and RRC indication for NB-IoT.

Proposal 4: RAN2 Introduces a capability and RRC indication for PWS reception in RRC connected mode for NB-IoT.

During the last meeting, RAN2 sent an LS to other WGs to inform that RAN2 is discussing PWS support for NB-IoT where RAN1 was not involved. Since there are related descriptions in RAN1 specifications that needs to be updated for direct indication information for NB-IoT, we need to send an LS to RAN1 as well so they can update the TS accordingly.

Proposal 5: Send an LS to RAN1 to inform them about RAN2 progress on PWS support for NB-IoT.

## 2.4 PWS acquisition and GNSS measurement

In Rel-18, GNSS measurement was introduced for IoT NTN. There may be a case that an NB-IoT UEs in RRC Connected mode needs to both receive ETWS/CMAS and perform GNSS measurement, e.g. upon reception of GNSS out-of-date indication, upon T390 expiry and upon reception of GNSS measurement MAC CE command. During the GNSS measurement, the UE will suspend the AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbor cell measurement, RACH, SR, and BSR) and not acquire PWS SIBs during GNSS measurement. So RAN2 needs to discuss the case when the acquisition of PWS SIBs collides with GNSS measurement.

Proposal 6: RAN2 to discuss the case when the acquisition of PWS SIBs collides with GNSS measurement.

# Conclusion

This contribution discussed the support of PWS for NB-IoT. Proposals are summarized as follows:

Proposal 1: Not introduce the etws-Indication and cmas-Indication in the Paging-NB message.

Proposal 2: After reception of PWS indication, NB-IoT UE re-acquires SIB1-NB immediately without waiting until the next system information modification period boundary, and acquires the PWS SIBs if present.

Proposal 3: RAN2 confirms NB-IoT UEs in RRC Connected mode can receive PWS related system information without state transition.

Proposal 4: RAN2 Introduces a capability and RRC indication for PWS reception in RRC connected mode for NB-IoT.

Proposal 5: Send an LS to RAN1 to inform them about RAN2 progress on PWS support for NB-IoT.

Proposal 6: RAN2 to discuss the case when the acquisition of PWS SIBs collides with GNSS measurement.

# References

RP-242397, Revised WID: Non-Terrestrial Networks (NTN) for IoT Phase 3.