3GPP TSG-RAN WG2 Meeting #128 	R2-2410306

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.3

Source:	Panasonic

Title:	L1 measurement event configuration and reporting

Document for:	Discussion

# Introduction

In last meeting, RAN2 achieved the following agreements on LTM measurement event evaluation and reporting.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this paper, we focus on addressing some remaining issues. In particular, we discuss how to ensure the same RS type for both serving and candidate cells, TTT operation and event trigger conditions when configured for both early synchronization and cell switch procedures.

# Discussion

## How to ensure the same RS type

As mentioned in section 1, the working assumption has been confirmed in the previous meeting i.e. same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. There were short discussions on how to achieve this. Some company had a view that this can rely on network configuration, i.e. same RS type is configured for both serving cell and all candidate cells. However, this option is too restrictive to the NW and we are not even sure whether this option is feasible in the first place. For example, although NW is able to configure only one single type of RS, e.g. CSI-RS, for all LTM candidate cells for L1 measurement, the measurement RS for serving cell depends on the current beam corresponding to indicated TCI state, which could be either SSB or CSI-RS, subject to intra-cell beam management. It is hard to imagine the NW only configure single type of RS for intra-cell beam management, because this restriction could significantly compromise the data transmission in serving cell.

To ensure the same RS type, the most straightforward option would be that NW configures both RS types for each LTM candidate cell, then the RS type follows the serving cell. In more details, for serving cell, UE measures the QCL RS of the indicated TCI state, and for candidate cells, UE measures the configured RS(s) with the same RS type as the QCL RS of the indicated TCI state of the serving cell.

Proposal 1. To ensure the same RS type between serving and candidate cells for event LTM3 and LTM5 evaluation, NW configures both types of RS (i.e. SSB and CSI-RS) as measurement RS for each candidate cell, and which type to use follows the current beam in serving cell (i.e. QCL RS associated with the indicate TCI state in the serving cell).

FFS: whether and how to address the case where NW configures single type of RS for candidate cells.

On the other hand, one may argue that NW may prefer the possibility to configure only single type of RS for L1 measurements for some (or all) candidate cells in some scenarios. Then the above proposal 1 may not be acceptable. First of all, we would like to understand the use case and benefit of such configuration. It may be able to save some signaling overhead and UE storage space by restricting to one single type RS configuration, but the saving is arguably quite small. Having said that, we are open to further discuss.

Observation 1. The use case for NW to configure only single type of RS for some or all candidate cells is unclear.

If the use case is justified (i.e. the RS configuration for candidate cells contains only one RS type), we may need something else than the above proposal 1. Because the serving beam is dynamically indicated by TCI state, it can happen that the semi-static configured RS type for candidate cells is different than the QCL RS of the indicated TCI state of the serving cell. Therefore, following serving cell might not be working. For example, the QCL RS of the indicate TCI state of serving cell is CSI-RS but the configured RS set for candidate cells only include SSB. We think it is feasible to use SSB as RS type since CSI-RS can be sourced to SSB eventually via QCL chain. This applies to candidate cell CSI-RS as well. The following table summarizes the RS type determination handling.

Table 1: The RS type determination handling

| QCL RS of indicated TCI state in serving cell | Configured RS in candidate cells | Which RS type to use for L1 measurement | Which RS to measure |
| --- | --- | --- | --- |
| CSI-RS | SSB | SSB | For serving cell: SSB QCLed with the QCL RS of the indicated TCI state For candidate cell: configured RS |
| SSB | CSI-RS | SSB | For serving cell: QCL RS of indicated TCI state For candidate cell: SSB QCLed with the configured RS |

There is another scenario where SSB needs to be used for evaluation is in case that no TCI state is indicated in the serving cell. This can happen after initial access and before any TCI state is indicated to the UE. UE just keeps using the SSB beam selected from the random access procedure in the serving cell. Then for candidate cell evaluation, SSB should also be used.

To summarize, we have the following proposals.

Proposal 2. If a single type of RS is configured for candidate cells, the RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell;

or if this is not possible (e.g. RS configuration for candidate cell contains only SSB but serving cell indicated TCI state has CSI-RS as QCL RS, or no TCI state is indicated in the serving cell), RS type is SSB.

Proposal 3. If SSB is decided as RS type for event evaluation but the indicated TCI state has CSI-RS (for serving cell) or configured RS set has only CSI-RS (for candidate cell), UE measures the SSB which is QCL source of the indicated or configured CSI-RS.

## TTT operation

In the previous meeting, it was discussed whether TTT is applied per beam or per cell without consensus. For example, beam A satisfies the event and TTT starts, during TTT, another beam B instead of beam A satisfies the event. Should TTT be re-started or just continued? From our perspective, it makes sense that in this case TTT is continued (i.e. not reset if candidate beam changes), because otherwise, the triggering of the event can be delayed too much. Furthermore, this is more aligned with the agreement “Any beam in candidate RS configuration can be used for LTM event evaluation.” Eventually, the LTM is to decide whether to change the serving cell to another cell. Therefore, TTT per cell is more aligned with such purpose.

In previous meeting discussion, some companies mentioned the compromised option where TTT operation is per CSI-RS set. We would like to clarify the intention of CSI-RS set is to include all configured RS for this candidate cell. We should not introduce another layer of configuration to apply TTT, e.g. to a subset of all configured RSs of a given candidate cell.

Proposal 4. TTT is per candidate cell (i.e. across all configured RSs for a given candidate cell) rather than per beam (i.e. per configured RS), meaning that TTT is not reset if candidate beam satisfying the same event changes within the same candidate cell.

There was another related discussion on whether TTT should be reset if current serving beam is changed in serving cell. In our opinion, as long as the new serving beam still meet the event condition, there is no need to reset TTT. Otherwise, the reporting of event would be delayed unnecessarily. This is also aligned with the same handling as candidate cell as proposal 4.

Proposal 5. TTT need not be reset if current serving beam is changed as long as new beam still satisfies the event condition.

## LTMx event reporting for early synchronization.

In RAN2#126 meeting the following agreement was made regarding LTM measurement reporting design.

Event triggered L1 measurement should be designed for the following LTM purposes:-

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

Accordingly, if event based L1 measurements are to be triggered for early synchronization and cell switch procedures, it is a common understanding that network configures such trigger conditions at UE to report L1 measurements of candidate beam/cell, when LTM procedures are expected to be performed (like early synchronization or cell switch) towards one (or more) candidate beam/cell. The following figure illustrates such design requirement in LTM signalling flow diagram.

Figure 1: LTM procedure (source – TS38.300v18.1; 9.2.3.5 L1/L2 triggered mobility)

Proposal 6: Signaling flow diagram for Rel-19 LTM to be updated with L1 measurement reporting flow from UE during early synchronization procedure as seen in Figure-1, possibly triggered and reported using event reporting conditions.

Further if network configures UE with event trigger condition to report L1 measurements during early synchronization and cell switch procedures, there could be multiple ways to provide such configurations to UE.

Proposal 7: RAN2 to consider following options to configure event trigger conditions to be reported during early synchronization and cell switch procedures.

Option1: One LTMx event can be configured at UE with event condition (i.e. entering condition) to report for both early synchronization and cell switch procedures.

- FFS: How entering condition is configured to trigger during different LTM procedure stages.

Option2: Multiple LTMx events can be used to configure event conditions at UE, where at least one LTMx event is configured dedicated to early synchronization procedure and the remaining (one or more) dedicated to cell switch procedure.

- FFS: How redundant measurement reports can be avoided when one beam/cell is triggered with multiple LTMx event conditions configured one for early synchronization and the other for cell switch.

# Conclusion

In this paper, we discuss some remaining issues for L1 measurement event. The observations and proposals are as follows.

Same RS type

Observation 1. The use case for NW to configure only single type of RS for some or all candidate cells is unclear.

Proposal 1. To ensure the same RS type between serving and candidate cells for event LTM3 and LTM5 evaluation, NW configures both types of RS (i.e. SSB and CSI-RS) as measurement RS for each candidate cell, and which type to use follows the current beam in serving cell (i.e. QCL RS associated with the indicate TCI state in the serving cell).

FFS: whether and how to address the case where NW configures single type of RS for candidate cells.

Proposal 2. If a single type of RS is configured for candidate cells, the RS type for event evaluation should follow the QCL RS associated with the indicated TCI state in the serving cell;

or if this is not possible (e.g. RS configuration for candidate cell contains only SSB but serving cell indicated TCI state has CSI-RS as QCL RS, or no TCI state is indicated in the serving cell), RS type is SSB.

Proposal 3. If SSB is decided as RS type for event evaluation but the indicated TCI state has CSI-RS (for serving cell) or configured RS set has only CSI-RS (for candidate cell), UE measures the SSB which is QCL source of the indicated or configured CSI-RS.

TTT operation

Proposal 4. TTT is per candidate cell (i.e. across all configured RSs for a given candidate cell) rather than per beam (i.e. per configured RS), meaning that TTT is not reset if candidate beam satisfying the same event changes within the same candidate cell.

Proposal 5. TTT need not be reset if current serving beam is changed as long as new beam still satisfies the event condition.

Triggering conditions for early sync and cell switch

Proposal 6: Signaling flow diagram for Rel-19 LTM to be updated with L1 measurement reporting flow from UE during early synchronization procedure as seen in Figure-1, possibly triggered and reported using event reporting conditions.

Proposal 7: RAN2 to consider following options to configure event trigger conditions to be reported during early synchronization and cell switch procedures.

Option1: One LTMx event can be configured at UE with event condition (i.e. entering condition) to report for both early synchronization and cell switch procedures.

- FFS: How entering condition is configured to trigger during different LTM procedure stages.

Option2: Multiple LTMx events can be used to configure event conditions at UE, where at least one LTMx event is configured dedicated to early synchronization procedure and the remaining (one or more) dedicated to cell switch procedure.

- FFS: How redundant measurement reports can be avoided when one beam/cell is triggered with multiple LTMx event conditions configured one for early synchronization and the other for cell switch.