3GPP TSG-RAN WG2 Meeting #128	R2-2410309

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-127bis meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT mechanism as well as the Diversity Slotted-Aloha (DSA).

Agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we provide our view on possible enhancements for CB-msg3 EDT, DSA as well as the enhancement for Msg4.

# 2	Discussion

## 2.1	Support of CB-Msg3 EDT

CB-Msg3 EDT resource configuration:

In the RAN2-127bis meeting, it was agreed the cell-specific PUSCH resource for CB-Msg3 transmission is configured at least via system information.  The CB-msg3 resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission. The UE selects the corresponding PUSCH resource from a proper CE level based on the measured RSRP level against the CE level RSRP-threshold. For each of the CE level, the configured CB-Msg3 resource is shared between UEs and multiple UEs may transmit Msg3 simultaneously, thus it may cause a collision which will bring some adverse consequence such as the increased signalling overhead, additional UE power consumption and waste of the resources. Since the CB-Msg3 EDT resource configuration is provided to UE via SIB, the NW has no idea how many idle UEs will use the resource simultaneously which may cause unexpected Msg3 collision. To avoid very high packet loss anyways due to a high collision rate, we believe the NW should have a mechanism to control the possible Msg3 collision in order to mitigate the Msg3 collision impact. The legacy access barring check (as defined in TS36.331 clause 5.3.3.11) to control the RACH collision can be used as baseline.

Proposal 1: If cell-specific CB-Msg3 EDT resource is provided via SIB, RAN2 to introduce a mechanism to control Msg3 collisions. The legacy access barring check mechanism to control RACH collision can be used as baseline. FFS on the details.

Since the CB-Msg3 resource is configured by NW, the presence of the resource configuration can be regarded as a common control flag to indicate that UEs can use the shared resource for direct Msg3 transmission in cell level.

Proposal 2: The presence/absence of the shared CB-Msg3 EDT resource configuration can be regarded as common control flag to enable/disable CB Msg3 transmission in cell level.

CB-RNTI and Contention Resolution:

For CB-Msg3 EDT transmission, since there is no Msg2 to schedule the Msg3 transmission, the TC-RNTI to be used for Msg3 transmission and Msg4 scheduling cannot be indicated in RAR. In the RAN2-127bis meeting, it was agreed the RNTI used to schedule Msg4 is derived from the PUSCH resource corresponding to the Msg3 transmission occasion while it is not clear whether the derived RNTI can be used for Msg3 transmission. According to TS36.213, in legacy, the TC-RNTI assigned by RAR or the UE’s C-RNTI can be used for scrambling initialization of Msg3 NPUSCH transmission. For an RRC idle UE which triggers the CB-Msg3 transmission, it is natural to use the RNTI derived based on the PUSCH resource used for the Msg3 transmission.

Excerpt from TS 36.213: ### 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel … If a Temporary C-RNTI is set by higher layers, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by Temporary C-RNTI. Otherwise, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by C-RNTI.

Proposal 3: The RNTI used for scrambling initialization of PUSCH for contention based Msg3 transmission is derived based on the resource associated to the PUSCH occasion used for the CB-Msg3 EDT transmission.

In legacy 4-step RACH the RACH collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 (i.e., first 48 bits of CCCH SDU) and NW acknowledgement in Msg4. In legacy PUR mechanism (Msg3 transmission without Msg1/RAR), there is no need for contention resolution since the PUR resource allocated via RRCConnectionRelease message is dedicated to a specific UE (i.e., contention free resource). For CB-Msg3 transmission in shared resource, there may occur Msg3 collision between UEs. Therefore, the contention resolution scheme should be supported. The most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 4: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

CB-Msg3 EDT Fallback:

In legacy EDT, the CP-EDT and UP-EDT procedure can be fallback by network to RRC connection establishment. For example, if the MO data trigger further downlink information from application or if the UE may send part of its data or first packet in EDT and include BSR that will trigger connection setup for remaining part of the packets, then NW may move the UE to RRC connection.

Figure 1: CB-Msg3 CP-EDT fallback to RRC connection establishment

For CB-Msg3 EDT, the same mechanism can also be applied.

Proposal 5: The CB-Msg3 EDT can fallback to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

If the UE moves to RRC Connected mode, the derived RNTI from the resource associated to the CB-Msg3 EDT PUSCH occasion cannot be used as dedicated C-RNTI for the UE for further dedicated communications between UE and NW as it is not a unique UE identifier. Therefore, a dedicated C-RNTI needs to be allocated and indicated in Msg4 for each UE that needs to establish the RRC connection and perform the further communication with the eNB.

Proposal 6: For CB-Msg3 EDT, a dedicated RNTI needs to be indicated in Msg4 for UE that fallback to legacy RRC connection establishment.

The CB-Msg3 procedure is subject to failures due to, e.g., RF conditions, interference or collision. Thus, another important issue to consider is the UE behavior in case of a CB-Msg3 failure or consecutive CB-Msg3 failures. The UE will initially select a CE level based on the measured RSRP and the configured RSRP threshold. After a failure in CB-Msg3 transmission, the UE could perform another attempt. If it fails multiple times in the selected CE level, the first issue is whether the UE should increase the CE level hence it can have multiple CB-Msg3 transmission retries using the next CE level. In our understanding, if the transmission failures are caused by RF conditions, it is reasonable to have a higher CE level which is typically configured with higher repetitions to support more robust transmission. While if the transmission failures are caused by resource collision in the limited CB-Msg3 resource, it seems not reasonable to use the next CE level which may consume more resources.

Observation 1: In case of consecutive CB-Msg3 transmission failures, the increase of the CE level should take into account whether it is caused by poor RF conditions or resource collisions.

After a number of CB-Msg3 transmission failures, the UE may be fallback to legacy RACH procedures such as 4-step RACH or legacy RACH-based EDT procedure. Furthermore, it is important to ensure that the UE may have a successful data transmission within a time limit, assuming that the UE has periodic data transmission requirement as applied for the legacy PUR. Compared to 4-step RACH or RACH-based EDT procedure, Msg1/Msg2 can be skipped in PUR procedure. Therefore, the fallback from CB-Msg3 EDT to PUR may also be considered.

Proposal 7: The UE may fallback from CB-Msg3 EDT to 4-step RACH, EDT and PUR.

## 2.2	Support of DSA for CB-Msg3 EDT

For DSA, the UE is allowed to transmit more than one copy of Msg3 on the contention-based resources (i.e., more than one replica). The NW individually decodes each replica without soft combining. If any of the replicas is successfully decoded by NW, the NW can successfully receive the packet accommodated in Msg3.  Firstly, we understand the DSA mechanism is not the same as a reattempt of the Msg3 transmission after a failure, i.e., it is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

Observation 2: It is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

As RAN2 agreed the RNTI used to schedule Msg4 is derived from the resource used for CB-Msg3 transmission, it may cause trouble in DSA in which multiple RNTIs may be derived based on multiple Msg3 replicas.

Figure 2: Msg3 transmission and Msg4 monitoring in DSA using Msg3 resource derived RNTI

As illustrated in Figure 2, the RNTI X, Y and Z may be derived based on the individual Msg3 replica resource. On one hand, it may cause the UE to monitor multiple RNTI(s) for Msg4 after it sends multiple replicas. On the other hand, NW should avoid duplicated/redundant Msg4 to different replica of Msg3 as different RNTI may be derived for different replica of same Msg3 and NW cannot identify the replica of the same UE just based on RNTI. Of course, NW may store and compare the decoded Msg3 SDU contents from different replicas to identify the same UE but it may result in complex NW implementation.

Observation 3: Due to multiple RNTIs may be derived in DSA for different replicas, it may cause two impacts to UE and NW: 1) UE has to monitor multiple RNTIs for Msg4. 2) NW has to avoid duplicated Msg4 delivery for different RNTI.

In our understanding, the DSA impact to UE and NW should be minimised as much as possible. Though multiple RNTIs may be derived in DSA, one RNTI should be selected for all the replicas with the same Msg3 content.

Proposal 8: One RNTI should be selected for all the replicas with the same Msg3 content.

Another important aspect is early terminating the replica transmissions once Msg3 is successfully decoded by NW. Due to repetitions may be configured together with DSA to extend the UL coverage for IoT, the resource utilization efficiency may be reduced with the increase of the replica number. Therefore, when the NW successfully decodes the Msg3, it’s better to respond with Msg4 as early as possible hence UE can stop the remaining Msg3 replica transmissions.

Proposal 9: Once Msg3 is successfully decoded by NW, early terminating the remaining replica transmission with the same Msg3 content should be supported.

In this sense, the UE should start the contention resolution timer after transmission of the first replica.

Proposal 10: The UE starts contention resolution timer after the first replica transmission.

Furthermore, the UE cannot monitor PDCCH for Msg4 when it transmits Msg3 replicas, because the IoT NTN UE is assumed as half-duplex hence the UE cannot transmit and receive simultaneously. This may affect when NW can schedule Msg4. The UE will miss the NW-scheduled Msg4 if the UE is transmitting the next Msg3 replicas at the same time. Therefore, RAN2 needs to discuss how to handle the potential collision between Msg3 replica transmission and the Msg4 reception.

Proposal 11: RAN2 to discuss how to handle the potential collision between Msg3 replica transmission and the Msg4 reception.

Since the NW handling for UE with or without Msg3 replica transmissions is different, the UE should indicate to NW in Msg3 whether DSA is used for CB-Msg3 EDT. FFS it is indicated implicitly or explicitly.

Proposal 12: UE should indicate to NW in Msg3 whether DSA is used for CB-Msg3 EDT. FFS it is indicated implicitly or explicitly.

## 2.3	Efficient Msg4 delivery

On the objective of efficient delivery of msg4, according to the WID [1], the intention is to identify methods to de-couple the UL from the DL as much as possible hence unlock the additional UL capacity potential. We understand it is to avoid the PDCCH/PDSCH congestion which may impact the UL scheduling as well as the UL capacity. Therefore, how to reduce the Msg4 overhead should be studied.

In legacy, a dedicated Msg4 with both PDCCH and PDSCH is always needed for the corresponding Msg3 for contention resolution. To reduce the Mgs4 overhead, one possible enhancement is to reduce the PDCCH usage for Msg4 scheduling. For example, it seems a possible optimization way-forward to support one Msg4 to deliver contention resolution result for multiple UEs or preconfigure the PDSCH for Msg4 delivery based on Msg3 scheduling to save the scheduling PDCCH at least. The other possible enhancement is to reduce the PDSCH overhead for Msg4 content delivery. For example, some of the IEs in Msg4 can be reduced or reused, especially considering the case when multiple UEs can be scheduled in a single Msg4.

Proposal 13: To support efficient delivery of Msg4, RAN2 to consider below options:

- Option1: Multiplex several UE contention-resolution identities into one Msg4.

- Option2: Reduce common IEs if multiple UEs are addressed in one Msg4.

- Option3: Preconfigure the PDSCH for Msg4 delivery based on Msg3 scheduling.

# 3	Conclusion

This document has made the following observations and proposals:

Support of CB-Msg3 EDT

Proposal 1: If cell-specific CB-Msg3 EDT resource is provided via SIB, RAN2 to introduce a mechanism to control Msg3 collisions. The legacy access barring check mechanism to control RACH collision can be used as baseline. FFS on the details.

Proposal 2: The presence/absence of the shared CB-Msg3 EDT resource configuration can be regarded as common control flag to enable/disable CB Msg3 transmission in cell level.

Proposal 3: The RNTI used for scrambling initialization of PUSCH for contention based Msg3 transmission is derived based on the resource associated to the PUSCH occasion used for the CB-Msg3 EDT transmission.

Proposal 4: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

Proposal 5: The CB-Msg3 EDT can fallback to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

Proposal 6: For CB-Msg3 EDT, a dedicated RNTI needs to be indicated in Msg4 for UE that fallback to legacy RRC connection establishment.

Observation 1: In case of consecutive CB-Msg3 transmission failures, the increase of the CE level should take into account whether it is caused by poor RF conditions or resource collisions.

Proposal 7: The UE may fallback from CB-Msg3 EDT to 4-step RACH, EDT and PUR.

Support of DSA for CB-Msg3 EDT

Observation 2: It is expected in DSA that all or at least multiple replicas of Msg3 may be transmitted before UE declaring the contention resolution failure.

Observation 3: Due to multiple RNTIs may be derived in DSA for different replicas, it may cause two impacts to UE and NW: 1) UE has to monitor multiple RNTIs for Msg4. 2) NW has to avoid duplicated Msg4 delivery for different RNTI.

Proposal 8: One RNTI should be selected for all the replicas with the same Msg3 content.

Proposal 9: Once Msg3 is successfully decoded by NW, early terminating the remaining replica transmission with the same Msg3 content should be supported.

Proposal 10: The UE starts contention resolution timer after the first replica transmission.

Proposal 11: RAN2 to discuss how to handle the potential collision between Msg3 replica transmission and the Msg4 reception.

Proposal 12: UE should indicate to NW in Msg3 whether DSA is used for CB-Msg3 EDT. FFS it is indicated implicitly or explicitly.

Efficient Msg4 delivery

Proposal 13: To support efficient delivery of Msg4, RAN2 to consider below options:

- Option1: Multiplex several UE contention-resolution identities into one Msg4.

- Option2: Reduce common IEs if multiple UEs are addressed in one Msg4.

- Option3: Preconfigure the PDSCH for Msg4 delivery based on Msg3 scheduling.

# Reference

[1] RP- RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3