3GPP TSG-RAN WG2 Meeting #128	R2-2410310

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on support of PWS for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The R19 WID [1] for IoT NTN has been updated in RAN plenary#105 meeting to cover the objective for support of PWS (Public Warning System). In details, the support of PWS messages for NB-IoT is expected to reuse the LTE mechanisms. Furthermore, the support of geo-fencing function for ETWS will be considered based on NR NTN outcome.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

Following agreements have been reached in RAN2-127bis meeting.

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this contribution, we elaborate more on remaining issues related to PWS for NB-IoT NTN.

# 2	Discussion

## 2.1	PWS indication in Paging-NB

The ETWS and CMAS notifications can occur at any point in time. In legacy PWS for eMTC, the NW may set the etws-indication and cmas-indication bits in Paging message or Direct Indication information to alert UEs to start receiving SIB1 and corresponding SIBs for ETWS and CMAS notifications. To enable PWS functionality for NB-IoT NTN, RAN2 has agreed the indications can be included in Direct Indication Information while it is FFS whether the indications are also added in Paging-NB message.

In our understanding, the same mechanism as eMTC to support the indications in both Paging-NB and Direct Indication information can be reused in NB-IoT NTN.  In both of the DCI to schedule paging message for eMTC (i.e., DCI format 6-2) and NB-IoT (i.e., DCI format N2), the DCI has a flag that indicates it is either for direct indication or scheduling of the paging message. So, if RAN2 only supports direct indication it means that when the network both has to page a UE and provide the PWS it cannot be done in one paging occasion, because the paging DCI can only indicate one of the two (paging message scheduling or direction indication). If RAN2 also supports PWS in the paging message the network will be able to schedule both the paging message and include the PWS there. Therefore, we believe that supporting the PWS indication in both Paging-NB and Direct Indication information messages can provide more flexibility to NW implementations.

#### TS36.212 6.4.3.3 DCI Format N2 DCI format N2 is used for paging, direct indication, scheduling of one NPDSCH codeword carrying SC-MCCH in one cell, and notifying SC-MCCH change. The following information is transmitted by means of the DCI format N2: - If the format N2 CRC is scrambled by P-RNTI: - Flag for paging/direct indication differentiation – 1 bit, with value 0 for direct indication and value 1 for paging - Else if the format N2 CRC is scrambled by a SC-RNTI: - Information for SC-MCCH change notification – 1 bit as defined in clause 5.8a of [6] - If the format N2 CRC is scrambled by P-RNTI and Flag=0: - Direct Indication information – 8 bits provide direct indication of system information update and other fields, as defined in [6] - Reserved information bits are added until the size is equal to that of format N2 with Flag=1 - If the format N2 CRC is scrambled by P-RNTI and Flag=1, or if the format N2 CRC is scrambled by SC-RNTI: - Resource assignment – 3 bits as defined in clause 16.4.1.3 of [3] - Modulation and coding scheme – 4 bits as defined in clause 16.4.1.5 of [3] - Repetition number – 4 bits as defined in clause 16.4.1.3 of [3] - DCI subframe repetition number – 3 bits as defined in clause 16.6 of [3]

Proposal 1: Add the PWS indication (i.e., etws-Indication, cmas-Indication) in Paging-NB message.

## 2.2	PWS acquisition for UE in connected mode

In RAN2-127bis meeting, there was some discussion on whether to support PWS for NB-IoT NTN UEs in RRC Connected mode but there is no consensus reached yet.

Continue the discussion in the next meeting(s) on whether and how to support PWS for NB-IoT NTN UEs in Connected mode

In Rel-16 eMTC, PWS for RRC Connected UE was introduced. The UE monitors MPDCCH in CSS with the SI-RNTI in which the ETWS indication and/or CMAS indication is included in the DCI (i.e., DCI format 6-1A/1B). The UE remains in RRC Connected after receiving the indication for corresponding SIB acquisition. We share the view that the eMTC UE should be able to receive using multiple narrowband hence it is intended for non-BL UEs operating in CE mode. However, for NB-IoT, currently the RRC Connected UE is not required to monitor the system information change or paging. The eMTC mechanism for RRC Connected may not be feasible for NB-IoT UE which targets cost-efficient and simple/low-complexity devices. Instead, we think the legacy procedure would be used in case PWS is to be delivered to UEs. The NW may release the UEs to RRC idle for the corresponding SIBs acquisition which is similar to the approach on how RRC Connected UE acquires essential system information change (as described in NOTE 2 below).

#### TS36.331 5.2.1.3 System information validity and notification of changes … NOTE 2: Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

One potential issue for the legacy procedure is the extra delay for scheduling of RRC Release messages which may result in the delay of ETWS/CMAS message delivery. For example, the delay requirement on ETWS primary notification (i.e., within 4 seconds) may not be fulfilled. In RAN2-127bis meeting, RAN2 has agreed to clarify that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios. In addition to the case where the UE is in eDRX mode, the case where the UE is in RRC Connected mode could be another case.

RAN2-127bis agreement:

=> Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

Proposal 2: ETWS/CMAS acquisition is not supported for UE in RRC Connected state.

Proposal 3: RAN2 to clarify the ETWS, CMAS, PWS delay requirement may not be met when the NB-IoT UE is in RRC Connected state.

## 2.3	Support PWS in S&F operation

When the cell capable of store-and-forward operates in normal mode the PWS transmission can be supported for the areas that is under the coverage of the cell at the time of normal mode operation. When the cell switches to SF mode the time of delivery of PWS depends on the actual service area and the time the cell takes to start serving the new service area.  This impacts the delivery time requirements of PWS.

Observation 1: PWS delivery when cell operates in SF mode may have impacts on meeting delay requirements of PWS services.

From RAN2 perspective the UE capability for SF and PWS support are independent features and it is up to network implementation to configure these services together. However whether such support is needed for PWS needs to be decided in SA2 based on service requirements and impacts.

Observation 2: RAN2 assumes no additional impacts in supporting PWS functionality in SF operation.

Based on the above analysis we propose that support of PWS in SF operation is decided by SA2. RAN2 can indicate that this can be supported without additional signalling impacts

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

## 2.4	PWS delivery during eDRX

eDRX is a key feature for NB-IoT devices that enhances their power efficiency and extends battery life. This makes it as a key enabler for NB-IoT in massive IoT deployments. Tough RAN2 agreed to relax the PWS delivery time for NB-IoT NTN, it still worths to further study efficient and fast delivery in order to reach the original delay requirement as close as possible, especially for ETWS primary notification which is time-critical.

Proposal 5: RAN2 to further study how to reduce the PWS delivery latency during eDRX.

Furthermore, to further reduce the energy consumption for paging monitoring towards a UE configured with eDRX, the wake-up signals (WUS) feature can be used. The typical scenario is that NW configure a short eDRX cycle configurations to UE to support the case where the network command expects a UE response within specific duration. However, if PWS is supported and the transmission is indicated via paging messages (NB-IoT paging) or direct indication information message, it may wake-up all the UEs in the satellite coverage area if the current WUS is used. For PWS scheduling in resource and energy efficient manner use of separate WUS for PWS services can be considered.

Proposal 6: RAN2 to consider support of PWS specific WUS configuration for energy efficient PWS scheduling.

## 2.5	Geofencing for ETWS

As part of the PWS objective for IoT NTN, there is a Note to consider supporting indication of intended service area for ETWS based on NR NTN discussions/outcome.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In Rel-19 NR NTN, there are similar discussions for ETWS notifications regarding whether the geofencing for an area smaller than an NTN cell should be supported. As a result, RAN2 decided to send LS to SA1 [2] in RAN2-127 meeting and asked for feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in an NTN cell, considering the delay requirements for ETWS primary notification. Therefore, for IoT NTN, we can wait for the conclusion from NR NTN first.

Proposal 7: Wait for NR NTN conclusion to decide whether to support indication of intended service area for ETWS for IoT NTN.

# 3	Conclusion

This document has made the following observations:

Observation 1: PWS delivery when cell operates in SF mode may have impacts on meeting delay requirements of PWS services.

Observation 2: RAN2 assumes no additional impacts in supporting PWS functionality in SF operation.

And proposed the following:

Proposal 1: Add the PWS indication (i.e., etws-Indication, cmas-Indication) in Paging-NB message.

Proposal 2: ETWS/CMAS acquisition is not supported for UE in RRC Connected state.

Proposal 3: RAN2 to clarify the ETWS, CMAS, PWS delay requirement may not be met when the NB-IoT UE is in RRC Connected state.

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

Proposal 5: RAN2 to further study how to reduce the PWS delivery latency during eDRX.

Proposal 6: RAN2 to consider support of PWS specific WUS configuration for energy efficient PWS scheduling.

Proposal 7: Wait for NR NTN conclusion to decide whether to support indication of intended service area for ETWS for IoT NTN.

# Reference

[1] RP- RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-2407823 LS on requirements for ETWS primary notification