3GPP TSG-RAN WG2 Meeting #128	R2-2410321

# Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on on-demand SIB1

Document for:	Discussion

Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2]. RAN2#127bis meeting is the first meeting of the normative phase, following are the related agreements [3]:

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we provide our views on the open issues related OD-SIB1.

Discussion

WUS configuration

In last meeting, RAN2 agreed to use a new SIB for WUS configuration delivery, but it’s not clear the  maximum number of WUS configurations. Considering that Cell A is more possibility to be deployed on a carrier frequency of bottom network, while NES cell is used as a hotspot. Therefore, it is likely that Cell A and NES cell are on the different carrier frequency. Based on this, the maximum number of inter frequency cells supported can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 1: The maximum number of inter frequency cells supported can be considered as the maximum  number of WUS configuration, i.e., 16.

As to the content of WUS configuration, at least RACH related configurations are included in WUS configuration, as discussed in both RAN1 and RAN2. However, except for these, the configurations for SIB1 reception should also be indicated to the UE. One way is to deliver the related parameters in RAR, and the other way is to include the parameters in the WUS configuration. In our view, the second way is preferred to avoid changes on RAR and to reduce the overhead on MAC PDU transmission. As to the detail parameters indicated in WUS configuration, it can be further discussed in RAN1.

Proposal 2: SIB1 reception related parameters can be indicated in WUS configuration, and the detail parameters included is decided by RAN1.

Cell barring related cases

In last meeting, RAN2 made some agreements on OD-SIB1 request failure:

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

Besides, in RAN2#126 meeting, following agreement was made for OD-SIB1 request failure [4]:

- UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

Since the RACH procedure in a NES cell without SIB1 delivery is only used for OD-SIB1request, but not for other purpose, therefore, we think after MAC layer indicated the RACH failure to upper layers, the UE upper layer can considered the cell as barred.

Proposal 3: After MAC layer indicated the RACH failure to upper layers, the UE upper layer can considered the cell as barred.

Besides, there’s some other cell barring related cases.

- Cell Barring in initial cell selection

Following is the description of SIB1 acquisition in TS 38.331[5]:

…… 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> perform the actions as specified in clause 5.2.2.5. ……

ssb-SubcarrierOffset  is a parameter corresponds to K_SSB defined in TS 38.213. And following agreements were made in RAN1#118bis meeting:

Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

During initial cell selection, the NES UE may identify the NES cell as a cell without SIB1 delivery as legacy UE, and treated NES cell as barred according to the K_SSB value of the cell. And during this phase, a NES UE cannot identify whether this cell is a NES cell or a legacy cell without SIB1 delivery.

Observation 1: UE cannot identify a cell is a NES cell or a legacy cell without SIB1 based on K_SSB.

Proposal 4: UE treats the NES cell as barred based on K_SSB as legacy in initial cell selection.

- Cell barring after camping on a non-NES cell

Based on Proposal 4, the UE would select to a non-NES cell, and if UE’s camping cell is an anchor cell, the UE may acquire WUS configuration. Once UE acquires the WUS configuration, it’s reasonable to treat the cell barred before which is configured with WUS configuration as not barred.

Proposal 5: If a cell is barred, but the UE acquires its WUS configuration for OD-SIB1 request and identify it’s a NES cell, the UE treats the cell as not barred.

Conclusions

In this contribution, we analyse the OD-SIB1 related open issues , following are the proposals and observations:

Proposal 1: The maximum number of inter frequency cells supported can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 2: SIB1 reception related parameters can be indicated in WUS configuration, and the detail parameters included is decided by RAN1.

Proposal 3: After MAC layer indicated the RACH failure to upper layers, the UE upper layer can considered the cell as barred.

Observation 1: UE cannot identify a cell is a NES cell or a legacy cell without SIB1 based on K_SSB..

Proposal 4: UE treats the NES cell as barred based on K_SSB as legacy in initial cell selection.

Proposal 5: If a cell is barred, but the UE acquires its WUS configuration for OD-SIB1 request and identify it’s a NES cell, the UE treats the cell as not barred.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #102;

- RP-242354, Revised WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #105;

- Report of RAN2 #127bis meeting

- Report of RAN2 #126 meeting;

- TS 38.331.