3GPP TSG-RAN WG2 Meeting #128	R2-2410324

# Orlando, USA, Nov. 18th – 22nd, 2024

# Agenda item:	8.6.4

# Source:	CMCC

# Title:	Discussion on Conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#127bis meeting, RAN2 reached the following agreements on conditional LTM [1]:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this paper, we further discuss the open issues considering different phase of conditional intra-CU LTM.

- Discussion

  - Preparation of conditional LTM

In last meeting, RAN2 agreed some general principles for conditional LTM design of execution conditions, but there’s still some FFS.

The first open issue about conditional LTM execution condition is whether CHO conditions can be used. Though one motivation of introduction Event LTM3 and Event LTM5 is for LTM decision, it’s the network, who has more information than measurement results reported by the UE to make mobility decision. When it comes to conditional LTM, the mobility decision is decided by the UE itself, if only one beam evaluation is considered, it may cause frequent conditional mobility. Therefore, we think UE should consider cell level measurement results in conditional LTM. To achieve cell level measurement results, one way is to reuse CHO condition, and the other way is evaluating more than one beam for conditional LTM execution.

Observation 1: If only one beam related event(s) is considered, it may cause frequent conditional LTM.

Considering the advantage of LTM compared with L3 mobility is that LTM using L1 measurement result without L3 filtering, which can facilitate a fast mobility procedure. If CHO condition is configured together with beam level events(Event-A3 like or Event-A5 like), it will extend the mobility decision latency compared with evaluation of multiple beams. Besides, as L1 measurement results and L3 measurement results change granularity is different, if trigger events based on L1 and L3 measurement results are configured together, it may be difficult for the two kinds of trigger events satisfied for conditional LTM execution. Therefore, We prefer to evaluate multiple beams for conditional LTM execution. As to the number of beams for evaluation, it can be configured by the network, and the maximum number can be further discussed.

Proposal 1: For conditional LTM execution, more than one beam should be evaluated.

Proposal 2: The network can configure the number of beams for evaluation, and the maximum number can be further discussed.

For CHO, an execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in TS 38.331). However, based on the discussion above, if cell level measurement results are considered, the number of execution conditions can be enlarged compared to CHO.

Proposal 3: The execution condition may consist of  one or two trigger condition(s) (Event LTM3-like, or Event LTM5-like).

What’s more, for CHO, Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell [2]. We think similar principle can be applied in conditional LTM, only one single RS type  (CSI-RS and SSB) is configured for beam level trigger conditions of the evaluation of CHO execution condition of a single candidate cell. .

Proposal 4: Only one single RS type (CSI-RS and SSB) is configured for beam level trigger conditions of the evaluation of CHO execution condition of a single candidate cell.

Besides, since that L1 measurement capability and LTM capability are decouple. RAN2 should clarify whether L3 measurement results triggered conditional LTM is supported, since this has impact not only on RAN2 execution condition discussion, but also on RAN4 conditional LTM related discussion, and based on RAN4 agreements [3], RAN4 will follow RAN2’s conclusion. We think there’s no reason to exclude LTM supporting UE without L1 measurement capability to support only legacy intra-CU LTM but not conditional intra-CU LTM.

Issue 1-1-3: Measurement requirements on conditional LTM < Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

Proposal 5: L3 measurement based conditional LTM is supported.

For LTM supporting UE without L1 measurement capability, only L3 measurement results can be used to trigger LTM, therefore, CHO trigger conditions can be reused for conditional LTM.

Proposal 6: For the LTM supporting UE without L1 measurement capability, CHO trigger conditions can be configured for conditional LTM.

Proposal 7: RAN2 sends LS to RAN4 on L3 measurement based conditional LTM related agreements.

The other issue in preparation phase of conditional LTM is about the candidate set. Since the UE can be configured with a set of candidate cells for legacy LTM, it is not clear whether the same set of candidate cells is configured for conditional LTM. In our view, it’s network implementation to configure the same or different cells for conditional LTM and legacy LTM.

Proposal 8: It’s network implementation to configure the same or different cells for conditional LTM and legacy LTM.

  - Early synchronization

Early synchronization is a key feature of LTM for mobility latency reduction. And RAN2 agreed that both UE-based TA measurement and PDCCH ordered early TA acquisition are supported for conditional LTM. UE-based TA measurement is up to UE implementation in Rel-18 LTM, it can be inherited here. But for PDCCH ordered TA acquisition, there’s some issues for further discussion.

How to ensure the network to trigger PDCCH order at a right time for a right candidate cell

For legacy LTM, when the source orders the UE for TA acquisition by PDCCH order is network implementation, based on UE’s measurement results. And once the source cell gets the TA value, it can send LTM cell switch command including the TA value. If the TA is invalid, the source cell can trigger LTM with indicating RACH related parameters.

Observation 2: In legacy LTM, the source cell may trigger PDCCH order based on UE’s measurement results.

Similarly, the same principle can be applied to conditional LTM, in other words, the source cell may send PDCCH order to the UE for early TA acquisition based on available measurement results. However, considering that conditional LTM is decided by the UE itself, to achieve a more accurate and valid TA value, and an extra condition can be configured to the UE to trigger TA acquisition, i.e. an offset based on the trigger condition of conditional LTM.

Proposal 9: The source cell may send PDCCH order based on UE’s measurement results or UE’s indication/request.

How can UE acquire the TA value from the network

In legacy intra-CU LTM, the TA value is delivered to the UE in LTM cell switch command MAC CE, But for conditional LTM, the cell switch command is omitted. In our view, there’s no need to introduce any other new MAC CE for the TA value delivery, UE can acquire the TA value via RAR. Considering the impact to serving cell data reception, it’s preferred that the UE acquires the TA value via serving cell’s RAR.

Proposal 10: UE  acquires the TA value via serving cell’s RAR.

  - Execution and Completion of conditional LTM

Besides, it was agreed to support RACH-based conditional LTM, it should be discussed that how to configure the RACH related configuration to the UE, as in legacy LTM, RACH parameters are indicated in LTM cell switch command. In our view, RACH configurations can be configured by RRC signalling as a part of conditional LTM configuration, similar to CHO. If UE can perform early TA acquisition, it performs RACH-less LTM, else, it can perform RACH-based LTM according to the configurations in RRC signalling.

Proposal 11: Configurations for RACH-based LTM can be pre-configured in the RRC signalling as a part of conditional LTM configuration.

Proposal 12: If UE can perform early TA acquisition, it performs RACH-less LTM, else, it can perform RACH-based LTM according to the configurations in RRC signalling.

  - Collision with other mobility mechanism(s)

Based on [4], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command or LTM cell switch command.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 13: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 14: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: If only one beam related event(s) is considered, it may cause frequent conditional LTM.

Observation 2: In legacy LTM, the source cell may trigger PDCCH order based on UE’s measurement results.

Observation 3: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1:For conditional LTM execution, more than one beam should be evaluated.

Proposal 2: The network can configure the number of beams for evaluation, and the maximum number can be further discussed.

Proposal 3: The execution condition may consist of  one or two trigger condition(s) (Event LTM3-like, or Event LTM5-like).

Proposal 4: Only one single RS type (CSI-RS and SSB) is configured for beam level trigger conditions of the evaluation of CHO execution condition of a single candidate cell.

Proposal 5: L3 measurement based conditional LTM is supported.

Proposal 6: For the LTM supporting UE without L1 measurement capability, CHO trigger conditions can be configured for conditional LTM.

Proposal 7: RAN2 sends LS to RAN4 on L3 measurement based conditional LTM related agreements.

Proposal 8: It’s network implementation to configure the same or different cells for conditional LTM and legacy LTM.

Proposal 9: The source cell may send PDCCH order based on UE’s measurement results or UE’s indication/request.

Proposal 11: Configurations for RACH-based LTM can be pre-configured in the RRC signalling as a part of conditional LTM configuration.

Proposal 12: If UE can perform early TA acquisition, it performs RACH-less LTM, else, it can perform RACH-based LTM according to the configurations in RRC signalling.

Proposal 13: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 14: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- References

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18);

- R4-2416863 WF on RRM requirements for NR_Mob_Ph4_Part2;

- TS 38.473 F1 application protocol (F1AP) (Release 18).