3GPP TSG RAN WG2 Meeting #128	R2-2410334

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.2.3

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#127bis meeting, The following progress are made for A-IoT paging, shown as:

Agreements - 1.The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - 2.Based on this information the device determines whether to skip sending the response to paging. - 3.From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - 4.If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Then in this contribution, we would like to provide some views on remaining issues for A-IoT paging between reader (gNB or intermediate UE) and A-IoT device(s).

Discussion

### 2.1 Message content

2.1.1  Service type

In RAN2#126 meeting, it is agreed that [1]:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

In RAN2, “inventory only”, “inventory and command” and “command only” procedure have been discussed. For “inventory only” procedure, the A-IoT device may transmit Msg1 for 3-step CBRA. While, for “command only” procedure, the A-IoT device may perform the command and transmit the command response directly. Thus, from A-IoT device side, it should be aware of the service type. From our perspective, non AS-layer service type is always existing and the reader can piggy back it in A-IoT paging directly. For AS-layer service type, we do not think it is necessary. Note that what we discuss here is only whether AS-layer service type should be contained in A-IoT paging message, whether the service type is visible to the reader is another issue which is discussed in this paper.

Observation 1: From A-IoT device side, it should be aware of the service type.

Proposal 1: From RAN2’s perspective, AS-layer service type is not necessary.

2.1.2  Identifier of multiple access method

In RAN1#118bis meeting, it has been agreed that [2]:

Agreement Add the following TP to the TR section on “D2R multiple access” for TDMA: 6.1.2.x D2R multiple access Time-domain multiple access, and frequency domain multiple access at least by using a small frequency-shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [CMCC, CATT, Ericsson, FUTUREWEI] state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [DOCOMO, LGE, MTK] state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration (….).

It can be seen that both TDMA and FDMA of D2R transmissions for Msg1 are agreed to be studied by RAN1. Note that different multiple access methods (e.g. TDMA only, FDMA only or TDMA and FDMA) may be applied for procedures related to different CN requests. The use cases, where “TDMA only” or “TDMA and FDMA” is applied, seem easy to be found. While for “FDMA only” use case, it is also possible. For example, when the number of target A-IoT devices is just a few, then only frequency domain information may be included in the A-IoT paging to indicate them accessing with FDMA. Based on above analysis, one identifier which marks which multiple access methods (such as: TDMA, FDMA, TDMA and FDMA etc.) is used, should be contained in A-IoT paging. However, whether the identifier is presented explicitly or implicitly in A-IoT paging needs further discussion.

Proposal 2: A-IoT paging can indicate target devices about the multiple access method at least for Msg1 (e.g. TDMA, FDMA, TDMA and FDMA, etc.). FFS through explicit or implicit parameter

2.1.3  Identifier of D2R transmission resources

In RAN2#126 meeting, it has been agreed that [1]:

Agreement - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/pre-configured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Based on the study in RAN1, the D2R transmission resources may include at least time-domain (such as access occasion) and/or frequency-domain (such as the frequency-shift). From our point of view, the D2R transmission resources can be shared D2R resources or dedicated D2R resources, in which dedicated D2R resources represents the access resources assigned to specific Ambient IoT device, while shared D2R resources denote the common resources which should be shared and competed by all paged Ambient IoT devices. In other words, Ambient IoT devices with dedicated D2R resources can access/respond without competition, while Ambient IoT devices with shared D2R resources will compete with each other and may collide when responding. Note that in order to allocate dedicated D2R resources, one device ID or a list of device IDs should be contained in A-IoT paging. For the A-IoT paging targeted to one A-IoT device, it is not necessary to contain explicit indication of D2R transmission resources. However for the A-IoT paging targeted to multiple specific A-IoT devices, explicit mapping rule between each device ID and dedicated transmission resource may be contained.

In terms of the transmission resource allocation, the number of paged devices is really helpful for gNB allocated related resources in A-IoT paging. For example, considering TDMA based D2R transmission, if ten devices are targeted, allocating at least ten access occasions is suitable to avoid collisions as soon as possible. Therefore the number of target A-IoT devices should be visible to gNB if available. For other case which CN can not provide the number of target A-IoT devices (such as paging all devices in coverage of a reader), the gNB should estimates the device number, whose method is given in our another papaer [3]. With the number of target A-IoT devices even if it is not very precise, gNB can allocated D2R resources for A-IoT radio more efficiently. Thus, RAN2 is asked to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

Proposal 3: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 4: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 5: For CFRA, the mapping ruled between each device ID and dedicated transmission resource, which is explicit or implicit, is supported.

Proposal 6: RAN2 is suggested to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

2.1.4 Identifier of random access type

In RAN2#127bis meeting, it has been agreed that [4]:

Agreement - For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

Up to now, 2-step CBRA, 3-step CBRA, CFRA are being discussed. Note that CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated. From our perspective, there are two options for CBRA:

- Option 1: Only support 3-step CBRA

For 2-step CBRA, it brings benefits only when the collision does not occur frequently. Considering that the Msg1 which contains both random number and device ID in 2-step CBRA is longer than that of 3-step CBRA, the reader have to wait for longer time when it detects collision during the reception of Msg1. Therefore, if collision occurs frequently in 2-step CBRA, more time will be wasted. Besides, some companies thinks that there may be some devices which can only support 2-step CBRA. However based on the agreement made in RAN2, 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA. Thus, we do not think it is difficult for A-IoT devices supporting 3-step CBRA. Based on above analysis, if down-selection is agreed by RAN2, 3-step CBRA is supported and 2-step CBRA is not supported in R19. To this end, no identifier of CBRA type is needed in A-IoT paging.

- Option 2: Mandatory to support both 3-step CBRA and 2-step CBRA by mandatory

For Option 2, both 3-step CBRA and 2-step CBRA are mandatory to support by all A-IoT devices. Under this circumstance, an identifier which indicates whether 3-step CBRA or 2-step CBRA is used in this procedure should be contained in A-IoT paging. To be specific, if an identifier representing 3-step CBRA is contained in A-IoT paging, the target device(s) performs 3-step CBRA. If an identifier representing 2-step CBRA is contained in A-IoT paging, the target device(s) should perform 2-step CBRA.

With the consideration that Option 2 may bring additional complexity to A-IoT devices and overhead to A-IoT paging message, Option 1 is supported.

Observation 2: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Observation 3: 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA.

Observation 4: If only 3-step CBRA is supported, no identifier of CBRA type is needed in A-IoT paging.

Observation 5: If both 3-step CBRA and 2-step CBRA are mandatory to support, an identifier which indicates whether 3-step CBRA or 2-step CBRA is used in this procedure should be contained in A-IoT paging.

Proposal 7: Before discussing the indication of CBRA type (e.g. 2-step CBRA or 3-step CBRA), RAN2 is asked to discuss the following options at first:

- Option 1: Only support 3-step CBRA

- Option 2: Mandatory to support both 3-step CBRA and 2-step CBRA

- Option 1 is supported in R19, thus no identifier of CBRA type is needed in A-IoT paging.

2.1.5 Identifier to avoid duplicate response

In RAN2#127bis meeting, the following agreements have been made:

Agreements - 1.The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - 2.Based on this information the device determines whether to skip sending the response to paging.

Considering the device unavailability caused by energy status, triggering multiple A-IoT paging messages which are associated with the same request from CN should be supported. In this case, each paged device is expected to respond only once for one request from CN, otherwise, duplicated response may case more collisions and greatly decrease the efficiency of the A-IoT system. To avoid the duplicated response from devices, there are following possible options which depend on parameters contained in A-IoT paging:

- Option 1: the number of expected subsequent A-IoT paging message is contained in A-IoT paging

For option 1, the number of expected subsequent A-IoT paging message is contained in A-IoT paging message. With that, if a device is accessed successfully after being paged by an A-IoT paging, it should set the value of a counter to the number contained in the A-IoT paging message. After that, if another A-IoT paging message is received, the counter of the device minus one. It will not respond to any A-IoT paging message until its counter equals to zero. If option 1 is selected, the A-IoT device need to save the number of expected subsequent A-IoT paging message in its RAM or ROM which increases devices’ complexity.

Observation 6: If the number of expected subsequent A-IoT paging message is contained in A-IoT paging, A-IoT device need to save it in its RAM or ROM which increases device’complexity.

Observation 7: If the number of expected subsequent A-IoT paging message is contained in A-IoT paging, A-IoT device initiate the value of counter and will not respond to any A-IoT paging message until its counter equals to zero.

- Option 2: an identifier reflecting the CN request is contained in A-IoT paging

For option 2, an identifier representing the CN request, such as transaction ID, etc., can be contained in A-IoT paging message. For the same request from CN, the same identifier should be contained in the A-IoT paging messages which are associated the mentioned CN request. With that, if a device is accessed successfully after being paged by an A-IoT paging, it should save the identifier reflecting the CN request contained in the A-IoT paging message. After that, if another A-IoT paging message which contains the same identifier as the saved one is received, the device will not respond to it. If option 2 is selected, the A-IoT device need to save the transaction ID in its RAM or ROM which increases devices’ complexity.  However, option 2 offers more flexibility for reader transmitting the subsequent A-IoT paging message than option 1.

Based on above analysis, Option 2 offers more pros than Option 1, thus Option 2 is preferred.

Observation 8: If an identifier reflecting the CN request is contained in A-IoT paging is contained in A-IoT paging, A-IoT device need to save it in its RAM or ROM which increases devices’complexity. However, it will not affect the flexibility for the reader transmitting subsequent paging messages.

Observation 9: If an identifier reflecting the CN request is contained in A-IoT paging is contained in A-IoT paging, A-IoT device will not respond to another A-IoT paging message which contains the same identifier after it accessed successfully.

Proposal 8: To avoid the duplicated response for multiple/subsequent A-IoT paging from devices, there are following options:

- Option 1: the number of expected subsequent A-IoT paging message is contained in A-IoT paging.

- Option 2: an identifier reflecting the CN request, e.g. transaction ID, is contained in A-IoT paging.

Option 2 is preferred.

2.1.6 Reader ID

For Ambient IoT, it has been approved that RAN2 assumes that the device will not support tracking/RAN area update procedure. To improve the efficiency of A-IoT paging, it is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated reader/area. For example, a command or inventory message can be sent to the reader associated with the target device which sends an A-IoT paging to target device as an initial attempt. If the response of target device is not received, that is, A-IoT paging failure occurs, more readers can be selected to send A-IoT paging. With the consideration of support the association, re-association, etc. efficiently, there may also be the identifier of the reader or something like A-IoT paging area contained in A-IoT paging. For example, when a reader ID is contained in A-IoT paging, the A-IoT device can identify its association relationship with reader by recording reader ID. Then when an A-IoT device finds the reader ID contained in A-IoT paging is different from the reader ID saved, it can perceive its association relationship with reader changes. With that, the reader can inventory the A-IoT device(s) whose association relationship with reader changes by transmitting specific A-IoT paging instead of always inventorying all A-IoT devices. Because inventorying all devices within a reader’s coverage each time wastes too much resources.

Observation 10: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Observation 11: If an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging and saved by devices,devices can perceive their association relationship changes. Otherwise, reader have to inventory all devices each time for gNB or CN updating the association relationship.

Proposal 9: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

Note that, from A-IoT device side, the behavior of A-IoT device for determining whether to respond or not based on reader ID is almost same as that based on the identifier to avoid duplicate response. To be specific, for both reader ID and identifier to avoid duplicate response, the A-IoT device save it after receiving an initial paging. After that, the A-IoT device compares the identifier in subsequent A-IoT paging with its saved one. If different, the A-IoT device responses.

Proposal 10: From A-IoT device side, the behavior of A-IoT device for determining whether to respond or not based on reader ID is almost same as that based on the identifier to avoid duplicate response.

### 2.2 Initiation

From our point of view, an A-IoT paging can be initiated by core network to inventory A-IoT device(s) or send read or write command to the A-IoT device(s). As for reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system saved in reader side. For example, if the association relationship is saved in gNB-reader, then it may trigger an A-IoT paging to update the association relationship between reader and A-IoT device(s). For the cases that reader triggers multiple/subsequent A-IoT paging messages associated with same request from CN, they should essentially be classified as CN-initiated A-IoT paging as well. Besides, for the periodical A-IoT paging which is discussed by RAN3, it also belongs to CN-initiated A-IoT paging, because the period for transmitting A-IoT paging is configured by CN. Based on above analysis, RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Observation 12: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system (such as the association relationship) saved in reader side.

Observation 13: Subsequent A-IoT paging associated with same CN request and periodical A-IoT paging should also be considered as CN-initiated A-IoT paging .

Proposal 11: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

### 2.3 Transmitting behavior

In RAN1#117, it has been agreed as following [5]:

Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

It is clear that PRDCH is the only physical channel for R2D, thus, the A-IoT paging is transmitted on PRDCH.

Proposal 12: The A-IoT paging is transmitted on PRDCH by a reader to A-IoT devices.

In RAN2#125-bis meeting, it has been approved that:

- RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

In RAN2#127-bis meeting, the support of segmentation is also considered as one of the functionalities of A-IoT MAC layer. From our point of view, the introduction of a new AS protocol on top of A-IoT MAC layer is not necessary but increasing the complexity and power consumption of A-IoT devices. To this end, A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 13: A-IoT paging is transmitted in A-IoT MAC layer.

### 2.4 Device unavailability due to energy harvesting

In RAN1#118 meeting, the following agreements has been made:

For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

From RNA1’s perspective, there are two directions for determining device unavailability. For direction 1, we do not see any RNA2’s impact since reader does not need to provide any information for determining device availability/unavailability. For direction 2, some information which indicates device become available or unavailable may need to be contained in A-IoT paging. From our point of view, direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1. To this end, RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy before designing the specific parameter(s) to be contained in A-IoT paging in work item, if needed.

Observation 14: For direction 1, there seems to be no RAN2’s impact. Direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1.

Proposal 14: RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy then design the specific parameter(s) to be contained in A-IoT paging in work item, if needed.

2.5 Determination of transmission/access occasion

For determining the corresponding D2R transmission/access occasion after A-IoT paging, it has been discussed in RAN2#127 meeting. The result is waiting for further RAN1 progress on indication of the start of access occasion, since it is closely related to the timing accuracy of device 1, 2a and 2b. Considering the fact that the initial clock accuracy of device 1 is about 10^4 ~ 10^5 ppm which is the evaluation assumption in RAN1, it should be really difficult for device 1 determining the transmission/access occasion efficiently. Thus, in order to have a harmonized design, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging. For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously. From RAN2’s perspective, we can assume there is a R2D message which indicates the boundary of transmission/access occasions as a baseline and follow RAN1 further progress.

Observation 15: With the consideration of device 1’s timing accuracy, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 16: For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Proposal 15: RAN2 can assume there is a R2D message which indicates the boundary of transmission/access occasions as a baseline and follow RAN1 further progress.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on paging for Ambient IoT:

Observation 1: From A-IoT device side, it should be aware of the service type.

Observation 2: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Observation 3: 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA.

Observation 4: If only 3-step CBRA is supported, no identifier of CBRA type is needed in A-IoT paging.

Observation 5: If both 3-step CBRA and 2-step CBRA are mandatory to support, an identifier which indicates whether 3-step CBRA or 2-step CBRA is used in this procedure should be contained in A-IoT paging.

Observation 6: If the number of expected subsequent A-IoT paging message is contained in A-IoT paging, A-IoT device need to save it in its RAM or ROM which increases device’complexity.

Observation 7: If the number of expected subsequent A-IoT paging message is contained in A-IoT paging, A-IoT device initiate the value of counter and will not respond to any A-IoT paging message until its counter equals to zero.

Observation 8: If an identifier reflecting the CN request is contained in A-IoT paging is contained in A-IoT paging, A-IoT device need to save it in its RAM or ROM which increases devices’complexity. However, it will not affect the flexibility for the reader transmitting subsequent paging messages.

Observation 9: If an identifier reflecting the CN request is contained in A-IoT paging is contained in A-IoT paging, A-IoT device will not respond to another A-IoT paging message which contains the same identifier after it accessed successfully.

Observation 10: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Observation 11: If an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging and saved by devices,devices can perceive their association relationship changes. Otherwise, reader have to inventory all devices each time for gNB or CN updating the association relationship.

Observation 12: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system (such as the association relationship) saved in reader side.

Observation 13: Subsequent A-IoT paging associated with same CN request and periodical A-IoT paging should also be considered as CN-initiated A-IoT paging .

Observation 14: For direction 1, there seems to be no RAN2’s impact. Direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1.

Observation 15: With the consideration of device 1’s timing accuracy, a R2D message, similar to QueryRep in RFID, which indicates the occasion boundary, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 16: For the case that A-IoT paging provides all subsequent scheduling information without any R2D message indicating the occasion boundary, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Proposal 1: From RAN2’s perspective, AS-layer service type is not necessary.

Proposal 2: A-IoT paging can indicate target devices about the multiple access method at least for Msg1 (e.g. TDMA, FDMA, TDMA and FDMA, etc.). FFS through explicit or implicit parameter

Proposal 3: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 4: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 5: For CFRA, the mapping ruled between each device ID and dedicated transmission resource, which is explicit or implicit, is supported.

Proposal 6: RAN2 is suggested to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

Proposal 7: Before discussing the indication of CBRA type (e.g. 2-step CBRA or 3-step CBRA), RAN2 is asked to discuss the following options at first:

- Option 1: Only support 3-step CBRA

- Option 2: Mandatory to support both 3-step CBRA and 2-step CBRA

- Option 1 is supported in R19, thus no identifier of CBRA type is needed in A-IoT paging.

Proposal 8: To avoid the duplicated response for multiple/subsequent A-IoT paging from devices, there are following options:

- Option 1: the number of expected subsequent A-IoT paging message is contained in A-IoT paging.

- Option 2: an identifier reflecting the CN request, e.g. transaction ID, is contained in A-IoT paging.

Option 2 is preferred.

Proposal 9: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

Proposal 10: From A-IoT device side, the behavior of A-IoT device for determining whether to respond or not based on reader ID is almost same as that based on the identifier to avoid duplicate response.

Proposal 11: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Proposal 12: The A-IoT paging is transmitted on PRDCH by a reader to A-IoT devices.

Proposal 13: A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 14: RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy then design the specific parameter(s) to be contained in A-IoT paging in work item, if needed.

Proposal 15: RAN2 can assume there is a R2D message which indicates the boundary of transmission/access occasions as a baseline and follow RAN1 further progress.

Reference

- “RAN2 Chair’s Notes”, RAN2#126, Fukuoka City, Japan.

- “RAN1 Chair’s Notes”, RAN1#118bis, Hefei, China.

- R2-2410315, “Further discussion on ambient IoT random access”, RAN2#128, Orlando, USA, CMCC.

- “RAN2 Chair’s Notes”, RAN2#127bis, Hefei, China.

- “RAN1 Chair’s Notes”, RAN1#117, Fukuoka City, Fukuoka, Japan.