3GPP TSG RAN WG2 Meeting #128	R2-2410336

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#127bis meeting, the following agreements have been made for random access in SBFD, shown as:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

Then in this contribution, we would like to provide our further views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

In RAN2#127 meeting, the following working assumptions are made for RA events:

- 1. Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Aim to reduce the random access latency and the PRACH collision probability, we do not find any strong reason to drop any RA event up to now. Thus, RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

Proposal 1: RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

For legacy NR, there are different CBRA and CFRA events and corresponding PRACH resource indication/configuration signallings as summarized in Table 1.

Table 1. RA events and corresponding PRACH resource indication/configuration signallings

| RA event | CBRA PRACH resource indication/configuration signalling | CFRA PRACH resource indication/configuration signalling |
| --- | --- | --- |
| Initial access from RRC_IDLE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| RRC Connection Re-establishment procedure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronize" | - CBRA resource configured by RACH-ConfigCommon - PDCCH order trigger | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronized" | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| SR failure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Request by RRC upon synchronous reconfiguration (e.g. handover) | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by RACH-ConfigDedicated |
| RRC Connection Resume procedure from RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| To establish time alignment for a secondary TAG | - N/A | - CFRA resource configured by RACH-ConfigCommon - PDCCH order indicating the used PRACH resource and preamble |
| Request for Other SI | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by SI-RequestConfig or SI-RequestConfigRepetition |
| Beam failure recovery | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource, preamble and root sequence configured by BeamFailureRecoveryConfig |
| SDT in RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| Early UL synchronization with an LTM candidate cell | - N/A | - CFRA resource configured by EarlyUL-SyncConfig - PDCCH order trigger |
| RACH-based LTM cell switch | - CBRA resource configured by RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger | - CFRA resource configured by RACH-ConfigDedicated or RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger |

From Table 1, it can be observed that there are multiple events which can trigger random access procedures  and different RA procedures (CBRA or CFRA) may be triggered by different RA events. Furthermore, it can also be found that CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events. From RAN2’s perspective, since random access in SBFD symbols is supported for all the existing RA events, we will discuss the resource configuration of CBRA and CFRA separately in section 2.2.

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

## 2.2 	Configuration of RACH resource

In RAN1#118 meeting, it has been agreed that [1]:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration. Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

In RAN1#118bis meeting, for RACH configuration Option 1, it is further agreed that [2]:

Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols -FFS: Details of the separate power control parameters

In RAN1, RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 has been agreed to support. To be specific, RACH configuration Option 1 with Alt 1-1 represents using one single RACH configuration, and only based on the existing parameters of the single RACH configuration except for power control parameters. RACH configuration Option 2 with Alt 2-3 represents using two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration, and the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. Since all existing RA events are supported by RAN2, one important issue to be discussed is whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported for each PRACH resource configuration signalling.

Proposal 2: RAN2 is asked to discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 are supported for each RA event.

For a comprehensive view, the detailed analysis of the 4-step CBAR and CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is given as following.

### 2.2.1 Configuration for 4-step CBRA

- Option 1 with Alt 1-1

- In RAN2#127bis meeting, it has been agreed that [2]:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

- Since SBFD is not considered as a new feature, 4-step CBRA configuration shares the existing parameters configured in RACH-ConfigCommon, while note that RAN1 support separate power control parameters or PRACH transmission in SBFD symbols and non-SBFD symbols. With the consideration that details of the separate power control parameters are still being discussed by RAN1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

- Option 2 with Alt 2-3

- For CBRA configuration with Option 2 with Alt 2-3, two separate RACH configurations are needed. Therefore, a new IE called rach-ConfigCommonSBFD-r19 can be introduced to indicate the additional 4-step CBRA configuration. An example is given as:

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

[[

rach-ConfigCommonSBFD-r19            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

- While, it still needs to be discussed that whether all parameters in RACH-ConfigCommon IE is needed for rach-ConfigCommonSBFD-r19.

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

Proposal 4:  For 4-step CBRA configuration Option 1 with Alt 2-3, RAN2 can introduce a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon. FFS details of parameters in ConfigCommonSBFD-r19 IE

### 2.2.2 Configuration for 4-step CFRA

- Option 1 with Alt 1-1

- Compared to the configuration for 4-step CBRA, which depends on whether SBFD is considered as a new feature, configuration for 4-step CFRA based on Option 1 with Alt 1-1 is much easier. To be specific, 4-step CFRA shares the same existing parameters in resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc. With consideration that there are multiple different resource configuration signallings for the 4-step CFRA configuration triggered by different RA events, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

- Option 2 with Alt 2-3

- For 4-step CFRA configuration based on Option 2 with Alt 2-3, additional CFRA configuration is needed. An easy solution is introducing a new IE parallel to each legacy IE. For example, a new IE named BeamFailureRecoveryConfigSBFD-r19 can be introduced to provide the additional 4-step CFRA configuration for SBFD-aware UEs to be used when beam failure occurs. While the behavior of SBFD-aware UEs when some parameters in new IE is absent needs further discussion. For example, if ra-Prioritization is absent in BeamFailureRecoveryConfig, legacy UE releases this field. While if ra-Prioritization is absent in BeamFailureRecoveryConfigSBFD-r19, whether the SBFD-aware UE can use the same ra-Prioritization as configured in BeamFailureRecoveryConfig, if available.

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

### 2.2.3 Enabling case of different RACH configuration options

In RAN1#116bis meeting, it has been agreed that UE is not required to support both options. Thus, from UE side, there may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1

- Cap 2: SBFD-aware UE only supports Option 2

- Cap 3: SBFD-aware UE supports both Option 1 and Option 2

Further in RAN1#118bis meeting, the following conclusion are made [3]:

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

For us, enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side is unnecessary.Therefore, from network side, there may be also three types of configuration cases as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

Based on above analysis, the behavior of SBFD-aware UEs with different capabilities under different network configurations is shown in Table 2.

Table 2. The behavior of SBFD-aware UEs with different capabilities under different network configurations

| gNB RACH configuration cases | SBFD-aware UE capability |  |  |
| --- | --- | --- | --- |
|  | Cap 1 | Cap 2 | Cap 3 |
| Case 1 | Can only use legacy ROs | Can only use legacy ROs | Can only use legacy ROs |
| Case 2 | Can only use legacy ROs and additional ROs configured by RACH configuration option 1 | Can use legacy ROs | Can only use legacy ROs and additional ROs configured by RACH configuration option 1 |
| Case 3 | Can use legacy ROs | Can use legacy ROs and additional ROs configured by RACH configuration option 2 | Can use legacy ROs and additional ROs configured by RACH configuration option 2 |

From Table 2, it can be found that the behavior of SBFD-aware UE under different configuration cases is very clear. However, there is one issue which needs to be discussed in RAN2 is that how can a SBFD-aware UE determine whether configuration case 1 or configuration case 2 is enabled, because existing parameters of the legacy RACH configuration may be reused for RACH configuration Option 1 with Alt 1-1. With the consideration that separate power control parameters is supported for RACH configuration Option 1 with Alt 1-1, one easy solution is that if the additional power control parameters PRACH transmission in SBFD symbols is configured, then RACH configuration Option 1 is enabled, otherwise, RACH configuration Option 1 is disabled. The other solution is to introduce an explicit indication to indicate whether RACH configuration Option 1 is enabled or not. While note that if RACH configuration Option 2 is configured through additional RACH configurations, SBFD-aware UE knows RACH configuration Option 1 is disabled naturally.

Observation 3: From UE side, there may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE only supports Option 2 with Alt 2-3

- Cap 3: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 4: From network side, there may be also three types of configuration cases as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

- Observation 5: One issue which needs to be discussed in RAN2 is that how can a SBFD-aware UE determine whether RACH configuration with Option1 is enabled or neither Option 1 nor Option 2 is enabled.

Proposal 7: From RAN2 perspective, enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side is not supported.

Proposal 8: For SBFD-aware UE determining whether RACH configuration with Option1 is enabled or neither Option 1 nor Option 2 is enabled, RAN2 should discuss following options:

- Option 1: If the additional power control parameters PRACH transmission in SBFD symbols is configured, then RACH configuration Option 1 is enabled, otherwise, RACH configuration Option 1 is disabled.

- Option 2: An explicit indication to indicate whether RACH configuration Option 1 is enabled or not.

## 2.3 	RACH procedure

### 2.3.1 Initial PRACH transmission attempt

In RAN2#127bis meeting, the following agreement is made:

Agreement - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

From our point of view, above agreement can also be extended to CFRA RACH procedure and can be applied to both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 with minimum standard effort. The remaining issue is that how the certain specified/configured conditions/prioritizations is designed. In legacy NR, the RSRP of the downlink pathloss reference is taken into consideration when selecting between 4-step RA and 2-step RA, if the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type Random Access Resources. For SBFD, whether/how to define the initial PRACH resource selection behaviour based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols. For example, if the motivation is to improve PRACH capacity and reduce PRACH collision probability, it would be better that cell centre SBFD-aware UEs use the additional ROs, which implies UE should select additional ROs if DL RSRP is higher than a threshold. If the motivation is to improve the PRACH coverage by using long PRACH format or using PRACH repetition, it is suitable that cell edge SBFD-aware UEs use the additional ROs, which implies UE should select additional-ROs if DL RSRP is lower than a threshold. Apart from this, whether any conditions/prioritizations except for DL RSRP should be supported needs discussion as well. Therefore, RAN2 is asked to discuss the uses cases when determining how to design the certain specified/configured conditions/prioritizations.

Observation 6: For SBFD, whether/how to define the initial PRACH resource selection behaviour based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols.

Observation 7: Whether any conditions/prioritizations except for DL RSRP should be supported needs discussion.

- Proposal 9: RAN2 is asked to discuss the uses cases when determining how to design the certain specified/configured conditions/prioritizations.

### 2.3.2 PRACH transmission re-attempt

Similar to initial PRACH transmission attempt, same solution can be applied to RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3. For PRACH transmission re-attempt, it has been agreed by RAN2 that:

Agreement For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- It can be found that RAN2 has agreed to support additional ROs fallback to legacy ROs, with the consideration that the interference level in SBFD symbols is often more serious than that in non-SBFD symbols. For the other direction, that is, legacy ROs fallback to additional ROs, further discussion is needed. For the factors which cause the failure of random access attempt includes high interference, collision between multiple UEs, etc. To this end, allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently. Therefore, RAN2 is asked to support SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs.

Observation 8: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Proposal 10: RAN2 is asked to support SBFD-aware UEs which select leagcy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

## 2.4 	Early indication

The gNB can identify a SBFD UE which transmits Msg1 in an additional RO since additional RO is in SBFD symbol. However, considering that legacy ROs are valid for both SBFD-aware UEs and non-SBFD UEs, one issue needs to be discussed by RAN2 is whether to support early indication of SBFD-aware UEs, which can be achieved by either Msg1-based or Msg3-based early indication. For Msg1-based early indication, in last RAN2 meeting, it has been agreed that there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework from RAN2’s perspective. Therefore, Msg1-based early indication in legacy ROs is not supported in RAN2. The remaining issue is whether Msg3-based early indication is supported. If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission in SBFD symbol(s) for SBFD-aware UE which brings benefits. Considering using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs.

- From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs. Apart from this, since Msg1-based early indication is agreed not to be supported in RAN2, if Msg3-based early indication is also not supported, then early indication is not supported for SBFD which may reduce the performance of SBFD operation.

Observation 9: With Msg3-based early indication, there is almost no impact on PRACH capacity of non-SBFD-aware UEs.

Observation 10: Since Msg1-based early indication is agreed not to be supported in RAN2, if Msg3-based early indication is also not supported, then early indication is not supported for SBFD which may reduce the performance of SBFD operation.

Proposal 11: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

Observation 3: From UE side, there may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE only supports Option 2 with Alt 2-3

- Cap 3: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 4: From network side, there may be also three types of configuration cases as following:

- Case 1: Neither Option 1 nor Option 2 is configured

- Case 2: Option 1 is configured

- Case 3: Option 2 is configured

- Observation 5: One issue which needs to be discussed in RAN2 is that how can a SBFD-aware UE determine whether RACH configuration with Option1 is enabled or neither Option 1 nor Option 2 is enabled.

Observation 6: For SBFD, whether/how to define the initial PRACH resource selection behaviour based on DL RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols.

Observation 7: Whether any conditions/prioritizations except for DL RSRP should be supported needs discussion.

Observation 8: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Observation 9: With Msg3-based early indication, there is almost no impact on PRACH capacity of non-SBFD-aware UEs.

Observation 10: Since Msg1-based early indication is agreed not to be supported in RAN2, if Msg3-based early indication is also not supported, then early indication is not supported for SBFD which may reduce the performance of SBFD operation.

Proposal 1: RAN2 confirms the working assumption “Random access procedure in SBFD symbols is supported for all the existing RACH trigger events”.

Proposal 2: RAN2 is asked to discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 are supported for each RA event.

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, RAN2 can wait for RAN1’s progress to determine the additional power control parameters for PRACH transmission in SBFD symbols.

Proposal 4:  For 4-step CBRA configuration Option 1 with Alt 2-3, RAN2 can introduce a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon. FFS details of parameters in ConfigCommonSBFD-r19 IE

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

Proposal 7: From RAN2 perspective, enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side is not supported.

Proposal 8: For SBFD-aware UE determining whether RACH configuration with Option1 is enabled or neither Option 1 nor Option 2 is enabled, RAN2 should discuss following options:

- Option 1: If the additional power control parameters PRACH transmission in SBFD symbols is configured, then RACH configuration Option 1 is enabled, otherwise, RACH configuration Option 1 is disabled.

- Option 2: An explicit indication to indicate whether RACH configuration Option 1 is enabled or not.

- Proposal 9: RAN2 is asked to discuss the uses cases when determining how to design the certain specified/configured conditions/prioritizations.

Proposal 10: RAN2 is asked to support SBFD-aware UEs which select leagcy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

Proposal 11: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Reference

- “RAN1 Chair’s Notes”, RAN1#118, Maastricht, NL.

- “Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD” RAN2#127bis, Hefei, China.

- “RAN1 Chair’s Notes”, RAN1#118bis, Hefei, China.