3GPP TSG-RAN WG2 Meeting #128	R2-2410340

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#127bis meeting we reached the following agreements [1]:

RAN2 #127bis: Agreements on L1 event triggered MR - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. - Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

At the RAN #105 meeting, it was agreed to support conditional Intra-CU LTM [2]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion

TTT operation per beam or per cell?

First, there is only one current beam for the serving cell, even if the current beam changes. So this question is about the candidate cell beams.

As for Event LTM3/4/5, in our opinion, Event LTM3/5 will be mainly used for LTM cell switching purposes. From this perspective, NW does not care which specific beam in the candidate cell triggers the measurement report, as long as there is a beam that meets the event condition within the TTT time, even if this beam may change.

However, in the RAN2#126 meeting, we agreed that Event triggered L1 measurement will also be used for early sync purposes:

RAN2#126 Agreement: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure.

In our opinion, Event LTM4 can be used to select appropriate candidate beams for early sync. From this perspective, NW should know the quality of specific candidate beams, so beam level TTT is a better choice.

Observation 1: Event LTM4 can be used to select appropriate candidate beams for early sync, so NW should know the quality of specific candidate beams, which means beam level TTT is a better choice.

In addition, in the discussion on C-LTM at the last meeting, we came to the following conclusions:

RAN2#127bis Agreement: - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

In C-LTM, it is unreasonable to make a handover decision just because Event LTM3 and/or LTM5 is triggered, because this only means that a single beam of a candidate cell is better than the current beam, not that the overall quality of the candidate cell is better.  Therefore, a possible C-LTM execution condition is that although Event LTM3 and/or LTM5 are still reused, the UE will only execute C-LTM switching when N beams meet the event triggering conditions. In our opinion, this is also the meaning of ‘Event LTM3-like and LTM5-like’. Then it is necessary to evaluate each candidate beam, which means beam level TTT is required. Therefore, for the purpose of compatibility with C-LTM, beam level TTT is a better choice.

Observation 2: For C-LTM, the UE may perform handover only when the quality of multiple beams in the candidate cell is good, which requires evaluation through beam level TTT.

Proposal 1: In order to take into account both early sync and C-LTM, beam level TTT is supported.

If it is beam level TTT, there may be multiple beams satisfying the same event condition at the same time. When this happens, triggering a measurement report for each beam will result in a very large resource overhead, so it is possible to trigger only one measurement report that carries all the measurement results.

Proposal 2: If multiple beams satisfying the same event condition at the same time, it is possible to trigger only one measurement report that carries all the measurement results.

TTT operation on current beam changing

First, we want to make it clear that if it is cell level TTT for candidate cell beams, then there is a problem of changes in the compared beams within the same candidate cell, but if beam level TTT for candidate cell beams is supported, then there is no candidate beam change, because each candidate beam has its own TTT.

Observation 3: If beam level TTT for candidate cell beams is supported, then there is no candidate beam change, because each candidate beam has its own TTT.

When the current beam of serving cell changes, there are two options:

Option 1: If the TTT timer is reset when the current beam of the serving cell changes, it will increase the difficulty of triggering the event, and the NW may not be able to obtain the L1 measurement results in time to trigger the LTM cell handover.

Option 2: Another option is the TTT timer is not reset. Actually, the purpose of event triggering is to send the measurement results to the NW when the beam quality of the serving cell deteriorates and/or the beam quality of the candidate cell is better than the beam quality of the serving cell. Therefore, it does not matter which beam is the current beam of the serving cell, so there is no need to reset the TTT timer when the current beam of the serving cell changes.

Proposal 3: TTT timer is not reset when the current beam of serving cell changes.

Proposal 4: When the current beam of serving cell changes, the UE carries the latest current beam measurement information in the measurement report.

Measurement RS type alignment

In RAN2#127bis meeting, the working assumption is confirmed that same RS type should be used for both serving and candidate cell for event LTM3 and event LTM5. Then, two issues are left for further discussion:

- Issue 1: How to ensure the same RS type for serving cell and candidate cell for LTM3 and LTM5?

- Issue 2: How to handle the case when only one TRS is configured in the indicated TCI state?

At the last meeting, we had the following options:

RAN2#127bis: - Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement. - Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement. - Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement. - Option 4: Rely on network configuration, i.e. same RS type is configured. - Option 5: Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement. - Option 6: The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

In RAN1#118bis meeting, RAN1 has agreed current beam is derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell.

RAN1#118bis Agreement - The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios

The above issues have been also discussed in Rel-19 MIMO UE initiated/event-driven beam reporting and the solution can be reused for LTM, where implicit manner is used to ensure same RS type for serving cell and candidate cell. If the RS(s) for candidate cell are CSI-RS configured in a CSI-RS resource set configured with repetition, the serving cell RS for event evaluation is the QCL RS of the indicated joint/DL TCI state, if the RS(s) for candidate cell are SSB, the serving cell RS for event evaluation is the SSB QCLed with the QCL RS of the indicated joint/DL TCI state. When TRS is configured in the indicated TCI state and the RS(s) for candidate cell are CSI-RS configured in a CSI-RS resource set configured with repetition, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

Proposal 5: Option 6 is prefered. If the RS(s) for candidate cell are CSI-RS configured in a CSI-RS resource set configured with repetition, the serving cell RS for event evaluation is the QCL RS of the indicated joint/DL TCI state, if the RS(s) for candidate cell are SSB, the serving cell RS for event evaluation is the SSB QCLed with the QCL RS of the indicated joint/DL TCI state.

Contents of MR MAC CE

In RAN2#127bis meeting agreed the following conclusions:

RAN2#127bis Agreement: - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

If ‘LTM configuration id + SSB/CSI-RS id’ is used, it is necessary to indicate in the MAC CE how many candidate cells are contained in the measurement report and how many beams each candidate cell contains, which will result in a larger MAC CE size.

If ‘SSBRI/CRI of N beams’ is used, since this is a global variable, there is no need for an additional field in the measurement report to indicate the candidate cell and the number of beams it contains.

Proposal 6: Support ‘SSBRI/CRI of N beams’ in MR MAC CE.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1: Event LTM4 can be used to select appropriate candidate beams for early sync, so NW should know the quality of specific candidate beams, which means beam level TTT is a better choice.

Observation 2: For C-LTM, the UE may perform handover only when the quality of multiple beams in the candidate cell is good, which requires evaluation through beam level TTT.

Observation 3: If beam level TTT for candidate cell beams is supported, then there is no candidate beam change, because each candidate beam has its own TTT.

Proposals:

Proposal 1: In order to take into account both early sync and C-LTM, beam level TTT is supported.

Proposal 2: If multiple beams satisfying the same event condition at the same time, it is possible to trigger only one measurement report that carries all the measurement results.

Proposal 3: TTT timer is not reset when the current beam of serving cell changes.

Proposal 4: When the current beam of serving cell changes, the UE carries the latest current beam measurement information in the measurement report.

Proposal 5: Option 6 is prefered. If the RS(s) for candidate cell are CSI-RS configured in a CSI-RS resource set configured with repetition, the serving cell RS for event evaluation is the QCL RS of the indicated joint/DL TCI state, if the RS(s) for candidate cell are SSB, the serving cell RS for event evaluation is the SSB QCLed with the QCL RS of the indicated joint/DL TCI state.

Proposal 6: Support ‘SSBRI/CRI of N beams’ in MR MAC CE.

References

- R2-2407571 Report from session on V2X/SL, R18/19 MOB, and R19 NES.

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4.