3GPP TSG-RAN WG2 Meeting #128		                          R2-2410341

Orlando, USA, Nov.  18th – 22nd , 2024

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda item:	8.4.3

Document for:	Discussion

- Introduction

At the RAN2#127bis meeting, we reached the following agreements[1]:

Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

At the RAN2#127 meeting, we reached the following agreements[2]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the LR measurement results.

- Discussion

In the LP-WUS discussion of RAN4, they listed several cases for measurement offloading/relaxation. In order to avoid duplication of work and conduct efficient discussions within a limited time, RAN2 should prioritize research based on the cases approved by RAN4.

RAN4 has already approved the following cases:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 1: RAN2 should prioritize research based on the cases approved by RAN4.

2.1 Measurement relaxation/offloading of serving cell

Case1: Serving cell measurement offloading

According to the conclusions of RAN4#111, measurement offloading is supported, and we reached the following agreements in RAN2#127:

RAN2#127 Agreement For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In the 'Fully offloading case' agreed by RAN4, MR serving and neighboring cell measurement are stopped. In our understanding, in order not to affect the existing cell (re)selection process in TS 38.304, this means that the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements. Otherwise, the MR cannot stop measuring neighboring cells. The relationship between the relevant thresholds is shown in Figure 1.

Figure 1 The relationship between the relevant thresholds

Proposal 2: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Considering that the latency of enabling MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

Case3: Serving cell measurement relaxation

Similarly as measurement offloading, an RSRP threshold can be predefined. In addition, since the MR is not turned off, the mobility of the UE may not need to be considered in this case. Since the MR in this case will perform relaxation measurements on the serving cell and the neighboring cell, the threshold for measurement relaxation should be less than the threshold for the UE to start neighboring cell measurement. As shown in Figure 1.

In addition, since the types and accuracy of MR and LR may be different, the measurement results of MR and LR should not be combined, but should be processed as independent measurement quantities. And considering that MR is crucial to the measurement of neighboring cells in cell (re)selection, the entry/exit conditions of measurement relaxation should be mainly based on the measurement quality of MR.

Proposal 4: For case3 agreed by RAN4, the serving cell measurement relaxation condition can be defined as:

- The entry conditions can be defined as at least MR measurement result is greater than a certain RSRP threshold, and LR measurement result could also be considered if LR threshold is configured.

- The exit condition of case3 to case1 is based on the MR and LR measurement results.

- The exit condition of case3 to no relaxion is based on the MR measurement results.

Proposal 5: For case3 agreed by RAN4, the threshold for measurement relaxation is less than the threshold for starting neighboring cell measurements.

Proposal 6: For case3 agreed by RAN4, serving cell measurement relaxation does not need to consider UE mobility state.

2.2 Measurement relaxation of neighboring cell

Case3: Neighboring cell measurement relaxation

In RAN2#126, we agreed that the RRM measurement of the neighboring cell can only be performed by MR. In RAN2#127bis, we reached the following agreements:

Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

For the ‘UE not at cell edge’ criterion, it is a condition based on the serving cell measurement result. In case 3, the serving cell measurement is performed by both MR and LR, so the condition for neighboring cell measurement relaxation should consider the measurement results of both MR and LR. Therefore, R16 criteria needs to be enhanced.

Proposal 7: For case 3 agreed by RAN4, if the serving cell has performed measurement relaxation, then the neighboring cell measurement relaxation condition can be defined as:

- The entry condition can be defined as both MR and LR measurement result being greater than a certain threshold, and the thresholds of MR and LR can be configured separately.

- The exit condition can be defined as either MR or LR measurement result being less than a certain threshold.

In addition, in the discussion of RAN4, most companies support the scaling factor(s) larger than that of legacy Rel-16 UE power saving WI (i.e., 3). Considering that LR cannot assist MR in performing neighbor cell measurement, in our opinion, R19 Neighboring cell measurement relaxation should have stricter evaluation conditions than R16. Therefore, the MR threshold in the entry condition of R19 Neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16. As shown in Figure 1.

Proposal 8: The MR threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and is up to RAN4 decision.

For the ‘UE with low mobility’ criterion, we think it is also necessary. Usually we think that a high-speed mobile UE has a higher handover probability, so the UE needs to perform accurate neighbor cell measurement. However, the relaxation factor currently discussed by RAN4 is very large. If a high-speed UE performs R19 neighbor cell measurement relaxation, it will bring risks in terms of mobility.

Proposal 9: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

2.3 Other issues

Is case2 supported?

RAN4 is still discussing whether to support case 2, that is, the MR performs relaxed measurements on the serving cell while not performing measurements on the neighboring cells. As shown in Figure 1, if case 2 is supported, its working area will be between case 1 and case 3.

UEs usually give priority to intra-frequency cell reselection, i.e. the network will configure the UE to perform intra-frequency measurements before inter-frequency measurements. Inter-frequency cell reselection is only used when there is a coverage problem or when the UE is located on a cell with a higher priority. Therefore, the configuration of SIntraSearchP can be quite conservative.

So in our opinion, whether case 2 is needed depends on the SIntraSearchP configured by the network. If SIntraSearchP is configured very conservatively, then case 1 is sufficient; if SIntraSearchP is configured more aggressively, which means that the UE does not need to perform neighbor measurements in a larger area, then it is reasonable to support case 2 as a transition between case 3 and case 1. In short, case 2 can be supported, but it depends on the network configuration.

Observation 1: If SIntraSearchP is configured conservatively, the available region of case2 is very small, so it is a rare case, and it seems redundant when case1 is already agreed.

Proposal 10: Postponing the discussion of case2, whether it makes sense depends on the SIntraSearchP configured by the network.

LP-SS measurement accuracy

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra in FR1.

TS 38.133 For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements:

RAN4 112bis Agreement Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: For FR1: - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB

For LP-WUS, a larger margin may be defined. A larger cell reselection margin implies that cell reselection is delayed.

Observation 2: For LP-WUS a larger cell reselection margin may be defined, which may cause cell reselection to be delayed.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are the observations and proposals.

Observations:

Observation 1: If SIntraSearchP is configured conservatively, the available region of case2 is very small, so it is a rare case, and it seems redundant when case1 is already agreed.

Observation 2: For LP-WUS a larger cell reselection margin may be defined, which may cause cell reselection to be delayed.

Proposals:

Proposal 1: RAN2 should prioritize research based on the cases approved by RAN4.

Proposal 2: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 4: For case3 agreed by RAN4, the serving cell measurement relaxation condition can be defined as:

- The entry conditions can be defined as at least MR measurement result is greater than a certain RSRP threshold, and LR measurement result could also be considered if LR threshold is configured.

- The exit condition of case3 to case1 is based on the MR and LR measurement results.

- The exit condition of case3 to no relaxion is based on the MR measurement results.

Proposal 5: For case3 agreed by RAN4, the threshold for measurement relaxation is less than the threshold for starting neighboring cell measurements.

Proposal 6: For case3 agreed by RAN4, serving cell measurement relaxation does not need to consider UE mobility state.

Proposal 7: For case 3 agreed by RAN4, if the serving cell has performed measurement relaxation, then the neighboring cell measurement relaxation condition can be defined as:

- The entry condition can be defined as both MR and LR measurement result being greater than a certain threshold, and the thresholds of MR and LR can be configured separately.

- The exit condition can be defined as either MR or LR measurement result being less than a certain threshold.

Proposal 8: The MR threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and is up to RAN4 decision.

Proposal 9: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

Proposal 10: Postponing the discussion of case2, whether it makes sense depends on the SIntraSearchP configured by the network.

- References

- draft R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD.