3GPP TSG-RAN WG2 Meeting #128       	                      R2-2410344

Orlando, US, November 18th-22nd, 2024

Agenda item:	8.1.5

Source:	CMCC, NTT Docomo, China Unicom, CATT, Apple

Title:	Discussion on AI/ML model transfer/delivery

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

During R18 study, RAN2 discussed the potential solutions for AI/ML model transfer/delivery and captured the analysis of current status, gaps and potential RAN specification impact for each solution in TR 38.843 [1].

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

RAN1 studies the following potential cases for model transfer/delivery considering the model format, model storage location and training location. And RAN1 agreed to de-prioritize case z2, z3 and z5 during R19 discussion.

Table 1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

In last meeting [2], RAN2 discussed the requirements on controllability and visibility for model transfer/delivery, but had no consensus. This contribution will provide further analysis of the requirements on controllability and visibility.

- Discussion

During the discussion on R18 and R19 AI/ML for air interface, one-sided model for CSI prediction, beam management, positioning use cases, and two-sided model for CSI compression use case are studied in RAN1 and RAN2. In this contribution, we will discuss the requirements on model transfer/delivery for one-sided model and two-sided model separately.

In general, controllability encompasses both the control exercised during the model transfer/delivery procedure and the control maintained after the model transfer/delivery procedure is completed. In our understanding, we think that control maintained after the model transfer/delivery procedure should be considered as part of LCM. Therefore, we will focus on the controllability during the model transfer/delivery procedure in this contribution.

## 2.1 Discussion on one-sided model

For UE-sided model for CSI prediction, beam management and positioning, RAN2 assumed that UE-side OTT server would be responsible for model training during study phase, and discussed the following potential solutions for data collection for UE-sided model training [2]. Considering the training location for one-sided model, we understand that case y and case z1 in Table 1 can be applied for UE-sided model transfer/delivery.

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Furthermore, RAN2 discussed the requirements on controllability and visibility for data collection, since there were some concerns on the privacy and data leakage issues from the perspective of operators, chip vendor, UE vendor and network vendor.

However, for model transfer/delivery, we think the case is different from data collection. In our understanding, when a model is trained by OTT server, it is straightforward to send the model to the UE directly (transparent to 3GPP, i.e. case y in Table 1), whether and when to transmit the model is up to implementation, which is similar to legacy user plane data transmission. Thus, there is no requirements on controllability or visibility for UE-sided model transfer/delivery case y.

Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly from OTT-server (transparent to 3GPP).

For case z1, the model is trained at UE side or neutral side but stored at network side, and then transferred/delivered to the UE. Due to the model format is proprietary, we don’t think that the network can be able to and need to comprehend the model in proprietary format.

Observation 1: For case z1, the model is in proprietary format which is not comprehend by network.

In last meeting, some companies proposed that meta info of the model should be aware by network during the model transfer procedure, e.g. to be used for further model management especially for monitoring. But we think this is unreasonable because during R19 discussion, RAN2 focuses on functionality-based LCM not model-based LCM for now, so the network doesn’t be aware of and doesn’t care which model is used at UE side, and there is no need to know the meta info of the model.

Observation 2: RAN2 focuses on functionality-based LCM during R19 discussion, the network doesn’t be aware of which model is used at UE side.

However, considering the model is stored in network for case z1, we understand that the network can be aware that this is a model, but no more than that. In addition, whether and when to transfer the model to the UE can be up to network, including network initiates autonomously or based on UE request. From this perspective, we think there may be requirements on visibility and controllability for model transfer/delivery case z1, i.e. the network can be aware this is the model (model content or meta info is not aware) and model transfer/delivery procedure. In other words, there is some requirements on partial visibility and controllability.

Proposal 2: For the visibility and controllability for UE-sided model transfer/delivery case z1, since the model is stored at network side,

- The network can be aware that this is a model (model content or meta info is not aware)

- Whether and when to transfer the model to the UE can be up to network, including network initiates autonomously or based on UE request

From the perspective of operator, we have no strong requirement on visibility and controllability for UE-sided model transfer/delivery actually, and we also have no idea on how to achieve the commercial deployment for case z1, e.g. which 3GPP node will store the AI models. Therefore, we prefer to de-prioritize case z1 for one-sided model transfer/delivery.

Proposal 3: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator.

## 2.2 Discussion on two-sided model

For two-sided model for CSI compression, RAN1 focuses on case z4 after discussion of several meetings, i.e. model training at NW side and model transfer in open format of a known model structure at UE. And RAN1 studied the alternatives for case z4, where the same step is that NW transfers the parameters to the UE for at least one supported known model structure.

RAN1#117 Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

Observation 3: RAN1 focuses on case z4 for two-sided model transfer/delivery, where NW transfers the parameters to the UE for the supported known model structure.

For two-sided model, it should be aligned between CSI generation part at UE side and CSI reconstruction part at network side for good model performance. For case z4, the whole model is trained at network, and then the CSI generation part is sent to the UE, so the model transfer procedure should be controlled by network. Furthermore, considering the model is open format and the structure is known at UE side, the network transfers the parameters of model to the UE, so the visibility should be also required.

Figure 1. Example for model transfer of two-sided model

Proposal 4: For two-sided model, controllability and visibility are required for model transfer/delivery case z4, considering the following aspects:

- The CSI generation part at UE side and the CSI reconstruction part at NW side should be aligned for good model performance

- The whole model is trained at NW and the CSI generation part is sent to UE

- The model is open format and the structure is known at UE side, NW transfers the parameters of the model to UE

# 3	Conclusion

Here are the observations and proposals for requirements on AI/ML model transfer/delivery.

Discussion on one-sided model

Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly from OTT-server (transparent to 3GPP).

Observation 1: For case z1, the model is in proprietary format which is not comprehend by network.

Observation 2: RAN2 focuses on functionality-based LCM during R19 discussion, the network doesn’t be aware of which model is used at UE side.

Proposal 2: For the visibility and controllability for UE-sided model transfer/delivery case z1, since the model is stored at network side,

- The network can be aware that this is a model (model content or meta info is not aware)

- Whether and when to transfer the model to the UE can be up to network, including network initiates autonomously or based on UE request

Proposal 3: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator.

Discussion on two-sided model

Observation 3: RAN1 focuses on case z4 for two-sided model transfer/delivery, where NW transfers the parameters to the UE for the supported known model structure.

Proposal 4: For two-sided model, controllability and visibility are required for model transfer/delivery case z4, considering the following aspects:

- The CSI generation part at UE side and the CSI reconstruction part at NW side should be aligned for good model performance

- The whole model is trained at NW and the CSI generation part is sent to UE

- The model is open format and the structure is known at UE side, NW transfers the parameters of the model to UE

# 4	Reference

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2407807 CR on RAN2 inputs to TR 38.843

- Draft_R2_127bis_Meeting_Report_v1