3GPP TSG-RAN WG2 Meeting #128        	                       R2-2410345

Orlando, US, November 18th-22nd, 2024

Agenda item:	8.3.5

Source:	CMCC

Title:	Discussion on other aspects of simulation assumption

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

As per Chairlady’s guidance, this contribution will discuss on simulations, including controversial aspects of email discussion or on aspects not covered in email discussion related to simulation assumptions for RLF, Event prediction, and system performance evaluation assumptions, and aspects and assumptions related to generalization study for RRM prediction.

- Discussion

- 2.1 Discussion on model generalization

- During R18 study on AI/ML for Air Interface, RAN1 discussed how to evaluate the model generalization and considered the various scenarios or configurations per use case. The conclusions on model generalization are captured in TR 38.843 [1], and the following text is for beam management use case:

Model generalization: In the context of model generalization, scenarios may mean various deployment scenarios, various outdoor/indoor UE distributions, various UE mobility assumptions. Similarly, configurations may mean various UE parameters, various gNB settings, Various Set B of beam(pairs). The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2). Specifically, the following generalizations could be considered and clause 6.3.2 presents those which have been actually simulated by companies: - Scenarios: - Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height; - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings): - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported Companies to report the selected scenarios/configurations for generalization verification. Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded. The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two. - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance.

- For RRM measurement prediction, temporal domain, frequency domain and spatial domain are studied in R19 and the following cases are considered for simulation evaluation.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- In our understanding, the temporal prediction and spatial prediction for RRM prediction are similar to BM-Case2 and BM-Case1 for beam management separately. Therefore, the cases agreed in RAN1 for generalization performance of an AI/ML model over various scenarios/configurations for beam management can be reused for RRM measurement prediction.

Proposal 1: Reuse the cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations agreed in RAN1-led AI/ML for Air Interface SI for RRM measurement prediction:

-	Case 1: The AI/ML model performs inference/test based on the same dataset as training, e.g. the dataset for both training and inference/test is constructed based on Scenario#B/Configuration#B

-	Case 2: The AI/ML model performs inference/test based on the different dataset from training, e.g., the dataset for training is constructed based on Scenario#A/Configuration#A, but the dataset for inference/test is constructed based on Scenario#B/Configuration#B

-	Case 3: The AI/ML model performs inference/test based on the subset of dataset for training, e.g.  the training dataset is constructed by mixing datasets from multiple scenarios/configurations including Scenario#B/Configuration#B and Scenario#A/Configuration#A, but the inference/test dataset is constructed only based on a single Scenario/Configuration (i.e. Scenario#B/Configuration#B)

To verify the generalization performance of an AI/ML model over various scenarios or configurations, the considered scenarios or configurations which may impact the model performance should be discussed per sub use case. During RAN1 discussion, various deployment scenarios, outdoor/indoor UE distributions and UE mobility, UE parameters, gNB settings, Set B of beam (pairs), and T1 for measurement /T2 for prediction for BM-Case2 are considered.

Based on the agreed simulation assumptions on FR1 and FR2, the deployment scenarios and UE distribution are fixed, but BS antenna configuration, UE antenna configuration, UE speed, MRRT, and OW/PW are various.

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz,15Khz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 500m ISD, 2-tier model (7 sites, 3 sectors/cells per site) | 200m ISD, 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 Fast fading is necessary for RRM sub case 1, 2 and 3. LOSsoft is optional modelled.(LOSsoft is not considered) Oxygen absorption, Time-varying Doppler shift , Explicit ground reflection model and blockage are not considered. | UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 as baseline channel modelling Fast fading is necessary for RRM sub case 1, 2 and 3. LOSsoft is optional modelled.(LOSsoft is not considered) Oxygen absorption, Time-varying Doppler shift , Explicit ground reflection model and blockage are not considered. |
| System BW | 20MHz | 80MHz |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Number of BS beams: 32 or 64 downlink Tx beams (max number of available beams) at NW side. Other values, e.g., 256 not precluded. |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Number of UE beams: 4 or 8 downlink Rx beams (max number of available beams) per UE panel at UE side. Other values, e.g., 16 not precluded. |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm for 20MHz | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10 m |
| UE Antenna height | Follow TR36.873, which is 1.5m | 1.5 m |
| Spatial consistency | At least forSpatial domain prediction, companies report the one of spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report the one of spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| L1/L3 filtering | - sliding L1/L3 filtering - non-sliding L1/L3 filtering | - sliding L1/L3 filtering - non-sliding L1/L3 filtering |
| the number of TX/RX | - Number of TX beams：{1,2,4} - Number of RX beams：{1} | - Number of TX beams：{8,16,32} - Number of RX beams：{4} |
| UE speed | - 90km/h - 30km/h | - 60km/h |
| OW/PW | - OW: {40ms~2000ms} - PW: {40ms~800ms} | - OW/PW: {5,4,3,2,1,1/2,1/3,1/4,1/5} |
| MRRT | - 50%~80% | - N/A |

For intra-frequency temporal prediction including FR1 and FR2, UE speed (e.g. 30km/h, 60km/h, 90km/h, etc.), the number of TX/RX (e.g. 1/1,1/2,1/4 for FR1, 8/4,16/4,32/4 for FR2), MRRT for FR1 (e.g. 50%, 80%), and OW/PW (e.g. 5/1, 4/1, 1/1, 1/2, etc.) are various, and may have impacts on the model performance based on the simulation results reported by companies so far. Therefore, we think they can be considered to verify the generalization performance of the model.

Proposal 2: For intra-frequency temporal prediction, the following aspects can be considered to verify the generalization performance of an AI/ML model over various configurations:

-  UE speed (e.g. 30km/h, 60km/h, 90km/h, etc.)

-  the number of TX/RX (e.g. 1/1,1/2,1/4 for FR1, 8/4,16/4,32/4 for FR2)

-  MRRT for FR1 (e.g. 50%, 80%)

-  OW/PW (e.g. 5/1, 4/1, 1/1, 1/2, etc.)

For inter-frequency prediction, RAN2 observed that the UE speed has minor impact on prediction accuracy for co-located scenario based on simulation results reported by companies in last meeting [2]. And some companies proposed that the correlation coefficient between input/output frequencies may have impact on prediction accuracy although we only agreed one pair of frequencies (i.e. 2GHz and 4GHz) for simulation evaluation. Therefore, the correlation coefficient between input/output frequencies may be considered to verify the generalization performance of the model.

- Companies should report with their simulation the correlation coefficient

- Higher-to-lower and lower-to-higher frequency prediction is comparable

- For co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy

Proposal 3: For inter-frequency prediction, the correlation coefficient between input/output frequencies may be considered to verify the generalization performance of an AI/ML model.

During RAN1 discussion on BM-Case1 in AI/ML for Air Interface, configuration of Set B and Set A are various, and different relations between Set A and Set B (e.g. Set A=Set B, Set B is a subset of Set A, Set A is different from Set B) are considered. For example, the results of 64 beams (Set A) can be predicted using actual measurements of 4 beams (Set B) where set B is subset of Set A, or the results of 64 beams (Set A) can be predicted using actual measurements of 4 beams (Set B) where set B is different with Set A, or the results of 64 beams (Set A) can be predicted using actual measurements of 8 beams (Set B) where set B is subset of Set A. In addition, the UE speed is also considered during RAN1 discussion on model generalization.

Case 1: Set B is different from Set A           Case 2: Set B is a subset of Set A

Figure 1. Examples of relations between Set A and Set B in BM-Case1

We understand that the case of FR2 intra-frequency spatial prediction is similar to BM-Case1, so the configuration of Set B and Set A, UE speed can be considered as variable to verify the generalization performance of an AI/ML model over various configurations.

Proposal 4: For FR2 intra-frequency spatial prediction, the following aspects can be considered to verify the generalization performance of an AI/ML model over various configurations:

-  Configuration of Set A and Set B (e.g. using 4 beams (Set B) to predict 64 beams (Set A) where Set B is subset of Set A, using 4 beams to predict 64 beams where Set B is different from Set A)

-  UE speed (e.g. 30km/h, 60km/h, 90km/h, etc.)

# 3	Conclusion

Here are the proposals for other aspects of simulation assumption.

Discussion on model generalization

Proposal 1: Reuse the cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations agreed in RAN1-led AI/ML for Air Interface SI for RRM measurement prediction:

-	Case 1: The AI/ML model performs inference/test based on the same dataset as training, e.g. the dataset for both training and inference/test is constructed based on Scenario#B/Configuration#B

-	Case 2: The AI/ML model performs inference/test based on the different dataset from training, e.g., the dataset for training is constructed based on Scenario#A/Configuration#A, but the dataset for inference/test is constructed based on Scenario#B/Configuration#B

-	Case 3: The AI/ML model performs inference/test based on the subset of dataset for training, e.g.  the training dataset is constructed by mixing datasets from multiple scenarios/configurations including Scenario#B/Configuration#B and Scenario#A/Configuration#A, but the inference/test dataset is constructed only based on a single Scenario/Configuration (i.e. Scenario#B/Configuration#B)

Proposal 2: For intra-frequency temporal prediction, the following aspects can be considered to verify the generalization performance of an AI/ML model over various configurations:

-  UE speed (e.g. 30km/h, 60km/h, 90km/h, etc.)

-  the number of TX/RX (e.g. 1/1,1/2,1/4 for FR1, 8/4,16/4,32/4 for FR2)

-  MRRT for FR1 (e.g. 50%, 80%)

-  OW/PW (e.g. 5/1, 4/1, 1/1, 1/2, etc.)

Proposal 3: For inter-frequency prediction, the correlation coefficient between input/output frequencies may be considered to verify the generalization performance of an AI/ML model.

Proposal 4: For FR2 intra-frequency spatial prediction, the following aspects can be considered to verify the generalization performance of an AI/ML model over various configurations:

-  Configuration of Set A and Set B (e.g. using 4 beams (Set B) to predict 64 beams (Set A) where Set B is subset of Set A, using 4 beams to predict 64 beams where Set B is different from Set A)

-  UE speed (e.g. 30km/h, 60km/h, 90km/h, etc.)

# 4	Reference

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- Draft_RAN2_127b_Meeting_Report_v1