# 3GPP TSG-WG2 Meeting #128              	   R2-2410346

# Orlando, United States, Nov 13 - 17, 2024

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#127bis meeting, the following agreements [1] were approved as below:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Discussion

Remaining time

In the RAN2#127 meeting, RAN2 had made a pending conclusion that：“Come back to this in the next meeting： UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast ”[2]. This aspect was not discussed in the last RAN2#127bis meeting due to limited time. In our point of view, providing remaining time information by broadcasting SIB message (e.g. SIB31) do have some benefits for both S&F capable UE and legacy UE. The following Table 1 gives a brief summary for using remaining time:

Table 1 The benefits for using remaining time for both S&F capable UE and legacy UE

| Benefits | S&F capable UE | Legacy UE |
| --- | --- | --- |
| Normal to S&F | Adjust configurations/services in advance to reduce performance degradation | UE can know when to go back to idle state/reselect to other cells in advance |
| S&F to normal | - Adjust configurations/services in advance to reduce performance degradation - UE can defer initial access procedure if the remaining time of S&F operation is short. | N/A |

From Table 1, we can see that remaining time is useful for both S&F capable UE and legacy UE including that:

- If the current mode is in normal NTN mode and soon switch to S&F operation:

- For S&F capable UEs, UE can adjust configurations/services in advance (e.g. release services with high QoS requirement or high latency requirements). So that UE can try to prevent performance degradation.

- For legacy UEs, UE can know when the service cell will be unavailable, thus UE also can adjust configurations/services for the interruption and try to reselect to other cells or change to other available satellite if any.

- If the current mode is in S&F operation and soon switch to normal NTN mode:

- For S&F capable UEs, similarly to normal NTN switching to S&F operation, UE can adjust configurations/services in advance (e.g. resume high services with high QoS requirement or high latency requirements); Besides, if UE wants to start initial access to the current satellite and the remaining time of S&F operation is short, then UE can wait till the cell switches back to normal NTN cell, then UE try initiate the access procedure to prevent multi rounds RA steps and save signaling overheads.

- For legacy UEs, no need to define legacy UE behaviour in S&F operation as per previous agreement in RAN2#127 meeting.

For the details of remaining time, we prefer to use a start point and duration time indication when indicating the remaining time. Since if just a relative value is indicated, there may be latency or loss during the transmission which may cause inconsistent remaining time between UE and satellite after the transmission. In this circumstance, UE may go back to idle state too early or too late and behave not as the network expects. Thus, we think a absolute value like start point plus duration time is a safer way to use

Proposal 1: UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

Proposal 2: For detail information of remaining time, it’s better to broadcast specific starting time (absolute value) and duration time together.

If the satellite knows there are frequently operation switching between normal operation and S&F operation before t-service timeout (e.g. the feeder link status is not stable), it’s better for network to indicate all switching patterns via SIB message (which means sending all indication information of every operation switching in SIB message at once). To be specific, if there are lots of operation switching between normal and S&F operation (e.g. more than 4 times), dynamic indication requires the network to send four indications when switching that will cause big signaling overheads. If UE failed to receive SIB message and there will be some latency and inconsistency. Thus, by using this kind of half-static method, the network can send all patterns of every operation switching in SIB message for the first time UE accesses in. When the patterns are not changed, network does not need to send it again. If changed, only sending changed part is enough. In this way, UE is clear to know the total numbers of operation transition and the duration time of each operation which will bring great convenience for UE to set related parameters and configurations in advance and also adjust service based on different patterns. If the S&F operation information does not change, the network also does not need to change the broadcast message and if some information is changed (e.g. duration time or S&F operation start time, etc), then the network only needs to broadcast the changed information in SIB message.

Proposal 3: For multi operation transition scenario, satellite can send all indication information of every S&F operation switching in SIB message when the UE first enters the cell .

Proposal 4: For multi operation transition scenario, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time).

Paging enhancement

When S&F capable UE work in the cell operating in S&F mode or out of satellite coverage, UE does not need to monitor paging message all the time. That means UE can relax the paging monitoring for some scenarios shown as below:

- When UE is not within the satellite coverage area:

  - The UE can determine when the next satellite carrying the DL data it needs (paging itself) will arrive based on the waiting timer in NAS rejection message. Therefore, when the UE is not within the coverage area of the satellite that means there is no satellite carrying downlink data to wake up the UE. Thus, the UE does not need to monitor the paging message.

- When UE is within the satellite coverage area and camps on a cell in S&F operation:

- The network has already transmitted stored DL data: as for the normal procedure, the satellite sends the stored downlink messages to the UE. If there is no data communication and the DRX timer runs out, the UE goes back to idle state. Therefore, during the remaining time of the S&F operation, the network will not send new downlink data to the UE since feederlink does not recover. Consequently, unless the UE actively needs to transmit UL data, it will not enter the connected state, and thus the UE does not need to monitor paging messages.

- The satellite never recover its feederlink: That means the current satellite does not have data transmission with ground EPC. Thus, the satellite only has stored UE uplink data and no new arrived downlink data. Therefore, even UE is within the coverage area, without recovered feederlink, UE does not need to monitor paging and only switch back to RRC connected state when has uplink data to send.

Proposal 5: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

Priority

- During the initial attach procedure, if UE is rejected with new reject message including wait timer, SA2 give the conclusion about the UE behaviour during the wait time as below:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

- From SA2 conclusion[3], it is clear to see that UE can still search for terrestrial or satellite in normal operation during the waiting time. Thus, in our opinion, if UE is rejected by the satellite during initial access procedure due to S&F operation, it can still try to camp on other normal cell or satellite if available before the wait timer expired. This operation can keep UE flexibility and do not need to only wait for S&F timer expired.

- Proposal 6: If UE is rejected by the satellite during initial access procedure due to S&F reason, it can still try to camp on other normal cell or satellite if available before the wait timer expired.

Data transmission

- In the RAN2#125bis meeting, RAN2 gave a conclusion on CP and UP solution as “The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)” [4]. About the UP solution, SA2 has already made the following conclusion:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

- From the conclusion, we can see that SA2 has agreed that legacy CP and UP solutions can be used for data traffic in S&F operation

- Observation 1: SA2 has confirmed that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Thus, from RAN2 point of view, it is nature to follow SA2 conclusion that legacy CP and UP solution can be reused.

- Proposal 7: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

- In the last RAN2#127 meeting, RAN2 has made an agreement that RAN2 will consider both MO and MT data within scope, it’s natural to support CIoT EPS optimisations and from Proposal 7, both CP and UP solution are supported. Thus when the satellite is moving away and makes service link unavailable, RRC connection suspend procedure in TS 36.300 as below [5] can be reused.

- Figure 1: RRC Connection Suspend procedure when the satellite is moving away

- During the initial access procedure, the wait timer is provided by the MME using rejection message to tell the UE how long should it wait for the next/current satellite coming back. Thus, in our opinion, the similar method can be also used for RRC connection state. That means when the satellite is moving away (the service link becomes available), eNB releases RRC connection using the legacy procedure with adding a new resumeTime IE in the RRCConnectionRelease message. In this way, UE can know how long it should wait until next/current satellite comes back and do not need to receive unnecessary paging messages to save power.

- Figure 2: RRC Connection Suspend procedure in S&F operation when the satellite is moving away

Proposal 8：RRCConnection suspend/resume procedure can be reused in S&F operation

Proposal 9：It is proposed to add a new IE to notify UE about the wait timer in RRCConnectionRelease message (e.g. resumeTime).

Conclusions

- Observation 1: SA2 has confirmed that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Remaining time

Proposal 1: UE is informed of the remaining time of the satellite’s current operation mode (normal or S&F) via System Information broadcast.

Proposal 2: For detail information of remaining time, it’s better to broadcast specific starting time (absolute value) and duration time together.

Proposal 3: For multi operation transition scenario, satellite can send all indication information of every S&F operation switching in SIB message when the UE first enters the cell .

Proposal 4: For multi operation transition scenario, the satellite only needs to broadcast the changed S&F operation information (e.g. starting time, duration time).

- Paging enhancement

Proposal 5: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

- Priority

- Proposal 6: If UE is rejected by the satellite during initial access procedure due to S&F reason, it can still try to camp on other normal cell or satellite if available before the wait timer expired.

- Data transmission

- Proposal 7: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

Proposal 8：RRCConnection suspend/resume procedure can be reused in S&F operation

Proposal 9：It is proposed to add a new IE to notify UE about the wait timer in RRCConnectionRelease message (e.g. resumeTime).

References

- 3GPP Chair_Notes, R2-127bis NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP Chair_Notes, R2-127 NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

- 3GPP Chair_Notes, R2-125bis NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage2 (Release 18)