3GPP TSG-RAN WG2 Meeting #128	R2-2410352

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on LP-WUS for RRC_CONNECTED mode in RAN2#127bis meeting [1].

- For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - Basic functionality for Option 1-2

    - PDCCH monitoring impacted by LP-WUS

In last meeting, the procedure for Option 1-2 was discussed and there was an FFS left as below.

FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

Based on the RAN1 agreement from RAN1-118: “UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS”, the intended UE behavior is not to trigger PDCCH monitoring, so the simplest way is to specify that the legacy drx-onDurationTimer is not started if LP-WUS is enabled/used for the UE. The new timer will be used to trigger MR PDCCH monitoring and the legacy drx-onDurationTimer will never be started.

Proposal 1: The legacy drx-onDurationTimer is not started if LP-WUS is enabled/used for the UE.

In some cases, there is no following data to be received before C-DRX Active Time related timers expire, it wastes power if UE keeps monitoring PDCCH. In legacy C-DRX, (Long) DRX Command MAC CE is introduced to overcome this issue, the NW can end the C-DRX Active Time if it finds no more data reception is needed to reduce power consumption. Based on the agreed procedure for Option 1-2, we understand similar mechanism can be supported naturally, e.g. MR (Long) DRX Command MAC CE can be used to stop drx- InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring. The example of procedure of Option 1-2 is illustrated in Figure 1 below.

Figure 1. Illustration for procedure of Option 1-2 based on timer and signalling

Proposal 2: RAN2 to confirm that legacy (Long) DRX Command MAC CE can be used to stop drx-InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring without additional specification impacts.

    - SRS transmission and CSI reporting

Considering LP-WUS, if LP-WUS triggers PDCCH monitoring for data transmission/reception, i.e. during C-DRX Active Time, normal SRS transmission and CSI reporting can be performed as legacy operation, which is beneficial for data scheduling. Based on the current agreements, when this new timer is running, UE is in C-DRX active time. Thus, during the time period that new timer is running, normal SRS transmission and CSI reporting are still performed to ensure the performance for data transmission.

Proposal 3: During C-DRX Active Time including the time period that new timer is running, normal SRS transmission and CSI reporting are performed as legacy operation in C-DRX Active Time without additional specification impacts.

During the time that is not C-DRX Active Time, in existing DRX and Rel-16 DCP operation, the UE behavior for SRS (periodic SRS and semi-persistent SRS) transmission and CSI reporting (periodic CSI that is L1-RSRP or not, and semi-persistent CSI) are additionally defined in order to save power, i.e. “not transmit periodic SRS and semi-persistent SRS, not report CSI on PUCCH and semi-persistent CSI configured on PUSCH”. It can also be applied without additional specification impact, if UE is currently monitoring LP-WUS, i.e. not in C-DRX Active Time.

Observation 1: If UE is currently monitoring LP-WUS, i.e. not in C-DRX Active Time, legacy operations for periodic/semi-persistent SRS, CSI reporting on PUCCH and semi-persistent CSI reporting configured on PUSCH, are applied without additional specification impact.

Moreover, RAN1 made an agreement on periodic CSI/L1-RSRP reporting in RAN1#118 meeting:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1)

  - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

  - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

For periodic CSI/L1-RSRP reporting, same behaviour as Rel-16 DCP is supported: within drx-onDurationTimer duration, there can be no PDCCH monitoring if UE is monitoring LP-WUS (during which the PDCCH monitoring is not triggered by legacy drx-onDurationTimer), in this case, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

Proposal 4: Within drx-onDurationTimer duration, if there is no PDCCH monitoring due to LP-WUS monitoring, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

    - RRM/RLM/BFD measurement

It assumes no impact on RRM/RLM/BFD measurement requirements according to RAN1 agreement. With C-DRX configured, RRM/RLM/BFD measurement requirements are clearly defined in RAN4 specification. Even with introduction of LP-WUS, it is combined UE operation in MR, e.g. PDCCH monitoring if during C-DRX Active Time, LP-WUS monitoring if outside C-DRX Active Time, legacy UE behaviours based on RAN4 requirements for RRM/RLM/BFD measurement are applied without RAN2 impact.

Observation 2: Legacy UE behaviors for RRM/RLM/BFD measurement are applied without RAN2 impact.

    - Configuration for LP-WUS

Regarding the issue how the NW provide the configuration to the UE, basically it can rely on the RRC measurement report. For example, the NW configures UE to report measurement results based on some events and periodically, the NW can know whether it is suitable for UE to start use of LP-WUS, e.g. the NW finds that the measurement result is good enough, or the change of the measurement result is small enough, the NW can configure UE to use LP-WUS. However, the NW only knows the measurement results from MR. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR. In this case, UE can inform the NW to stop using LP-WUS since the UE knows the link quality of LR, and also indicate whether the LP-WUS can be used again if it finds that the LR link quality is good enough. For example, UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS.

Proposal 5: UE can send assistance information to the network indicating to disable the LP-WUS or whether the LP-WUS can be enabled again.

After the NW provides the LP-WUS configuration to the UE by RRC signaling, UE considers the LP-WUS is enabled and start to use the LP-WUS function. However, one issue needs discussion is whether the UE starts LP-WUS monitoring once receives the LP-WUS configuration or not.

One option is to start LP-WUS monitoring after receiving the LP-WUS configuration. However, if UE applies LP-WUS monitoring ahead of the NW, ongoing services from the NW may be affected which causes UE’s data loss. On another hand, if the LP-WUS function takes effect on the NW before the UE, UE may miss the LP-WUS which leads to LR/MR state mismatch between UE and NW. Thus, it is beneficial that the UE and NW can align the start point for monitoring LP-WUS, e.g. UE and NW start LP-WUS monitoring after a time period after the UE sends RRC reconfiguration complete message.

Another option is to keep UE stay in C-DRX Active Time (i.e., using MR) after it receives the LP-WUS configuration. As UE receives RRC reconfiguration message and needs to receive PDCCH scheduling PUSCH for sending RRC reconfiguration complete message, UE can naturally stay in C-DRX Active Time. And there is no interruption for ongoing service if any. After the data transmission is finished and C-DRX Active Time ends, the UE goes back to LP-WUS monitoring as existing behavior.

Proposal 6: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring after it receives the LP-WUS configuration, alignment of the start point for LP-WUS monitoring needs further studied;

- Alt2: UE keeps in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE behavior.

  - Combination of solutions

Option 1-1 can be considered as one special case of Option 1-2, if the LP-WUS monitoring occasions are configured according to C-DRX cycle, LP-WUS triggers drx-onDurationTimer for Option 1-1 and triggers a timer controling PDCCH monitoring for Option 1-2. drx-InactivityTimer will be further started if there is data scheduled. UE stops MR PDCCH monitoring and switches back to LR if there is no more data. Additionally, Option 1-2 can have a more flexible configuration and better performance. Thus, it is unnecessary to configure Option 1-1 and Option 1-2 simultaneously for the same UE.

Proposal 7: Option 1-1 and Option 1-2 are not configured simultaneously for the same UE.

In RAN2#126 meeting, we had the following agreement:

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The same principle can be applied to Option 1-2, since Option 1-2 can technically provide better performance for power saving and latency compared with DCP, we don’t see the motivation to configure both legacy DCP and Option 1-2, besides, such combination gives additional complexity for the specification.

Proposal 8: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

  - Other higher layer impacts

    - SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 9: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 10: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

    - LP-WUS with CA

There were the following agreements in RAN1 on CA scenario.

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

Based on latest RAN1 agreement, LP-WUS is supported in CA scenario, PDCCH monitoring on different serving cells need to be further studied. For example, UE only monitors LP-WUS on PCell, or multiple LP-WUS monitoring occasions on PCell and SCell(s) can be defined to control different serving cell groups. Once the LP-WUS is received targeted to the UE, UE starts MR PDCCH monitoring in all activated serving cells, or only on specific serving cell (e.g. PCell) and relies on other signaling to activate PDCCH monitoring in other serving cell. Anyway, whether/how LP-WUS works when UE is configured with CA needs to be further studied with taking RAN1 progress into account.

Proposal 11: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Basic functionality for Option 1-2

Proposal 1: The legacy drx-onDurationTimer is not started if LP-WUS is enabled/used for the UE.

Proposal 2: RAN2 to confirm that legacy (Long) DRX Command MAC CE can be used to stop drx-InactivityTimer, which may terminate the C-DRX Active Time and trigger LP-WUS monitoring without additional specification impacts.

Proposal 3: During C-DRX Active Time including the time period that new timer is running, normal SRS transmission and CSI reporting are performed as legacy operation in C-DRX Active Time without additional specification impacts.

Proposal 4: Within drx-onDurationTimer duration, if there is no PDCCH monitoring due to LP-WUS monitoring, UE reports periodic CSI on PUCCH that is L1-RSRP/not L1-RSRP based on NW configuration, configuration fields like ps-TransmitPeriodicL1-RSRP and ps-TransmitOtherPeriodicCSI can be introduced.

Proposal 5: UE can send assistance information to the network indicating to disable the LP-WUS or whether the LP-WUS can be enabled again.

Proposal 6: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring after it receives the LP-WUS configuration, alignment of the start point for LP-WUS monitoring needs further studied;

- Alt2: UE keeps in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE behavior.

Combination of solutions

Proposal 7: Option 1-1 and Option 1-2 are not configured simultaneously for the same UE.

Proposal 8: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

Other higher layer impacts

Proposal 9: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 10: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

Proposal 11: RAN2 waits for more RAN1 conclusion for further study on LP-WUS with CA.

- References

R2-2409212, Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.