3GPP TSG-RAN WG2 Meeting #128                                                           R2-2410364

# Orlando, USA, November 18th - 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5161 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on NTN in FR2 bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, vivo, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per RAN1 LS in R1-2407406, RAN2 discussed the required specs impact in RAN2 and reached below agreements: R2-2408654 Miscellaneous corrections on NTN in FR2 bands ZTE Corporation, Sanechips CR Rel-18 38.331 18.3.0 5026 - F NR_NTN_enh-Core - We take this CR as a basis, apart from the fist change where we consider the proposal in R2-2408012 - Come back in the next meeting (a single CR from ZTE and vivo is expected in the next meeting). In R2-2408012, it is proposed to add FR2-NTN in the field description of subCarrierSpacing specified in several places to indicate the applicable SCS values applicable for FR2-NTN, which is the same as FR2-1 as defined in 38101-5. Additionally, below reasons for changes are proposed in R2-2408654: p-max is maximum power requirement defined for FR1 for cell (re)selection. This parameter is not used in FR2 due to lack of R4 requirement, which is the same for VSAT UEs operating in FR2-NTN. Therefore, a VSAT UE shall ignore this field and apply the requirement specified in TS 38101-5, and such reference is missing the field description of p-max. dl-SchedulingOffset-PDSCH-TypeA/dl-SchedulingOffset-PDSCH-TypeB are mandatory features with FR1/2 and TDD/FDD differentiation. IoT bits have been introduced to indicate whether UE has been IoT tested or not for both features. It was agreed in RAN2#103bis in R2-2105647/R2-2105648 to introduce separate indications in UERadioPagingInformation message so that NW can know whether UE has been IoT tested to support K0>0 for TDD/FDD FR1 and TDD-FR2, in order to allow NW to know if it can schedule K0 > 0 in Paging PDCCH. Such capabilities are missing in FDD FR2-NTN bands, which means NW cannot provide proper K0 configuration for UEs operating in FDD-FR2-NTN bands. Furthermore, it is observed that ‘fr2-Add-UE-NR-CapabilitiesNTN-r18’ is introduced for UE capability signalling in FR2 NTN bands which is only applicable for VSAT UEs. Considering VSAT UEs cannot operate in TN and FR1-NTN bands, therefore fr2-Add-UE-NR-CapabilitiesNTN-r18 shall always be present when it is used to report UE capability in FR2 NTN, there is no case UE will reuse fr2-Add-UE-NR-Capabilities when fr2-Add-UE-NR-CapabilitiesNTN is absent. Last but not least, RAN4 has defined a new default value of n-TimingAdvanceOffset for FDD FR2-NTN bands. The field description of corresponding fields n-TimingAdvanceOffset should be updated to reflect this. |
| f |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add FR2-NTN in the field description of SCS in several places to indicate the applicable SCS range for FR2-NTN 2. Clarify in the field description of p-max in SIB2/4/FrequencyInfoUL /FrequencyInfoUL-SIB that UE ignores this field in FR2 bands and applies the requirement as specified in TS 38101-5. 3. Separate indications are introduced in UERadioPagingInformation message to indicate whether UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A/B in FDD FR2-NTN bands. 4. Delete the ‘if absent..’ sentence from the field description of fr2-Add-UE-NR-CapabilitiesNTN. 5. Clarify the VSAT UEs to determine the applicable values of n-TimingAdvanceOffset for FR2-NTN based on table Table 7.1C.2-4 of TS 38.133. Impact analysis Impacted functionality: NTN operation in FR2-NTN Inter-operability analysis: There is no inter-operatibility issue foreseen for the first 4 changes. For the 5th change, if the network is implemented according to the CR and the UE is not, the network and the UE may have different understandings of the N_TA-Offset value, and vice versa. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 spec is not clear on UE requirement/UE capability for FR2-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.3.1, 6.3.2, 6.3.3, 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.306 CR1200 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1st CHANGE

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs                          OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}                            OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                                  OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                             OPTIONAL,      -- Need S

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-UEIdentityRemote-r17             RNTI-Value                                                          OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-Inactive-r17             SetupRelease { SRS-PosRRC-Inactive-r17 }                            OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17                                             OPTIONAL    -- Cond RANPaging

]],

[[

ncd-SSB-RedCapInitialBWP-SDT-r17    SetupRelease {NonCellDefiningSSB-r17}                               OPTIONAL    -- Need M

]],

[[

resumeIndication-r18                ENUMERATED {true}                                                   OPTIONAL,   -- Need N

srs-PosRRC-InactiveEnhanced-r18     SetupRelease { SRS-PosRRC-InactiveEnhanced-r18 }                    OPTIONAL,   -- Need M

ran-ExtendedPagingCycleConfig-r18   ExtendedPagingCycleConfig-r18                                       OPTIONAL,  -- Cond RANPaging

multicastConfigInactive-r18         SetupRelease { MulticastConfigInactive-r18 }                        OPTIONAL   -- Need M

]]

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                               OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                  OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...,

[[

freqPriorityListDedicatedSlicing-r17 FreqPriorityListDedicatedSlicing-r17                               OPTIONAL        -- Need M

]]

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-DL-r17        BWP-DownlinkDedicatedSDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17         TimeAlignmentTimer                                            OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17          RSRP-Range                                                    OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17        SetupRelease { CG-SDT-TA-ValidationConfig-r17 }               OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                    RNTI-Value                                                    OPTIONAL,   -- Need M

...,

[[

cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-RestrictionExt-v1800

OPTIONAL,   -- Need N

cg-MT-SDT-MaxDurationToNextCG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                 OPTIONAL    -- Need R

]]

}

CG-SDT-TA-ValidationConfig-r17 ::=  SEQUENCE {

cg-SDT-RSRP-ChangeThreshold-r17     ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22,

dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1}

}

BWP-DownlinkDedicatedSDT-r17 ::=    SEQUENCE {

pdcch-Config-r17                    SetupRelease { PDCCH-Config }                                       OPTIONAL,   -- Need M

pdsch-Config-r17                    SetupRelease { PDSCH-Config }                                       OPTIONAL,   -- Need M

...

}

BWP-UplinkDedicatedSDT-r17 ::=      SEQUENCE {

pusch-Config-r17                    SetupRelease { PUSCH-Config }                                       OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r17                 ConfiguredGrantConfigToAddModList-r16             OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r17                ConfiguredGrantConfigToReleaseList-r16            OPTIONAL,   -- Need N

...

}

CG-SDT-ConfigLCH-Restriction-r17 ::= SEQUENCE {

logicalChannelIdentity-r17          LogicalChannelIdentity,

configuredGrantType1Allowed-r17     ENUMERATED {true}                                                   OPTIONAL,   -- Need R

allowedCG-List-r17                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL    -- Need R

}

CG-SDT-ConfigLCH-RestrictionExt-v1800 ::= SEQUENCE {

cg-SDT-MaxDurationToNextCG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                    OPTIONAL    -- Need R

}

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

RSRP-ChangeThreshold-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16        OPTIONAL -- Need N

}

SRS-PosRRC-InactiveEnhanced-r18 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveEnhancedConfig-r18)

SRS-PosRRC-InactiveEnhancedConfig-r18 ::=    SEQUENCE {

srs-PosRRC-AggBW-InactiveConfigList-r18      SetupRelease { SRS-PosRRC-AggBW-InactiveConfigList-r18 }         OPTIONAL,  -- Need M

srs-PosTx-Hopping-r18                        SetupRelease { SRS-PosTx-Hopping-r18 }                           OPTIONAL,  -- Need M

srs-PosRRC-InactiveValidityAreaPreConfigList-r18 SetupRelease { SRS-PosRRC-InactiveValidityAreaPreConfigList-r18 } OPTIONAL, -- Need M

srs-PosRRC-InactiveValidityAreaNonPreConfig-r18  SetupRelease { SRS-PosRRC-InactiveValidityAreaConfig-r18 }        OPTIONAL, -- Need M

...

}

SRS-PosRRC-InactiveValidityAreaPreConfigList-r18  ::= SEQUENCE (SIZE(1..maxNrOfVA-r18)) OF SRS-PosRRC-InactiveValidityAreaConfig-r18

SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE {

srs-PosConfigValidityArea-r18                 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity,

srs-PosConfigNUL-r18                          SRS-PosConfig-r17                                              OPTIONAL,   -- Need R

srs-PosConfigSUL-r18                          SRS-PosConfig-r17                                              OPTIONAL,   -- Need R

bwp-NUL-r18                                   BWP                                                            OPTIONAL,   -- Need S

bwp-SUL-r18                                   BWP                                                            OPTIONAL,   -- Need S

areaValidityTA-Config-r18                     AreaValidityTA-Config-r18                                      OPTIONAL,   -- Need R

...,

[[

srs-PosConfigValidityAreaExt-v1830            SEQUENCE (SIZE(1..maxNrOfCellsInVA-Ext-r18)) OF CellIdentity   OPTIONAL    -- Need R

]]

}

AreaValidityTA-Config-r18 ::=             SEQUENCE {

inactivePosSRS-ValidityAreaTAT-r18        ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity},

inactivePosSRS-ValidityAreaRSRP-r18       RSRP-ChangeThreshold-r17                                           OPTIONAL,   -- Need R

autonomousTA-AdjustmentEnabled-r18        ENUMERATED {true}                                                  OPTIONAL    -- Need R

}

SRS-PosRRC-AggBW-InactiveConfigList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResSetCombInactive-r18)) OF

SRS-InactivePosResourceSetLinkedForAggBW-List-r18

SRS-InactivePosResourceSetLinkedForAggBW-List-r18 ::=  SEQUENCE (SIZE (2..maxNrOfLinkedSRS-PosResourceSet-r18)) OF

SRS-PosResourceSetLinkedForAggBW-r18

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

ExtendedPagingCycleConfig-r18 ::=  SEQUENCE {

extendedPagingCycle-r18            ENUMERATED {hf2, hf4, hf8, hf16, hf32, hf64, hf128,hf256, hf512, hf1024,

spare6, spare5, spare4, spare3, spare2, spare1},

pagingPTWLength-r18                ENUMERATED {ms1280, ms2560, ms3840, ms5120, ms6400, ms7680, ms8960, ms10240, ms11520,

ms12800, ms14080, ms15360, ms16640, ms17920, ms19200, ms20480, ms21760,

ms23040, ms24320, ms25600, ms26880, ms28160, ms29440, ms30720, ms32000,

ms33280, ms34560, ms35840, ms37120, ms38400, ms39680, ms40960}

}

MulticastConfigInactive-r18::=         SEQUENCE {

inactivePTM-Config-r18                 OCTET STRING (CONTAINING MBSMulticastConfiguration-r18)     OPTIONAL, -- Need S

inactiveMCCH-Config-r18                OCTET STRING (CONTAINING SystemInformation)                 OPTIONAL  -- Need N

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cellReselectionPriorities Dedicated priorities to be used for cell reselection as specified in TS 38.304 [20]. The maximum number of NR carrier frequencies that the network can configure through FreqPriorityListNR and FreqPriorityListDedicatedSlicing together is eight. If the same frequency is configured in both FreqPriorityListNR and FreqPriorityListDedicatedSlicing, the frequency is only counted once.

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

srs-PosRRC-InactiveEnhanced Contains the SRS for positioning configuration in RRC_INACTIVE state that is applicable for a validity area. The field also contains bandwidth aggregation (see TS 38.214 [19], clause 6.2.1.4.2) and frequency hopping configurations (see TS 38.214 [19], clause 6.2.1.4.1) for SRS for positioning in RRC_INACTIVE state.

measIdleConfig Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to mps-PriorityAccess for a new connection following a redirect to NR or set the resume cause to mps-PriorityAccess for a resume following a redirect to NR. If the target RAT is E-UTRA, see TS 36.331 [10]. The gNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the gNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCRelease message.

multicastConfigInactive Indicates whether the UE is configured to receive MBS multicast in RRC_INACTIVE. The presence of this field indicates the UE is configured to receive MBS multicast in RRC_INACTIVE; otherwise, the UE is not configured to receive MBS multicast in RRC_INACTIVE.

noLastCellUpdate Presence of the field indicates that the last used cell for PEI shall not be updated. When the field is absent, the PEI-capable UE shall update its last used cell with the current cell. The UE shall not update its last used cell with the current cell if the AS security is not activated.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. Based on UE capability, the network may include redirectedCarrierInfo in RRCRelease message with suspendConfig if this message is sent in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer (see 5.3.1.4 in TS 24.501 [23]).

srs-PosRRC-Inactive Contains the SRS for positioning configuration in RRC_INACTIVE state.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

voiceFallbackIndication Indicates the RRC release is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

[…]

#### –	SIB4

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1760      InterFreqCarrierFreqList-v1760              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1800      InterFreqCarrierFreqList-v1800              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v1700 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqList-v1730 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1730

InterFreqCarrierFreqList-v1760 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1760

InterFreqCarrierFreqList-v1800 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,   -- Need R

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellList           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

...

}

InterFreqCarrierFreqInfo-v1610 ::=  SEQUENCE {

interFreqNeighCellList-v1610        InterFreqNeighCellList-v1610                                OPTIONAL,    -- Need R

smtc2-LP-r16                        SSB-MTC2-LP-r16                                             OPTIONAL,    -- Need R

interFreqAllowedCellList-r16        InterFreqAllowedCellList-r16                                OPTIONAL,    -- Cond SharedSpectrum2

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                                OPTIONAL,    -- Cond SharedSpectrum

interFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF InterFreqCAG-CellListPerPLMN-r16   OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1700 ::=  SEQUENCE {

interFreqNeighHSDN-CellList-r17     InterFreqNeighHSDN-CellList-r17                             OPTIONAL,    -- Need R

highSpeedMeasInterFreq-r17          ENUMERATED {true}                                           OPTIONAL,    -- Need R

redCapAccessAllowed-r17             ENUMERATED {true}                                           OPTIONAL,    -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,    -- Cond SharedSpectrum

interFreqNeighCellList-v1710        InterFreqNeighCellList-v1710                                OPTIONAL     -- Cond SharedSpectrum2

}

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1730 ::=  SEQUENCE {

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1760 ::=  SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL,    -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1800 ::=  SEQUENCE {

dl-CarrierFreq-r18                  ARFCN-ValueNR                                               OPTIONAL,    -- Cond LessThan5MHz

frequencyBandList-r18               MultiFrequencyBandListNR-SIB                                OPTIONAL,    -- Cond LessThan5MHz

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                     OPTIONAL,    -- Need S

mobileIAB-CellList-r18              PCI-Range                                                   OPTIONAL,    -- Need R

mobileIAB-Freq-r18                  ENUMERATED {true}                                           OPTIONAL,    -- Need R

eRedCapAccessAllowed-r18            ENUMERATED {true}                                           OPTIONAL,    -- Need R

tn-AreaIdList-r18                   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF TN-AreaId-r18    OPTIONAL,    -- Need R

accessAllowed2RxXR-r18              ENUMERATED {true}                                           OPTIONAL     -- Need R

}

InterFreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PCI-Range

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighCellList-v1710 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1710

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                              OPTIONAL,   -- Need R

...

}

InterFreqNeighCellInfo-v1610 ::=    SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqNeighCellInfo-v1710 ::=    SEQUENCE {

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqExcludedCellList ::=       SEQUENCE (SIZE (1..maxCellExcluded)) OF PCI-Range

InterFreqAllowedCellList-r16 ::=    SEQUENCE (SIZE (1..maxCellAllowed)) OF PCI-Range

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

accessAllowed2RxXR Indicates if the cells on the frequency support 2Rx XR UEs. If present, 2Rx XR UEs shall consider only these NR frequencies in cell reselection evaluation.

channelAccessMode2 If present, this field indicates that the neighbor cells on the inter-frequency apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the neighbor cells on the inter-frequency do not apply any channel access procedure.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

dl-CarrierFreq This field indicates center frequency of the SS block of the neighbour cells, where the frequency corresponds to a GSCN value as specified in TS 38.101-1 [15] or TS 38.101-5 [75]. For a neighbouring carrier frequency when dl-CarrierFreq-r18 is included, the network sets the corresponding value of dl-CarrierFreq (without suffix) to 250, and the UE applies dl-CarrierFreq-r18 instead of dl-CarrierFreq (without suffix). In such case, if the UE does not support the GSCN value corresponding to the dl-CarrierFreq-r18, it ignores the corresponding neighbour cell.

eRedCapAccessAllowed Indicates whether eRedCap UEs are allowed to access cells on the frequency.

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. For a neighbouring carrier frequency when frequencyBandList-r18 is included, the network sets the corresponding value of freqBandIndicatorNR in frequencyBandList (without suffix) to 200, and the UE applies frequencyBandList-r18 instead of frequencyBandList (without suffix).

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.5.

highSpeedMeasInterFreq If the field is set to true and UE supports high speed inter-frequency IDLE/INACTIVE measurements, the UE shall apply the enhanced inter-frequency RRM requirements on the inter-frequency carrier to support high speed up to 500 km/h in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [14].

interFreqAllowedCellList List of allow-listed inter-frequency neighbouring cells, see TS 38.304 [20], clause 5.2.4.

interFreqCAG-CellList List of inter-frequency neighbouring CAG cells (as defined in TS 38.304 [20] per PLMN.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information. If interFreqCarrierFreqList-v1610, interFreqCarrierFreqList-v1700, interFreqCarrierFreqList-v1720, interFreqCarrierFreqList-v1730, interFreqCarrierFreqList-v1760 or interFreqCarrierFreqInfo-v1800 are present, they shall contain the same number of entries, listed in the same order as in interFreqCarrierFreqList (without suffix).

interFreqExcludedCellList List of exclude-listed inter-frequency neighbouring cells.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters. If interFreqNeighCellList-v1610 is present, it shall contain the same number of entries, listed in the same order as in interFreqNeighCellList (without suffix).

interFreqNeighHSDN-CellList List of inter-frequency neighbouring HSDN cells as specified in TS 38.304 [20].

mobileIAB-CellList Contains a PCI range on which mobile IAB cells may be deployed.

mobileIAB-Freq If present, it indicates that a mobile IAB node may be deployed on the inter-frequency carrier.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

plmn-IdentityIndex Index of the PLMN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

p-Max Value in dBm applicable for the neighbouring NR cells on this carrier frequency. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

q-OffsetCell Parameter "Qoffsets,n" in TS 38.304 [20].

q-OffsetFreq Parameter "Qoffsetfrequency" in TS 38.304 [20].

q-QualMin Parameter "Qqualmin" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffsetCell Parameter "Qqualminoffsetcell" in TS 38.304 [20]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20].

q-RxLevMinOffsetCell Parameter "Qrxlevminoffsetcell" in TS 38.304 [20]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter "QrxlevminoffsetcellSUL" in TS 38.304 [20]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20].

redCapAccessAllowed Indicates whether RedCap UEs are allowed to access cells on the frequency.

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference.

smtc2-LP Measurement timing configuration for inter-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in InterFreqCarrierFreqInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in InterFreqCarrierFreqInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the inter-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no inter-frequency neighbour cells with a Long Periodicity.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

ssb-PositionQCL Indicates the QCL relation between SS/PBCH blocks for a specific neighbor cell as specified in TS 38.213 [13], clause 4.1. If provided, the cell specific value overwrites the common value signalled by ssb-PositionQCL-Common in SIB4 for the indicated cell.

ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for inter-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}                                      OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                           OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                                                             OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                                                         OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing              SubcarrierSpacing                                                         OPTIONAL  -- Need M

]],

[[

ra-PrioritizationTwoStep-r16        RA-Prioritization                                                         OPTIONAL, -- Need R

candidateBeamRSListExt-v1610        SetupRelease{ CandidateBeamRSListExt-r16 }                                OPTIONAL  -- Need M

]],

[[

spCell-BFR-CBRA-r16                 ENUMERATED {true}                                                         OPTIONAL  -- Need R

]]

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

CandidateBeamRSListExt-r16::=       SEQUENCE (SIZE(1.. maxNrofCandidateBeamsExt-r16)) OF PRACH-ResourceDedicatedBFR

-- TAG-BEAMFAILURERECOVERYCONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

candidateBeamRSList, candidateBeamRSListExt-v1610 Set of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. This set includes all elements of candidateBeamRSList (without suffix) and all elements of candidateBeamRSListExt-v1610. The UE maintains candidateBeamRSList and candidateBeamRSListExt-v1610 separately: Receiving candidateBeamRSListExt-v1610 set to release releases only the entries that were configured by candidateBeamRSListExt-v1610, and receiving candidateBeamRSListExt-v1610 set to setup replaces only the entries that were configured by candidateBeamRSListExt-v1610 with the newly signalled entries. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

msg1-SubcarrierSpacing Subcarrier spacing for contention free beam failure recovery (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	BWP

The IE BWP is used to configure generic parameters of a bandwidth part as defined in TS 38.211 [16], clause 4.5, and TS 38.213 [13], clause 12.

For each serving cell the network configures at least an initial downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) initial uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The uplink and downlink bandwidth part configurations are divided into common and dedicated parameters.

BWP information element

-- ASN1START

-- TAG-BWP-START

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

-- TAG-BWP-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all subcarrier spacings and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see TS 38.211 [16], clause 4.2). Except for SUL, the network ensures the same cyclic prefix length is used in active DL BWP and active UL BWP within a serving cell.

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz For the initial DL BWP and operation in licensed spectrum this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Except for SUL, the network ensures the same subcarrier spacing is used in active DL BWP and active UL BWP within a serving cell. For the initial DL BWP and operation with shared spectrum channel access, the value of this field corresponds to the subcarrier spacing of the SSB associated to the initial DL BWP.

[…]

#### –	CSI-RS-ResourceConfigMobility

The IE CSI-RS-ResourceConfigMobility is used to configure CSI-RS based RRM measurements.

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=        SEQUENCE {

csi-RS-Index                        CSI-RS-Index,

slotConfig                          CHOICE {

ms4                                 INTEGER (0..31),

ms5                                 INTEGER (0..39),

ms10                                INTEGER (0..79),

ms20                                INTEGER (0..159),

ms40                                INTEGER (0..319)

},

associatedSSB                       SEQUENCE {

ssb-Index                           SSB-Index,

isQuasiColocated                    BOOLEAN

}                                                                                                           OPTIONAL, -- Need R

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

sequenceGenerationConfig            INTEGER (0..1023),

...,

[[

slotConfig-r17                      CHOICE {

ms4                                 INTEGER (0..255),

ms5                                 INTEGER (0..319),

ms10                                INTEGER (0..639),

ms20                                INTEGER (0..1279),

ms40                                INTEGER (0..2559)

}                                                                                                           OPTIONAL  -- Need R

]]

}

CSI-RS-Index ::=                    INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB and the support of increasedNumberofCSIRSPerMO capability (see TS 38.214 [19], clause 5.1.6.1.3).

density Frequency domain density for the 1-port CSI-RS for L3 mobility. See TS 38.211 [16], clause 7.4.1.

nrofPRBs Allowed size of the measurement BW in PRBs. See TS 38.211 [16], clause 7.4.1.

startPRB Starting PRB index of the measurement bandwidth. See TS 38.211 [16], clause 7.4.1.

CSI-RS-ResourceConfigMobility field descriptions

csi-RS-CellList-Mobility List of cells for CSI-RS based RRM measurements.

refServCellIndex Indicates the serving cell providing the timing reference for CSI-RS resources without associatedSSB. The field may be present only if there is at least one CSI-RS resource configured without associatedSSB. If this field is absent, the UE shall use the timing of the PCell for measurements on the CSI-RS resources without associatedSSB. The CSI-RS resources and the serving cell indicated by refServCellIndex for timing reference should be located in the same band.

subcarrierSpacing Subcarrier spacing of CSI-RS. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4 and DRBs.

allowedPHY-PriorityIndex This restriction applies only when the UL grant is a dynamic grant. If the field is present and the dynamic grant has a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to the dynamic grants indicating PHY-priority index equal to the values configured by this field. If the field is present and the dynamic grant does not have a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to this dynamic grant if the value of the field is p0, see TS 38.213 [13], clause 9. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any dynamic grants. Corresponds to "allowedPHY-PriorityIndex" as specified in TS 38.321 [3].

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3]. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements and/or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                          OPTIONAL,   -- Need R

nrofCSI-RS-ResourcesToAverage       INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)                   OPTIONAL,   -- Need R

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

offsetMO                            Q-OffsetRangeList,

cellsToRemoveList                   PCI-List                                                        OPTIONAL,   -- Need N

cellsToAddModList                   CellsToAddModList                                               OPTIONAL,   -- Need N

excludedCellsToRemoveList           PCI-RangeIndexList                                              OPTIONAL,   -- Need N

excludedCellsToAddModList           SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

allowedCellsToRemoveList            PCI-RangeIndexList                                              OPTIONAL,   -- Need N

allowedCellsToAddModList            SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement      OPTIONAL,   -- Need N

...,

[[

freqBandIndicatorNR                 FreqBandIndicatorNR                                             OPTIONAL,   -- Need R

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

]],

[[

smtc3list-r16                       SSB-MTC3List-r16                                                OPTIONAL,   -- Need R

rmtc-Config-r16                     SetupRelease {RMTC-Config-r16}                                  OPTIONAL,   -- Need M

t312-r16                            SetupRelease { T312-r16 }                                       OPTIONAL    -- Need M

]],

[[

associatedMeasGapSSB-r17            MeasGapId-r17                                                   OPTIONAL,   -- Need R

associatedMeasGapCSIRS-r17          MeasGapId-r17                                                   OPTIONAL,   -- Need R

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

measCyclePSCell-r17                 ENUMERATED {ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, spare1}

OPTIONAL,   -- Cond SCG

cellsToAddModListExt-v1710          CellsToAddModListExt-v1710                                      OPTIONAL    -- Need N

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]]

}

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

T312-r16 ::=                        ENUMERATED { ms0, ms50, ms100, ms200, ms300, ms400, ms500, ms1000}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

[[

ssb-PositionQCL-Common-r16              SSB-PositionQCL-Relation-r16                                OPTIONAL,   -- Cond SharedSpectrum

ssb-PositionQCL-CellsToAddModList-r16   SSB-PositionQCL-CellsToAddModList-r16                       OPTIONAL,   -- Need N

ssb-PositionQCL-CellsToRemoveList-r16   PCI-List                                                    OPTIONAL    -- Need N

]],

[[

deriveSSB-IndexFromCellInter-r17    ServCellIndex                                                   OPTIONAL,   -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,   -- Cond SharedSpectrum2

ssb-PositionQCL-Cells-r17           SetupRelease {SSB-PositionQCL-CellList-r17}                     OPTIONAL    -- Need M

]],

[[

cca-CellsToAddModList-r17           PCI-List                                                        OPTIONAL,   -- Need N

cca-CellsToRemoveList-r17           PCI-List                                                        OPTIONAL    -- Need N

]],

[[

ssb-ToMeasureAltitudeBasedList-r18  SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 }             OPTIONAL    -- Need M

]]

}

Q-OffsetRangeList ::=               SEQUENCE {

rsrpOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrqOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

sinrOffsetSSB                       Q-OffsetRange               DEFAULT dB0,

rsrpOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

rsrqOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0,

sinrOffsetCSI-RS                    Q-OffsetRange               DEFAULT dB0

}

ThresholdNR ::=                     SEQUENCE{

thresholdRSRP                       RSRP-Range                                                      OPTIONAL,   -- Need R

thresholdRSRQ                       RSRQ-Range                                                      OPTIONAL,   -- Need R

thresholdSINR                       SINR-Range                                                      OPTIONAL    -- Need R

}

CellsToAddModList ::=               SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddModListExt-v1710 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1710

CellsToAddModListExt-v1800 ::=      SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddModExt-v1800

CellsToAddMod ::=                   SEQUENCE {

physCellId                          PhysCellId,

cellIndividualOffset                Q-OffsetRangeList

}

CellsToAddModExt-v1710 ::=          SEQUENCE {

ntn-PolarizationDL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL,  -- Need R

ntn-PolarizationUL-r17              ENUMERATED {rhcp,lhcp,linear}                                   OPTIONAL   -- Need R

}

CellsToAddModExt-v1800 ::=          SEQUENCE {

ntn-NeighbourCellInfo-r18           NTN-NeighbourCellInfo-r18                                       OPTIONAL   -- Cond NeighbourCell

}

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...,

[[

rmtc-Bandwidth-r17                  ENUMERATED {mhz100, mhz400, mhz800, mhz1600, mhz2000}           OPTIONAL,   -- Need R

measDurationSymbols-v1700           ENUMERATED {sym140, sym560, sym1120}                            OPTIONAL,   -- Need R

ref-SCS-CP-v1700                    ENUMERATED {kHz120, kHz480, kHz960}                             OPTIONAL,   -- Need R

tci-StateInfo-r17               SEQUENCE {

tci-StateId-r17                  TCI-StateId,

ref-ServCellId-r17               ServCellIndex                                                  OPTIONAL   -- Need R

} OPTIONAL -- Need R

]],

[[

ref-BWPId-r17                   BWP-Id                                                              OPTIONAL   -- Need R

]]

}

SSB-PositionQCL-CellsToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-CellsToAddMod-r16

SSB-PositionQCL-CellsToAddMod-r16 ::= SEQUENCE {

physCellId-r16                        PhysCellId,

ssb-PositionQCL-r16                   SSB-PositionQCL-Relation-r16

}

SSB-PositionQCL-CellList-r17 ::= SEQUENCE (SIZE (1..maxNrofCellMeas)) OF SSB-PositionQCL-Cell-r17

SSB-PositionQCL-Cell-r17         ::= SEQUENCE {

physCellId-r17                        PhysCellId,

ssb-PositionQCL-r17                   SSB-PositionQCL-Relation-r17

}

SSB-ToMeasureAltitudeBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofAltitudeRanges-r18)) OF SSB-ToMeasureAltitudeBased-r18

SSB-ToMeasureAltitudeBased-r18 ::=     SEQUENCE {

altitudeRange-r18                      SEQUENCE {

altitudeMin-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeMax-r18                        Altitude-r18                                             OPTIONAL,  -- Need S

altitudeHyst-r18                       HysteresisAltitude-r18                                   OPTIONAL   -- Need R

},

ssb-ToMeasure-r18                      SSB-ToMeasure                                                OPTIONAL   -- Need S

}

NTN-NeighbourCellInfo-r18 ::=          SEQUENCE {

epochTime-r18                          EpochTime-r17,

ephemerisInfo-r18                      EphemerisInfo-r17,

referenceLocation-r18                  ReferenceLocation-r17                                        OPTIONAL   -- Need R

}

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

CellsToAddMod field descriptions

cellIndividualOffset Cell individual offsets applicable to a specific cell.

physCellId Physical cell identity of a cell in the cell list.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per CSI-RS resource as described in 5.5.5.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The field is used for the derivation of cell measurement results as described in 5.5.3.3 and the reporting of beam measurement information per SS/PBCH block index as described in 5.5.5.2.

allowedCellsToAddModList List of cells to add/modify in the allow-list of cells. It applies only to SSB resources.

allowedCellsToRemoveList List of cells to remove from the allow-list of cells.

associatedMeasGapSSB Indicates the associated measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapSSB2 Indicates the associated additional measurement gap for SSB measuring identified by ssb-ConfigMobility in this measurement object for NTN deployments. When multiple MeasObjectNR with the same SSB frequency are configured, the network configures the same measurement gap ID in this field for each MeasObjectNR. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapSSB.

associatedMeasGapCSIRS Indicates the associated measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object. If this field is absent, the associated measurement gap is the gap configured via gapFR1, gapFR2, or gapUE.

associatedMeasGapCSIRS2 Indicates the associated additional measurement gap for CSI-RS measuring identified by csi-rs-ResourceConfigMobility in this measurement object for NTN deployments. If this field is absent, the associated measurement gap is the gap indicated by associatedMeasGapCSIRS. In this release of the specification, this field is not configured for NTN deployments.

cellsToAddModList List of cells to add/modify in the cell list. If the network includes cellsToAddModListExt-v1710 and/or cellsToAddModListExt-v1800, it contains the same number of entries listed in the same order as in cellsToAddModList (i.e without suffix).

cellsToRemoveList List of cells to remove from the cell list.

excludedCellsToAddModList List of cells to add/modify in the exclude-list of cells. It applies only to SSB resources.

excludedCellsToRemoveList List of cells to remove from the exclude-list of cells.

freqBandIndicatorNR The frequency band in which the SSB and/or CSI-RS indicated in this MeasObjectNR are located and according to which the UE shall perform the RRM measurements. This field is always provided when the network configures measurements with this MeasObjectNR.

measCyclePSCell The parameter is used only when the PSCell is configured on the frequency indicated by the measObjectNR and the SCG is deactivated, see TS 38.133 [14]. The field may also be configured when the PSCell is not configured on that frequency. The network always configures measCyclePSCell for the measObjectNR associated with the PSCell if bfd-and-RLM is set to true and the SCG is deactivated. Value ms160 corresponds to 160 ms, value ms256 corresponds to 256 ms and so on.

measCycleSCell The parameter is used only when an SCell is configured on the frequency indicated by the measObjectNR and is in deactivated state, see TS 38.133 [14]. gNB configures the parameter whenever an SCell is configured on the frequency indicated by the measObjectNR, but the field may also be signalled when an SCell is not configured. Value sf160 corresponds to 160 sub-frames, value sf256 corresponds to 256 sub-frames and so on.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell associated with this MeasObject.

ntn-NeighbourCellInfo Includes satellite assistance information of an NTN neighbour cell.

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

ntn-PolarizationUL If present, this parameter indicates polarization information for uplink transmission on service link. If not present and ntn-PolarizationDL is present, UE assumes the same polarization for UL and DL.

offsetMO Offset values applicable to all measured cells with reference signal(s) indicated in this MeasObjectNR.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

referenceSignalConfig RS configuration for SS/PBCH block and CSI-RS.

refFreqCSI-RS Point A which is used for mapping of CSI-RS to physical resources according to TS 38.211 [16] clause 7.4.1.5.3.

smtc1 Primary measurement timing configuration. (see clause 5.5.2.10).

smtc2 Secondary measurement timing configuration for SS corresponding to this MeasObjectNR with PCI listed in pci-List. For these SS, the periodicity is indicated by periodicity in smtc2 and the timing offset is equal to the offset indicated in periodicityAndOffset modulo periodicity. periodicity in smtc2 can only be set to a value strictly shorter than the periodicity indicated by periodicityAndOffset in smtc1 (e.g. if periodicityAndOffset indicates sf10, periodicity can only be set of sf5, if periodicityAndOffset indicates sf5, smtc2 cannot be configured).

smtc3list Measurement timing configuration list for SS corresponding to IAB-MT. This is used for the IAB-node's discovery of other IAB-nodes and the IAB-Donor-DUs.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10.

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

ssb-PositionQCL-Common Indicates the QCL relationship between SS/PBCH blocks for all measured cells as specified in TS 38.213 [13], clause 4.1.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...,

[[

ra-PrioritizationForAccessIdentity-r16  SEQUENCE {

ra-Prioritization-r16                   RA-Prioritization,

ra-PrioritizationForAI-r16              BIT STRING (SIZE (2))

}                                                                                                       OPTIONAL,   -- Cond InitialBWP-Only

prach-RootSequenceIndex-r16             CHOICE {

l571                                    INTEGER (0..569),

l1151                                   INTEGER (0..1149)

}   OPTIONAL   -- Need R

]],

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                          OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]]

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 4-step RA. The network does not configure this list to have more than 16 entries.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2). This field is set to the same value for different repetition numbers associated with a specific FeatureCombination.

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).

[…]

#### –	RACH-ConfigCommonTwoStepRA

The IE RACH-ConfigCommonTwoStepRA is used to specify cell specific 2-step random-access type parameters.

RACH-ConfigCommonTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-START

RACH-ConfigCommonTwoStepRA-r16 ::=                   SEQUENCE {

rach-ConfigGenericTwoStepRA-r16                      RACH-ConfigGenericTwoStepRA-r16,

msgA-TotalNumberOfRA-Preambles-r16                   INTEGER (1..63)                                    OPTIONAL, -- Need S

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16   CHOICE {

oneEighth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                              ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                                 INTEGER (1..16),

eight                                                INTEGER (1..8),

sixteen                                              INTEGER (1..4)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-CB-PreamblesPerSSB-PerSharedRO-r16              INTEGER (1..60)                                                OPTIONAL, -- Cond SharedRO

msgA-SSB-SharedRO-MaskIndex-r16                      INTEGER (1..15)                                                OPTIONAL, -- Need S

groupB-ConfiguredTwoStepRA-r16                       GroupB-ConfiguredTwoStepRA-r16                                 OPTIONAL, -- Need S

msgA-PRACH-RootSequenceIndex-r16                     CHOICE {

l839                                                 INTEGER (0..837),

l139                                                 INTEGER (0..137),

l571                                                 INTEGER (0..569),

l1151                                                INTEGER (0..1149)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-TransMax-r16                                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL, -- Need R

msgA-RSRP-Threshold-r16                              RSRP-Range                                                     OPTIONAL, -- Cond 2Step4Step

msgA-RSRP-ThresholdSSB-r16                           RSRP-Range                                                     OPTIONAL, -- Need R

msgA-SubcarrierSpacing-r16                           SubcarrierSpacing                                              OPTIONAL, -- Cond 2StepOnlyL139

msgA-RestrictedSetConfig-r16                         ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB}                                OPTIONAL, -- Cond 2StepOnly

ra-PrioritizationForAccessIdentityTwoStep-r16        SEQUENCE {

ra-Prioritization-r16                                RA-Prioritization,

ra-PrioritizationForAI-r16                           BIT STRING (SIZE (2))

}                                                                                                                   OPTIONAL, -- Cond InitialBWP-Only

ra-ContentionResolutionTimer-r16                     ENUMERATED {sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}     OPTIONAL, -- Cond 2StepOnly

...,

[[

ra-PrioritizationForSlicingTwoStep-r17               RA-PrioritizationForSlicing-r17              OPTIONAL, -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL  -- Cond AdditionalRACH

]]

}

GroupB-ConfiguredTwoStepRA-r16 ::=                       SEQUENCE {

ra-MsgA-SizeGroupA-r16                               ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800,

b1000, b72, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r16                         ENUMERATED {minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r16                       INTEGER (1..64)

}

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigCommonTwoStepRA field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 2-step RA. The network does not configure this list to have more than 16 entries.

groupB-ConfiguredTwoStepRA Preamble grouping for 2-step random access type. If the field is absent then there is only one preamble group configured and only one msgA PUSCH configuration.

msgA-CB-PreamblesPerSSB-PerSharedRO Number of contention-based preambles used for 2-step RA type from the non-CBRA 4-step type preambles associated with each SSB for RO shared with 4-step type RA. The number of preambles for 2-step RA type shall not exceed the number of preambles per SSB minus the number of contention-based preambles per SSB for 4-step type RA. The possible value range for this parameter needs to be aligned with value range for the configured SSBs per RACH occasion in ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. The field is only applicable for the case of shared ROs with 4-step type random access.

msgA-PRACH-RootSequenceIndex PRACH root sequence index. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e., not within AdditionalRACH-Config), the UE applies the value in field prach-RootSequenceIndex in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the corresponding value of prach-RootSequenceIndex in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the value of restrictedSetConfig in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config) and both 2-step and 4-step are configured for the BWP, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. If the field is not configured in AdditionalRACH-Config and both 2-step and 4-step are configured in AdditionalRACH-Config, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon in the same AdditionalRACH-Config. The field is not present when RACH occasions are shared between 2-step and 4-step type random access in the BWP.

msgA-SSB-SharedRO-MaskIndex Indicates the subset of 4-step type ROs shared with 2-step random access type for each SSB. This field is configured when there is more than one RO per SSB. If the field is absent, and 4-step and 2-step has shared ROs, then all ROs are shared.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If the field is absent, the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) in case of 2-step only BWP, otherwise the UE applies the same SCS as Msg1 derived from RACH-ConfigCommon. The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

[…]

#### –	RateMatchPattern

The IE RateMatchPattern is used to configure one rate matching pattern for PDSCH, see TS 38.214 [19], clause 5.1.4.1.

RateMatchPattern information element

-- ASN1START

-- TAG-RATEMATCHPATTERN-START

RateMatchPattern ::=                SEQUENCE {

rateMatchPatternId                  RateMatchPatternId,

patternType                         CHOICE {

bitmaps                             SEQUENCE {

resourceBlocks                      BIT STRING (SIZE (275)),

symbolsInResourceBlock              CHOICE {

oneSlot                             BIT STRING (SIZE (14)),

twoSlots                            BIT STRING (SIZE (28))

},

periodicityAndPattern               CHOICE {

n2                                  BIT STRING (SIZE (2)),

n4                                  BIT STRING (SIZE (4)),

n5                                  BIT STRING (SIZE (5)),

n8                                  BIT STRING (SIZE (8)),

n10                                 BIT STRING (SIZE (10)),

n20                                 BIT STRING (SIZE (20)),

n40                                 BIT STRING (SIZE (40))

}                                                                                           OPTIONAL,   -- Need S

...

},

controlResourceSet                  ControlResourceSetId

},

subcarrierSpacing                   SubcarrierSpacing                                               OPTIONAL,   -- Cond CellLevel

dummy                               ENUMERATED { dynamic, semiStatic },

...,

[[

controlResourceSet-r16              ControlResourceSetId-r16                                        OPTIONAL    -- Need R

]]

}

-- TAG-RATEMATCHPATTERN-STOP

-- ASN1STOP

RateMatchPattern field descriptions

bitmaps Indicates rate matching pattern by a pair of bitmaps resourceBlocks and symbolsInResourceBlock to define the rate match pattern within one or two slots, and a third bitmap periodicityAndPattern to define the repetition pattern with which the pattern defined by the above bitmap pair occurs.

controlResourceSet This ControlResourceSet is used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it. In frequency domain, the resource is determined by the frequency domain resource of the CORESET with the corresponding CORESET ID. Time domain resource is determined by the parameters of the associated search space of the CORESET. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

periodicityAndPattern A time domain repetition pattern at which the pattern defined by symbolsInResourceBlock and resourceBlocks recurs. This slot pattern repeats itself continuously. Absence of this field indicates the value n1 (see TS 38.214 [19], clause 5.1.4.1).

resourceBlocks A resource block level bitmap in the frequency domain. A bit in the bitmap set to 1 indicates that the UE shall apply rate matching in the corresponding resource block in accordance with the symbolsInResourceBlock bitmap. If used as cell-level rate matching pattern, the bitmap identifies "common resource blocks (CRB)". If used for MBS broadcast CFR, the bitmap identifies "physical resource blocks" inside the MBS broadcast CFR. If used as BWP-level rate matching pattern, the bitmap identifies "physical resource blocks" inside the BWP or MBS multicast CFR. The first/ leftmost bit corresponds to resource block 0, and so on (see TS 38.214 [19], clause 5.1.4.1).

subcarrierSpacing The SubcarrierSpacing for this resource pattern. If the field is absent, the UE applies the SCS of the associated BWP. The value kHz15 corresponds to µ=0, the value kHz30 corresponds to µ=1, and so on. Only the following values are applicable depending on the used frequency (see TS 38.214 [19], clause 5.1.4.1): FR1: 15, 30 or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

SCS-SpecificCarrier field descriptions

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2).

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

txDirectCurrentLocation Indicates the downlink Tx Direct Current location for the carrier. A value in the range 0..3299 indicates the subcarrier index within the carrier. The values in the value range 3301..4095 are reserved and ignored by the UE. If this field is absent for downlink within ServingCellConfigCommon and ServingCellConfigCommonSIB, the UE assumes the default value of 3300 (i.e. "Outside the carrier"). (see TS 38.211 [16], clause 4.4.2). Network does not configure this field via ServingCellConfig or for uplink carriers.

subcarrierSpacing Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                                       OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16          ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}                   OPTIONAL, -- Need R

ssb-PositionQCL-r16                     SSB-PositionQCL-Relation-r16                                    OPTIONAL, -- Cond SharedSpectrum

highSpeedConfig-r16                     HighSpeedConfig-r16                                             OPTIONAL  -- Need R

]],

[[

highSpeedConfig-v1700               HighSpeedConfig-v1700                                               OPTIONAL, -- Need R

channelAccessMode2-r17              ENUMERATED {enabled}                                                OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}  OPTIONAL, -- Need R

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                        OPTIONAL, -- Cond SharedSpectrum2

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                              OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommon-v1700                                            OPTIONAL, -- Need R

ntn-Config-r17                      NTN-Config-r17                                                      OPTIONAL  -- Need R

]],

[[

featurePriorities-r17               SEQUENCE {

redCapPriority-r17                  FeaturePriority-r17                                             OPTIONAL, -- Need R

slicingPriority-r17                 FeaturePriority-r17                                             OPTIONAL, -- Need R

msg3-Repetitions-Priority-r17       FeaturePriority-r17                                             OPTIONAL, -- Need R

sdt-Priority-r17                    FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17            ENUMERATED {enabled}                                                OPTIONAL -- Cond SharedSpectrum2

]],

[[

featurePriorities-v1800             SEQUENCE {

msg1-Repetitions-Priority-r18       FeaturePriority-r17                                             OPTIONAL, -- Need R

eRedCapPriority-r18                 FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL, -- Need R

atg-Config-r18                      ATG-Config-r18                                                      OPTIONAL   -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.1) and uplink (TS 38.211 [16], clause 6.4.1.1.3).

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters configured by MIB and SIB1 (if provided) of the serving cell, with the exception of controlResourceSetZero and searchSpaceZero which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-r17 is applicable to SCS 480 kHz and SCS 960 kHz.

featurePriorities Indicates priorities for features, such as (e)RedCap, Slicing, SDT, MSG1-Repetitions, and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on this serving cell if n-TimingAdvanceOffset2 is not configured. If tag2 is configured for this serving cell, this field is to be applied to all uplink transmissions associated to tag-id configured for this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns. Rate match patterns defined here on cell level apply only to PDSCH of the same numerology (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

shortBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-periodicityServingCell The SSB periodicity in ms for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssb-PositionQCL Indicates the QCL relation between SSB positions for this serving cell as specified in TS 38.213 [13], clause 4.1.

ssb-PositionsInBurst For operation in licensed spectrum, indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access, the UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst (see TS 38.213 [13], clause 4.1). If the k-th bit of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the discovery burst transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k – 1 may be transmitted; if the kt-th bit is set to 0, the UE assumes that the corresponding SS/PBCH block(s) are not transmitted. The k-th bit is set to 0, where k > ssb-PositionQCL and the number of actually transmitted SS/PBCH blocks is not larger than the number of 1's in the bitmap. The network configures the same pattern in this field as in the corresponding field in ServingCellConfigCommonSIB. For operation with shared spectrum channel access in FR1, only mediumBitmap is used, and for FR2-2, longBitmap is used.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

[…]

#### –	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI). The network does not configure SRS specific power control parameters alpha (without suffix) or pathlossReferenceRS if unifiedTCI-StateType is configured for the serving cell.

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId              OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                   OPTIONAL,   -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16  OPTIONAL  -- Need N

]],

[[

dci-TriggeringPosResourceSetLink-r18    ENUMERATED { enabled }                                                  OPTIONAL  -- Need R

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]],

[[

usagePDC-r17                            ENUMERATED {true}                                              OPTIONAL, -- Need R

availableSlotOffsetList-r17             SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17               OPTIONAL, -- Need R

followUnifiedTCI-StateSRS-r17           ENUMERATED {enabled}                                           OPTIONAL  -- Need R

]],

[[

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL  -- Cond FollowUTCI

]]

}

AvailableSlotOffset-r17 ::=   INTEGER (0..7)

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL    -- Need R

]],

[[

spatialRelationInfo-PDC-r17             SetupRelease { SpatialRelationInfo-PDC-r17 }                   OPTIONAL,   -- Need M

resourceMapping-r17                     SEQUENCE {

startPosition-r17                       INTEGER (0..13),

nrofSymbols-r17                         ENUMERATED {n1, n2, n4, n8, n10, n12, n14},

repetitionFactor-r17                    ENUMERATED {n1, n2, n4, n5, n6, n7, n8, n10, n12, n14}

}                                                                                                      OPTIONAL,   -- Need R

partialFreqSounding-r17                 SEQUENCE {

startRBIndexFScaling-r17                CHOICE{

startRBIndexAndFreqScalingFactor2-r17   INTEGER (0..1),

startRBIndexAndFreqScalingFactor4-r17   INTEGER (0..3)

},

enableStartRBHopping-r17                ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

transmissionComb-n8-r17                 SEQUENCE {

combOffset-n8-r17                       INTEGER (0..7),

cyclicShift-n8-r17                      INTEGER (0..5)

}                                                                                                      OPTIONAL,   -- Need R

srs-TCI-State-r17                       CHOICE {

srs-UL-TCI-State                        TCI-UL-StateId-r17,

srs-DLorJointTCI-State                  TCI-StateId

}                                                                                                      OPTIONAL    -- Need R

]],

[[

repetitionFactor-v1730                  ENUMERATED {n3}                                                OPTIONAL,   -- Need R

srs-DLorJointTCI-State-v1730            SEQUENCE {

cellAndBWP-r17                          ServingCellAndBWP-Id-r17

}                                                                                                 OPTIONAL -- Cond DLorJointTCI-SRS

]],

[[

nrofSRS-Ports-n8-r18                    ENUMERATED {ports8, ports8tdm}                                 OPTIONAL,   -- Need R

combOffsetHopping-r18                   SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n4                       BIT STRING (SIZE (4)),

transmissionComb-n8                       BIT STRING (SIZE (8))

}                                                                                                  OPTIONAL,   -- Need R

hoppingWithRepetition-r18               ENUMERATED {symbol, repetition}                            OPTIONAL    -- Need R

}                                                                                                      OPTIONAL,   -- Need R

cyclicShiftHopping-r18                  SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                          OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n2                       BIT STRING (SIZE (8)),

transmissionComb-n4                       BIT STRING (SIZE (12)),

transmissionComb-n8                       BIT STRING (SIZE (6))

}                                                                                                  OPTIONAL,   -- Need R

hoppingFinerGranularity-r18             ENUMERATED {enable}                                        OPTIONAL    -- Need R

}                                                                                                      OPTIONAL    -- Need R

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-sp-Ext-r16           SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

...,

[[

periodicityAndOffset-p-Ext-r16            SRS-PeriodicityAndOffsetExt-r16                      OPTIONAL    -- Need R

]],

[[

srs-PosPeriodicConfigHyperSFN-Index-r18   ENUMERATED {even0, odd1}                             OPTIONAL     --Need R

]]

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...,

[[

txHoppingConfig-r18                       TxHoppingConfig-r18                                          OPTIONAL    --Need R

]]

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SRS-PeriodicityAndOffsetExt-r16 ::=     CHOICE {

sl128                                   INTEGER(0..127),

sl256                                   INTEGER(0..255),

sl512                                   INTEGER(0..511),

sl20480                                 INTEGER(0..20479)

}

SpatialRelationInfo-PDC-r17 ::=   SEQUENCE {

referenceSignal                   CHOICE {

ssb-Index                         SSB-Index,

csi-RS-Index                      NZP-CSI-RS-ResourceId,

dl-PRS-PDC                        NR-DL-PRS-ResourceID-r17,

srs                               SEQUENCE {

resourceId                        SRS-ResourceId,

uplinkBWP                         BWP-Id

},

...

},

...

}

TxHoppingConfig-r18 ::=             SEQUENCE {

overlapValue-r18                    ENUMERATED {zeroRB, oneRB, twoRB, fourRB},

numberOfHops-r18                    INTEGER(1..6),

slotOffsetForRemainingHopsList-r18  SEQUENCE (SIZE (1..maxNrofHops-1-r18) ) OF SlotOffsetForRemainingHops-r18,

...

}

SlotOffsetForRemainingHops-r18 ::=  SEQUENCE {

slotOffsetRemainingHops-r18         CHOICE {

aperiodic-r18                       SEQUENCE {

slotOffset-r18                      INTEGER (1..32)                                            OPTIONAL,   -- Need S

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

semi-persistent-r18                 SEQUENCE {

periodicityAndOffset-sp-r18         SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-sp-Ext-r18     SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need R

...

},

periodic-r18                        SEQUENCE {

periodicityAndOffset-p-r18          SRS-PeriodicityAndOffset-r16                               OPTIONAL,   -- Need R

periodicityAndOffset-p-Ext-r18      SRS-PeriodicityAndOffsetExt-r16                            OPTIONAL,   -- Need R

startPosition-r18                   INTEGER (0..13)                                            OPTIONAL,   -- Need S

...

},

...

}

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

dci-TriggeringPosResourceSetLink Indicates whether the single DCI-triggering SRS positioning resource sets across the linked carriers is enabled or not for bandwidth aggregation.

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n8 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

combOffsetHopping Configures UE with comb offset hopping. The hoppingId is used to initialize pseudo random comb offset hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingWithRepetition configures time-domain hopping behavior for repetition factor R>1. The hoppingSubset indicates a set of comb offset by a bit string (see clause 6.4.1.4.3 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scohSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

cyclicShiftHopping Configures UE with cyclic shift hopping. The hoppingId is used to initialize pseudo random cyclic shift hopping. If UE is configured with both comb offset and cyclic shift hopping, only one hoppingId is configured. The hoppingFinerGranularity enables finer granular hopping, see TS 38.211 [16], clause 6.4.1.4.2. If hoppingSubset is configured, hoppingFinerGranularity is not configured. The hoppingSubset indicates a set of cyclic shift by a bit string (see clause 6.4.1.4.2 of TS 38.211 [16]). The i-th bit in the bit string is set to 1 to indicate the scshSRSt=i-1, where t is determined by its ordinary position among the positive bits in bit string, i.e., if the i-th bit is a first positive bit, t=0; if the i-th bit is a second positive bit, t=1 , and so on.

enableStartRBHopping When this RRC parameter is configured, start RB location hopping is enabled for partial frequency sounding in different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS as described in clause 6.4.1.4 in TS 38.211.

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area. c-SRS Indicates the maximum bandwidth. When TxHoppingConfig is configured the valid values for c-SRS are such that the maximum bandwidth is: 104 PRBs, 48 PRBs, 132 PRBs, 64 PRBs, for 15,30,60,120 KHz respectively. The same value for c-SRS is configured for all the hops when TxHoppingConfig is configured.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'. For SRS for positioning configuration in multiple cells, the value of this field applies to all cells in the validity area.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

nrofSRS-Ports-n8 Number of ports if the number of antenna ports is 8. The value 'ports8' configures UE with 8 antenna ports and the value 'ports8tdm' configures the UE with 8 antenna ports which are partitioned into 2 subsets with each subset having 4 different ports, and the subsets are mapped to different OFDM symbols, see TS 38.211 [16], clause 6.4.1.4.2. If combOffsetHopping-r18 or cyclicShiftHopping-r18 is configured, this field is not set to ports8tdm. If this field is present UE ignores the field nrofSRS-Ports.

periodicityAndOffset-p, periodicityAndOffset-p-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured. For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz except when periodicity of 20480ms is configured. When periodicityAndOffset-p-Ext is present, periodicityAndOffset-p shall be ignored by the UE.

periodicityAndOffset-sp, periodicityAndOffset-sp-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For SRS-PosResource, values sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, values sl40960 and sl81920 cannot be configured for SCS=30kHz, and value sl81920 cannot be configured for SCS=60kHz. When periodicityAndOffset-sp-Ext is present, periodicityAndOffset-sp shall be ignored by the UE.

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). If resourceMapping-r17 is signalled, resourceMapping-r16 is not signalled and the UE shall ignore the resourceMapping (without suffix) and only the values of nrofSymbols which are integer multiples of the configured repetitionFactor can be configured. The network can only signal repetitionFactor-v1730 if resourceMapping-r17 is signalled. When repetitionFactor-v1730 is signalled, the UE shall ignore repetitionFactor-r17. For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area. nrofSymbols is same for all the hops when TxHoppingConfig is configured.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource, or slot offset for aperiodic SRS resource for positioning (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-PosResource. If the field is absent the UE applies no offset (value 0).

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement. This field is not configured if unifiedTCI-StateType is configured for the serving cell.

spatialRelationInfo-PDC Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS-PDC (see TS 38.214 [19], clause 6.2.1). The field is present in case of resourceType=periodic and usagePDC-r17=true in the SRS-ResourceSet, otherwise the field is absent.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31.

srs-PosPeriodicConfigHyperSFN-Index Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480m. If this field is not configured, the UE assumes that SRS for positioning periodictity longer than one Hyper SFN is not configured.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request" in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-TCI-State Configuration of either a UL TCI state or a joint TCI state for the SRS resource. In case of UL TCI-State, refers to the TCI state defined in ul-TCI-StateList in the BWP-UplinkDedicated where the SRS-Config is configured. In case of joint TCI state, refers to a TCI state defined in dl-OrJointTCI-StateList in pdsch-Config of the BWP-DownlinkDedicated and serving cell indicated by cellAndBWP. This field is absent when the SRS resource is in an SRS-ResourceSet configured with followUnifiedTCI-StateSRS-r17 or applyIndicatedTCI-State, or when the field unifiedTCI-StateType is not configured to the serving cell which the SRS resource is located in.

startRBIndexAndFreqScalingFactor Configures the UE with the startRBIndex and freqScalingFactor for partial frequency sounding as described in Clause 6.4.1.4 in TS 38.211. The startRBIndexForFScaling2 gives the startRBIndex when freqScalingFactor is 2 and the startRBIndexForFScaling4 gives the startRBIndex when FreqScalingFactor is 4

transmissionComb, transmissionComb-n2, transmissionComb-n4, transmissionComb-n8 Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1). If network configures field transmissionComb-n8, the UE ignores transmissionComb. If srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

SRS-ResourceSet, SRS-PosResourceSet field descriptions

alpha alpha value for SRS power control (see TS 38.213 [13], clause 7.3). When the field is absent the UE applies the value 1. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

aperiodicSRS-ResourceTriggerList An additional list of DCI "code points" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6). When the field is not included during a reconfiguration of SRS-ResourceSet of resourceType set to aperiodic, UE maintains this value based on the Need M; that is, this list is not considered as an extension of aperiodicSRS-ResourceTrigger for purpose of applying the general rule for extended list in clause 6.1.3.

aperiodicSRS-ResourceTrigger The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration (see TS 38.214 [19], clause 6).

applyIndicatedTCI-State This field indicates, for an SRS-ResourceSet, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.2.1.

associatedCSI-RS ID of CSI-RS resource associated with this SRS resource set in non-codebook based operation (see TS 38.214 [19], clause 6.1.1.2).

availableSlotOffsetList Indicates a list of up to four different available slot offset values from slot n+k to the slot where the aperiodic SRS resource set is transmitted, where slot n is the slot with the triggering DCI, and k is the slotOffset (without suffix) as described in clause 6.2.1 of TS 38.214 [19].

csi-RS ID of CSI-RS resource associated with this SRS resource set (see TS 38.214 [19], clause 6.1.1.2).

dl-PRS This field indicates a PRS configuration.

followUnifiedTCI-StateSRS When set to enabled, for SRS resource Set, the UE applies the "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 5.1.5. This parameter may be configured for aperiodic SRS for BM or SRS of any time-domain behavior for codebook, non-codebook, and antenna switching.

p0 P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed (see TS 38.213 [13], clause 7.3). If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

pathlossReferenceRS A reference signal (e.g. a CSI-RS config or a SS block) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRS-Pos A reference signal (e.g. a SS block or a DL-PRS config) to be used for SRS path loss estimation (see TS 38.213 [13], clause 7.3).

pathlossReferenceRSList Multiple candidate pathloss reference RS(s) for SRS power control, where one candidate RS can be mapped to SRS Resource Set via MAC CE (clause 6.1.3.27 in TS 38.321 [3]). The network can only configure this field if pathlossReferenceRS is not configured in the same SRS-ResourceSet.

resourceType Time domain behavior of SRS resource configuration, see TS 38.214 [19], clause 6.2.1. The network configures SRS resources in the same resource set with the same time domain behavior on periodic, aperiodic and semi-persistent SRS. The aperiodic SRS is not applicable for the UE in RRC_INACTIVE. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, the value of this field applies to all cells in the validity area.

slotOffset An offset in number of slots between the triggering DCI and the actual transmission of this SRS-ResourceSet. If the field is absent the UE applies no offset (value 0).

srs-PowerControlAdjustmentStates Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) or separate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH. If absent or release, the UE applies the value sameAs-Fci1 (see TS 38.213 [13], clause 7.3).

srs-ResourceIdList, srs-PosResourceIdList The IDs of the SRS-Resources/SRS-PosResource used in this SRS-ResourceSet/SRS-PosResourceSet. If this SRS-ResourceSet is configured with usage set to codebook, the srs-ResourceIdList contains at most 2 entries. If this SRS-ResourceSet is configured with usage set to nonCodebook, the srs-ResourceIdList contains at most 4 entries. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceIdList is commonly configured across cells within the validity area.

srs-ResourceSetId, srs-PosResourceSetId The ID of this resource set. It is unique in the context of the BWP in which the parent SRS-Config is defined. If srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured, srs-PosResourceSetId is commonly configured across cells within the validity area.

ssb-IndexServing Indicates SSB index belonging to a serving cell where the SRS is configured.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

usage Indicates if the SRS resource set is used for beam management, codebook based or non-codebook based transmission or antenna switching. See TS 38.214 [19], clause 6.2.1. Reconfiguration between codebook based and non-codebook based transmission is not supported.

usagePDC If configured, it indicates that this SRS resource set is used for propagation delay compensation. The field can be present in only one SRS-ResourceSet.

SRS-SpatialRelationInfoPos field descriptions

csi-RS-IndexServing Indicates CSI-RS index belonging to a serving cell.

dl-PRS This field indicates a PRS configuration.

resourceSelection Indicates whether the configured SRS spatial relation resource is a SRS-Resource or SRS-PosResource.

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

ssb-IndexServing Indicates SSB index belonging to a serving cell.

ssb-Ncell This field indicates a SSB configuration from neighboring cell.

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

DL-PRS-Info field descriptions

dl-PRS-ID This field specifies the UE specific TRP ID (see TS 37.355 [49]) for which PRS configuration is provided.

dl-PRS-ResourceSetId This field specifies the PRS-ResourceSet ID of a PRS resourceSet.

dl-PRS-ResourceId This field specifies the PRS-Resource ID of a PRS resource. If this field is absent, the UE determines the dl-PRS-ResourceID based on its PRS measurement from the TRP (see TS 37.355 [49]) and DL-PRS Resource Set.

SSB-Configuration field descriptions

halfFrameIndex Indicates whether SSB is in the first half or the second half of the frame. Value zero indicates the first half and value 1 indicates the second half.

integerSubframeOffset Indicates the subframe boundary offset of the cell in which SSB is transmited.

sfn0-Offset Indiactes the time offset of the SFN0 slot 0 for the cell with respect to SFN0 slot 0 of serving cell.

sfn-Offset Specifies the SFN offset between the cell in which SSB is transmited and serving cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of serving cell to the beginning of the closest subsequent radio frame #0 of the cell in which SSB is transmitted.

sfn-SSB-Offset Indicates the SFN offset of the transmitted SSB relative to the start of the SSB period. Value 0 indicates that the SSB is transmitted in the first system frame, value 1 indicates that SSB is transmitted in the second system frame and so on. The network configures this field according to the field ssb-Periodicity such that the indicated system frame does not exceed the configured SSB periodicity.

ssb-Freq Indicates the frequency of the SSB.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity Indicates the periodicity of the SSB. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1)

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

# 2nd CHANGE

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need N

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]]

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

}

RangeToBestCell    ::= Q-OffsetRange

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

cellEdgeEvaluation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.2).

cellEdgeEvaluationWhileStationary Indicates the criteria for a UE to detect that it is not at cell edge while stationary, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.4).

cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 38.304 [20].

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

combineRelaxedMeasCondition When both lowMobilityEvalutation and cellEdgeEvalutation criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when either or both of the criteria are met. (See TS 38.304 [20], clause 5.2.4.9.0)

combineRelaxedMeasCondition2 When both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when only the stationary criteria is met. (See TS 38.304 [20], clause 5.2.4.9.0)

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14].

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3.

highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies. If the field is absent, the UE shall not relax measurements on high priority frequencies beyond "Thigher_priority_search" unless both low mobility and not at cell edge criteria are fulfilled (see TS 38.133 [14], clauses 4.2.2.7, 4.2.2.10 and 4.2.2.11).

intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells.

lowMobilityEvaluation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.1).

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20].

p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

[…]

#### –	SIB4

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1760      InterFreqCarrierFreqList-v1760              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1800      InterFreqCarrierFreqList-v1800              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v1700 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqList-v1730 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1730

InterFreqCarrierFreqList-v1760 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1760

InterFreqCarrierFreqList-v1800 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,   -- Need R

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellList           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

...

}

InterFreqCarrierFreqInfo-v1610 ::=  SEQUENCE {

interFreqNeighCellList-v1610        InterFreqNeighCellList-v1610                                OPTIONAL,    -- Need R

smtc2-LP-r16                        SSB-MTC2-LP-r16                                             OPTIONAL,    -- Need R

interFreqAllowedCellList-r16        InterFreqAllowedCellList-r16                                OPTIONAL,    -- Cond SharedSpectrum2

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                                OPTIONAL,    -- Cond SharedSpectrum

interFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF InterFreqCAG-CellListPerPLMN-r16   OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1700 ::=  SEQUENCE {

interFreqNeighHSDN-CellList-r17     InterFreqNeighHSDN-CellList-r17                             OPTIONAL,    -- Need R

highSpeedMeasInterFreq-r17          ENUMERATED {true}                                           OPTIONAL,    -- Need R

redCapAccessAllowed-r17             ENUMERATED {true}                                           OPTIONAL,    -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,    -- Cond SharedSpectrum

interFreqNeighCellList-v1710        InterFreqNeighCellList-v1710                                OPTIONAL     -- Cond SharedSpectrum2

}

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1730 ::=  SEQUENCE {

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1760 ::=  SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL,    -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760                          OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1800 ::=  SEQUENCE {

dl-CarrierFreq-r18                  ARFCN-ValueNR                                               OPTIONAL,    -- Cond LessThan5MHz

frequencyBandList-r18               MultiFrequencyBandListNR-SIB                                OPTIONAL,    -- Cond LessThan5MHz

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                     OPTIONAL,    -- Need S

mobileIAB-CellList-r18              PCI-Range                                                   OPTIONAL,    -- Need R

mobileIAB-Freq-r18                  ENUMERATED {true}                                           OPTIONAL,    -- Need R

eRedCapAccessAllowed-r18            ENUMERATED {true}                                           OPTIONAL,    -- Need R

tn-AreaIdList-r18                   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF TN-AreaId-r18    OPTIONAL,    -- Need R

accessAllowed2RxXR-r18              ENUMERATED {true}                                           OPTIONAL     -- Need R

}

InterFreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PCI-Range

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighCellList-v1710 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1710

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                              OPTIONAL,   -- Need R

...

}

InterFreqNeighCellInfo-v1610 ::=    SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqNeighCellInfo-v1710 ::=    SEQUENCE {

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqExcludedCellList ::=       SEQUENCE (SIZE (1..maxCellExcluded)) OF PCI-Range

InterFreqAllowedCellList-r16 ::=    SEQUENCE (SIZE (1..maxCellAllowed)) OF PCI-Range

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

accessAllowed2RxXR Indicates if the cells on the frequency support 2Rx XR UEs. If present, 2Rx XR UEs shall consider only these NR frequencies in cell reselection evaluation.

channelAccessMode2 If present, this field indicates that the neighbor cells on the inter-frequency apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the neighbor cells on the inter-frequency do not apply any channel access procedure.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in TS 38.133 [14].

dl-CarrierFreq This field indicates center frequency of the SS block of the neighbour cells, where the frequency corresponds to a GSCN value as specified in TS 38.101-1 [15] or TS 38.101-5 [75]. For a neighbouring carrier frequency when dl-CarrierFreq-r18 is included, the network sets the corresponding value of dl-CarrierFreq (without suffix) to 250, and the UE applies dl-CarrierFreq-r18 instead of dl-CarrierFreq (without suffix). In such case, if the UE does not support the GSCN value corresponding to the dl-CarrierFreq-r18, it ignores the corresponding neighbour cell.

eRedCapAccessAllowed Indicates whether eRedCap UEs are allowed to access cells on the frequency.

frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. For a neighbouring carrier frequency when frequencyBandList-r18 is included, the network sets the corresponding value of freqBandIndicatorNR in frequencyBandList (without suffix) to 200, and the UE applies frequencyBandList-r18 instead of frequencyBandList (without suffix).

frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.5.

highSpeedMeasInterFreq If the field is set to true and UE supports high speed inter-frequency IDLE/INACTIVE measurements, the UE shall apply the enhanced inter-frequency RRM requirements on the inter-frequency carrier to support high speed up to 500 km/h in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [14].

interFreqAllowedCellList List of allow-listed inter-frequency neighbouring cells, see TS 38.304 [20], clause 5.2.4.

interFreqCAG-CellList List of inter-frequency neighbouring CAG cells (as defined in TS 38.304 [20] per PLMN.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information. If interFreqCarrierFreqList-v1610, interFreqCarrierFreqList-v1700, interFreqCarrierFreqList-v1720, interFreqCarrierFreqList-v1730, interFreqCarrierFreqList-v1760 or interFreqCarrierFreqInfo-v1800 are present, they shall contain the same number of entries, listed in the same order as in interFreqCarrierFreqList (without suffix).

interFreqExcludedCellList List of exclude-listed inter-frequency neighbouring cells.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters. If interFreqNeighCellList-v1610 is present, it shall contain the same number of entries, listed in the same order as in interFreqNeighCellList (without suffix).

interFreqNeighHSDN-CellList List of inter-frequency neighbouring HSDN cells as specified in TS 38.304 [20].

mobileIAB-CellList Contains a PCI range on which mobile IAB cells may be deployed.

mobileIAB-Freq If present, it indicates that a mobile IAB node may deployed on the inter-frequency carrier.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20].

plmn-IdentityIndex Index of the PLMN across the plmn-IdentityInfoList and npn-IdentityInfoList fields included in SIB1.

p-Max Value in dBm applicable for the neighbouring NR cells on this carrier frequency. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79].

[…]

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

additionalSpectrumEmission-v1760    AdditionalSpectrumEmission-v1760                        OPTIONAL    -- Need S

]],

[[

additionalSpectrumEmissionAerial-r18   AdditionalSpectrumEmission-r18                       OPTIONAL   -- Need S

]]

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If both additionalSpectrumEmission (without suffix) and additionalSpectrumEmission-v1760 are absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], tables 6.2.3.1-1A, 6.2A.3.1.1-2 and 6.2A.3.1.2-2, TS 38.101-2 [39], tables 6.2.3.1-2 and 6.2A.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured and if signalled, the same vaue in additionalSpectrumEmission-v1760 for all uplink carrier(s) of the same band with UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for these uplink carriers, and NS_01 for the remaining uplink carrier(s) of band n77. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

additionalSpectrumEmissionAerial The additional spectrum emission requirements to be applied by the aerial UE on this uplink (see TS 38.101-1 [15], clause 6.2K). If the field is absent, the aerial UE uses value indicated by the field additionalSpectrumEmission/ additionalSpectrumEmission-v1760.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. Value in dBm. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emission requirements as specified in TS 38.106 [79].

[…]

#### –	FrequencyInfoUL-SIB

The IE FrequencyInfoUL-SIB provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL-SIB information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-SIB-START

FrequencyInfoUL-SIB ::=             SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR-SIB                            OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18                 OPTIONAL    -- Need S

]]

}

FrequencyInfoUL-SIB-v1760 ::=       SEQUENCE {

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760

}

-- TAG-FREQUENCYINFOUL-SIB-STOP

-- ASN1STOP

FrequencyInfoUL-SIB field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

frequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

frequencyBandListAerial Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for aerial UE as defined in TS 38.101-1 [15], clause 6.2K. If the field is present, the aerial UE shall apply the first listed band which it supports in the frequencyBandListAerial field. If the field is absent, frequencyBandList applies.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emission requirements as specified in TS 38.106 [79].

# 3rd CHANGE

#### –	UERadioPagingInformation

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

Direction: gNB to/ from 5GC and gNB to/from gNB

UERadioPagingInformation message

-- ASN1START

-- TAG-UE-RADIO-PAGING-INFORMATION-START

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v15e0-IEs                      OPTIONAL

}

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v1700-IEs          OPTIONAL

}

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v18xx-IEs                   OPTIONAL

}

UERadioPagingInformation-v18xx-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                          OPTIONAL

}

-- TAG-UE-RADIO-PAGING-INFORMATION-STOP

-- ASN1STOP

UERadioPagingInformation field descriptions

supportedBandListNRForPaging Indicates the UE supported NR frequency bands which are derived by the gNB from UE-NR-Capability.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR2.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR2.

halfDuplexFDD-TypeA-RedCap Indicates whether the (e)RedCap UE only supports half-duplex operation for FDD in the indicated band(s).

inactiveStatePO-Determination Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state.

numberOfRxERedCap Indicates the number of Rx branches supported by an eRedCap UE.

numberOfRxRedCap Indicates the number of Rx branches supported by a RedCap UE.

supportOf2RxXR Indicates whether the UE is a 2Rx XR UE.

ue-RadioPagingInfo The field is used to transfer UE capability information used for paging. The gNB generates the ue-RadioPagingInfo and the contained UE capability information is absent when not supported by the UE.

# 4th CHANGE

#### –	NTN-Parameters

The IE NTN-Parameters is used to convey the subset of UE Radio Access Capability Parameters that apply to NTN access when there is a difference compared to TN access.

NTN-Parameters information element

-- ASN1START

-- TAG-NTN-PARAMETERS-START

NTN-Parameters-r17 ::= SEQUENCE {

inactiveStateNTN-r17                ENUMERATED {supported}                                OPTIONAL,

ra-SDT-NTN-r17                      ENUMERATED {supported}                                OPTIONAL,

srb-SDT-NTN-r17                     ENUMERATED {supported}                                OPTIONAL,

measAndMobParametersNTN-r17         MeasAndMobParameters                                  OPTIONAL,

mac-ParametersNTN-r17               MAC-Parameters                                        OPTIONAL,

phy-ParametersNTN-r17               Phy-Parameters                                        OPTIONAL,

fdd-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddXDD-Mode                           OPTIONAL,

fr1-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL,

ue-BasedPerfMeas-ParametersNTN-r17  UE-BasedPerfMeas-Parameters-r16                       OPTIONAL,

son-ParametersNTN-r17               SON-Parameters-r16                                    OPTIONAL

}

NTN-Parameters-v1820 ::= SEQUENCE {

fr2-Add-UE-NR-CapabilitiesNTN-r18   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL

}

-- TAG-NTN-PARAMETERS-STOP

-- ASN1STOP

NTN-Parameters field descriptions

fdd-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fdd-Add-UE-NR-Capabilities applies to NTN.

fr1-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr1-Add-UE-NR-Capabilities applies to NTN.

fr2-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN.

mac-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, mac-Parameters applies to NTN.

measAndMobParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, measAndMobParameters applies to NTN.

phy-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, phy-Parameters applies to NTN.

son-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, son-Parameters-r16 applies to NTN.

ue-BasedPerfMeas-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, ue-BasedPerfMeas-Parameters-r16 applies to NTN.

# 5th CHANGE

[…]

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                                       OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16          ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}                   OPTIONAL, -- Need R

ssb-PositionQCL-r16                     SSB-PositionQCL-Relation-r16                                    OPTIONAL, -- Cond SharedSpectrum

highSpeedConfig-r16                     HighSpeedConfig-r16                                             OPTIONAL  -- Need R

]],

[[

highSpeedConfig-v1700               HighSpeedConfig-v1700                                               OPTIONAL, -- Need R

channelAccessMode2-r17              ENUMERATED {enabled}                                                OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}  OPTIONAL, -- Need R

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                        OPTIONAL, -- Cond SharedSpectrum2

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                              OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommon-v1700                                            OPTIONAL, -- Need R

ntn-Config-r17                      NTN-Config-r17                                                      OPTIONAL  -- Need R

]],

[[

featurePriorities-r17               SEQUENCE {

redCapPriority-r17                  FeaturePriority-r17                                             OPTIONAL, -- Need R

slicingPriority-r17                 FeaturePriority-r17                                             OPTIONAL, -- Need R

msg3-Repetitions-Priority-r17       FeaturePriority-r17                                             OPTIONAL, -- Need R

sdt-Priority-r17                    FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17            ENUMERATED {enabled}                                                OPTIONAL -- Cond SharedSpectrum2

]],

[[

featurePriorities-v1800             SEQUENCE {

msg1-Repetitions-Priority-r18       FeaturePriority-r17                                             OPTIONAL, -- Need R

eRedCapPriority-r18                 FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL, -- Need R

atg-Config-r18                      ATG-Config-r18                                                      OPTIONAL   -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

ServingCellConfigCommon field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

dmrs-TypeA-Position Position of (first) DM-RS for downlink (see TS 38.211 [16], clause 7.4.1.1.1) and uplink (TS 38.211 [16], clause 6.4.1.1.3).

downlinkConfigCommon The common downlink configuration of the serving cell, including the frequency information configuration and the initial downlink BWP common configuration. The parameters provided herein should match the parameters configured by MIB and SIB1 (if provided) of the serving cell, with the exception of controlResourceSetZero and searchSpaceZero which can be configured in ServingCellConfigCommon even if MIB indicates that they are absent.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-r17 is applicable to SCS 480 kHz and SCS 960 kHz.

featurePriorities Indicates priorities for features, such as (e)RedCap, Slicing, SDT, MSG1-Repetitions, and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

longBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1.

lte-CRS-ToMatchAround Parameters to determine an LTE CRS pattern that the UE shall rate match around.

mediumBitmap Bitmap when maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1.

n-TimingAdvanceOffset The N_TA-Offset to be applied for all uplink transmissions on this serving cell if n-TimingAdvanceOffset2 is not configured. If tag2 is configured for this serving cell, this field is to be applied to all uplink transmissions associated to tag-id configured for this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2/Table 7.1C.2-4.

[…]

#### –	ServingCellConfigCommonSIB

The IE ServingCellConfigCommonSIB is used to configure cell specific parameters of a UE's serving cell in SIB1.

ServingCellConfigCommonSIB information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMONSIB-START

ServingCellConfigCommonSIB ::=      SEQUENCE {

downlinkConfigCommon                DownlinkConfigCommonSIB,

uplinkConfigCommon                  UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

supplementaryUplink                 UplinkConfigCommonSIB                                       OPTIONAL, -- Need R

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                           OPTIONAL, -- Need S

ssb-PositionsInBurst                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                      OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                               OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16      ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}               OPTIONAL, -- Need R

highSpeedConfig-r16                 HighSpeedConfig-r16                                         OPTIONAL  -- Need R

]],

[[

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-v1700    ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25} OPTIONAL,  -- Need R

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                      OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommonSIB-v1700                                 OPTIONAL  -- Need R

]],

[[

enhancedMeasurementNGSO-r17         ENUMERATED {true}                                           OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17                ENUMERATED {enabled}                                        OPTIONAL  -- Cond SharedSpectrum2

]],

[[

downlinkConfigCommon-v1760          DownlinkConfigCommonSIB-v1760                               OPTIONAL, -- Need R

uplinkConfigCommon-v1760            UplinkConfigCommonSIB-v1760                                 OPTIONAL  -- Need R

]]

}

-- TAG-SERVINGCELLCONFIGCOMMONSIB-STOP

-- ASN1STOP

ServingCellConfigCommonSIB field descriptions

channelAccessMode If present, this field indicates which channel access procedures to apply for operation with shared spectrum channel access as defined in TS 37.213 [48]. If the field is configured as "semiStatic", the UE shall apply the channel access procedures for semi-static channel occupancy as described in clause 4.3 in TS 37.213. If the field is configured as "dynamic", the UE shall apply the channel access procedures as defined in TS 37.213, clause 4.1 and 4.2.

channelAccessMode2 If present, this field indicates that the UE shall apply channel access procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure. The network always configures this field if channel access procedures are required for the serving cell within this region by regulations.

discoveryBurstWindowLength Indicates the window length of the discovery burst in ms (see TS 37.213 [48]). The field discoveryBurstWindowLength-v1700 is applicable to SCS 480 kHz and SCS 960 kHz.

enhancedMeasurementNGSO If the field is present and UE supports the enhanced cell reselection requirements for NTN NGSO in RRC_IDLE/RRC_INACTIVE, the UE shall apply the enhanced cell reselection requirements for NTN NGSO as specified in TS 38.133 [14], clauses 4.2C.2.3 and 4.2C.2.4.

groupPresence This field is present when maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1. The first/leftmost bit corresponds to the SS/PBCH index 0-7, the second bit corresponds to SS/PBCH block 8-15, and so on. Value 0 in the bitmap indicates that the SSBs according to inOneGroup are absent. Value 1 indicates that the SS/PBCH blocks are transmitted in accordance with inOneGroup.

inOneGroup When maximum number of SS/PBCH blocks per half frame equals to 4 as defined in TS 38.213 [13], clause 4.1, only the 4 leftmost bits are valid; the UE ignores the 4 rightmost bits. When maximum number of SS/PBCH blocks per half frame equals to 8 as defined in TS 38.213 [13], clause 4.1, all 8 bits are valid. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. When maximum number of SS/PBCH blocks per half frame equals to 64 as defined in TS 38.213 [13], clause 4.1, all 8 bit are valid; The first/ leftmost bit corresponds to the first SS/PBCH block index in the group (i.e., to SSB index 0, 8, and so on); the second bit corresponds to the second SS/PBCH block index in the group (i.e., to SSB index 1, 9, and so on), and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this serving cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this serving cell. See TS 38.133 [14], table 7.1.2-2/Table 7.1C.2-4.

ra-ChannelAccess If present, this field indicates that the UE shall apply channel access procedures before msg1/msgA transmission for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4.5 for FR2-2.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR1, only inOneGroup is used and the UE interprets this field same as mediumBitmap in ServingCellConfigCommon. The UE assumes that a bit in inOneGroup at position k > NSSBQCL is 0, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1. For operation with shared spectrum channel access in FR2-2, the m-th bit in groupPresence is set to 0 for m > NSSBQCL/8, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| SharedSpectrum | This field is mandatory present if this cell operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this cell operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |