3GPP TSG-RAN WG2 Meeting #128	R2-2410372

Orlando, USA, 18th – 22nd November, 2024

Agenda item:	8.7.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on DSR enhancements

Document for:	Discussion and Decision

# 1. Introduction

For DSR enhancements, RAN2 made the following agreements in RAN2#127 and RAN2#127bis:

- RAN2#127: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR - RAN2#127bis: Thresholds configuration We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. DSR MAC CE details For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, we will further discuss DSR enhancement according to the current progress.

# 2.	Discussion

## 2.1	Multiple reporting thresholds for DSR enhancements

Following the updated WI to specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG, RAN2 further discussed whether there are any constraints on how the NW configures DSR triggering and reporting thresholds in the last meeting. However, no agreement was made on this other than that the definition of delay-critical data will not change and request companies to further analyze if there is any impact on the DSR procedure in terms of triggering and cancellation of setting the triggering threshold with value lower than the largest reporting threshold.

Thresholds configuration We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

(i) Analysis on value of triggering threshold < largest reporting threshold

If the triggering threshold is set to be lower than the largest reporting threshold, then the information reported in DSR MAC CE may include the information of delay-critical data, and the information of non-delay-critical data that may become delay critical soon. However, there is concern that this may not be compatible to the current DSR triggering as follow:

If an LCG is configured for delay status reporting, the MAC entity shall: 1> if the smallest remaining value of the running PDCP discardTimers among all the SDUs buffered for the LCG that has not been transmitted in any MAC PDU and has not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the LCG: 2> trigger a DSR for the LCG.

Note that the “data volume” in the first bullet 1> above is just referring to the delay critical data, since only delay critical data is reported in legacy MAC CE. The analysis below assumes that this is not changed.

In the following, some analysis is made taking the following Figure (1) with a single LCH as an example where the buffered data of the LCH includes data1-data7.

Assuming that the triggering threshold is set to the smallest value among the multiple reporting thresholds, as time elapsed, the remaining time of data1 and data2 becomes below the configured triggering threshold at t1, then DSR is triggered by the LCH if there is no pending DSR for the LCH and UE reports the DSR MAC CE according to the configured multiple reporting thresholds. Figure (2a) shows the remaining time of each data on the LCH at t1, where the remaining time of delay-critical data “data1” and “data2” is below the triggering threshold, i.e., within range1, and the remaining time of “data3” and “data4” is below the reporting threshold2 and above reporting threshold1, i.e., within range2, and the remaining time of “data5” and “data6” is within range3. If the pending DSR is not cancelled, UE needs to include the data volume information corresponding to each time range (i.e., range1, range2 and range3 in Figure 2a) in the DSR MAC CE and include the DSR MAC CE for transmission if there is UL grant after t1 to accommodate it.

Scenario A (DSR is cancelled (as the associated DSR MAC CE is transmitted), while the buffered data is not all transmitted): Assuming there is only sufficient UL grant to transmit the DSR MAC CE (i.e., the pending DSR is cancelled) but no sufficient UL grant after t1 to transmit the buffered data and the buffered data on the LCH is still the same as Figure (1). At t2, the remaining time of data3 becomes below the configured DSR triggering threshold as shown in Figure (2b), DSR will again be triggered for the buffered data for the LCH since data3 is not part of the associated SDU and there is no pending DSR. Nothing is broken here since the UE just sends DSR with updated information of the data volume and this will provide the up-to-date data volume in each range to the gNB included any new data (e.g. data8 and above) that enters the reporting range. Some minor updates may be needed to make it clear that the “data volume” in the first bullet 1> in the triggering condition is just for delay critical data. There is no change in the legacy UE behaviour. The TP for such change can be very minor as follow:

If an LCG is configured for delay status reporting, the MAC entity shall: 1> if the smallest remaining value of the running PDCP discardTimers among all the SDUs buffered for the LCG that has not been transmitted in any MAC PDU and has not been reported as delay critical data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the LCG: 2> trigger a DSR for the LCG.

Observation 1:	R18 DSR is triggered if the delay-critical data has not been reported as data volume of R18 DSR MAC CE, where the data volume in R18 DSR MAC CE is only for delay-critical data. For R19 DSR triggering, same mechanism can be applied: a PDCP SDU that has not been reported as delay-critical data in DSR MAC CE can trigger DSR.

Scenario B (DSR is still pending, while the buffered data is not transmitted): Assuming there is no UL grant after t1 to transmit the pending DSR (i.e., the DSR MAC CE in t1) and the buffered data on the LCH is still the same as Figure (1). At t2, the remaining time of data3 becomes below the configured DSR triggering threshold as shown in Figure (2b). Figure (2b) shows the remaining time of each data on the LCH at t2, where the remaining time of delay-critical data “data1”, “data2” and “data3” is below the triggering threshold, i.e., within range1, and the remaining time of “data4”, “data5” and “data6” is below the reporting threshold2 and above reporting threshold1, i.e., within range2, and the remaining time of “data7” is within range3. If the pending DSR is not cancelled, UE needs to include the data volume information corresponding to each time range at t2 (i.e., range1, range2 and range3 in Figure 2b) in the DSR MAC CE and include the DSR MAC CE for transmission if there is UL grant after t2 to accommodate it. Any new data (e.g. data8 and above) that comes into reporting range will also be included into range 3. There again does not seem to be any downside for this scenario.

Observation 2:	For R19 DSR, when DSR is pending, UE includes in DSR MAC CE the up-to-date information same as in legacy.

Figure (1). Data arrival on the LCH                                  Figure (2a). Remaining time of each data on the LCH at t1

Figure (2b). Remaining time of each data on the LCH at t2

In existing DSR procedure, the cancellation of DSR is specified as follows in 38.321:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. …… After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Note that again from the cancellation criteria, the SDU associated to the DSR is just related to the delay critical data. Taking the above Figure (2a) and Figure (2b) as examples, in Figure (2a), data1 and data2 are regarded as the “PDCP SDUs associated with the DSR”, meaning that the triggered DSR is cancelled if data1 and data2 are discarded, or information of data1 and data2 are sent by the corresponding DSR MAC CE, or data1 and data2 are transmitted. Similarly, in Figure (2b), data1, data2 and data3 are regarded as the “PDCP SDUs associated with the DSR”, meaning that the triggered DSR is cancelled if data1, data2 and data3 are discarded or are transmitted, or information of them are sent by the corresponding DSR MAC CE. Even though the data volume information for ranges above the triggering thresholds may not be reported in some cases when only data1 and data2 (at t1 in Figure (2a)) or data1, data2 and data3 (at t2 in Figure (2b)) are transmitted or discarded without sending the DSR MAC CE, it will eventually be reported when their remaining time of those data (data4 and above) falls below the triggering threshold.

Observation 3:	The R18 DSR cancellation is only about the PDCP SDUs associated with the DSR, where the PDCP SDUs associated with the DSR only include delay-critical data. This mechanism can be reused for R19 DSR.

In addition, with the configuration of DSR triggering threshold smaller than the largest reporting threshold, the information of non-delay-critical data with remaining time of discardTimer larger than the DSR triggering threshold can be included in the DSR MAC CE, which can avoid the case where most of the data becomes delay-critical before UE transmits it, and leaving less time for the gNB to schedule it and causing more DSRs to be triggered.

Observation 4:	The DSR triggering threshold smaller than the largest DSR reporting thresholds can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB, leaving more time for the gNB to schedule.

Based on the above analysis, it can be seen that the legacy trigger and cancellation procedure can be reused even if the DSR triggering threshold is set with value smaller than the largest reporting threshold, and the case where leaving less time for the gNB to schedule can be avoided.

Conclusion 1:	For DSR triggering threshold less than the largest DSR reporting threshold: From our analysis, legacy DSR trigger condition and cancellation can be reused without any issues, and non-delay critical data that is sent in a previous enhanced DSR can trigger another DSR when it becomes delay critical data. In addition, providing non-delay critical data in the DSR MAC CE can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB, leaving more time for the gNB to schedule.

(ii) Analysis on value of triggering threshold >= largest reporting threshold

For the case where the triggering threshold is the largest one among all the configured reporting thresholds, the information reported in DSR MAC CE includes the information of delay-critical data with finer granularity. It is obvious that the legacy DSR trigger condition and cancellation can be reused considering that the DSR MAC CE only include the information of delay-critical data, and the only difference is the finer granularity of information of delay-critical data in DSR MAC CE. Although such configuration can provide a finer information of the delay-critical data to gNB, gNB can estimate the remaining time range of the previous reported delay-critical data. Therefore, it may cause signalling overhead if the triggering threshold is the largest one among all the reporting thresholds. However, if there are no network overhead issues, it is still valuable to configure a triggering threshold larger than or equal to the largest threshold to obtain the finer granularity information about delay-critical data.

Conclusion 2:	For DSR triggering threshold larger than or equal to the largest DSR reporting threshold: legacy DSR trigger condition and cancellation can be reused, but it may lead to signalling overhead as UE unnecessarily reports information of the previous delay-critical data again whenever DSR is triggered for the LCG. However, it can be left to network implementation to decide whether such overhead is acceptable.

Based on the above analysis and conclusions, it is obvious that there is no impact on DSR triggering and cancellation no matter how the DSR triggering threshold and reporting thresholds are configured, and it is more reasonable to leave it up to gNB implementation on how to configure the multiple DSR reporting thresholds and the DSR triggering threshold.

Observation 5:	The multiple thresholds only impact the content of DSR MAC CE. The legacy DSR triggering and cancellation can be reused with minor updates to clarify the associated SDUs are still referring to the delay critical data (no change to the UE behaviour) no matter how the reporting thresholds and trigger threshold is configured.

Proposal 1:	There is no constraints on how the NW configures DSR triggering and reporting thresholds, the configuration can be left to gNB implementation.

In conclusion, based on the above analysis on the relationship between the multiple reporting thresholds and the triggering threshold, it is obvious that UE should report the data according to the configured reporting thresholds, and for the case where reporting threshold is larger than triggering threshold, the information of non-delay critical data with remaining time of discardTimer larger than the threshold should also be reported.

Proposal 2:	UE should report the data according to the configured reporting thresholds even when this data is non-delay critical, i.e., in the case reporting threshold is larger than triggering threshold.

## 2.2	Other cases of Non-delay critical data in enhanced DSR

RAN2#127 discussed the non-delay data related issues for enhanced DSR and agreed the following.

For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

The non-delay critical data ahead of delay critical data in transmission buffer size may consume the resources scheduled for delay-critical data, making the delay-critical data to be discarded. If the transmission of non-delay-critical data has impact on transmission of delay-critical data, but the information is not indicated through any signaling, e.g., DSR or BSR, the gNB may not know the existence of the non-delay-critical data and will not schedule sufficient grant to the UE. The cases where non-delay-critical data has impact on transmission of delay-critical data include the following:

- When PSI-based discard is activated, low importance data which is in front of the delay-critical data impacts the transmission of delay-critical data. Currently, when DSR is triggered and PSI-based discard is activated, the non-delay-critical data information in front of the delay-critical data is not included in DSR MAC CE. However, RAN2 agreed in the previous meeting that intra-LCH scheduling is not in RAN2 scope, this means that data within an LCH is assembled in a first-in and first-out manner. Therefore, there is no doubt that non-delay-critical data in front of the delay-critical data will be assembled firstly and consume some UL grant, under which case the delay-critical data transmission may be impacted and even be discarded when there is not enough UL grant. Therefore, the non-delay-critical data information which is in front of the delay-critical data should be reported to gNB through enhanced DSR MAC CE, so that UE can request more UL grant from gNB and avoid the delay-critical data being discarded.

Observation 6:	When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Proposal 3:	The data volume of the low importance PDU sets which is ahead of delay-critical data in the UE buffer when PSI-based discard is activated should also be included in the enhanced DSR.

## 2.3	DSR MAC CE format

In RAN2#127bis, the issues of new DSR MAC CE format are discussed and RAN2 agreed the following, where the use of one-bit indication related to the pair of remaining time and buffer size information needs further discussion.

A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

With respect of the one-bit indication for remaining time and buffer size information pair (name as information pair for short) associated with a reporting threshold, RAN2 needs to further discuss whether this one-bit indication is used to indicate the existence of a certain information pair associated with a reporting threshold (Option 1), or to indicate the existence of next information pair associated with a reporting threshold (Option 2).

For Option 1, a separate field for each information pair in an LCG needs to be introduced, indicating the existence of the remaining time and buffer size information pair for a specific threshold range. Figure (3a) show an example of the new DSR MAC CE format in Option 1, assuming that 4 multiple thresholds are configured for each LCG, e.g., a one-byte length new field is introduced for each LCG. For LCG0, if the value of Rangex bit is 1, then the new DSR MAC CE includes the information pair of Rangex for LCG0, if the value of Rangex bit is 0, then the new DSR MAC CE doesn’t include the information pair of Rangex for LCG0, where Rangex can be regarded as the remaining time range between the two configured thresholds. For example, if the value of Range bit is 00001100, then the new DSR MAC CE includes the information pair of Range4 and Range3 for LCG0, and doesn’t include the information pair of other Ranges for LCG0. With such information, NW can know whether each information pair of each LCG exists and the corresponding starting bit if an information pair exist, and thus enabling the parallel decoding. In other words, the NW can obtain the full picture of the reported information as soon as receiving the DSR MAC CE, rather than waiting for the decoding of the MAC CE, so that the NW can schedule the transmission resources for UE in a timely manner.

Figure (3a). Example of new DSR MAC CE format in Option 1

Observation 7:	A one-bit indication to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, can enable the parallel decoding of the new DSR MAC CE by NW.

For Option 2, the one-bit indication is included in the reported information pair in each LCG, indicating whether the next information pair for the LCG exist or not. Taking the following Figure (3b) as an example, where 4 multiple thresholds are configured for each LCG. For LCG0, if the value of One-bitx bit is 1, then the new DSR MAC CE includes further information pair for LCG0, otherwise, the new DSR MAC CE doesn’t include further information pair for LCG0, meaning that the current information pair is the last one for LCG0. For example, if the value of One-bit1 to One-bit3 in LCG0 is 110, then the new DSR MAC CE includes the information pair of remaining time range 1, 2 and 3 for LCG0, and the next information pair is for next LCG. Applying the reserved bit in such way, can reduce the signaling overhead by 1 byte for each LCG for the new DSR MAC CE format, compared with Option 1. However, with respect of decoding, NW cannot have full picture of the reported information as soon as reception of the DSR MAC CE, considering that NW doesn’t know starting bit of each information pair for each LCG with such dynamic one-bit indication, and thus NW should decode in a serial way. Moreover, NW does not know which range is included in the DSR MAC CE until it decodes the remaining time value.

Figure (3b). Example of new DSR MAC CE format in Option 2

Observation 8:	A one-bit indication to indicate whether a further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, will cause decoding latency (e.g. NW does not know which range has been included until it decodes the remaining time etc.), and impact the transmission of delay-critical data.

Considering that the transmission latency is the key factor for XR traffic, it is better to adopt a DSR MAC CE format which is beneficial for XR scheduling, thus we propose the following:

Proposal 4:	Include a bitmap in a MAC CE where each bit indicates whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

## 2.4	Other issues for DSR

In RAN2#127bis, the issue of how to use the new DSR MAC CE format is discussed and RAN2 agree the following FFS:

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

If NW configures multiple reporting thresholds for at least one of the LCGs, UE can report the delay information using the new DSR MAC CE when DSR is triggered for an LCG configured with multiple reporting thresholds, and the reported information is for multiple reporting range. If DSR is triggered for an LCG configured with multiple reporting thresholds, and UE only has information for one threshold, then the legacy DSR MAC CE format is used (note that if data volume is reported for only one reporting range, it has to be the one containing delay critical data).

Proposal 5:	Use the new DSR MAC CE in case there is at least one LCG configured with multiple reporting thresholds has triggered DSR and the reported information is for multiple thresholds, in other cases, legacy DSR format is used.

In addition, even though the new DSR MAC CE format has larger signalling overhead compared with R18 DSR MAC CE, the new DSR MAC CE at most carries around 100 bytes information, which is not a burden for NW to schedule. Hence, in our view, there is no need to consider the case where UL grant is not enough to carry the new DSR MAC CE.

Proposal 6:	No need to consider the case where UL grant is not enough to carry the new DSR MAC CE or the truncated DSR MAC CE issues.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for DSR enhancements. The following observations and proposals were made:

Multiple reporting thresholds for DSR enhancements:

Observation 1:	R18 DSR is triggered if the delay-critical data has not been reported as data volume of R18 DSR MAC CE, where the data volume in R18 DSR MAC CE is only for delay-critical data. For R19 DSR triggering, same mechanism can be applied: a PDCP SDU that has not been reported as delay-critical data in DSR MAC CE can trigger DSR.

Observation 2:	For R19 DSR, when DSR is pending, UE includes in DSR MAC CE the up-to-date information same as in legacy.

Observation 3:	The R18 DSR cancellation is only about the PDCP SDUs associated with the DSR, where the PDCP SDUs associated with the DSR only includes delay-critical data. This mechanism can be reused for R19 DSR.

Observation 4:	The DSR triggering threshold smaller than the largest DSR reporting thresholds can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB, leaving more time for the gNB to schedule.

Conclusion 1:	For DSR triggering threshold less than the largest DSR reporting threshold: From our analysis, legacy DSR trigger condition and cancellation can be reused without any issues, and non-delay critical data that is sent in a previous enhanced DSR can trigger another DSR when it becomes delay critical data. In addition, providing non-delay critical data in the DSR MAC CE can provide more information (e.g. non-delay critical data that may become delay critical soon etc.) to the gNB, leaving more time for the gNB to schedule.

Conclusion 2:	For DSR triggering threshold larger than or equal to the largest DSR reporting threshold: legacy DSR trigger condition and cancellation can be reused, but it may lead to signalling overhead as UE unnecessarily reports information of the previous delay-critical data again whenever DSR is triggered for the LCG. However, it can be left to network implementation to decide whether such overhead is acceptable.

Observation 5:	The multiple thresholds only impact the content of DSR MAC CE. The legacy DSR triggering and cancellation can be reused with minor updates to clarify the associated SDUs are still referring to the delay critical data (no change to the UE behaviour) no matter how the reporting thresholds and trigger threshold is configured.

Proposal 1:	There is no constraints on how the NW configures DSR triggering and reporting thresholds, the configuration can be left to gNB implementation.

Proposal 2:	UE should report the data according to the configured reporting thresholds even when this data is non-delay critical, i.e., in the case reporting threshold is larger than triggering threshold.

Other cases of non-delay critical data in enhanced DSR:

Observation 6:	When PSI-based discard is activated, if the information of non-delay-critical data which is in front of the delay-critical data is not reported to gNB, then delay-critical data transmission will be impacted by these non-delay-critical data.

Proposal 3:	The data volume of the low importance PDU sets which is ahead of delay-critical data in the UE buffer when PSI-based discard is activated should also be included in the enhanced DSR.

DSR MAC CE format:

Observation 7:	A one-bit indication to indicate whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, can enable the parallel decoding of the new DSR MAC CE by NW.

Observation 8:	A one-bit indication to indicate whether a further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG, will cause decoding latency (e.g. NW does not know which range has been included until it decodes the remaining time etc.), and impact the transmission of delay-critical data.

Proposal 4:	Include a bitmap in a MAC CE where each bit indicates whether a certain pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Proposal 5:	Use the new DSR MAC CE in case there is at least one LCG configured with multiple reporting thresholds has triggered DSR and the reported information is for multiple thresholds, in other cases, legacy DSR format is used.

Proposal 6:	No need to consider the case where UL grant is not enough to carry the new DSR MAC CE or the truncated DSR MAC CE issues.

# 4. Reference

[1]. RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3