3GPP TSG-RAN WG2 Meeting #128	R2-2410373

Orlando, USA, 18th – 22nd November, 2024

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on LCP enhancements

Document for:	Discussion and Decision

# 1. Introduction

For LCP enhancements, RAN2 needs to specify additional logical channel priority handling using delay/deadline information of packets as in the updated WID [1]. With respect of the additional priority handling, RAN2#127bis discussed the related issues and had the following agreements:

How/when to apply additional priority As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation Remaining time threshold Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. DISCUSSION on terminology used for data with adjusted priority: For now, we will use “LCH priority-adjusted data”

In this contribution, we will further discuss the additional priority handling based on the current progress.

# 2.	Discussion

## 2.1	Additional priority handling

### 2.1.1	Triggering of additional priority

In the last meeting, a separate per LCH remaining time threshold is agreed for the additional priority handling:

Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

However, it has not been discussed how this new LCP threshold is applied to the remaining time of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel.

In existing DSR procedure, the trigger condition for DSR is applied to the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel (highlighted below) as follow and this can also be applied for the LCP remaining time threshold, and should be applied to the buffered PDCP SDUs that have or have not been transmitted in any MAC PDU as in the DSR triggering.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Proposal 1:	When LCP procedure is initiated for a new transmission, additional priority is applied when the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have or have not been transmitted in any MAC PDU is below the LCP remaining time threshold for the LCH.

### 2.1.2	Configuration of additional priority

To configure the additional priority, one approach is to configure only one additional priority for each LCH as discussed in [2] and [3]. In this approach, as long as there is LCH priority-adjusted data buffered on an LCH, the single additional priority for the LCH is applied. However, such additional priority is configured based on the remaining time threshold for each LCH, regardless of the data arrival time for each LCH, which may result in an LCH with lower remaining time using a lower priority just because of its higher remaining time LCP threshold.

Taking the following Figure (1) and Table (1) as an example, assuming that the data arrival on each LCH is shown in Figure (1), and the default priority, additional priority for each LCH is shown in Table (1), e.g., considering that LCH1 has higher remaining time threshold than LCH2 (18ms and 12ms), then LCH1 can be configured with lower additional priority than LCH2. For LCH1 and LCH2, assuming that data#1 arrives at LCH1 at T1, data#2 arrives at LCH2 at T2. Upon reception of the UL grant at T3, the remaining time of data#1 and data#2 are both below the configured remaining time threshold. UE performs the LCP procedure based on the additional priority of LCH1 and LCH2. Considering that LCH2 has higher priority, UE transmit data#2 first, and then data#1. However, at T3, the remaining time of data#1 is 8ms, and the remaining time of data#2 is 12ms, data#1 which has less remaining time should be transmitted first, otherwise, data#1 may be discarded if the UL grant is not enough.

To avoid such case, NW can configure an additional priority list for UE, based on which UE determines the additional priority for each LCH by considering the remaining time of data in each LCH, e.g., NW can configure an additional priority list and the remaining time range for UE, where each remaining time range corresponds to an additional priority in the list. With such solution, the LCH priority-adjusted data with lower remaining time can be allocated with resources first.

|  | Default priority | Additional priority |
| --- | --- | --- |
| LCH1 | 5 | 4 |
| LCH2 | 5 | 3 |

Figure (1)                                                                                                Table (1)

Observation 1:	Depending on when the grant is provided from the network, there may be a situation where the “LCH priority-adjusted data” on an LCH with lower additional priority has lower remaining time than the data in another LCH with higher additional priority. If NW configures only one additional priority for each LCH based on the remaining time threshold, the “LCH priority-adjusted data” on the LCH with lower remaining time may be impacted, e.g. its PDB may expire.

Proposal 2:	To maximize the chances that the resource allocation goes to LCH with lower remaining time, NW can configure a list of additional priorities, UE determines the additional priority for each LCH based on the smallest remaining time of data buffered on each LCH according to the configured list of additional priorities.

### 2.1.3	Additional priority fallback

In existing LCP procedure during resource allocation for new transmission, resources are allocated based on logical channel priority of the selected logical channels in two phases: first one when resources are initially allocated (which we will refer to as Initial resource allocation) and another when there are remaining resources to be allocated (which we will refer to as Remaining resource allocation).

##### 5.4.3.1.3 Allocation of resources 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); [Initial resource allocation] 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. [Remaining resource allocation] NOTE 1: The value of Bj can be negative.

UE performs resource allocation at an LCH granularity. For resource allocation, RAN2#126 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions” which means that the data within the logical channel is not reshuffled or priority is not changed based on whether it is LCH priority-adjusted data or not, while performing resource allocation during Initial resource allocation or Remaining resource allocation. However, we think that LCH priority can change before Remaining resource allocation during LCP resource allocation (i.e. it can be different than the LCH priority at Initial resource allocation). The reason is that if the additional priority is still used when there is no LCH priority-adjusted data, it would negatively impact other LCHs without LCH priority-adjusted data whose priority level is normally higher than default priority of the LCH for which the priority is adjusted. For example, default priority of LCH1 is 5, default priority of LCH2 is 3, UE adjusts the priority of LCH1 which has LCH priority-adjusted data to the additional priority, e.g., 2. If additional priority “2” is still used after all the LCH priority-adjusted data are assembled, e.g., during the remaining resource stage when the non-LCH priority-adjusted data of LCH1 still uses the additional priority “2” even though all the LCH priority-adjusted data is already assembled, LCH2 (which has higher priority than default priority of LCH1) would be deprioritized unnecessarily.

Observation 2:	If the additional priority is still used for an LCH when there is no LCH priority-adjusted data on the LCH at Remaining resource stage, it would have negative impact on other LCHs without LCH priority-adjusted data whose priority level is higher than the default priority of the LCH for which the priority is adjusted.

RAN2#127bis discussed the fairness issues, and had concerns about UE complexity as shown in the FFS below:

FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

To address the complexity and tight timeline concerns, instead of using the real-time LCH priority-adjusted data information, MAC layer can decide the LCH priority-adjusted data volume on each LCH before the resource allocation procedure (e.g. MAC layer can be informed by higher layer the LCH priority-adjusted data volume) MAC layer may perform both the first round of resource allocation (i.e., Initial resource allocation) and the second round of resource allocation (i.e., Remaining resource allocation) based on this LCH priority-adjusted data volume and doesn't need to check the real-time information of LCH priority-adjusted data volume when performing the Remaining resource allocation, meaning that the priority fallback for the second round (i.e., Remaining resource allocation) will not impact the tight timeline of LCP procedure and will not add UE complexity.

Observation 3:	MAC layer may perform the first round of resource allocation (i.e., Initial resource allocation) and the second round of resource allocation (i.e., Remaining resource allocation) based on the LCH priority-adjusted data volume information during the first round of resource allocation during LCP. MAC layer doesn't need to check the real-time information of LCH priority-adjusted data when performing the second round of resource allocation and hence it will not impact the tight timeline of LCP procedure and will not add UE complexity.

To generalize, the MAC entity should determine the LCH priority for an LCH before the resource allocation procedure based on whether the LCH contains LCH priority-adjusted data for Initial resource allocation and Remaining resource allocation separately. If there is, the additional priority is used, otherwise, the default priority is used. In addition, considering that the additional priority is used only when there is LCH priority-adjusted data, it is obvious to fall back to the default priority if there is no LCH priority-adjusted data in the LCH while deciding the priority for remaining resource allocation.

Proposal 3:	The MAC entity should determine the LCH priority based on whether the LCH contains LCH priority-adjusted data before each of Initial resource allocation and Remaining resource allocation phases.

Proposal 4:	For the remaining resource allocation phase, it is up to UE implementation whether to use the data volume information obtained before initial resource allocation phase or use an updated information obtained before remaining resource allocation phase.

Based on Proposal 6, if data was assembled in the MAC PDU in the Initial resource allocation stage, then the data should not be considered in the priority determination in the Remaining resource allocation stage (i.e., the additional priority is applied for an LCH in the Remaining resource allocation stage only if there are still PDCP SDUs which are delay critical and are not in the current MAC PDU construct. A text proposal for TS 38.321 specification can be found in Appendix to specify the triggering condition for applying the additional priority for an LCH in the Remaining resource allocation.

### 2.1.4	Bj enhancement

In existing LCP procedure, resource allocation procedure is performed with consideration of Bj, i.e., tokens in the bucket. In the Initial resource allocation stage, only LCH(s) with Bj > 0 are allocated resources in a decreasing priority order, which means that the LCH priority-adjusted data may not be allocated with resources in the Initial resource allocation procedure if Bj is smaller than the LCH priority-adjusted data volume. In such case, the LCH priority-adjusted data may be further delayed or even be discarded because of the Bj limitation if there are no remaining resources.

To avoid the Bj limitation impact on delay-critical data transmission, some companies proposed to raise Bj [4] or set PBR to infinity [5]. There is no doubt that the LCH priority-adjusted data transmission can be assured if Bj is temporarily raised or PBR is set to infinity. Our understanding of Bj and PBR is that it is calculated in real time at every resource allocation during LCP procedure. It is unclear how Bj can be temporarily raised and what value it should be raised to. For PBR setting to infinity, which is related to LCH configured with LCP enhancement, it is not fair to other LCH(s) without LCP enhancement configured which can also be of high priority.

Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, it may be a corner case that the configured PBR and BSD differ greatly from the buffered LCH priority-adjusted data volume. Therefore, even if the LCH priority-adjusted data cannot be allocated resources because of Bj limitation, the difference between Bj and LCH priority-adjusted data is small. If any enhancement is needed, it should not impact UE complexity or fairness.

Observation 4:	Delay critical data may not get through if Bj is not enough and could cause further delay to delay critical data, if such case occurs.

Observation 5:	Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, enhancement methods based on setting PBR to infinity or temporarily raising the Bj may result in either unfairness to other LCHs or complexity to the LCP procedure.  If an enhancement is needed, fairness and low complexity need to be considered.

In current spec, Bj can be negative as described in 38.321 below:

##### 5.4.3.1.3 Allocation of resources 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); [Initial resource allocation] 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. [Remaining resource allocation] NOTE 1: The value of Bj can be negative.

To ensure the LCH priority-adjusted data transmission with the constraint of Bj limitation, similar mechanism can also be applied. UE can keep performing the resource allocation for an LCH with LCH priority-adjusted data in the Initial resource allocation stage, if Bj < 0 and there is still LCH priority-adjusted data in the buffer. After that, Bj will be increased as time elapses, which has less impact on data on other LCH without LCH priority-adjusted data.

Proposal 5:	LCH priority-adjusted data on the LCH shall still be transmitted even if Bj is negative.

Based on the above Observations and Proposals, a corresponding text procedure is provided in Appendix.

Proposal 6:	Considering the TP in Appendix as baseline for LCP enhancement.

### 2.1.5	Impact on SRB

With respect to impacts on traffic from SRBs, NW can configure a higher priority for the LCH associated with SRBs. For example, by NW implementation, NW can ensure that the priority for LCH associated with SRBs is higher than that of delay-critical data, even when the priority of the LCH with delay-critical data has been adapted.

Proposal 7:	For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

## 2.2	Impact of additional priority handling on Intra-UE prioritization

With respect of additional LCH priority, RAN2#127 discussed the issues of intra-UE prioritization, and have the following FFS:

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3).

In legacy intra-UE prioritization procedure, for the MAC entity configured with lch-basedPrioritization, the following mechanism is used to determine the priority of an UL grant as defined in 38.321. There is also similar mechanism on deciding on the priority of a SR based on the priority of the logical channel triggering the SR.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Considering that an LCH may be adjusted to use the additional priority when there is delay-critical data on the LCH, it is more reasonable to use the LCH’s additional priority when deciding the priority of its associated uplink grant when the LCH contains delay critical data. The overlapping cases involve not only the UL-SCH grant, but also the SR resources. Therefore, it is obvious that additional LCH priority should be considered when deciding the priority of SR which is triggered by LCH with delay-critical data, as well as when deciding the priority of an UL-SCH grant which is associated with LCH(s) with delay-critical data. Otherwise, the LCH priority actually used for resource scheduling is different from the LCH priority used when deciding the UL grant and SR priority, which would result in delay critical data being delayed further if its UL grant is not selected.

Moreover, we think that this should be considered for new transmission and retransmission case. And in the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data. This should be considered also when determining the priority of the UL grant.

Observation 6:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data. This should be considered also when determining the priority of UL grant.

Proposal 8:	The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for LCP enhancements. The following observations and proposals were made:

Triggering of additional priority

Proposal 1:	When LCP procedure is initiated for a new transmission, additional priority is applied when the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have or have not been transmitted in any MAC PDU is below the LCP remaining time threshold for the LCH.

Configuration of additional priority

Observation 1:	Depending on when the grant is provided from the network, there may be a situation where the “LCH priority-adjusted data” on an LCH with lower additional priority has lower remaining time than the data in another LCH with higher additional priority. If NW configures only one additional priority for each LCH based on the remaining time threshold, the “LCH priority-adjusted data” on the LCH with lower remaining time may be impacted, e.g. its PDB may expire.

Proposal 2:	To maximize the chances that the resource allocation goes to LCH with lower remaining time, NW can configure a list of additional priorities, UE determines the additional priority for each LCH based on the smallest remaining time of data buffered on each LCH according to the configured list of additional priorities.

Additional priority fallback

Observation 2:	If the additional priority is still used for an LCH when there is no LCH priority-adjusted data on the LCH at Remaining resource stage, it would have negative impact on other LCHs without LCH priority-adjusted data whose priority level is higher than the default priority of the LCH for which the priority is adjusted.

Observation 3:	MAC layer may perform the first round of resource allocation (i.e., Initial resource allocation) and the second round of resource allocation (i.e., Remaining resource allocation) based on the LCH priority-adjusted data volume information during the first round of resource allocation during LCP. MAC layer doesn't need to check the real-time information of LCH priority-adjusted data when performing the second round of resource allocation and hence it will not impact the tight timeline of LCP procedure and will not add UE complexity.

Proposal 3:	The MAC entity should determine the LCH priority based on whether the LCH contains LCH priority-adjusted data before each of Initial resource allocation and Remaining resource allocation phases.

Proposal 4:	For the remaining resource allocation phase, it is up to UE implementation whether to use the data volume information obtained before initial resource allocation phase or use an updated information obtained before remaining resource allocation phase.

Bj enhancement

Observation 4:	Delay critical data may not get through if Bj is not enough and could cause further delay to delay critical data, if such case occurs.

Observation 5:	Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, enhancement methods based on setting PBR to infinity or temporarily raising the Bj may result in either unfairness to other LCHs or complexity to the LCP procedure.  If an enhancement is needed, fairness and low complexity need to be considered.

Proposal 5:	LCH priority-adjusted data on the LCH shall still be transmitted even if Bj is negative.

TP for Additional priority handling

Proposal 6:	Considering the TP in Appendix as baseline for LCP enhancement.

Impact on SRB

Proposal 7:	For LCP prioritisation enhancement, if there is a concern on traffic from SRBs delayed due to LCHs with delay-critical data, it can be left to network implementation to ensure LCH of SRBs are always higher than LCHs of delay critical data.

Impact of additional priority handling on Intra-UE prioritization

Observation 6:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the multiplexed data in the MAC PDU may become delay critical data. This should be considered also when determining the priority of UL grant.

Proposal 8:	The additional LCH priority should be considered for intra-UE prioritization while determining the priority of UL grant for new transmission, UL grant for retransmission and the priority of SR.

# 4. Reference

[1]. RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2]. R2-2408426	Discussion on LCP enhancements of XR traffic	Xiaomi Communications	RAN2#127bis 202410

[3]. R2-2408916	LCP enhancements	Ericsson	RAN2#127bis 202410

[4]. R2-2408121	Discussion on LCP enhancement for XR	vivo	RAN2#127bis 202410

[5]. R2-2409149	Discussion on LCP enhancement for XR	LG Electronics Inc.	RAN2#127bis 202410

# 5. Appendix

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

-	additionalPriorityList which is used if there is data with remaining time of discardTimer below the configured additionalPriorityThreshold.

-	additionalPriorityThreshold which sets the remaining time threshold to use the additional priority for an LCH.

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	determine the priority for the logical channels according to clause 5.4.3.1.4.

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or with LCH priority-adjusted data even if Bj < 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain:

3>	determine the priority for the logical channels according to clause 5.4.3.1.4.

3>	all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

##### 5.4.3.1.4	Priority determination

For each logical channel selected in clause 5.4.3.1.2:

1>	if the logical channel is configured with additionalPriorityThreshold:

2>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have or have not been transmitted in any MAC PDU is below additionalPriorityThreshold of the LCH for the new transmission and the PDCP SDUs are not yet in the MAC PDU generation; and

3>	select one additional priority from the configured additionalPriorityList for the LCH based on the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel;

2>	else:

3>	use the priority for the logical channel.

1>	else:

2>	use the priority for the logical channel.

NOTE:	UE may select one additional priority from the configured additionalPriorityList for an LCH based on the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel to maximize the chances that the resource allocation goes to LCH with lower remaining time.