3GPP TSG-RAN WG2 Meeting #128	R2-2410379

Orlando, US, November 18th – 22nd  , 2024

Agenda Item: 	8.4.4

Source:		Sony

Title:			Considerations on LP-WUS/WUR in RRC Connected mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline Specify necessary RRM requirements-

At RAN2#126 [3], the following agreements were made related to LP-WUS in RRC Connected mode:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

At RAN2#127 [4], the following agreements were made;

For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

The options 1-1 and 1-2 has also been discussed in RAN1#118 [6] and also RAN plenary.

At RAN#105 [7] plenary the following proposal was discussed but rejected.

Proposal 1:

-	RAN2 TU is kept unchanged.

-	For LP-WUS procedure in connected mode, following are clarified for RAN2:

- 	Option 1-1 should be supported based on legacy DCP mechanism.

- Option 1-2 can be supported by leveraging the RAN1#118 agreement, if it fits within the RAN2 TU budget.

At RAN1#118 [7], the following was agreed as a working assumption:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

At RAN2#127bis [8], the following agreements were made:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we discuss our view on some RAN2 related aspects related to, LP-WUS and DCP and LP-WUS in relation to drx_onDuration, and UE capabilities.

# Discussions on LP-WUS/WUR in RRC Connected mode

## LP-WUS related to drx_onDuration

In [2], it was discussed that there could be a benefit for power saving purpose for XR traffic to let the UE monitor LP-WUS instead of PDCCH monitoring while the drx_onDurationTimer is running, i.e., during C-DRX active time. The benefit would also be to handle unpredictable jitter, and hence reduce the uncertainty period of PDCCH monitoring. Though, since the Connected mode DRX periodicity is more statically configured, and that jitter potentially could also imply early or late arrival of packets, even outside PDCCH monitoring period, the potential wake-up signal should preferably not be related to the PDCCH monitoring period.

At RAN2#127 it was agreed that the timer for PDCCH monitoring is started a time offset after triggered by LP-WUS. I.e. option 1-2.

At RAN2#127bis it the difference between option 1-1 and option 1-2 was discussed

As we see it the first option consider a fixed legacy -C-DRX scheme, where the LP-WUS is monitored a time offset before the start of PDCCH monitoring. The other option is where the C-DRX scheme is adjustable, i.e. advanced or delay based on LP-WUS

Observation 1: The practical differences between option 1-1 and option 1-2, is whether the start of PDCCH monitoring follows a fixed scheme or is adjustable.

At RAN2#127bis the following agreements were made:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

For Option 1-2, we cannot see any rational for introducing a new timer. The same concept and functionality could clearly be used after PDCCH monitoring has been triggered, and therefor also the legacy drx-onDuration timer can be used. The only thing that differs is the timing for when to start PDCCH monitoring, i.e., when to start the On-duration timer.

Observation 2: We do not see any need for any new timer.

Proposal 1: The legacy drx-onDuration timer is sufficient to also cover option 1-2, and no new timer is needed.

## UE capabilities

At RAN1#117 meeting [4], it was agreed to support UE capability reporting according to the following;

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

In RAN2, no discussion or agreements have been made related to UE capabilities.

Proposal 2: RAN2 to follow up RAN1 agreements and define capabilities accordingly.

# Summary

In this contribution, we have discussed our views on aspects related to.LP-WUS and DCP and LP-WUS in relation to drx_onDuration and UE capabilities.

Observation 1: The practical differences between option 1-1 and option 1-2, is whether the start of PDCCH monitoring follows a fixed scheme or is adjustable.

Observation 2: We do not see any need for any new timer.

Proposal 1: The legacy drx-onDuration timer is sufficient to also cover option 1-2, and no new timer is needed.

Proposal 2: RAN2 to follow up RAN1 agreements and define capabilities accordingly.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power wake up signal and receiver for NR

- Minutes RAN2#126

- Minutes RAN1#117

- Minutes RAN2#127

- Minutes RAN#105

- Minutes RAN1#118

- RAN2#127bis