3GPP TSG-RAN WG2 Meeting # 128 	                R2-2410385

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.11.2

Source: 	Sony

Title:	Random access for SBFD Operation

Document for:	Discussion and Decision

# Introduction

3GPP has been working for enhancing duplexing operation for TDD based on subband duplex (SBFD) operation at gNB side within a TDD carrier [1-2]. The motivations of enhancing duplexing operation at least for TDD are for improving system capacity, reducing delays, and improving uplink coverage.

A Rel-19 Work Item (WI) on Duplex Evolution is therefore agreed to specify the requirements for FD-TDD where FD-TDD is performed at the gNB, i.e., the gNB can transmit and receive data/signals to/from the UEs at the same time on the same frequency band, whilst the UE is maintained as HD-TDD. That is, full duplex TDD is achieved at the gNB by scheduling a UE in the DL and scheduling another UE in the UL within the same OFDM symbol.

RAN1#116 Agreement:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier.​

RAN1#117 Agreement:

- For RRC connected mode UEs, SBFD subband time period:

  - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- For cell-specific indication of SBFD subband frequency location, adopt the following option:

  - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#116bis Agreement:

- Confirm the working assumption:​

  - Working assumption​

    - For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.​

RAN1#117 Agreement:

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

RAN1#117 Agreement:

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

RAN1#118 Working Assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

RAN2#127 Agreements:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

FFS whether/how early indication is used during a SBFD RA procedure.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

RAN2#127bis Agreements:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration.

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we discuss some details of random access in SBFD operation and provide some proposals at the end.

## RACH configuration for SBFD Subband

Work Item (WI) captures to specify SBFD operation to support random access in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage. RAN2 agreed that 4-step RACH is supported while 2-step RACH is FFS. Based on the above justification of reducing the delays and increasing the capacity, it is reasonable to support 2-step RACH in UL SBFD symbols as well.

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 1: Support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Early Indication:

In the last meeting, RAN2 discussed whether/how early indication is used during a SBFD RA procedure. For one case, it is agreed that RAN2 understands that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

However, for the other case, when SBFD-aware UE selects non-SBFD ROs, then the network can not identify the SBFD-aware UE, hence the network should schedule message 3 to non-SBFD PUSCH resources. Propopents for early idication think that the early indication is needed in order the network to be able to schedule message 3 of SBFD-aware UE to non-SBFD PUSCH resources. However, most of the companies questioned the merit of this, because for at least two reasons [3]: a) MAC CE would increase unnecessary design efforts for RAN2, and b) early indication via message 3 would provide marginal gain given that the UE will anyway provide its capability report to the gNB in message 5.

Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure.

RO Type selection:

For SBFD-aware UE, after selecting the SSB, the SSB may be associated with both SBFD RO and legacy RO, that is there are two types of RO for a PRACH transmission. The SBFD-aware UE needs to decide which type of RO to use for its PRACH transmission.

It is obvious that legacy UEs are not able to use SBFD ROs (i.e., they can only use legacy non-SBFD ROs) whilst SBFD-aware UEs can select both SBFD ROs and non-SBFD ROs, and that may result in less use of SBFD ROs and more congestion in non-SBFD ROs. For example, for early deployment of the SBFD feature where number of SBFD-aware UEs are small. In this case of early deployment of the SBFD feature, where SBFD-aware UEs are small, a higher selection probability for SBFD ROs may be needed so that SBFD-aware UEs can select SBFD ROs more often compared to legacy non-SBFD ROs. On the contrary, when the deployment of the SBFD feature is popular (i.e., a significant number of UEs supports the feature), equal selection probability for SBFD ROs and non-SBFD ROs may be needed. Hence, a mechanism to perform some kind of load balancing in the system is needed.

Furthermore, cell-edge UEs may be susceptible to interferences for both legacy ROs and SBFD ROs, but for UE-to-UE cross-link interferences (CLI) in the adjacent subbands in the serving cell, the interference certainly impacts SBFD ROs but not in the legacy ROs. This emits from high transmission power for the cell-edge UEs in the UL subband that contains SBFD ROs, which creates interferences in the adjacent DL subbands and as a result affects the UE’s DL coverage. Hence, in this case, it is preferable that SBFD-aware UE at the cell-edge should have a lower probability for selecting SBFD ROs compared to SBFD-aware UEs in the cell-centre, so that interference at the cell-edge is significantly reduced in practice. The cell-edge UEs can be defined a configurable RSRP threshold.

In the last meeting, RAN2 agreed that when initiating CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication is from network.

The additional indication from network can be used for load balancing by employing a probability PSBFD for a UE to use the additional RO (a SBFD RO). For example, if PSBFD = 0.7, then the UE has a probability of 70% in selecting an SBFD RO and 30% probability of selecting a legacy RO. The probability PSBFD can be configured in the SIBs. This will enable the network to perform load balancing on the two different types of ROs.

Proposal 3: RO Type selection based on load balancing, network configures/indicates probability PSBFD, where the UE has a probability of PSBFD in selecting an additional RO and a probability of 1 - PSBFD in selecting a legacy RO. The value PSBFD can be configured in the SIB.

Proposal 4: At the cell-edge, SBFD-aware UE should have a lower probability for selecting SBFD ROs compared to SBFD-aware UEs in the cell-centre, so that interference at the cell-edge is significantly reduced in practice. A configurable RSRP threshold defines the cell-edge UEs.

For CFRA, it is reasonable PDCCH order indicates whether to use additional RO (SBFD RO) or legacy RO. The main reason is that the network has an overall picture of the resource usage and what service triggers the PDCCH order.

Proposal 5: For CFRA, PDCCH order indicates whether to use additional RO (SBFD RO) or legacy RO for RA Procedure.

RNTI collision/confusion:

For configuration 2, how to handle the case where the legacy ROs (e.g. in Flexible Symbols) overlap with additional ROs needs to be discussed in RAN1 or RAN2. As the possible impact relates to RNTI collision/confusion, it should be discussed RAN2.

The legacy RA-RNTI/MsgB-RNTI are calculated as follows (TS 38.321):

The RA-RNTI/ MSGB-RNTI associated with the PRACH occasion in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

In our view, the above issue can be simply resolved by introducing a configurable offset for the symbol index (s_id) in the legacy RNTI equation. The offset will shift the s_id into another unused symbol index (0 ≤ s_id < 14). This will increase the number of available RNTIs as well as avoiding RNTI collisions, without increasing RNTI space. The offset should be signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

## Conclusions

In this contribution, we have discussed some details of random access in SBFD operation. We have the following observations and proposals:

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 1: Support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure.

Proposal 3: RO Type selection based on load balancing, network configures/indicates probability PSBFD, where the UE has a probability of PSBFD in selecting an additional RO and a probability of 1 - PSBFD in selecting a legacy RO. The value PSBFD can be configured in the SIB.

Proposal 4: At the cell-edge, SBFD-aware UE should have a lower probability for selecting SBFD ROs compared to SBFD-aware UEs in the cell-centre, so that interference at the cell-edge is significantly reduced in practice. A configurable RSRP threshold defines the cell-edge UEs.

Proposal 5: For CFRA, PDCCH order indicates whether to use additional RO (SBFD RO) or legacy RO for RA Procedure.

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

## References

# [1] RP-232763, 3GPP TR 38.858 V2.0.0, Study on Evolution of NR Duplex Operation.

# [2] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104.

# [3] R2-2408855, “SBFD RA aspects,”	Ericsson.