3GPP TSG RAN WG2 Meeting #128		R2-2410387

Orlando, U.S., 18th – 22nd November, 2024

Agenda Item: 	8.7.3

Source: 	Sony

Title: 	Discussion on enabling TX/RX for XR during RRM measurements

Document for: 	Discussion & Decision

# Introduction

eXtended Reality (XR) for NR phase 3 work item (WI) was revised in RANP#105 [1]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In RAN1#118bis meeting, the following agreements were made [2]:

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Clarify the note from previous agreement as follows: - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell. From RAN1 perspective, the case where an occasion(s) of gap/restrictions that is caused by RRM measurements is cancelled/skipped partially is not supported in Release 19.

At RAN2#127bis [3], the following agreements were made related to RRM measurement gap skipping [x].

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

This contribution provides our views on enabling transmission/reception in measurement gaps / scheduling restrictions period to support XR application.

# Discussion

Background:

Apart from performing data transmission and reception, a UE is also required to perform cell measurements, RRM. For example, if the signal quality of the serving cell is deteriorating, such as below certain threshold then the UE starts neighbor-cell measurements. A UE shall measure and report the neighboring cells signal and other carrier components to prepare for mobility when needed.

The UE can perform RRM measurements on the neighbor cells SSB signal when the signal is transmitted on the same frequency (i.e., intra-frequency measurements) and still being able to transmit and receive data from the serving cell (e.g., all signals are within a bandwidth part) without the need of a measurement gap. However, a scheduling restriction is still applied. The UE is not expected to transmit/receive around the SSB symbols. When the measurement shall be performed in other BWP then the UE have to suspend the communication as the UE requires a measurement gap for the RRM measurement.

The time duration during which mobile suspends its communication with serving cell to perform RRM measurement (e.g., inter frequency neighbor or other RAT measurement) is known as measurement gap. The measurement operation is illustrated in Figure 1.

Figure 1: Illustration of Legacy Measurement Gap using SSBs

During the measurement gap / scheduling restriction, the UE can perform cell signal measurement by using SS/PBCH Block (SSB), which is composed of Synchronizations Signal (SS) and Physical Broadcast Channel (PBCH)..

In legacy NR, the UE has scheduling restrictions so that the UE is expected not to transmit and/or receive during measurement gap. During the measurement gap, the UE is expected to perform RRM operation, such as RSRP measurement of the received or measured SSB. The measurement gap occasions can occur at the same time as the UE has been allocated for data Tx/Rx, such as the downlink configured grant (CG) transmission for XR traffic as shown in Figure 2.Then the UE is expected to prioritize the measurement and thereby skip the colliding Tx/Rx.

Figure 2: Collision of Measurement During MGL and Data Tx/Rx

In order to ensure the RRM measurement (e.g., measurement gap) does not affect the XR transmission, some signaling mechanism is needed, where the UE can temporarily modify the configured measurement gap, such as to skip the RRM measurement and continue data transmission/reception, particularly for the XR-specific traffic type. The gNB can provide an indication for the UE to skip measurement gap. It can be indicated explicitly or implicitly.

DCI

In RAN1#118, it was agreed as a working assumption on the solution based on the dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is provided from gNB to UE via downlink control information (DCI) carried out by PDCCH. This approach is illustrated in

Figure 3: Illustration of Skip / Release Measurement to support data Tx/Rx

The dynamic indication is basically an explicit indication by DCI to skip a particular gap(s)/restriction(s). It indicates the nearest gap(s)/restriction(s). This is known as Alt1-1 in the previous meeting agreement.

The other alternative, such as semi-static solution may also be supported. However, the use-cases and scenarios for that solution needs to be justified. It can also be the case when the network (gNB) wants to have flexibility in supporting and deploying the preferred solution(s).

RAN1 has made agreements on the supported DCI format, such as 0_1, 1_1, 0_2, 1_2 [2]. The usage of DCI is mainly for RAN1 to discuss, but potential configuration of DCI format and parameters may involve RAN2.

Proposal 1: RAN2 to investigate impact of DCI format and parameter configuration.

Partial skipping:

The XR traffic that may collide with RRM measurement may not necessarily be a video transmission which may require a large payload. It can also be the motion control information which require a small payload size. For example, it can be transmitted within one slot. If the UE perform transmission / reception for such short period then in practice, the UE can still continue to perform RRM measurement for the remaining measurement gap. The UE is not necessarily needed to skip the entire measurement gap. The UE can still perform both XR transmission / reception and RRM measurement with reduced measurement gap length. The RRM measurement with reduced measurement gap length is basically the operation of partial skipping of gap(s) / restriction(s).

RAN1 has expressed their view that from RAN1 perspective this feature is not supported in Rel-19 [2]. We think the primary reason is non-technical (e.g., limited time, etc). In our view, RAN2 at least need to discuss as this operation could still beneficial, particularly if the Tx/Rx for data transmission is relatively small. Hence, the UE can still perform RRM measurement.

Proposal 2: RAN2 to discuss supporting partial skipping of gap(s)/restriction(s) to allow RRM measurement (i.e., for the non-colliding data-channel and SSB).

UE assistance:

For the case of uplink transmission, UE may have better information than gNB in terms of the uplink transmission characteristics. The UE has direct interaction with the upper layer (e.g., application layer) so that the UE can predict the upcoming uplink transmission characteristics. The uplink transmission can be in a form of an aperiodic transmission that require spontaneous transmission from the UE. It can also be in a form of semi-static / periodic transmission. Furthermore, the UE may have various types of uplink transmission. Some uplink transmission can be compressed, such that the legacy gap/restriction can still be applied. In this case, the UE can prioritize the measurement over the transmission. Some other uplink transmission (e.g., XR traffic for video, control, data) has higher priority over RRM measurement. In this case, the UE skips RRM measurement and performs UL transmission instead. Based on these reasonings, UE assistance information to serving gNB on indicating an upcoming temporary measurement gap modification (e.g., skipping) shall be supported. The gNB can take that information into account and decide whether to grant the temporary measurement gap modification or not. This mechanism would also ensure that the gNB is still in full control of the UE behaviour. Hence, any potential system performance degradation (e.g., due to the measurement gap skipping) can be avoided.

In RAN1#117bis [5], RAN1 could not reach consensus to support UE assistance information (UAI) related to measurements occasions. The reason is due to lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). Hence, RAN1 sends an LS to RAN4 requesting their views on UAI [6]. Here, we express our observation related to UAI.

Though the use and design of UE assistance information is within RAN2 domain, and therefor these aspects should be discussed by RAN2.

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 3: Support UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE.

At last RAN2 meeting, RAN2#127bis, it was agreed to that “RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution”

Proposal 4: RAN2 to start discussing RRC based solution without wait RAN1/RAN4 input.

RRM measurements quality:

In case of the RRM measurement is affected by the temporary measurement gap modification, for example when one or more of the measurement gaps has been skipped, then the UE can also indicate that the reported RRM measurement has been affected. Skipping one or more measurement gaps is expected to reduce the quality of the reported RRM measurement from the UE, such as reducing the mobility performance. This may require further study in other WG(s), such as RAN2 and RAN4.

Observation 3: Skipping RRM measurement may affect the quality of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Last meeting:

At RAN2#127bis a number of agreements were made:

For the first one related to DSR, we think there should be no impact.

For the second one, we agree we see no impact on the LCP functionality

For the third one, DRX. We think there may be some impact than can be further investigated.

Observation 4: Potential impact on DRX may be further investigated.

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The observations and proposals are listed below.

Proposal 1: RAN2 to investigate impact of DCI format and parameter configuration.

Proposal 2: RAN2 to discuss supporting partial skipping of gap(s)/restriction(s) to allow RRM measurement (i.e., for the non-colliding data-channel and SSB).

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 3: Support UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE.

Proposal 4: RAN2 to start discussing RRC based solution without wait RAN1/RAN4 input.

Observation 3: Skipping RRM measurement may affect the quality of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Observation 4: Potential impact on DRX may be further investigated.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- RAN1 #118bis Chairman’s notes, Hefei, China, October 2024

- RAN2#127bis, Hefei

- RAN1#118bis, Hefei

- RAN1#117bis

- R1-2405736 “LS on UE assistance information related to measurement occasions”, Nokia