3GPP TSG-RAN WG2 Meeting # 128 	                R2-2410389

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.6.4

Source: 	Sony

Title:	Conditional Intra-CU LTM

Document for:	Discussion and Decision

# Introduction

In RAN#104, a revised WI of mobility enhancements for NR has been agreed which contains the following bullet point [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM.

  - Aim to support conditional LTM including subsequent LTM.

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC.

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint.

RAN2#127bis Agreements on conditional Intra-CU-LTM:

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this contribution, based on the WID, we discuss the remaining issues to support the conditional Intra-CU LTM.

## Conditional Intra-CU LTM

Rel-18 LTM (L1/L2 triggered mobility) is specified as shown on Figure 1 below. The LTM procedure is started from the network side after receiving L1 measurement reports from a UE, and then the network sends an LTM command (via MAC CE) to the UE to change the serving cell. The target cell is already prepared where the UE received all necessary configurations via RRC signalling. The main motivation is to reduce the mobility latency.

Rel-18 detailed procedure for LTM is as follows based on Figure 1 below, copied here from [2]:

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

3.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

4a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB.

4b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

5.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

6.	The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

7.	The UE performs the random-access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

8.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random-access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

The steps 4-8 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 2.

Figure 1. Rel-18 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

In Rel-19, the WID contains to specify support of conditional Intra-CU LTM in order to increase robustness, further reduce the latency and mobility failures. It is understood that the conditional LTM for intra-CU is like legacy CHO where the network configures some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed.

In the last meeting (RAN2#127bis), RAN2 agreed that the source cell sends the conditional LTM configuration via RRCReconfiguration to the UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

However, we believe that there are at least two options how to achieve and support of conditional Intra-CU LTM. As shown on Figure 3, the following options cane be envisioned [3]:

Option A: RRC Initiated C-LTM - Initially the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure, and the UE continuously evaluates the execution conditions, and if the execution condition(s) is satisfied for one target cell, the UE executes LTM cell switch.

Option B: LTM CSC Initiated C-LTM - Initially the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure. The UE carries out early DL and UL synchronizations with the candidate cells. The UE performs L1 measurements for the conditional LTM cells and reports to the serving cell. Based on the reported L1 measurements, the UE receives LTM command in advance from the serving cell that carries some conditions to evaluate for a target cell, and if the conditions for the LTM command are satisfied for the target cell, the UE executes LTM cell switch command.

Figure 2. Options for conditional Intra-CU LTM [3]

Observation 1: There are at least two options how to achieve and support of conditional Intra-CU LTM:  RRC Initiated C-LTM and LTM CSC Initiated C-LTM.

Comparing these two options, it seems Option A does not carry out synchronization with the candidate cells early enough. For example, the UL synchronization is performed after UE evaluates the execution conditions and decides to do LTM execution with a target cell which will increase the interruption time. However, for Option B, the DL and UL synchronizations are carried out before UE evaluates the execution conditions which will decrease the interruption time.

Another point is that in Option A, a UE reports L1 measurements to the serving cell, and the serving cell decides how many candidate cells the UE evaluates for conditional LTM via cell-switch command which will reduce the complexity at the UE as the number of cells to evaluate for conditional LTM are small in principle.

Observation 2: For RRC Initiated C-LTM, the UL synchronization is done after UE evaluates the execution conditions and decides to do LTM execution with a target cell which will increase the interruption time.

Observation 3: For LTM CSC Initiated C-LTM, the DL and UL synchronizations are carried out before UE evaluates the execution conditions which will decrease the interruption time. In addition, the serving cell decides how many candidate cells the UE evaluates for conditional LTM via cell-switch command which will reduce the complexity at the UE as the number of cells to evaluate for conditional LTM are small in principle.

Proposal 1: RAN2 to discuss options to achieve and support for conditional Intra-CU LTM:

Option A: RRC Initiated C-LTM - Initially the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure, and the UE continuously evaluates the execution conditions, and if the execution condition(s) is satisfied for one target cell, the UE executes LTM cell switch.

Option B: LTM CSC Initiated C-LTM – After the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure, the UE carries out early DL and UL synchronizations with the candidate cells and performs L1 measurements for the conditional LTM cells to report to the serving cell. The UE receives LTM command in advance from the serving cell that carries some conditions to evaluate for a target cell, and if the conditions for the LTM command are satisfied for the target cell, the UE executes LTM cell switch command.

## Conclusions

In this contribution, we have discussed how to support the conditional Intra-CU LTM, and we have the following observations and proposals:

Observation 1: There are at least two options how to achieve and support of conditional Intra-CU LTM:  RRC Initiated C-LTM and LTM CSC Initiated C-LTM.

Observation 2: For RRC Initiated C-LTM, the UL synchronization is done after UE evaluates the execution conditions and decides to do LTM execution with a target cell which will increase the interruption time.

Observation 3: For LTM CSC Initiated C-LTM, the DL and UL synchronizations are carried out before UE evaluates the execution conditions which will decrease the interruption time. In addition, the serving cell decides how many candidate cells the UE evaluates for conditional LTM via cell-switch command which will reduce the complexity at the UE as the number of cells to evaluate for conditional LTM are small in principle.

Proposal 1: RAN2 to discuss options to achieve and support for conditional Intra-CU LTM:

Option A: RRC Initiated C-LTM - Initially the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure, and the UE continuously evaluates the execution conditions, and if the execution condition(s) is satisfied for one target cell, the UE executes LTM cell switch.

Option B: LTM CSC Initiated C-LTM – After the network provides the conditional LTM configurations of one or more C-LTM candidate cells to the UE via RRC configuration procedure, the UE carries out early DL and UL synchronizations with the candidate cells and performs L1 measurements for the conditional LTM cells to report to the serving cell. The UE receives LTM command in advance from the serving cell that carries some conditions to evaluate for a target cell, and if the conditions for the LTM command are satisfied for the target cell, the UE executes LTM cell switch command.

## References

[1] RP-242356, “Revised Work Item: NR mobility enhancements Phase 4,” RAN#105.

[2] TS 38.300 V18.0.0, Section 9.2.3.5.

[3] R2-2408608, “Discussion on conditional LTM”, LG Electronics, RAN2#127bis.