3GPP TSG-RAN WG2 Meeting #R2-128		R2-2410399 Orlando, USA, Nov 12-22, 2024

Agenda Item:	8.6.3

Source: 	Rakuten

Title:  	Discussion on L1 event triggered measurement reporting

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID objective includes Event based L1 measurements for LTM.

In this contribution, we discuss the feasibility of configuring Event based L1 measurements and using them for LTM candidate cell addition/release. Further, we discuss redundancy of L3 and L1 measurements for LTM.

# Discussion

## Open issues from RAN2 #127-bis

## L1 Event Evaluation

In RAN2 #127, RAN2 made an agreement that, the beam level measurement result, but not cell level measurement result, is used for LTM event evaluation. For this purpose, Time to trigger (TTT) mechanism has to be applied. This TTT could be evaluated per beam or per cell.

In case the TTT is applied per beam, the UE starts TTT for a candidate beam whose quality meets the entering condition of the LTM event. Evaluation of multiple TTTs in parallel is also possible, if multiple beams of the candidate cell satisfy the entering condition. If one evaluated beam’s quality fails to satisfy the entering condition while the TTT is running, the UE shall stop the event evaluation for the beam and reset the TTT.

The TTT criteria may not be satisfied continuously, and therefore the triggering of the L1 measurement report and execution of LTM cell switch may be delayed due to the TTT reset.

In case that the TTT is evaluated per cell, the UE shall continue the event evaluation even if at least one beam of the candidate cell satisfies the entering condition while the TTT is running, i.e the TTT is not reset. Even the change of the best beam shall not impact the event evaluation for a candidate cell. Even, the change of the current beam of the serving cell may also reset the LTM event evaluation.

In this way, the triggering of L1 measurement reporting shall not be delayed as long as there is at least one beam of the cell satisfying the entering condition while the TTT is running.

For the LTM event evaluation, TTT is evaluated per cell, and not per beam. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

## Measurement Report for reportOnLeave

In L3 measurement reporting, the reportOnLeave is used to indicate whether to initiate a measurement report when the leaving condition of the event is satisfied. It can help the NW to know the signal quality deterioration of the concerned beam/cell in time, so that the NW can take appropriate decisions.

A similar mechanism is also beneficial for the L1 measurement report for the LTM. This information can be used to decide to, e.g., cancel the cell switch or remove the LTM candidate cell.

The reportOnLeave is configurable for event-triggered L1 measurement report and configurable by the gNB.

## L1 Measurement Report contents

The below agreement was made in RAN2 #128bis.

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

The L1 measurement report of Rel 18 LTM included the L1-RSRP and the corresponding candidate beam and cell information. The candidate beam and cell information are implicitly indicated by the index (i.e. SSBRI) in the LTM-CSI-ResourceConfig, which indicates the association between the SSB index and the LTM-CandidateId.

In case of event-triggered measurement report, L1 measurement results for multiple beams and cells can be included in one measurement report. The Index (i.e. SSBRI) in the LTM-CSI-ResourceConfig can be reused to indicate the candidate beam and cell information in the event-triggered measurement report.

Hence, we prefer that the LTM Configuration ID + SSB/CSI-RS ID is used to report the beam information.

LTM Configuration ID + SSB/CSI-RS ID is used to report the beam information in the event-triggered measurement report.

## Scheduling of L1 measurement report MAC CE

The below agreement was made in RAN2 #128bis.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

While it was agreed that legacy SR procedure for resource allocation is the baseline, requesting resources using BSR to send event-triggered L1 measurements MAC CE was not agreed. We believe there is a need to include some information of MR MAC CE size, otherwise if MR MAC CE size is variable and large, there is no way for the  network to assign a grant to guarantee max size.

The legacy BSR procedure for resource allocation is also used to request resources to send the event-triggered L1 measurements MAC CE.

## Scheduling First UL transmission in RACH-less conditional LTM

Conditional LTM is triggered by UE and the target beam selection is also performed by UE. Hence, the candidate cell is not aware of the beam the UE will select during conditional LTM execution.

Unlike Rel 18 LTM, there is no notification from source gNB-DU to the target gNB-DU (via gNB-CU) about the LTM cell switch execution where the selected beam is indicated. Therefore, there is no possibility of allocating dynamic grant to the UE at the target cell.

During conditional RACH-less LTM, if the NW is not aware of the candidate cell selected as target cell and the beam of this candidate cell selected by UE, there is no possibility for the candidate cell to know when and on which beam to perform dynamic scheduling for the first PUSCH transmission.

An obvious solution could be that each candidate cell keeps sending the dynamic scheduling information on each candidate beam or keeps monitoring the UL transmission among all CG occasions after the preparation phase. This is very inefficient and sub-optimal.

Hence, we propose that whenever C-LTM is configured, the UE periodically sends an L1 measurement report to the source cell, which indicates the potential target cells/beams. The source cell can inform the all the corresponding potential target gNB-DUs to send dynamic scheduling information on each candidate beam and/or monitor the UL transmission among all CG occasions in those beams.

Additionally, whenever the L1 execution condition of a C-LTM candidate cell is satisfied and the target cell beam selection is performed, if the UE’s radio link with the source cell is intact, the UE shall indicate the selected target cell and beam information to the source cell. In such a case, the Rel 18 mechanism can be re-used.

UE is configured to periodically report the potential target cell and beam information to the source cell during C-LTM.

UE is configured to report the selected target cell and beam information to the source cell, when the L1 execution condition of a C-LTM candidate cell is satisfied and the target cell beam selection is performed.

## gNB-DU initiated LTM candidate cell addition/release based on L1 measurement event

Following agreements were made in R2#126 regarding Measurement enhancements for LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In Release 18 LTM, RAN2 has agreed that the candidate cell preparation is performed by the gNB-CU based on the L3 measurements, while the LTM cell switch execution is performed by the gNB-DU based on the periodic L1 measurements.

In Release 18 LTM, the L1 measurements was possible to be configured only for LTM candidate cells and was only periodic in nature. With the introduction of event triggered L1 measurements, discovered and unconfigured cells are also feasible to be reported. For example, if a newly discovered cell meets the configured event criteria, it could be reported using L1 measurements. Hence, a new LTM candidate cell addition could be triggered based on these L1 measurements.

A maximum of 8 LTM candidate/target cells are feasible to be configured per UE and it is important that the candidate configuration slot isn’t wasted due to obvious reasons. It creates resource crunch at the network side and holding on to an unwanted candidate cell may deprive the UE of a valuable next candidate cell. Therefore, it is important to release unwanted and irrelevant LTM candidate cells.

In summary, currently since the LTM candidate cell addition and release is based only on L3 measurements, it implies that an LTM UE has to be configured for both L3 and L1 measurements, for the purpose of preparation and execution of LTM. But this could be triggered using event based L1 measurements also, which enhances benefits in terms of both signaling and latency.

LTM3 and LTM5 events could be used for new candidate cell addition and a measurement event configured for “reportOnLeave” could be used for candidate cell release.

With the introduction of event based L1 measurements, newly discovered cells can be reported to the gNB using an L1 measurement report.

LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

Configuring both L3 and L1 measurements is redundant in our view and causes unnecessary overhead. On the contrary, there are other RAN2 related study items in R19 where the objective of the study is to reduce measurements by using AI-ML methods. Hence, we think that RAN2 should explore the possibility of configuring UEs with only one kind of measurements for LTM, for example, using L1 event based measurements to configure addition and release of LTM candidate cells. This does not preclude the UE from being configured with L3 measurements for other purposes.

RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# Conclusion

We have the following observations:

Observation 1	With the introduction of event based L1 measurements, newly discovered cells can be reported to the gNB using an L1 measurement report.

Observation 2	LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

We have the following proposals:

Proposal 1	For the LTM event evaluation, TTT is evaluated per cell, and not per beam. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

Proposal 2	The reportOnLeave is configurable for event-triggered L1 measurement report and configurable by the gNB.

Proposal 3	LTM Configuration ID + SSB/CSI-RS ID is used to report the beam information in the event-triggered measurement report.

Proposal 4	The legacy BSR procedure for resource allocation is also used to request resources to send the event-triggered L1 measurements MAC CE.

Proposal 5	UE is configured to periodically report the potential target cell and beam information to the source cell during C-LTM.

Proposal 6	UE is configured to report the selected target cell and beam information to the source cell, when the L1 execution condition of a C-LTM candidate cell is satisfied and the target cell beam selection is performed.

Proposal 7	RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

Proposal 8	RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes

RAN2 Chairman Notes R2_126_ChairNotes