3GPP TSG-RAN WG2 Meeting #128	R2-2410405

Orlando, USA, 18 – 22 November, 2024

Agenda item:	8.4.4

Source:	InterDigital

Title:	LP-WUS in CONNECTED mode

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses the issues with LP-WUS in RRC_CONNECTED mode.

# 2	Discussion

## 2.1	LP-WUS capability

As it has been agreed that the LP-WUS monitoring is configured for the UE through dedicated RRC signaling, it is evident that UE capability is required for gNB to be able to trigger the configuration for a UE.

Proposal 1: UE capability for supporting LP-WUS in RRC_CONNECTED is supported.

## 2.2	Entry/exit conditions

RAN2 agreed the following baseline for entry/exit conditions for RRC_IDLE/INACTIVE mode in RAN2#126 and RAN2#127 meetings, respectively:

RAN2#126:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2#127:

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

This baseline seems to work as a reasonable foundation for entry/exit conditions also for RRC_CONNECTED mode where the UE monitors PDCCH solely based on C-DRX when the serving cell quality does not satisfy the condition to monitor LP-WUS (entry condition not satisfied); and where the UE monitors PDCCH based on both the LP-WUS and C-DRX when the serving cell quality satisfies the entry condition. Given the dedicated configuration of the LP-WUS monitoring for RRC_CONNECTED mode UE agreed, it seems reasonable to allow the configuration of these thresholds separately for RRC_CONNECTED UE from RRC_IDLE/INACTIVE conditions, or the same can be applied when no dedicated configuration is provided. To avoid ping-pong effect at the UE (especially, if there would be a requirement to report out-of-coverage events to the NW), UE could be required to have a threshold number of measurement events (in or out-of-coverage) before entering/leaving the LP-WUS monitoring.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

It is worth discussing whether only the serving cell quality based entry/exit condition is enough to maintain synchronization between UE and the gNB and avoid any service interruptions if the UE cannot receive the LP-WUS signal regardless of entry/exit condition being satisfied. Similarly to data inactivity timer (which protects the system from RRC mismatch), a timer could be exploited to avoid an issue where the entry/exit condition is satisfied but the UE cannot receive the LP-WUS signal while gNB tries to wake up the UE. While the timer is running (and entry/exit condition satisfied), the UE can activate LP-WUS monitoring whereas when the timer expires, the UE deactivates LP-WUS monitoring and monitors PDCCH based on the other rules in the specification (i.e., C-DRX).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

While the UE activating the LP-WUS monitoring based on the entry/exit condition being satisfied doesn’t need to be known to the gNB, the UE deactivating the LP-WUS monitoring based on the events as above would seem reasonable to be made known to the gNB, especially, if it happens through the timer while the entry/exit condition was still satisfied.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

## 2.3	PDCCH monitoring triggered by LP-WUS

In the previous RAN2#127-Bis meeting, the following was agreed wrt. the LP-WUS operation and C-DRX:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

RAN1#118 further agreed the following:

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

RAN2 left an FFS if the legacy drx-onDurationTimer would be started with LP-WUS option 1-2 (ie. not based on receiving the LP-WUS signal). It seems RAN1 already agreed previously the timer is not started which seems the most reasonable approach – LP-WUS option 1-1 can be used for that purpose (if NW intends to also configure such option).

Proposal 6: The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

RAN1#118-Bis confirmed the following working assumption:

Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

The main issue to discuss is whether to allow simultaneous configuration of Option 1-1 and 1-2. The usefulness of Option 1-2 LP-WUS is the possibility to configure shorter LP-WUS monitoring periodicity compared to Option 1-1 and, hence, enable faster scheduling when data appears in gNB DL buffer – for instance, the monitoring pattern could be configured to follow some application data activity. On the other hand, the usefulness of configuring Option 1-1 and Option 1-2 together is that the NW can exploit both the drx-onDurationTimer and the New timer for its scheduling and have flexibility in the configuration (e.g., configure shorter timer value for the New timer if the data activity is rather deterministic and use drx-onDurationTimer mainly for other data activity). From UE point of view, there should be no complexity issue even though it follows two LP-WUS monitoring configurations.

Proposal 7: Support simultaneous configuration of Option 1-1 and Option 1-2.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal 1: UE capability for supporting LP-WUS in RRC_CONNECTED is supported.

Proposal 2: As a baseline, support the same entry/exit condition criteria based on serving cell quality to activate/deactivate LP-WUS monitoring in RRC_CONNECTED mode as agreed for RRC_IDLE/INACTIVE mode.

Proposal 3: UE can be configured with a threshold number of LP-WUS receptions for determining LP-WUS in coverage (e.g., successful reception) and out-of-coverage (e.g., reception failure).

Proposal 4: Support a timer as additional condition to activate/deactivate LP-WUS monitoring.

Proposal 5: UE reports the events of autonomous deactivation of LP-WUS monitoring to the gNB.

Proposal 6: The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

Proposal 7: Support simultaneous configuration of Option 1-1 and Option 1-2.