3GPP TSG-RAN WG2 Meeting #128	R2-2410406
Orlando, U.S.A, November 18th- 22nd, 2024

Agenda Item:	8.7.3

Source:	InterDigital

Title:	RRM measurement gap related enhancements for XR

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on XR has the following objective on RRM measurement gap related enhancements [1]:

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

The topic was discussed for the first time last RAN2 meeting, and the following was agreed:

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

The contribution discusses the open L2 issues on RRM measurement gap related enhancements for XR.

# Discussion

In RAN2#127bis, this objective on RRM measurement gap related enhancements for XR was discussed. Two topics for enhancements were discussed, the first was DSR triggering related enhancements, i.e. to consider the cancellable measurement gap in the remaining time DSR trigger.

Several companies expressed differing views on addressing DSR enhancement, with the majority thinking proper network configuration can already handle the issue; and thus there is no need to pursue enhancements unless critical issues arise. The consensus is that the impact on DSR can be mitigated through network implementation, though further study is needed to determine whether specification changes are necessary.

After analysis of the timeline of the DCI based indication of cancellable measurement gaps, the network can subtract the value of the cancelled measurement gap occasion from the remaining time and also configure the value of the remaining time DSR trigger appropriately considering the length of the measurement gap occasion. Therefore, no further impact which would require specification changes/enhancements is foreseen.

Proposal 1:	Rely on proper network configuration of the DSR remaining time triggering threshold, considering possible measurement gaps.

The other enhancement discussed was the impact of measurement gap cancellation on C-DRX.

When the UE is configured with C-DRX, the DCI for skipping a gap is not received outside of C-DRX active period or when the DRX timers expire. Since the UE does not monitor PDCCH during the C-DRX non-active period (e.g. before the start of C-DRX active period), the DCI for skipping gap is not delivered and the UE delays any pending data to only after the measurements with the gap, thereby impacting QoS. Nevertheless, this issue can be addressed by proper NW configuration of the PDCCH monitoring occasions such that they are properly aligned with the gap occasions and C-DRX active duration.

In the last RAN2 meeting, a proposal was discussed whereby the UE extends its active time once it receives a DCI indicating cancellation of a measurement gap occasion (e.g. assuming the measurement gap falls outside the active time). However, the UE will start the drx-InactivityTimer once the DCI is received, which will anyway extend the Active Time. There is no reason why the network would then wait beyond the expiry of the InactivityTimer to schedule the UE.

Proposal 2:	No C-DRX enhancements specific for MG skipping is introduced.

# Conclusion

The following proposals are provided on RRM measurement gap related enhancements for XR:

Proposal 1:	Rely on proper network configuration of the DSR remaining time triggering threshold, considering possible measurement gaps.

Proposal 2:	No C-DRX enhancements specific for MG skipping is introduced.